3GPP TSG-RAN WG2 Meeting #119e	R2-2208599

# Electronic meeting, 17th-29th Aug, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 1394 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on UE behavior in LCP considering PSFCH reception capability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Rel-16, when the UE performs logical channel selection in a resource pool configured with PSFCH, the logical channels are selected according to the value of sl-HARQ-FeedbackEnabled of the highest priority logical channel. 1> select the logical channels satisfying all the following conditions among the logical channels belonging to the selected Destination: 2> SL data is available for transmission; and 2> sl-configuredGrantType1Allowed, if configured, is set to true in case the SL grant is a Configured Grant Type 1; and. 2> sl-AllowedCG-List, if configured, includes the configured grant index associated to the SL grant; and 3> if PSFCH is configured for the sidelink grant associated to the SCI: 4> sl-HARQ-FeedbackEnabled is set to enabled, if sl-HARQ-FeedbackEnabled is set to enabled for the highest priority logical channel satisfying the above conditions; or 4> sl-HARQ-FeedbackEnabled is set to disabled, if sl-HARQ-FeedbackEnabled is set to disabled for the highest priority logical channel satisfying the above conditions. 3> else: 4> sl-HARQ-FeedbackEnabled is set to disabled.. For Rel-17 SL Power Saving, a new UE type (i.e., Type A UE) which is not capable of PSFCH and S-SSB reception has been specified, and the PSFCH reception capability is introduced as FG 32-2, see in R1-2202928. However, according to the current specification, PSFCH reception capability is not cosidered. In other words, a Type A UE may select logcial channels with sl-HARQ-FeedbackEnabled being set to enabled, and transmit the multiplexed TB with HARQ feedback enabled indicator in SCI, but cannot actually receive the HARQ feedback because of its capability. This may decrese the reliablity of data in the HARQ-enabled logcial channels or make the TX UE fail to perform necessary retransmissions. Therefore, the impact of UE capability with or w/o PSFCH reception should be captured. |
| Summary of change: |  | In 5.22.1.4.1.2, the LCH selection considering not only the PSFCH resource configuration, but also the UE capability of PSFCH reception. Impact analysis Impacted 5G architecture options: Standalone Impacted functionality: Sidelink MAC Inter-operability: no inter-operability issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If HARQ-Feedback is enabled for TBs transmitted from a UE non-capable of PSFCH reception, it will either deteriorate SL QoS performance or cause enegy waste. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.22.1.4.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

###### 5.22.1.4.1.2	Selection of logical channels

The MAC entity shall for each SCI corresponding to a new transmission:

1>	if sl-BWP-DiscPoolConfig or sl-BWP-DiscPoolConfigCommon is configured according to TS 38.331 [5]:

2>	if the new transmission is associated to a sidelink grant in sl-DiscTxPoolSelected or sl-DiscTxPoolScheduling configured in sl-BWP-DiscPoolConfig or sl-BWP-DiscPoolConfigCommon:

3>	select a Destination associated with sidelink discovery as specified in TS 23.304 [26], among the logical channels that satisfy all the following conditions for the SL grant associated to the SCI:

4>	SL data for discovery is available for transmission; and

4>	SBj > 0, in case there is any logical channel having SBj > 0; and

4>	sl-configuredGrantType1Allowed, if configured, is set to true in case the SL grant is a Configured Grant Type 1; and

4>	sl-AllowedCG-List, if configured, includes the configured grant index associated to the SL grant.

2>	else:

3>	select a Destination associated to one of unicast, groupcast and broadcast (excluding the Destination(s) associated with sidelink discovery as specified in TS 23.304 [26]), having at least one of the MAC CE and the logical channel with the highest priority, among the logical channels that satisfy all the following conditions and MAC CE(s), if any, for the SL grant associated to the SCI:

4>	SL data for non-discovery is available for transmission; and

4>	SBj > 0, in case there is any logical channel having SBj > 0; and

4>	sl-configuredGrantType1Allowed, if configured, is set to true in case the SL grant is a Configured Grant Type 1; and

4>	sl-AllowedCG-List, if configured, includes the configured grant index associated to the SL grant; and

4>	sl-HARQ-FeedbackEnabled is set to disabled, if PSFCH is not configured for the SL grant associated to the SCI.

1>	else:

2>	select a Destination associated to one of unicast, groupcast and broadcast, that is in the SL Active time for the SL transmission occasion if SL DRX is applied for the destination, and having at least one of the MAC CE and the logical channel with the highest priority, among the logical channels that satisfy all the following conditions and MAC CE(s), if any, for the SL grant associated to the SCI:

3>	SL data is available for transmission; and

3>	SBj > 0, in case there is any logical channel having SBj > 0; and

3>	sl-configuredGrantType1Allowed, if configured, is set to true in case the SL grant is a Configured Grant Type 1; and

3>	sl-AllowedCG-List, if configured, includes the configured grant index associated to the SL grant; and

3>	sl-HARQ-FeedbackEnabled is set to disabled, if PSFCH is not configured for the SL grant associated to the SCI.

NOTE 1:	If multiple Destinations have the logical channels satisfying all conditions above with the same highest priority or if multiple Destinations have either the MAC CE and/or the logical channels satisfying all conditions above with the same priority as the MAC CE, which Destination is selected among them is up to UE implementation.

1>	select the logical channels satisfying all the following conditions among the logical channels belonging to the selected Destination:

2>	SL data is available for transmission; and

2>	sl-configuredGrantType1Allowed, if configured, is set to true in case the SL grant is a Configured Grant Type 1; and.

2>	sl-AllowedCG-List, if configured, includes the configured grant index associated to the SL grant; and

3>	if PSFCH is configured for the sidelink grant associated to the SCI and the UE is capable of PSFCH reception:

4>	sl-HARQ-FeedbackEnabled is set to enabled, if sl-HARQ-FeedbackEnabled is set to enabled for the highest priority logical channel satisfying the above conditions; or

4>	sl-HARQ-FeedbackEnabled is set to disabled, if sl-HARQ-FeedbackEnabled is set to disabled for the highest priority logical channel satisfying the above conditions.

3>	else:

4>	sl-HARQ-FeedbackEnabled is set to disabled.

NOTE 2:	HARQ feedback enabled/disabled indicator is set to disabled for the transmission of a MAC PDU only carrying CSI reporting MAC CE or Sidelink DRX Command MAC CE or Sidelink Inter-UE Coordination Request MAC CE or Sidelink Inter-UE Coordination Information MAC CE.

End of Change