3GPP TSG-RAN WG2 Meeting #119 Electronic	R2-2208605

Elbonia, 15 – 26 August 2022

Agenda item:	6.15.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Down selection of SR configuration for SL DRX MAC Command CE

WID/SID:	NR_SL_enh-Core - Release 17

Document for:	Discussion and Decision

# 1	Introduction

During RAN2#118, it was agreed to postpone the down-selection of SR configuration for SL DRX Command MAC CE, to one of the two best options selected during the outcome of the email discussion [712]

Proposal 5a RAN2 aims at down-selection of SR configuration for SL DRX Command MAC CE between the following 2 options in the next meeting: - Option 1: Reuse the SR configuration for SL logical channel, and which logical channel triggered a Scheduling Request for SL DRX command indication is up to UE implementation. - Option 2: Reuse the SR configuration for the CSI report.

This paper proposes a way forward when selecting the two options

# 2	Discussion

To make a qualified selection between option 1, and option 2, it is first necessary to be aware of the purpose of the SL DRX Command MAC CE, being, that when a receiving UE receives SL DRX command MAC CE from a transmitting UE, the receiving UE can stop the running on-duration timer and inactivity timer associated with a unicast link. In short, the SL DRX command is included as to indicate to the receiving UE that there are no more data to be transferred, and only in unicast.

Generally, we agree that since the SL DRX Command MAC CE is not related to data transfer, but rather the end of data transfer, the data is not time critical. Furthermore, selecting option 2 would indeed revert the previous RAN2#116bis-e agreement, as stated by ASUSTek in the follow-up discussion. However, allowing the UE to select by itself on selecting a logical channel to trigger the SR, will indeed make it difficult for the network to know about whether the SR is for data or for MAC CE, we propose to select option 2, and handle potential reverting a previous agreement.

Proposal 1: RAN2 to agree on selecting the SR configuration of the CSI report to trigger the scheduling request.

# 3	Conclusion

In this paper, we have set the following proposal:

Proposal 1: RAN2 to agree on selecting the SR configuration of the CSI report to trigger the scheduling request.