3GPP TSG-RAN WG2 Meeting #119-e	R2-2208618

Electronic Meeting, August 15 – August 26, 2022

Agenda item:		8.5.2

Source:		Futurewei

Title:	Discussion on XR traffic characteristics

Document for:		Discussion and Decision

# Introduction

EXtended Reality (XR) traffics typically involve various concurrent streams of video, audio, and other types of data (such as sensor/pose update and control command), each stream having its own periodicity, jitters, and QoS requirements. And within the video stream, packets of different types of application data units (ADU) (or PDU sets) produced by the video CODEC, such as H.264 SVC and H.265, may have different characteristics and different QoS requirements.

In this contribution, we focus on some QoS characteristics of the video traffics in XR applications, as the audio traffics and other types of data traffics in XR applications are not as challenging as the video traffics and may be handled well by the existing mechanisms.

# Discussions

## QoS characteristics

The current 5G QoS model is based on QoS flows, where the QoS parameters configured for a QoS flow apply to all data packets of that QoS flow. However, it may be beneficial to handle packets of different types of PDU set within a same video stream by applying different QoS parameters, e.g., different QoS parameters for intra-coding (I) picture vs. for predictive coding (P) picture, or different QoS parameters for base layer vs. for enhancement layer.

Taking the priority level for an example, as I picture (or base layer of a picture) is used as a reference picture for P picture (or enhancement layer of the picture), packets belonging to I picture (or base layer) could have a higher priority level than packets belonging to P picture (or enhancement layer).

For the same reason of inter-PDU-set dependency, a higher transmission reliability may be required for packets of I picture (or base layer) than packets of P picture (or enhancement layer). Hence, the packet error rate (PER) (or PDU set error rate (PSER) being introduced by SA2 for XRM) for I picture (or base layer) could be lower than that for P picture (or enhancement layer).

Due to better compression efficiency that can be achieved in compressing video data of P picture (or enhancement layer), compared to I picture (or base layer), the PDU set size (in either bytes or number of packets) varies more significantly between PDU sets of different types of PDU set than between PDU sets of a same type of PDU set. Therefore, if packets of different types of PDU set are mapped to a same QoS flow by NAS filtering and a same QoS profile with same QoS parameters being applied for all data packets of the QoS flow, then the Guaranteed Flow Bit Rate (GFBR), the Maximum Flow Bit Rate (MFBR), and the Maximum Data Burst Volume (MDBV) parameters may have to be set according to the I picture (or base layer), which may result in wasted resources when a burst of packets actually belong to a Picture (or enhancement layer).

To address above issues, one should avoid using a single QoS profile with same set of QoS parameters for packets of different types of PDU set within the same video stream.

Proposal 1: Avoid using a single QoS profile with same set of QoS parameters for packets of different types of PDU set within the same video stream.

## Candidate solutions

One solution can be mapping packets of different types of PDU set within the same video stream to different QoS flows by NAS filtering, where each QoS flow then needs only one unique QoS profile (and set of QoS parameters) and is identified by one unique QFI for packet handling in RAN, as today.

On the other hand, if packets of different types of PDU set within the same video stream are mapped to a same QoS flow by NAS filtering, one needs different QoS profiles with different sets of QoS parameters to be applied to packets of different types of PDU set. In this case, within the QoS flow, packets of different types of PDU set can be identified with a unique sub-QoS flow ID, which is associated to (and conveyed to RAN along with) each of the packets in a similar way as the QFI is today.

As SA2 are still evaluating the above two approaches, RAN2 may need to further evaluate any potential impacts on RAN spec by either approach (e.g., introducing the use sub-QoS flow ID in RAN), provide feedback to SA2, and work with SA2 to make a decision.

Proposal 2: RAN2 further evaluate any potential impacts on RAN by the two approaches as described above, provide feedback to SA2 via LS, and work with SA2 to make a decision.

# Conclusions

The followings are proposed:

Proposal 1: Avoid using a single QoS profile with same set of QoS parameters for packets of different types of PDU set within the same video stream.

Proposal 2: RAN2 further evaluate any potential impacts on RAN by the two approaches as described above, provide feedback to SA2 via LS, and work with SA2 to make a decision.

# References

RP-220285, Revised SID : Study on XR Enhancements for NR, Nokia (Rapporteur).