3GPP TSG-RAN WG2 Meeting #119-e	R2-2208621

Electronic Meeting, August 15 – August 26, 2022

Agenda item:		8.5.4

Source:		Futurewei

Title:	Layer 2 based XR capacity enhancement

Document for:		Discussion and Decision

# Introduction

EXtended Reality (XR) traffics typically involve various concurrent streams of video, audio, and other types of data (such as sensor/pose update and control command), each stream having its own periodicity, jitters, and QoS requirements.

The DL video stream in XR applications present a unique challenge to 5G RAN, in not only the peak data rate and throughput required being very high but also the associated latency requirements being very stringent. For example, 4k video is required to achieve the retina resolution with a head mounted display (HMD) unit, which is only a few centimeters away from the human eyes, compared to 1080p video required to achieve the same on a smartphone screen. For another example, due to the interactive nature of XR applications and human physiology, an end-to-end delay, which is referred to as the motion-to-photon delay, needs to be very short, e.g., can be no longer than 20 msec. Otherwise, the user may experience cyber-sickness such as feeling dizzy, disoriented, and nauseous.

Although RAN1 is the leading WG for XR capacity enhancement, XR capacity enhancement is not limited to the physical layer enhancement only. In this contribution, we propose a candidate solution for Layer 2 based XR capacity enhancement.

# Discussions

## Inter-PDU-set dependency

An XR application typically uses advanced video CODEC, such as H.264 or H.265, for the video stream, wherein different PDU sets produced by the CODEC may be of different types (such as I picture vs P picture, or base layer vs. enhancement layer), may be associated with different levels of importance, and/or may have different statuses of dependency on another PDU set within the same video stream.

For example, in a group of pictures (GOP), an intra-coded (I) picture is a picture that is encoded independently of all other pictures. An I picture is usually followed by a number of predictive coded (P) pictures until the same pattern is repeated at the next I picture. The P pictures are compressed and encoded either by directly using the I picture as a reference picture or by using another preceding P picture to reconstruct its reference picture for the inter-picture prediction. Although, there is a third type of video ADU, referred to as the bi-predictive coded (B) picture, we have intentionally left it out here because the buffering delay associated with the use of the B pictures will almost ensure the violation of the motion-to-photon delay requirement for XR applications, therefore making the use of the B pictures unsuitable in XR video.

For another example, an H.264 SVC codec divides a video stream into a sub-stream of base layer PDU sets and a number of sub-streams of enhancement layer PDU sets, where each base layer PDU set is self-decodable and provides a lower spatial resolution picture, which is used as a reference picture for the enhancement layer PDU sets that are dependent on it [3]. A spatial enhancement layer PDU sets provides better spatial resolution and a temporal enhancement layer PDU set provides better temporal resolution.

## A candidate solution for avoiding unnecessary transmissions and hence increasing system XR capacity

The successful decompression of the P pictures (to reconstruct their associated full pictures) depends on the successful reception and decompression of their respective reference pictures. If the I picture is lost in the transmission, all the subsequently P pictures (until the next I picture) become useless to the video decoder, even if they are received successfully. Therefore, in this situation, it becomes unnecessary to transmit them in the first place.

Similarly, the successful decompression of a spatial or temporal enhancement layer PDU set is dependent upon the successful reception of the associated base layer PDU set. If the base layer PDU set is lost in the transmission, the subsequent spatial and temporal enhancement layer PDU sets (until the next base layer PDU set) become useless to the video decoder, even if they are received successfully. Therefore, in this situation, it also becomes unnecessary to transmit them in the first place.

The inter-PDU-set dependency based packet dropping, as described above, can help to avoid wasting valuable radio resources on unnecessary transmissions that do not improve the user experience, thereby increasing the XR capacity for the system. To enable it, certain inter-PDU-set dependency information needs to be provided from the UPF to the gNB. RAN2 need to work with SA2 together to identify such information and to request SA2 to specify signaling means to convey such information from the UPF to the gNB.

Proposal 1: RAN2 study the support of inter-PDU-set dependency based packet dropping to avoid wasting radio resources on unnecessary transmissions that do not improve the user experience.

Proposal 2: RAN2 work with SA2 together to identify the inter-PDU-set dependency information needed.

Proposal 3: RAN3 request SA2 to specify signaling means to convey the inter-PDU-set dependency information from the UPF to the gNB.

# Conclusions

The followings are proposed:

Proposal 1: RAN2 study the support of inter-PDU-set dependency based packet dropping to avoid wasting radio resources on unnecessary transmissions that do not improve the user experience.

Proposal 2: RAN2 work with SA2 together to identify the inter-PDU-set dependency information needed.

Proposal 3: RAN3 request SA2 to specify signaling means to convey the inter-PDU-set dependency information from the UPF to the gNB.

# References

RP-220285, Revised SID : Study on XR Enhancements for NR, Nokia (Rapporteur).

Packet level video quality evaluation of extensive H.264/AVC and SVC transmission simulation, Journal of Internet Services and Applications, September 2011.

ITU-T Standard H.264.