3GPP TSG-RAN WG2 Meeting #119-e                                                                         	R2-2208629

E-meeting, 17th-29th August 2022

Agenda item:	8.14.4

Source:	China Telecom

Title:	On RAN visible QoE parameters for new services

Document for:	Discussion and Decision

- Introduction

The work item of NR QoE enhancement was revised at RAN#96[1]. The detailed objectives include:

- Support for new service type, such as AR, MR, MBS and other new service type defined or to be supported by SA4. Support RAN-visible parameters for the additional service types, and the existing service if needed, and the coordination with SA4 is needed [RAN3, RAN2].

- Left-over features from Rel-17, as well as the enhancements of existing features which are not included in Rel-17 normative phase, should be supported in Rel-18 if consensus on benefits are reached [RAN3, RAN2].

  - Specify per-slice QoE measurement configuration enhancement.

  - Specify RAN visible QoE enhancements for QoE value, RAN visible QoE trigger event, RAN visible QoE Report over F1.

  - Specify QoE reporting handling enhancement for overload scenario.

To optimize service performance, some QoE measurement information needs to be RAN visible, namely RAN visible QoE information [2]. The intention of this contribution is to analyse RAN visible QoE parameters for the new service types, and provide some proposals.

- Discussion

Per TS 38.331, the network sends the configuration of application layer measurements to UE via the RRCReconfiguration or RRCResume message

AppLayerMeasConfig-r17 ::=           SEQUENCE {

measConfigAppLayerToAddModList-r17   SEQUENCE (SIZE (1..maxNrofAppLayerMeas-r17)) OF MeasConfigAppLayer-r17     OPTIONAL, -- Need N    ...

}

MeasConfigAppLayer-r17 ::=           SEQUENCE {

measConfigAppLayerId-r17             MeasConfigAppLayerId-r17,

measConfigAppLayerContainer-r17      OCTET STRING (SIZE (1..8000))                                              OPTIONAL, -- Need N

serviceType-r17                      ENUMERATED {streaming, mtsi, vr, spare5, spare4, spare3, spare2, spare1}   OPTIONAL, -- Need M

ran-VisibleParameters-r17            SetupRelease {RAN-VisibleParameters-r17}                                   OPTIONAL, -- Need M

...

}

RAN-VisibleParameters-r17 ::=        SEQUENCE {

ran-VisiblePeriodicity               ENUMERATED {ms120, ms240, ms480, ms640, ms1024}                            OPTIONAL, -- Need S

numberOfBufferLevelEntries           INTEGER (1..8)                                                             OPTIONAL, -- Need R

reportInitialPlayOutDelay            BOOLEAN,

...

}

Specifically, if the AppLayerMeasConfig contains the ran-VisibleParameters, the UE need report the RAN visible QoE parameters, such as ran-VisiblePeriodicity, numberOfBufferLevelEntries, and reportInitialPlayOutDelay, to the gNB for network optimization. Up to now, the RAN visible QoE measurements are only supported for DASH streaming and VR services [2]. Given that the additional service types, e.g., MBS, will be supported in the R18 WID on NR QoE enhancement, the existing RAN visible QoE measurement framework specified in Rel-17 can be reused. In addition, the ran-VisibleParameters can be extended to introduce new metrics to support the new services

Proposal 1: The existing RAN visible QoE measurement framework specified in Rel-17 can be reused in Rel-18.

Proposal 2: The ran-VisibleParameters can be extended to introduce new metrics to support the new services

To support the new service types, existing RAN visible QoE parameters may be extended in Rel-18. For example, in the initial analysis of steaming services, the following parameters are considered as RAN visible QoE metrics, such as round-trip time, jitter duration, average throughput, initial playout delay, buffer level, etc [3]. In Rel-18, based on the progress in SA4 [4], the following metrics may be considered visible to the RAN to support MBS service.

Table 1: Initial analysis on QoE Report visibility at RAN for MBS

| Metric | Description | Initial analysis |
| --- | --- | --- |
| Active periods | Period of time during which the MBS User Data Ingest Session is active in the MBS System. If omitted, the data ingest session is intended to be active until further notice. | When the MBS enters into an active period, RAN can optimize the resource allocation, e.g, improving the transmit power, to satisfy the user requirements. |
| State | The current state of the MBS Distribution Session: INACTIVE, ESTABLISHED, ACTIVE or DEACTIVATING | RAN can adjust the resource allocation according to the MBS state to satisfy the user’s requirements while maintaining low power consumption. |
| QoS information | A 5G QoS Identifier (5QI) [5] to be applied to the traffic flow for this MBS Distribution Session. | RAN can use the QoS information to adjust resource allocation, such as the mapping between an MBS flow and an MRB (MBS Radio Bearer), to improve the quality of service. |
| Maximum bit rate | The maximum bit rate for the MBS Distribution Session. | If Maximum bit rate is small, RAN could adjust the resource allocation, e.g. bandwidth, of the UE to improve the bit rate and satisfy the user experience. |
| Maximum delay | The maximum end-to-end distribution delay that is tolerated for this MBS Distribution Session by the MBS Application Provider. | If the RAN can know the metrics, the RAN can try to compensate based on the RAN part delay, but the cost might be significant. |
| Service class | The class of the MBS User Service, expressed as a term identifier from a controlled vocabulary. | RAN can use the service class information to adjust the resource allocation for different services to improve user performance. |
| Start date–time | The point in time from which this MBS User Service Announcement is valid. If not present, the announcement is already valid | If the RAN can know this metric, the RAN can adjust the resource allocation of the UE to satisfy the user experience |
| End date–time | The point in time after which this MBS User Service Announcement is no longer valid. If not present, the announcement is valid indefinitely. | If the RAN can know this metric, the RAN can adjust the resource allocation of the UE to reduce resource waste. |

Proposal 3: The QOE metrics specified in SA4, such as Active periods, State, QoS information, Maximum bit rate, Maximum delay, Service class, Start date–time, and End date–time, can be added as RAN visible QoE parameters in Rel-18.

- Conclusions

The paper analyses the RAN visible QoE metrics for MBS, and concludes the following proposal:

Proposal 1: The existing RAN visible QoE measurement framework specified in Rel-17 can be reused in Rel-18.

Proposal 2: The ran-VisibleParameters can be extended to introduce new metrics to support the new services

Proposal 3: The QOE metrics specified in SA4, such as Active periods, State, QoS information, Maximum bit rate, Maximum delay, Service class, Start date–time, and End date–time, can be added as RAN visible QoE parameters in Rel-18.

- References

- RP-221803, WID update for Enhancement on NR QoE

- 3GPP, TS 38.300 V17.1.0, NR, NR and NG-RAN Overall Description

- 3GPP TR 38.890 V17.0.0, NR, Study on NR QoE (Quality of Experience) management and optimizations for diverse services

- 3GPP TS 26.502 V17.1.0, NR, 5G multicast–broadcast services; User Service architecture

- 3GPP TS 23.501: "System architecture for the 5G System (5GS)".