3GPP TSG-RAN WG2 Meeting #119-e                                                                         	R2-2208640

E-meeting, 17th-29th August 2022

Agenda item:	6.6.2

Source:	China Telecom

Title:	Discussion on UDC continuity in SDT

Document for:	Discussion and Decision

- Introduction

NR UDC has been studied in Rel-17 and frozen in RAN#95. Similar to RoHC, UDC is designed to compress packets and improve the network coverage. In RAN2#116bis meeting, it proposed:

Agreements: 1 UDC continuity can be configured for the same cases as RoHC continuity.

Since the RoHC continuity has been supported for SDT in Rel-17, the contribution intends to support the UDC continuity for SDT in Rel-17.

- Discussion

RoHC as an useful feature is widely used to reduce the packet header overhead and improve the uplink/downlink capacity by compressing the packet headers. In Rel-17 SDT WI, the network may configure UE to apply ROHC continuity for SDT. Compared to RoHC, UDC can be regarded as a more efficient compressing solution by compressing both header and payload of the packets in the compression buffer. Similarly, both UDC and RoHC are performed at the PDCP layer and are designed to compress packets. And RAN2 has agreed that “UDC continuity can be configured for the same cases as RoHC continuity”.

Given that RoHC continuity was supported in SDT in Rel-17 to reduce packet size and improve resource efficiency, we propose to support the UDC continuity in SDT in Rel-17.

Proposal 1: UDC continuity should be supported in SDT in Rel-17.

To support the UDC continuity for SDT in Rel-17, the corresponding CRs related to the TS38.300 and TS 38.331 can be seen in [4] and [5].

Proposal 2: To approve the corresponding CRs related to TS 38.300 and TS 38.331 to support the UDC continuity in SDT in Rel-17.

- Conclusion

The paper analyses the UDC continuity in SDT, and adopt the following proposals:

Proposal 1: UDC continuity should be supported in SDT in Rel-17.

Proposal 2: To approve the corresponding CRs related to TS 38.300 and TS 38.331 to support the UDC continuity in SDT in Rel-17.

- References

- R2-2202102, Report of 3GPP TSG RAN WG2 meeting #116bis-e

- 3GPP, TS 38.300 V17.1.0, NR, NR and NG-RAN Overall Description

- 3GPP TS 38.323 V17.1.0, NR, Packet Data Convergence Protocol (PDCP) specification