3GPP TSG RAN WG2 #119-e	R2-2208649

Online, 17 - 29 August 2022

Source: 	Huawei, HiSilicon

Title: 	Triggering of multiple cells for conditional reconfiguration execution

Agenda Item:	6.2.3.2

Document for:	Discussion and decision

# 1	Introduction

In Rel-16, the case in which multiple cells are triggered for conditional reconfiguration is handled by selecting one cell. In Rel-17, we discuss how the current specification addresses all the cases.

# 2	Discussion

Conditional reconfiguration execution occurs in TS 38.331 as follows;

##### 5.3.5.13.4 Conditional reconfiguration evaluation The UE shall: 1> for each condReconfigId within the VarConditionalReconfig: [...] 2> if event(s) associated to all measId(s) within condTriggerConfig for a target candidate cell within the stored condRRCReconfig are fulfilled: 3> consider the target candidate cell within the stored condRRCReconfig, associated to that condReconfigId, as a triggered cell; 3> initiate the conditional reconfiguration execution, as specified in 5.3.5.13.5; NOTE 1: Up to 2 MeasId can be configured for each condReconfigId. The conditional reconfiguration event of the 2 MeasId may have the same or different event conditions, triggering quantity, time to trigger, and triggering threshold. NOTE 2: If multiple NR cells are triggered in conditional reconfiguration execution, it is up to UE implementation which one to select, e.g. the UE considers beams and beam quality to select one of the triggered cells for execution. ##### 5.3.5.13.5 Conditional reconfiguration execution The UE shall: 1> if more than one triggered cell exists: 2> select one of the triggered cells as the selected cell for conditional reconfiguration execution; 1> else: 2> consider the triggered cell as the selected cell for conditional reconfiguration execution; 1> for the selected cell of conditional reconfiguration execution: 2> apply the stored condRRCReconfig of the selected cell and perform the actions as specified in 5.3.5.3; NOTE: If multiple NR cells are triggered in conditional reconfiguration execution, it is up to UE implementation which one to select, e.g. the UE considers beams and beam quality to select one of the triggered cells for execution.

This can be for CHO in NR, CPA in NR, intra-SN CPC in NR-DC or in EN-DC, and inter-SN CPC in NR-DC.

Conditional reconfiguration execution occurs in TS 36.331 as follows:

##### 5.3.5.9.4 Conditional reconfiguration evaluation If AS security has been activated successfully, the UE shall: 1> if VarConditionalReconfiguration includes at least one condReconfigurationId: 2> perform conditional reconfiguration evaluation; 1> for each condReconfigurationId within the VarConditionalReconfiguration: [...] 2> if trigger conditions for all associated measId(s) within triggerCondition are fulfilled: 3> consider the target cell candidate within the stored condReconfigurationToApply, associated to that condReconfigurationId, as a triggered cell; 3> initiate the conditional reconfiguration execution, as specified in 5.3.5.9.5; ##### 5.3.5.9.5 Conditional reconfiguration execution The UE shall: 1> if more than one triggered cell exists: 2> select one of the triggered cells as the selected cell for conditional reconfiguration; 1> else: 2> consider the triggered cell as the selected cell for conditional reconfiguration; 1> for the selected cell of conditional reconfiguration: 2> apply the stored condReconfigurationToApply associated to that condReconfigurationId and perform the actions as specified in 5.3.5.4, or perform the actions as specified in 5.3.5.3;

The execution of 5.3.5.9.5 in TS 36.331 can also be triggered in TS 38.331:

##### 5.3.5.13.4a Conditional reconfiguration evaluation of SN initiated inter-SN CPC for EN-DC The UE shall: 1> for each condReconfigurationId within the VarConditionalReconfiguration specified in TS 36.331[10]: [...] 2> if trigger conditions for all events associated with the measId(s) indicated in the CondReconfigExecCondSCG contained in the triggerConditionSN as specified in TS 36.331[10]), are fulfilled: 3> consider the target cell candidate within the RRCReconfiguration message contained in nr-SecondaryCellGroupConfig in the RRCConnectionReconfiguration message, as specified in TS 36.331[10], contained in the stored condReconfigurationToApply, associated to that condReconfigurationId as specified in TS 36.331[10]), clause 5.3.5.9.4, as a triggered cell; 3> initiate the conditional reconfiguration execution, as specified in TS 36.331[10]), clause 5.3.5.9.5; NOTE: If multiple NR cells are triggered in conditional reconfiguration execution, it is up to UE implementation which one to select, e.g. the UE considers beams and beam quality to select one of the triggered cells for execution.

So 5.3.5.9.5 can be executed for CHO in NR, CPA in NR and inter-SN CPC in EN-DC.

We noticed that, in TS 38.331, in 5.3.5.13.4 and 5.3.5.13.4a, there is no selection of one cell, if several cells are triggered but there is a note written as if it would be the case:

NOTE: If multiple NR cells are triggered in conditional reconfiguration execution, it is up to UE implementation which one to select, e.g. the UE considers beams and beam quality to select one of the triggered cells for execution.

This note is somehow relevant to 5.3.5.13.5 but not to 5.3.5.13.4 or. 5.3.5.13.4a.

Proposal 1: Remove the note on "selection of a triggered cell" in 5.3.5.13.4 or. 5.3.5.13.4a (since there is no selection in these subclauses, it is in 5.3.5.13.5).

When the UE is in NR-DC, "select one of the triggered cells" in TS 38.331 5.3.5.13.5 may mean to select an MCG cell (CHO configuration) or an SCG cell (CPA or CPC configuration).

When the UE is in EN-DC, "select one of the triggered cells" in TS 36.331 5.3.5.13.5 may mean to select an MCG cell (CHO configuration) or an SCG cell (CPA or CPC configuration) and, in EN-DC, TS 38.331 could, in parallel, trigger execution on an SCG cell for intra-SN CPC.

On this kind of aspect, there is the following note in TS 38.331 clause 5.3.5.13.5:

NOTE: If multiple NR cells are triggered in conditional reconfiguration execution, it is up to UE implementation which one to select, e.g. the UE considers beams and beam quality to select one of the triggered cells for execution.

Supposing there are triggered MCG and triggered SCG cells, comparing an MCG cell beams with an SCG cell beams seems rather meaningless, only the comparison of cells for the same cell group would make sense. Besides, the UE could also prioritize MCG cells, as connection to the PCell is more critical than connection to the PSCell.

Observation 1: The note on selection of a triggered cell for conditional reconfiguration is not accurate if there are triggered cells for the MCG (CHO) and for the SCG (CPA or CPC).

There is also nothing mentioned, for EN-DC, in case of parallel triggering of intra-SN CPC, by TS 38.331, and of inter-SN CPC by TS 36.331.

Observation 2: For EN-DC, there is no description related to parallel triggering of of intra-SN CPC, by TS 38.331, and of inter-SN CPC by TS 36.331.

However, this may not be a serious problem: if the UE triggers conditional reconfiguration execution for CHO, this will release CPC configurations, and if the UE triggers execution of CPC/CPA while there are CHO configurations that are not released, it is not really a problem that CHO is triggered while conditional reconfiguration execution for CPC was triggered already.

Proposal 2: There is no need to specify anything about parallel triggering of of intra-SN CPC, by TS 38.331, and of inter-SN CPC by TS 36.331.

As for the existing note in TS 38.331, it could be corrected as followed, or simply removed:

NOTE: If multiple NR cells are triggered in conditional reconfiguration execution, it is up to UE implementation which one to select, e.g. the UE considers beams and beam quality to select one of the triggered cells within a cell group for execution, the UE prioritizes target PCells over target PSCells.

Proposal 3: Discuss whether to remove the note in TS 38.331 clause 5.3.5.13.5 or to update it (see example). If it is updated, it could be captured in TS 36.331 clause 5.3.5.9.5 for consistency.

# 3	Conclusion

We discussed triggering of multiple cells for conditional reconfiguration execution and make the following proposals:

Proposal 1: Remove the note on "selection of a triggered cell" in 5.3.5.13.4 or. 5.3.5.13.4a (since there is no selection in these subclauses, it is in 5.3.5.13.5).

Proposal 2: There is no need to specify anything about parallel triggering of of intra-SN CPC, by TS 38.331, and of inter-SN CPC by TS 36.331.

Proposal 3: Discuss whether to remove the note in TS 38.331 clause 5.3.5.13.5 or to update it (see example). If it is updated, it could be captured in TS 36.331 clause 5.3.5.9.5 for consistency.