3GPP TSG-RAN WG2 #119e	R2-2208667

Electronic meeting, 2022-08-17 - 2022-08-29

Agenda Item:	7.2.6

Source:	Ericsson

Title:	R17 IoT NTN stage 2 corrections

Document for:	Discussion, Decision

# 1	Introduction

Here we discuss the coarse UE location reporting stage 2 description.

# 2	Discussion

## 2.1	UE location aspects

The UE location reporting was added for eMTC at the latest meeting, but nothing was added in the stage 2 to describe this.

In 38.300 they added a section on this

### 16.14.8	UE location aspects

Upon network request, after AS security is established in connected mode, a UE can report its coarse UE location information (coarse GNSS coordinates with accuracy around 2km) to the NG-RAN. If the UE cannot provide its coarse UE location, it may report "no coarse GNSS location available".

Editor's note: If/when SA3 will provide feedback that this is not recommended, this above text would be removed. Otherwise RAN2 can further update the above text, e.g. considering any other feedback from SA3.

We can for eMTC add a similar text.

### 23.21.X	UE location aspects

Upon network request, after AS security is established in connected mode, BL UEs and UEs in enhanced coverage can report its coarse UE location information (coarse GNSS coordinates with accuracy around 2km) to the eNB. If the UE cannot provide its coarse UE location, it may report "no coarse GNSS location available".

Editor's note: If/when SA3 will provide feedback that this is not recommended, this above text would be removed. Otherwise RAN2 can further update the above text, e.g. considering any other feedback from SA3.

Consider adding 23.21.X description for the eMTC coarse location reporting

# 3	Conclusion

In the previous sections we made the following proposals:

Proposal 1	Consider adding 23.21.X description for the eMTC coarse location reporting