3GPP TSG-RAN WG2 #119e	R2-2208671

Electronic meeting, 2022-08-17 – 2022-08-29

Agenda Item:	8.7.4

Source:	Ericsson

Title:	R18 NR NTN Idle mode Mobility enhancements

Document for:	Discussion, Decision

# 1	Introduction

The work item for NR NTN enhancements in Rel-18 [1] emphasizes NTN-TN and NTN-NTN mobility and service continuity as an important area for improvements. The following objectives are relevant for mobility:

- Specify NTN-TN and NTN-NTN measurement/mobility and service continuity enhancements [RAN2,RAN3,RAN4]

  - For NTN-NTN mobility, specify cell reselection enhancements for earth moving cell, the timing based and location-based cell reselection for quasi-earth fixed cell in Rel-17 can be considered as the starting point. [RAN2, RAN3, RAN4]

  - Specify NTN-NTN handover enhancement for RRC_CONNECTED UEs in the quasi-earth-fixed cell and earth-moving cell to reduce the signalling overhead. [RAN2, RAN3]

  - Specify cell reselection enhancements for RRC_IDLE/INACTIVE UEs to reduce UE power consumption (NTN-TN mobility is prioritized). [RAN2, RAN3, RAN4]

  - Study and, if needed, specify enhancement to Xn[/NG] signalling to support feeder link switch-over, CHO, e.g. exchange of necessary information between gNBs. [RAN3]

In this contribution, we aim to address the third bullet above.

# 2	Discussion

## 2.1	Support for location-based cell reselection

In RAN2#116-e, the first agreements to adopt the location criteria in idle mode procedures were taken. The following was agreed regarding location-based measurements initiation, and it was captured in the specification adopting the proposal in [1].

- Location information can be used to determine when to start measurement.

- UE may choose not to perform neighbour cell measurements of “NR intra-freq or inter-freq with equal or lower priority, or inter-RAT freq with lower priority”, if (the distance between UE and serving cell reference location is shorter than a threshold) and (legacy Srxlev/Squal condition is met, i.e., serving cell’s Srxlev/Squal is better than a threshold).

- Location-based measurement initiation is only applied if the cell broadcasts location-related parameters (e.g. a threshold) and by implementation the UE has location information.

However, in RAN2#118-e, after much discussion and due to the multiple alternatives and a lack of clear proposals [3], the following was agreed with regard to location-based cell reselection.

- Location based cell reselection method is not introduced in NR NTN Rel-17.

This agreement refers only to the ranking of cells in the cell reselection evaluation procedure, i.e., the cells that are candidates for cell reselection, that is described in section 5.2.4.2 of TS 38.304. In the email discussion [3], the following options were given based on previous discussions and contributions:

- Option 1: Introduce a distance threshold. Cell ranked on R-criterion first and then the distance threshold applies to down scope the candidate cells for reselection.

  - For cells not provided with reference location:

    - Alt.1: Not considered as candidate cell for reselection

    - Alt.2: Considered as candidate cell for reselection

- Option 2: Introduce a distance threshold. Distance threshold applies to decide the candidate cells and then rank the candidate cells based on R-criterion to decide the target cell for reselection.

  - For cells not provided with reference location:

    - Alt.1: Not considered as candidate cell for reselection

    - Alt.2: Considered as candidate cell for reselection

- Option 3: Cell ranked on R-criterion first and then the distance criteria applies to decide the target cell for reselection.

Among the options presented above, the last should be excluded. The R criterion, which is based on channel quality, should be prevalent over the newly introduced distance criterion and be the final decision factor for choosing the target cell for reselection in NTN. Therefore, the distance threshold should be taken as an optimization to enhance the procedure in NTN and assist in filtering out the potential candidates to be ranked.

The R criterion should be more important than the distance criterion for cell ranking.

The distance threshold is used to down scope candidate cells.

Regarding the two other options, they are in practice similar. Option 2 appears to be slightly better as it results in a lower number of neighbour cells to be measured which can increase the efficiency of the procedure. In addition, cells without reference location should not be excluded from the ranking. Thus, the distance criterion should only apply to those cells which the UE knows their reference location. Finally, if no cell fulfils the distance criterion, all cells should be ranked based on the R criterion and the UE should reselect to the cell with the highest R value.

A UE only applies the distance criterion to the cells with reference location.

In the absence of cells that comply to the distance criterion, cells are ranked based on the R criterion.

Introduce a threshold of the distance between UE and the cell reference location for the ranking of neighbour cells during cell reselection.

# 3	Conclusions

In the previous sections we made the following observations:

Observation 1	The R criterion should be more important than the distance criterion for cell ranking.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The distance threshold is used to down scope candidate cells.

Proposal 2	A UE only applies the distance criterion to the cells with reference location.

Proposal 3	In the absence of cells that comply to the distance criterion, cells are ranked based on the R criterion.

Proposal 4	Introduce a threshold of the distance between UE and the cell reference location for the ranking of neighbour cells during cell reselection.

# References

RP-221819, NR NTN (Non-Terrestrial Networks) enhancements, Thales, RAN#96, June 6-9, 2022

R2-2203725, Discussion on measurement rules for cell re-selection in NTN, OPPO, RAN2#117-e, March 2022.

R2-2206201, Report of [AT117-e][111][NTN] Idle mode, ZTE.