3GPP TSG-RAN WG2 #119e	R2-2208673

Electronic meeting, 2022-08-17 – 2021-08-29

Agenda Item:	8.6.3

Source:	Ericsson

Title:	R18 IoT NTN Mobility enhancements

Document for:	Discussion, Decision

# 1	Introduction

The work item for IoT NTN enhancements in Rel-18 [1] includes an objective on mobility enhancements:

4.1.2	Mobility enhancements

The following mobility enhancements objectives are listed.

-	Support of neighbour cell measurements and corresponding measurement triggering before RLF, using Rel-17 (TN) NB-IoT, eMTC as a baseline. [RAN2]

-	Re-use the solutions introduced in Rel-17 NR NTN for mobility enhancements for eMTC, with minimum necessary changes to adapt them to eMTC [RAN2]

- Define UE RRM core requirements for the above mobility enhancement features [RAN4].

In this paper we present our views on the first and second bullets.

# 2	Discussion

## 2.1	Neighbour cell measurements before RLF

NB-IoT does not support RRC_CONNECTED mode mobility, so moving to another cell involves a long service interruption as it requires to first declare RLF and then initiate the RRCConnectionReestablishment procedure. In Release 17, an optimization to reduce this time and improve mobility was introduced: neighbour cell measurements in connected mode. The intention was to speed up the cell selection process so that a UE might be able to have identified a “suitable” cell by the time it declares RLF. This is beneficial to “mobile” UEs which are moving towards the cell border, so a power criterion, configurable by the network, was standardized to prevent stationary UEs from triggering unnecessary connected mode measurements and waste power.

NB-IoT UEs can trigger connected mode measurements upon fulfilment of a power criterion.

This optimization is based on the premise that, as the UE moves towards the cell border, there is a clear difference in RSRP compared to the cell centre and was captured in the specification as follows:

--------- 36.331 ---------

1>	if neighCellMeasCriteria is not present in SystemInformationBlockType3-NB; or

1>	if T326 is running:

2>	if (NRSRP – nrs-PowerOffsetNonAnchor) < s-MeasureIntra, perform intra-frequency measurements as defined in TS 36.133 [16];

2>	if (NRSRP – nrs-PowerOffsetNonAnchor) < s-MeasureInter, perform inter-frequency measurements as defined in TS 36.133 [16].

--------- 36.331 ---------

However, as described in [2], such an effect may not be as pronounced in NTN cells, resulting in a small difference in signal strength between two beams in a region of overlap. The smaller margins would impose tighter power thresholds (s-MeasureDeltaP, s-MeasureIntra and s-MeasureInter) that could lead to errors in the differentiation between mobile and stationary UEs, i.e., small power fluctuations could be interpreted as a UE moving. Thus, the criterion above is not sufficient in NTN and can be complemented with information specific to this kind of deployment.

The power criterion alone might not be sufficient for NTN.

In NTN, the UE may know the characteristics of the cell it is connected to, e.g., stop serving time, cell reference location, cell radius or satellite ephemeris. In addition to the power criterion, this knowledge together with configurable distance or/and time thresholds can be leveraged to assist in the differentiation between mobile and stationary UEs to trigger connected mode measurements.

The existing criteria combined with a distance and/or time threshold can be used to improve service interruption in NB-IoT mobility.

To some extent, the use of this information (e.g., cell reference location, distance threshold, etc.) has been already considered in the solutions created to deal with discontinuous coverage (SIB32-NB) and location-based cell reselection in NR NTN (SIB3). However, SIB31 does not provide information about the cell’s footprint and, thus, it is insufficient for the purpose of measurements in connected mode. Therefore, RAN2 should consider transmitting the serving cell’s footprint information either by introducing a new SIB or extending an existing one such as SIB3-NB.

Cell footprint information is broadcast in a new or existing SIB to assist measurements in connected mode.

## 2.2	Conditional Handover

Conditional Handover (CHO) has been introduced in Rel-16 for terrestrial networks. Specification has happened in parallel for LTE and 5G NR.

CHO has been introduced in Rel-16 in parallel for LTE and 5G NR.

In Rel-17 NR NTN, CHO was enhanced with location-based and time-based trigger conditions.

- The location-based trigger is based on a new RRM event D1, which uses two pairs of reference location and threshold. The current serving cell is associated with a referenceLocation1 and threshold1, and the candidate target cell with a referenceLocation2 and threshold2. The event is fulfilled if the distance between the UE and referenceLocation1 becomes larger than the configured threshold1 and the distance between the UE and referenceLocation2 becomes shorter than configured threshold2.

- The time-based event T1 is defined by a certain point in time t1 and a duration. The event is fulfilled between the times t1 and t2, i.e., after the time t1 has passed at the UE, but before the time t2 passes, where t2 is t1 + duration.

In addition, the existing event A4 has been enabled to be used in CHO. Both the time-based and the location-based triggers have to be used together with a signal-strength based event (e.g. event A4).

In the RRC specification [3], these three new events have been added to the ReportConfigNR IE:

- CondEvent A4: Conditional reconfiguration candidate becomes better than absolute threshold;

- CondEvent D1: Distance between UE and a reference location referenceLocation1 becomes larger than configured threshold distanceThreshFromReference1 and distance between UE and a reference location referenceLocation2 of conditional reconfiguration candidate becomes shorter than configured threshold distanceThreshFromReference2;

- CondEvent T1: Time measured at UE becomes more than configured threshold t1-Threshold but is less than t1-Threshold + duration;

--------- 38.331 ---------

...

condEventA4-r17                  SEQUENCE {

a4-Threshold-r17                 MeasTriggerQuantity,

hysteresis-r17                   Hysteresis,

timeToTrigger-r17                TimeToTrigger

},

condEventD1-r17                  SEQUENCE {

distanceThreshFromReference1-r17  	INTEGER(0.. 65525),

distanceThreshFromReference2-r17  	INTEGER(0.. 65525),

referenceLocation1-r17           	ReferenceLocation-r17,

referenceLocation2-r17           	ReferenceLocation-r17,

hysteresis-r17                   	HysteresisLocation-r17,

timeToTrigger-r17                	TimeToTrigger

},

condEventT1-r17                  SEQUENCE {

t1-Threshold-r17                 INTEGER (0..549755813887),

duration-r17                     INTEGER (1..6000)

}

...

The new field descriptions are given below:

distanceThreshFromReference1, distanceThreshFromReference2 Distance from a reference location configured with referenceLocation1 or referenceLocation2. Each step represents 50m.

referenceLocation1, referenceLocation2 Reference locations used for location based event. The referenceLocation1 is associated to serving cell and referenceLocation2 is associated to candidate target cell.

duration This field is used for defining the leaving condition T1-2 for conditional HO event condEvent T1. Each step represents 100ms.

t1-Threshold The field counts the number of UTC seconds in 10 ms units since 00:00:00 on Gregorian calendar date 1 January, 1900 (midnight between Sunday, December 31, 1899 and Monday, January 1, 1900).

--------- 38.331 ---------

In Rel-17 NR NTN, CHO has been enhanced with time-based and location-based trigger conditions.

Both time-based and location-based CHO are essential for ensuring reliable mobility in NTN. Location-based CHO is thought to be used for example when a UE moves out of coverage of an Earth-fixed NTN cell, where the signal strength alone may not be enough to distinguish which cell a UE should be served from, but where this is clear from the UE location. An example use case of the time-based event is a service or feeder link switch in Earth-fixed NTNs, when the serving cell is going to disappear and be replaced by another cell at a well-known time. In addition to conveying information to the UEs about when the switch will occur, the time window can potentially be used to spread the HO executions in time such that not all UEs would attempt to access the upcoming cell at the same time.

Both time-based and location-based CHO are essential for ensuring reliable mobility in NTN.

Given that CHO has been introduced in parallel in LTE and 5G NR, it seems straightforward to adapt the enhancements from the Rel-17 NR NTN work to IoT NTN. In particular, the new condEvents A4, D1, and T1 should be added to the ReportConfigEUTRA IE in TS 36.331.

Specify condEventA4, condEventD1, and condEventT1 for eMTC.

In addition, for the location-based conditional handover to function, it becomes necessary to provide at least the serving cell reference location. The IE ReferenceLocation has already been defined for NR NTN in 38.331 and can be adopted. Besides, its value can be broadcast via System Information.

Adopt the ReferenceLocation IE from NR NTN.

Provide the serving cell reference location via System Information.

# 3	Conclusions

In the previous sections we made the following observations:

Observation 1	NB-IoT UEs can trigger connected mode measurements upon fulfilment of a power criterion.

Observation 2	The power criterion alone might not be sufficient for NTN.

Observation 3	CHO has been introduced in Rel-16 in parallel for LTE and 5G NR.

Observation 4	In Rel-17 NR NTN, CHO has been enhanced with time-based and location-based trigger conditions.

Observation 5	Both time-based and location-based CHO are essential for ensuring reliable mobility in NTN.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The existing criteria combined with a distance and/or time threshold can be used to improve service interruption in NB-IoT mobility.

Proposal 2	Cell footprint information is broadcast in a new or existing SIB to assist measurements in connected mode.

Proposal 3	Specify condEventA4, condEventD1, and condEventT1 for eMTC.

Proposal 4	Adopt the ReferenceLocation IE from NR NTN.

Proposal 5	Provide the serving cell reference location via System Information.

# References

RP-221806, WID on IoT NTN enhancements, MediaTek, RAN#96, June 6-9, 2022

TR 38.821, Solutions for NR to support non-terrestrial networks (NTN), V16.1.0, June 2021

TS 38.331, Radio Resource Control (RRC) protocol specification, V17.1.0, June 2022