3GPP TSG-RAN WG2 Meeting #119 electronic	R2-2208703

Online, August, 2022

Source: 	Session Chair (InterDigital)

Title:	Report for Rel-17 Small data and URLLC/IIoT

Email discussions:

CLOSED

[AT119-e][300] Organizational Diana – URLLC/IIoT, Small data]

Scope:

    - Share plans for the meetings and list of ongoing email discussions for the sessions related to Rel-17 URLLC/IIoT, Small data, RA Partitioning, R15-16 UP, Rel-18 UAV and NES

    - Share meetings notes and agreements for review and endorsement

[AT119-e][302][Sdata] UP open issues and CR to 38.321 (Huawei)

UP open issues and CR capturing agreed corrections

Deadline: To be set by rapporteur aiming to have company inputs and proposals by Friday

[AT119-e][303][Sdata] CR 38.300 (Nokia)

CR capturing agreed corrections

Deadline:

[AT119-e][304][IIoT] CR 38.300 (Nokia)

CR capturing agreed corrections

Deadline:

[AT119-e][305][IIoT] CP open issues and CR 38.331 (Ericsson)

CP open issues and CR capturing agreed corrections

Deadline: To be set by rapporteur

[AT119-e][306][IIoT] UP open issues and CR 38.321 (Samsung)

UP open issues and CR capturing agreed corrections

Deadline: To be set by rapporteur

[AT119-e][308][RA Part] UP open issues and CR 38.321 (ZTE)

UP open issues and CR capturing agreed corrections

Deadline: To be set by rapporteur

[AT119-e][309][R15/16 UP] CRs on UP (Nokia)

UP open issues and agreable CRs capturing agreed corrections

Deadline: To be set by rapporteur

[AT119-e][310][R18 Others - Low Latency] LS on Low latency (Huawei)

Discuss LS response on Low latency

Deadline: To be set by rapporteur

[AT119-e][311][SDT-Positioning] Config Transfer (Google)

Discuss LS response on Low latency

Deadline: To be set by rapporteur

[AT119-e][312][SDT] TA validation for CG-SDT (ZTE)

Discuss LS response

Deadline: To be set by rapporteur

Post email discussion

[POST119-e][301][Sdata] CP Open issues and CR to 38.331 (ZTE)

CP open issues and CR capturing agreed corrections

Deadline: short

[POST119-e][307][RA Part] CP open issues and CR 38.331 (Ericsson)

CP open issues and CR capturing agreed corrections

Deadline: To be set by rapporteur

[POST119-e][NES][313] Details of solutions - Huawei

- Capture more RAN2 details/impact/benefit on the solution groups and additional things to study

- Attempt prioritization of solutions

Deadline: Long

# 5	NR Rel-15 and Rel-16

Essential corrections only.

Tdoc Limitation: 11 tdocs in total for all sub agenda items.

### 5.1.2	User Plane corrections

User Plane corrections will be handled in a break out session

#### 5.1.2.1	MAC

R2-2208008	SPS HARQ feedback dropping for TDD	Nokia, Nokia Shanghai Bell	discussion	Rel-15	NR_newRAT-Core

-	Ericsson agrees with the principle and wants to review offline.

-	Samsung thought that the agreement was that the timer is started after feedback is transmitted.  In MBS the HARQ feedback can be disabled and we didn’t change the existing text.   Huawei, OPPO agrees with Samsung.  Nokia is concerned that if PUCCH is dropped in PHY due to prioritization than the MAC wouldn’t know.  Samsung thinks that there is an interaction and the PHY should notify.  Qualcomm explains that the UE can easily implement this MAC/PHY interaction and Rel-15 specs already implemented.

-	Qualcomm thinks that there is nothing broken if this is not agreed.  The gNB is aware the feedback is dropped and can act accordingly.

-	Mediatek agrees with the proposal.

-	CATT explains that cross layer check of actual L1 transmission is already supported in R16 MAC.  Nokia explains that the concern is that if the UE doesn’t start the timer the UE cannot retransmit.

-	Qualcomm further indicates this was also discussed in NR-U where LBT may fail for PUCCH. The gNB is not aware. Yet, even in that case, we kept the current spec.

-	LG wonders what the gNB might think if the feedback is not received, it could be lost and the UE will not start the timer anyways and it is dangerous for the gNB to assume that the feedback is lost or disabled.

-	Nokia asks if the same understanding applies to PUSCH.

=>	The CR is not agreed

Straight to email discussion 309

R2-2208969	Report for [AT119-e][309][R15/16 UP] CRs on UP (Nokia)

Proposal 1: The changes in R2-2207896 and R2-2207897 are not agreed. (Yes 0/14, Neutral 3/14, No 11/14)

Proposal 2: The changes in R2-2207898 and R2-2207899 are not agreed. (Yes 3/14, Neutral 4/14, No 7/14)

Proposal 3: To confirm that the MAC checks whether the configuredGrantTimer and/or cg-RetransmissionTimer is running at the time of corresponding PUSCH transmission, i.e., regardless of when the UE starts processing of the UL grant. The changes in R2-2208024 and R2-2208025 are not agreed. (Yes 1/14, Neutral 2/14, No 10/14)

Proposal 4: The changes in R2-2208254, R2-2208261, and R2-2208263 are not agreed. (Yes 2/14, Neutral 0/14, No 12/14)

Proposal 5: The changes in R2-2208689 (updated from R2-2206980) are not agreed. (Yes 1/14, Neutral 0/14, No 13/14)

=>	Noted

Agreements:

- Confirm that the MAC checks whether the configuredGrantTimer and/or cg-RetransmissionTimer is running at the time of corresponding PUSCH transmission, i.e., regardless of when the UE starts processing of the UL grant.  No spec changes are needed

R2-2207896	Clarification on BFD while SCell is deactivated	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.321	16.9.0	1347	-	F	NR_eMIMO-Core

=>	The CR is not pursued

R2-2207897	Clarification on BFD while SCell is deactivated	Nokia, Nokia Shanghai Bell	CR	Rel-17	38.321	17.1.0	1348	-	A	NR_eMIMO-Core

=>	The CR is not pursued

R2-2207898	Clarification on the matching TB size for 2-step RA	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.321	16.9.0	1349	-	F	NR_2step_RACH-Core

=>	The CR is not pursued

R2-2207899	Clarification on the matching TB size for 2-step RA	Nokia, Nokia Shanghai Bell	CR	Rel-17	38.321	17.1.0	1350	-	A	NR_2step_RACH-Core

=>	The CR is not pursued

R2-2208009	Clarification on HARQ RTT timer in case of HARQ feedback dropping	Nokia, Nokia Shanghai Bell	CR	Rel-15	38.321	15.13.0	1358	-	F	NR_newRAT-Core

=>	The CR is not pursued

R2-2208010	Clarification on HARQ RTT timer in case of HARQ feedback dropping	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.321	16.9.0	1359	-	A	NR_newRAT-Core

=>	The CR is not pursued

R2-2208011	Clarification on HARQ RTT timer in case of HARQ feedback dropping	Nokia, Nokia Shanghai Bell	CR	Rel-17	38.321	17.1.0	1360	-	A	NR_newRAT-Core

=>	The CR is not pursued

R2-2208024	Clarification on configuredGrantTimer and cg-RetransmissionTimer	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.321	16.9.0	1362	-	F	TEI16, NR_unlic-Core

=>	The CR is not pursued

R2-2208025	Clarification on configuredGrantTimer and cg-RetransmissionTimer	Nokia, Nokia Shanghai Bell	CR	Rel-17	38.321	17.1.0	1363	-	A	TEI16, NR_unlic-Core, NR_SmallData_INACTIVE-Core

=>	The CR is not pursued

R2-2208254	Correction on RA Resource Selection in Rel-15	vivo	CR	Rel-15	38.321	15.13.0	1373	-	F	NR_newRAT-Core

=>	The CR is not pursued

R2-2208261	Correction on RA Resource Selection in Rel-16	vivo	CR	Rel-16	38.321	16.9.0	1375	-	F	NR_newRAT-Core, NR_2step_RACH-Core

=>	The CR is not pursued

R2-2208263	Correction on RA Resource Selection in Rel-17	vivo	CR	Rel-17	38.321	17.1.0	1376	-	A	NR_newRAT-Core, NR_2step_RACH-Core

=>	The CR is not pursued

#### 5.1.2.2	RLC PDCP SDAP BAP

R2-2206980	Retransmission SDU choice under double-no condition When T-PollRetransmit expiration	PML	discussion

=> Revised in R2-2208689

R2-2208689	Correction on RLC retransmission SDU choice when T-PollRetransmit expiration	Purple Mountain Laboratories	discussion

=>	The CR is not pursued

#### 5.1.2.3	Other

User plane related corrections that should be handled in User plane break out session.

## 6.5	NR IIoT URLLC

(NR_IIOT_URLLC_enh-Core; leading WG: RAN2; REL-17; WID: RP-210854)

Tdoc Limitation: 3 tdocs

### 6.5.1	Organizational

Including LSs, rapporteur correction CR, and any rapporteur inputs (e.g. from ASN.1 ad-hoc meeting).

R2-2206922	LS on Rel-17 URLLC/IIoT RRC parameter updates (R1-2205507; contact: Nokia)	RAN1	LS in	Rel-17	NR_IIOT_URLLC_enh	To:RAN2

=>	Noted

R2-2208012	Correction on PUCCH sSCell for TDD	Nokia, Nokia Shanghai Bell, Ericsson, Qualcomm, Samsung, ZTE Corporation	CR	Rel-17	38.300	17.1.0	0524	-	F	NR_IIOT_URLLC_enh-Core

-	LG and Mediatek explains that if we say in addition may cause confusion that the UE may transmit at the same time.  Nokia explain that it is clear with the next sentence.

=>	The CR is agreed

### 6.5.2	Control Plane

A single CR with miscelaneous corrections is encouraged.  Small editorial corrections should be sent directly to rapporteur.  Big open issues can be discussed with contributions with CR in the appendix of the contribution

R2-2208060	Correction to the field description of usage-pdc	Huawei, HiSilicon	CR	Rel-17	38.331	17.1.0	3351	-	F	NR_IIOT_URLLC_enh-Core

=>	The CR is agreed

R2-2208556	CR on 38.331 for field description of PUCCH-Config for PUCCH Carrier Switch	ZTE Corporation,Sanechips, Nokia, Nokia Shanghai Bell, Ericsson, Samsung, Qualcomm	CR	Rel-17	38.331	17.1.0	3440	-	F	NR_IIOT_URLLC_enh-Core

=>	The CR is agreed

### 6.5.3	User Plane

A single CR with miscelaneous corrections is encouraged.  Small editorial corrections should be sent directly to rapporteur.  Big open issues can be discussed with contributions with CR in the appendix of the contribution

R2-2208926 UP Issue summary 	Samsung

Proposal 1. RAN2 to discuss: When a CG-PUSCH transmission is cancelled by a DG-PUSCH without UL-SCH (i.e. MAC PDU is not delivered to PHY) in Rel-17,

- Option 1. de-prioritization relies on existing Rel-16 LCH-based Prioritization. The CG is not considered as a de-prioritized uplink grant. (no specification change).

- Option 2. the uplink grant associated with the cancelled CG is considered as a de-prioritized grant.

=>	de-prioritization relies on existing Rel-16 LCH-based Prioritization. The CG is not considered as a de-prioritized uplink grant. (no specification change)

Proposal 2. RAN2 to discuss the following options for simultaneous transmission of SR and PUSCH over different PUCCH groups:

- Option 1. All Rel-17 UEs mandatorily supports simultaneous transmissions of overlapping SR and PUSCH over different PUCCH groups.

- Option 2. Define a capability and RRC configuration parameter of simultaneous transmissions of overlapping SR and PUSCH over different PUCCH groups.

- Option 3. Rel-17 MAC does not allow simultaneous transmission of SR and UL-SCH over different PUCCH groups. (No specification change)

-	Mediatek thinks that we should down select between option 1 and 2, exclude option 3.  Samsung thinks we can exclude option 3.  Nokia thinks that even with option 1 we may need some explicit configuration and the question is whether we have a capability or not.  Samsung agrees and are fine with RRC parameters.  Oppo agrees to exclude 3

=>	Option 3 is excluded and continue discussion on option 1 and 2 (including RRC explicit config and UE capability discussion for option 1).

=>	Noted

R2-2209005	Offline 306: User Plane Issues in IIOT/URLLC		Samsung

=>Noted

Agreements:

1. Define a capability and an RRC configuration parameter of simultaneous transmissions of overlapping SR and PUSCH over different PUCCH groups. MAC spec will be updated accordingly.

- R2-2207794 (RRC) and R2-2207795 (UE capability) are baselines for phase-2 review.

- Texts for MAC are updated Rapporteur MAC CR review

2	 R2-2208014 is not pursued.

3	 MAC relies on existing LCH-based Prioritization (no spec change) for the following case:

- a subset of symbols of a CG-PUSCH has been replaced to receive downlink transmission

- a TB is generated for a CG, but it is not transmitted due to collision with a measurement gap

4	 Proposed changes in R2-2208355 are not pursued.

5	 If simultaneousPUCCH-PUSCH or simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup is configured, when a SR transmission is considered as a prioritized SR transmission, the overlapping uplink grant(s) which can be transmitted with the SR simultaneously shall not be considered as de-prioritized uplink grant(s).

5b (all). If simultaneousPUCCH-PUSCH or simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup is configured, when a PUSCH transmission is considered as a prioritized PUSCH transmission, the overlapping SR(s) which can be transmitted with the PUSCH simultaneously shall not be considered as de-prioritized SR(s).

6	RAN2 confirms “When the maximum allowed deferral time of HARQ feedback is reached drx-RetransmissionTimerDL is not started. (no specification change)”

R2-2207432	Discussion on MAC layer operation at PUSCH cancellation	Apple	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

R2-2207433	Draft CR for MAC layer operation at PUSCH cancellation	Apple, Ericsson	CR	Rel-17	38.321	17.1.0	1316	-	F	NR_IIOT_URLLC_enh-Core

R2-2207506	Consideration on CG-PUSCH cancellation for UCI -only case	CATT	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

R2-2207507	Simultaneous transmission of SR and PUSCH over different PUCCH groups	CATT	CR	Rel-17	38.321	17.1.0	1321	-	F	NR_IIOT_URLLC_enh-Core

R2-2207796	Issue on a CG transmission cancelled by a DG without UL-SCH	OPPO	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

R2-2208013	MAC impact on PHY prioritization	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

R2-2208014	Correction on TB generated for UCI multiplexing	Nokia, Nokia Shanghai Bell	CR	Rel-17	38.321	17.1.0	1361	-	F	NR_IIOT_URLLC_enh-Core

R2-2208061	Discussion on deprioritized CG-PUSCH with UCI only TB	Huawei, HiSilicon	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

R2-2208062	Discussion on simultaneous transmissions of SR and PUSCH	Huawei, HiSilicon	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

R2-2208122	Open Issues in IIOT UP	Qualcomm Incorporated	discussion	Rel-17

R2-2208355	Discussion on SR error handling on PUCCH Cells	ASUSTeK	discussion	Rel-16	38.321	NR_IIOT_URLLC_enh-Core

R2-2208588	Correction for De-prioritizatin of Overlapping Resources	Samsung	draftCR	Rel-17	38.321	17.1.0	F	NR_IIOT_URLLC_enh-Core

Moved from TEI17

R2-2207792 Clarification on SR and PUSCH collision-Alt1        OPPO, Samsung         CR       Rel-17 38.321 17.1.0  1341    -           F          TEI17

R2-2207793 Clarification on SR and PUSCH collision-Alt2        OPPO, Samsung         CR       Rel-17 38.321 17.1.0  1342    -           F          TEI17

R2-2207794 Clarification on SR and PUSCH collision-Alt2        OPPO, Samsung         CR       Rel-17 38.331 17.1.0  3315    -           F          TEI17

R2-2207795 Clarification on SR and PUSCH collision-Alt2        OPPO, Samsung         CR       Rel-17 38.306 17.1.0  0778    -           F          TEI17

## 6.6	Small Data enhancements

(NR_SmallData_INACTIVE-Core; leading WG: RAN2; REL-17; WID: RP-212594)

Tdoc Limitation: 3 tdocs

### 6.6.1	Organizational

Including LSs, rapporteur correction CR and any rapporteur inputs (e.g. from ASN.1 ad-hoc meeting).

R2-2206907	Reply LS on Small Data Transmission (C1-224149; contact: Apple)	CT1	LS in	Rel-17	NR_SmallData_INACTIVE-Core	To:RAN2

=>	Noted

R2-2206931	LS on transferring SDT configuration and SRS positioning Inactive configuration from DU to CU (R3-223955; contact: Google)	RAN3	LS in	Rel-17	NR_SmallData_INACTIVE-Core, NR_pos_enh	To:RAN2

=>	Noted

=>	Response moved to email discussion 311

R2-2208973	 Report of [AT119-e][311][SDT-Positioning] Config Transfer 	Google

=>	Noted

R2-2208974	Reply LS on transferring SDT configuration and SRS positioning Inactive configuration from DU to CU	Google

=>	The LS is approved in R2-2208915

Moved to email discussion

R2-2208596	Discussion on RRC IEs in the RAN3 specification	Google Inc.	discussion	Rel-17

R2-2207120	Response to RAN3 LS on SDT containers for F1-AP	Intel Corporation	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2208072	On transferring SDT configuration and SRS positioning Inactive configuration from DU to CU

R2-2206953	Reply LS on TA validation for CG-SDT (R4-2211122; contact: ZTE)	RAN4	LS in	Rel-17	NR_SmallData_INACTIVE-Core	To:RAN2

-	LG thinks that RAN4 misunderstood the RAN2 agreement and we should clarify it in the response.  ZTE thinks that all cases should be covered by the release but can check. LG explains that they only consider the case from Connected in Inactive and should add the inactive place.   Huawei agrees with LG but points out that RAN4 is still discussing this and haven’t yet agreed.  LG points out that RAN2 already agreed and RAN4 should incorporate.

=>	Include clarification on RAN2 agreement INACTIVE state

=>	Noted

R2-2208990	Reply LS to the RAN4 LS in R2-2206953 	ZTE

=>	The LS is revised in R2-2209013

R2-2209013	Reply LS to the RAN4 LS in R2-2206953 	RAN2

=>	The LS is approved

R2-2207976	draft reply LS on TA validation for CG-SDT	ZTE Corporation, Sanechips	LS out	To:RAN4

=>	Moved to email discussion 312

R2-2207900	Corrections on SDT	Nokia, Nokia Shanghai Bell, Samsung	CR	Rel-17	38.300	17.1.0	0519	-	F	NR_SmallData_INACTIVE-Core

=>	remove the change in bullet 8 “The receiving gNB sends the RRCRelease message including suspend indication to the UE”

-	Vivo thinks we should change direct to keeps in inactive.  Vodafone, ZTE, and Intel prefer Nokia wording.

=>	The CR is agreed in R2-2208911 with the change above

R2-2207928	Editor's correction to MAC spec for Small Data Transmission	Huawei, HiSilicon	CR	Rel-17	38.321	17.1.0	1357	-	F	NR_SmallData_INACTIVE-Core

=> use as baseline for email discussion CR

### 6.6.2	User plane common aspects

A single CR with miscelaneous corrections is encouraged.  Small editorial corrections should be sent directly to rapporteur.  Big critical issues can be discussed in a contribution with CR in the appendix of the contribution

2-stepRACH during CG-SDT

Included in email discussion

R2-2207004  Issues for RA during CG-SDT procedure Samsung Electronics Co., Ltd     discussion Rel-17    NR_SmallData_INACTIVE-Core

R2-2207001	cg-SDT-TimeAlignmentTimer Handling	Samsung Electronics Co., Ltd	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2207359  cg-SDT-TimeAlignmentTimer maintenance during 2-step RA   Langbo   CR  Rel-17 38.321    17.1.0     1311       -      F NR_SmallData_INACTIVE-Core

R2-2207360  cg-SDT-TimeAlignmentTimer handling for RA-SDT Langbo   CR  Rel-17    38.321 17.1.0     1312       -      F NR_SmallData_INACTIVE-Core

LCH-restriction for CG-SDT

R2-2207901  LCH restrictions at SDT mode selection Nokia, Nokia Shanghai Bell, Ericsson, Huawei, HiSilicon, LGE CR  Rel-17    38.321 17.1.0     1351       -      F NR_SmallData_INACTIVE-Core

-	InterDigital thinks it is a good clarification but is wondering why the network would configure it as False.  Nokia explains that the configuration is for each logical channel so we can set it.   ZTE, Vivo, NEC agrees with InterDigital that the network should avoid this configuration.

-	ZTE thinks that the optimization is not very ideal.

-	Lenovo also thinks that if it configures CG resources then it should configure everything else appropriately and should be handled by NW configuration.  The assumption is that NW is aware of the data traffic and periodicity as it must configure CG resources and periodicity accordingly. Samsung also has a similar view and we have discussed this in the past.

-	Qualcomm thinks that we can maybe capture in the field description that the network configures it consistently.  And if we want to go with mix mode then option A can make sense.  ZTE doesn’t think the field description needs any clarification.

-	Ericsson, Huawei and Sony agrees with Nokia.

-	Samsung, for CG-SDT, even if DRBs do not use CG, CG is used for CCCH and DGs are used for DRBs

-	LG and Lenovo think that the CR is useful.

-	CATT also thinks that there is no issue, CG resource is configured and allowed for all the logical channel for SDT Bearers. Then no issue here.

=>	Continue by email discussion to see if companies can support and which option should be supported.

R2-2208117  LCH restrictions for CG-SDT       Ericsson discussion     Rel-17 NR_SmallData_INACTIVE-Core

=>	Noted

cg-SDT-TAT maintenance after receiving TAC MAC CE

R2-2207930  TAT maintenance for CG-SDT when receiving TAC MAC CE Huawei, Ericsson, HiSilicon, Nokia, Nokia Shanghai Bell, ZTE corporation    discussion     Rel-17 NR_SmallData_INACTIVE-Core

-	Lenovo supports the proposal, Intel, QC, CATT, Apple, Oppo

-	LG explains that we discussed this several times and we don’t see any new argument.  InterDigital agrees with LG.  Huawei thinks that there was some confusion before.

-	Intel asks which of the stored configurations are used and last meeting the assumption was that we are using everything stored.

=>	Move to email discussion

CG-SDT retransmission on different CG configuration

R2-2207902  MAC procedure issues Nokia, Nokia Shanghai Bell CR  Rel-17    38.321    17.1.0 1352       -      F NR_SmallData_INACTIVE-Core

To discuss whether it is allowed to use another CG configuration for CG-SDT retransmission different from the CG config used for initial transmission.

R2-2208912		Summary of email discussion 302	Huawei

Proposal5: It is not needed that, before initiating CG-SDT UE checks for the availability of CG occasion corresponding to a SSB with SS-RSRP above cg-SDT-RSRP-ThresholdSSB during the remaining time of cg-SDT-TimeAlignmentTimer. If CG occasion is not available, UE does not initiate CG-SDT. (16/18)

Discussions

Proposal1: CG-SDT can be selected if configuredGrantType1Allowed, if configured, is configured as “true” for all the SDT RBs available for data transmission. (15/18)

-	InterDigital is concerned that the meaning of CG restriction, only use CG for LCH that are configured as true, otherwise we might as well not have the configuration at all and prefers option B.  ZTE doesn’t thinks any option should be adopted and we are agreeing sub-optimal.  If we have a dedicated resource we always use it, neither option.

-	Nokia thinks that the UE has to follow LCH configuration and not waste data with CCCH.  Also fine with option B.

-	Ericsson explains that there is no resume cause and the network wouldn’t be able to determine.   ZTE explains that transmitting on CG-SDT will indicate it.  Ericsson thinks that it could be a request for larger data.  ZTE explains that if it is for larger data than SDT DVT SDT should not be used at all.

-	LG thinks that option A is better but both work.

Agreements

- For msgB reception, (a) if the C-RNTI MAC CE was included in MSGA, (b) if the cg-SDT-TimeAlignmentTimer is running, and (c) if the PDCCH transmission is addressed to the C-RNTI and contains a UL grant for a new transmission, UE considers Random Access Response reception and random access procedure successfully completed.

- cg-SDT-TimeAlignmentTimer should be restarted at successful completion of 2-step RACH while legacy TAT should not be started/restarted when CG-SDT procedure is on-going.

- Start/Restart the CG-SDT TAT from RRC’s perspective whenever the UE receives the CG-SDT configuration in RRCRelease message.

- Proposal 5a:     The condition to check TAT running for CG-SDT is updated as follows: '2>              if CG-SDT is configured on the selected UL carrier, and TA of the configured grant Type 1 resource is valid in the first available CG occasion according to clause 5.27.2; and'

- Any configured uplink grant with the same transport format and the HARQ process ID as the initial CG-SDT transmission can be used for CG-SDT retransmission.

- Add the condition “at least one RB configured for SDT having data available for transmission” as condition to initiate RACH for CG-SDT.

- Do not restore measObject configuration when the UE derives the pathloss reference upon receiving CG-SDT configuration. (17/18). FFS how to address the issue that measObject is not always configured when the UE receives RRCRelease with CG-SDT config, but current spec needs it.

The MAC entity shall, upon the reception of CG-SDT configuration:

- store the RSRP of the downlink pathloss reference derived based on the measObject configured for the Serving Cell as in TS 38.331 [5].

- CG-SDT can be selected if configuredGrantType1Allowed, if configured, is configured as “true” for all the SDT RBs available for data transmission.

- Legacy TAT is not started/restarted when TAC MAC CE is received during CG-SDT procedure

- Separate conditions checking ACK of initial CG-SDT and checking SSB index as initial CG-SDT’s

- Conditions for checking availability of the SSB with SS-RSRP above cg-SDT-RSRP-ThresholdSSB are not redundant.  No changes to current spec

- Capture in the RRC spec in the field description of uci-OnPUSCH that it cannot be configured for CG-SDT

- No need to trigger RACH when none of the SSB is above the threshold for RA-SDT

- FFS to check in this meeting - Allow the field harq-ProcID-Offset can be configured for CG-SDT (add the case for CG-SDT).  Double confirm that there is nothing that breaks in the MAC when CG retx timer is configured.

Proposal need further discussion

Proposal11: R2 to further discuss whether the field harq-ProcID-Offset can be configured for CG-SDT. (9/16)

-	InterDigital understands that previous releases allow this configuration for NR-U and IIoT and we should be able to reuse it.  ZTE explains that current RRC has no restriction for CG-SDT.  Huawei indicates that there is in a field description a restriction.   Huawei also thinks that we should introduce it as it is the same reason we introduced in NR-U.   Qualcomm also think there is no restriction.

Proposal14: R2 to further discuss whether to add the note for Bj increment. (7/15)

-	NOTE: UE doesn’t update Bj in RRC_INACTIVE when there is no SDT procedure ongoing.

-	LG indicates that we made the agreement last meeting and we are just capturing it in the specification.

To be discussed over email

R2-2207416	Analysis on remaining issues for SDT	CATT	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2207571	Correction on SSB selection for CG-SDT	LG Electronics Inc.	discussion	NR_SmallData_INACTIVE-Core

R2-2207572	CR for correction on SSB selection for CG-SDT	LG Electronics Inc.	CR	Rel-17	38.321	17.1.0	1325	-	F	NR_SmallData_INACTIVE-Core

R2-2207573	Clarification of Bj increment	LG Electronics Inc.	discussion	NR_SmallData_INACTIVE-Core

R2-2207815	Correction on the stored RSRP for TA validation	Xiaomi	draftCR	Rel-17	38.321	17.1.0	F	NR_SmallData_INACTIVE-Core

R2-2207906	User plane issues for SDT	NEC	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2207929	Text Proposal for RSRP-based TA validation	Huawei, HiSilicon	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2208266	Correction on CG-SDT Transmisson	vivo	CR	Rel-17	38.321	17.1.0	1377	-	F	NR_SmallData_INACTIVE-Core	Late

R2-2208356	Correction on SR delay timer	ASUSTeK	discussion	Rel-16	NR_SmallData_INACTIVE-Core

R2-2208660	Clarification on uci-onPUSCH for CG-SDT	vivo	CR	Rel-17	38.331	17.1.0	3462	-	F	NR_SmallData_INACTIVE-Core

### 6.6.3	Control plane common aspects

A single CR with miscelaneous corrections is encouraged.  Small editorial corrections should be sent directly to rapporteur.

Big critical issues can be discussed in a contribution with CR in the appendix of the contribution

T319 delayed start

R2-2207907	Issues due to delay of the start of T319a	NEC	discussion	Rel-17	NR_SmallData_INACTIVE-Core

Proposal  RAN2 need to update the “while T319a is running” and “if T319a is not running” in TS 38.331, for example by changing them to “while SDT is being performed” and “if SDT is not being performed” respectively. The proposed changes to TS38.331 is provided in Annex.

-	Nokia supports the CR

-	Intel thinks we should also treat Samsung and it seems a bit odd we are changing legacy behavior.

-	ZTE thinks the problem is valid but also thinks we can consider language and we can maybe add a NOTE

-	LG thinks that only issue when RA-SDT is starting.  NEC thinks it is also possible for CG case.

-	Nokia explains that it is specified also in MAC that "SDT is initiated", maybe we could align with that.

=>	Include the possible solution and new wording in CP email discussion.

UDC for SDT moved from 6.6.3

R2-2208640	Discussion on UDC continuity in SDT	China Telecom	discussion

-	LG thinks that SDT is mainly for small data and UDC is typically compression for large data.  We should make SDT simple and not support.  Huawei agrees with LG, and in addition UDC is usually useful for repetitive data and it is an optimization.   Apple, Ericsson, agrees with LG.  CATT thinks we should support UDC and the packet size should be reduced with UDC especially for CG SDT.

=>	The proposal is not agreed

Not treated

R2-2208655	CR for TS38.300 on Support of UDC in SDT	China Telecom	CR	Rel-17	38.300	17.1.0	0545	-	B	NR_SmallData_INACTIVE-Core

R2-2208656	CR for TS38.331 on Support of UDC in SDT	China Telecom	CR	Rel-17	38.331	17.1.0	3461	-	B	NR_SmallData_INACTIVE-Core

Move to email discussion

R2-2208987	Summary of email discussion 301 CP 	ZTE

Proposal 4: Changes proposed in R2-2207417 can be agreed – to be merged into rapporteur CR (16/17)

-	Intel is concerned that companies have different interpretation of the spec on whether the suspend config is or not stored in INACTIVE AS context.  Vodafone asks if we have a list of what is stored. Intel explains that in Rel-15 it was done in a messy way but it is stored.

=>	Noted

Agreements

- The T319a Synchronization issue in R2-2207003 is not pursued

- The issue with delayed start of T319a is solved in accordance with the proposal in R2-2207907 – changes merged into rapporteur CR

- Changes proposed in R2-2208357 are not pursued

- Changes proposed in R2-2207417 can be agreed – to be merged into rapporteur CR

- Per R2-2207418 and R2-2208269, remove the redundancy of re-establishment of PDCP entities for SRB1 (details to be discussed in the merged rapporteur CR)

- No changes needed to remove the redundancy of re-establishment of PDCP entities for SRB2 handling as proposed in R2-2207418

- Specify the ROHC handling in normative text as noted in R2-2207977 and R2-2207988 (details to be discussed as part of merged CR)

- Other changes in R2-2207977 and R2-2207988 (apart from the change about sdt-DRB-ContinueROHC) can be merged into rapporteur CR, taking into account the comments received during the email discussion

- For R2-2208218, Do not support the enhancement for UE assistance information to indicate its RRC state preference (IDLE or INACTIVE) during SDT

- Clarify in the spec that UDC/EHC is not applicable in SDT (discuss over email to capture in RRC)

- No need to further specify in SDT section how UAC for non-SDT data is performed as the NAS layer performs UAC according to current procedures

For online discussion:

Proposal 8: No further clarification is needed for handling of UDC/EHC (10/16)

-	ZTE explains that these configurations are store but when we restore PDCP in INACTIVE we need to make sure that these are not used.

-	Samsung and LG think it should be specified.  LG thinks it can be done in stage 2.

-	CATT thinks we can clarify this in RRC.

-	Ericsson thinks we need to discuss where it would go in RRC as it could go in many places.  Maybe stage 2 would be easier.

Proposal 10b: For R2-2207965, no need to perform UAC for non-SDT data indication (13/17)

- InterDigital flagged this due to CT1 LS where SDT operation is transparent to the AS.  Then the UAC is broken with SDT and don’t understand why we have to skip UAC.  NEC agrees. ZTE explains that UAC is performed when SRB1 is resumed and this is how it works in CONNECTED.  We don’t do it again for every packet otherwise we would double bar.

- Intel agrees that we don’t need to specify anything and if anything needs to be changed it should raised in CT1

- LG indicates that UAC is performed for non-SDT data. But spec change is not needed because UAC is specified in different section

- Google explains that there is a fundamental difference in INACTIVE as NAS is not aware and cannot perform UAC.  ZTE explains that NAS does UAC for every packet and the requirements are applicable for all states and doesn’t need to know the AS state.

R2-2207977	RRC corrections for SDT	ZTE Corporation, Sanechips	CR	Rel-17	38.331	17.1.0	3340	-	F	NR_SmallData_INACTIVE-Core

=>	The CR is revised in R2-2208989

R2-2208989	RRC corrections for SDT	ZTE Corporation, Sanechips	CR	Rel-17	38.331	17.1.0	3340	-	F	NR_SmallData_INACTIVE-Core

R2-2207003	T319a synchronisation issue	Samsung Electronics Co., Ltd	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2207417	Handling of sdt-Config upon reception of RRCRelease message	CATT	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2207418	PDCP Re-establishment for SRB(s) upon initiation of SDT	CATT	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2207907	Issues due to delay of the start of T319a	NEC	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2207965	UAC for non-SDT initiation during SDT	Google Inc.	CR	Rel-17	38.331	17.1.0	3337	-	F	NR_SmallData_INACTIVE-Core

R2-2207988	ROHC continuity and initial BWP related corrections	Huawei, HiSilicon	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2208130	BWP for CG-SDT	Ericsson	discussion	Rel-17	38.331	NR_SmallData_INACTIVE-Core

R2-2208218	RRC state preference during SDT procedure	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2208269	Correction on SRB1 Handling in SDT	vivo	CR	Rel-17	38.331	17.1.0	3393	-	F	NR_SmallData_INACTIVE-Core

R2-2208357	Correction on T319a	ASUSTeK	discussion	Rel-16	NR_SmallData_INACTIVE-Core

## 6.18	RACH indication and partitioning

Tdoc Limitation: 2 tdocs

Expected to cover WIs SDT, CovEnh, RedCap, RAN slicing.  RA specific aspects from the different WI should be covered in this AI given the RA experts are all there.

### 6.18.1	Common signalling framework

A single CR with miscelaneous corrections is encouraged.  Small editorial corrections should be sent directly to rapporteur.  Big open issues can be discussed in a contributions with CR in the appendix of the contribution

Move to email discussion

R2-2208995	[AT119-e][307][RA Part] CP open issues and CR 38.331 (Ericsson)

[5]	Based on the input from companies, the rapporteur proposes to accept change 2, 5, 8, 10. But not change 1 and 3. The proponent argues that indeed change 4, 6, 7 and 9 are essential and should be supported. The rapporteur proposes that these are quickly discussed online to see if the changes are essential given the motivation in this discussion.

•	For change 9 a proposal has been made to select from the following alternatives:

1.	Change the need code to Need R as proposed in the CR (this is aligned with groupBconfigured field in legacy specs)

2.	Keep Need S code and add the description for groupBconfigured field and capture UE behavior in this field description

[6]	There is no consensus to pursue changes proposed in [6]. The Rapporteur thinks that this discussion needs more time to conclude on if a limit is explicitly needed: i.e. determine what a NW may configure in terms of number or partitions, what reasonable SIB or testing limitation may exist - and if a change is pursued, weather a NBC or field description change is to be pursued including the actual max number of partitions. Proposal: Postpone. Discuss shortly online whether:

•	the maximum number of configurable partitions is essential to be corrected, and if yes

•	To what maximum number is this limit set to, if it can be same/different for maxAdditionalRACH-r17 or maxFeatureCombPreamblesPerRACHResource-r17

•	If consensus to a and b - how to correct this. E.g. field description additions also possible.

-	Mediatek this is essential for UE complexity and we should limit the number of partitions that the UE is expected to support, especially for RedCap UEs.  Huawei is ok to limit the number (e.g. 64)

-	Qualcomm agrees with Mediatek that this is very important to address as it is a big UE implementation problem as it has to dimension for a large number of partions.  Apple agrees as it saves memory.   Huawei explains that we can clarify it in field description and avoid NBC.  ZTE understands the concerns but we should allow the protocol to be extendable but we can limit it for some UEs.  Can we have it in RAN5 as a test for RedCap?  Mediatek explains that this is a common problem and not just redcap.  Why do we need 100+ partitions, it won’t fit in SIB signaling anyways?

-	Ericsson and Nokia agree that we should limit and not have a lot of flexibility. It’s also a problem for the network.

Agreements

1	There is no consensus to pursue changes proposed in [1]. Change is not aligned with agreements in the common RICS framework.

2	An update to the field description name numberOfPreamblesForThisPartition will be made as proposed in [2].

3	The value range of startPreambleForThisPartition is changed from 1-64 to 0-63 as proposed in [3]. Note that the encoded ASN.1 is unchanged, this may be seen as a so called “functionally NBC” change.

4	There is no consensus to pursue changes proposed in [4]. As the specification already includes the sentence “The network associates all possible preambles of an additional RACH configuration to a feature or feature combination.”, the NW behaviour according to the change proposal seems in place.

5	on [5] Change the need code to Need R as proposed in the CR (this is aligned with groupBconfigured field in legacy specs).  Ask question in the email if changes 4, 6, 7 are needed

6	Reduce max number of configurable partitions in the spec.  FFS the max limit and how to implement it in a BC way (e.g. field description)

7	The changes proposed in [7] are accepted and will be merged into a Rapporteur CR.

8	With a few exceptions the correction in [8] are supported by a large number of companies and can be included in a Rapporteur CR. Final text may be adjusted in that version.

9	The correction in [8] are supported by all except one company. Unless additional issues with the change can be determined, the correction can be included in a Rapporteur CR.

R2-2207679	Miscellaneous corrections to slice-specific RACH configuration	Spreadtrum Communications	discussion	Rel-17

R2-2207820	Correction on TS 38 331 for RACH common	CATT	CR	Rel-17	38.331	17.1.0	3317	-	F	NR_cov_enh-Core, NR_slice-Core, NR_SmallData_INACTIVE-Core, NR_redcap-Core

R2-2207981	Correction on startPreambleForThisPartition	ZTE Corporation, Sanechips, Ericsson	CR	Rel-17	38.331	17.1.0	3341	-	F	NR_redcap-Core

R2-2207982	Configuration of preambles for feature combination	ZTE Corporation, Sanechips	discussion

R2-2207989	RRC corrections to common RACH framework	Huawei, HiSilicon	draftCR	Rel-17	38.331	17.1.0	NR_SmallData_INACTIVE-Core, NR_slice-Core, NR_redcap-Core, NR_cov_enh-Core

R2-2207997	On the number of RACH partitions	MediaTek Inc.	discussion	Rel-17	NR_cov_enh-Core, NR_slice-Core, NR_SmallData_INACTIVE-Core, NR_redcap-Core

R2-2208240	Miscellaneous corrections to common signalling for RACH partitioning	Nokia, Nokia Shanghai Bell	CR	Rel-17	38.331	17.1.0	3389	-	F	NR_SmallData_INACTIVE-Core, NR_cov_enh-Core, NR_redcap-Core, NR_slice-Core

R2-2208399	Correction on Feature Combination	LG Electronics Inc.	CR	Rel-17	38.331	17.1.0	3415	-	F	NR_SmallData_INACTIVE-Core, NR_slice-Core, NR_redcap-Core, NR_cov_enh-Core

R2-2208910	Correction on the featurePriorities	Huawei, HiSilicon	discussion	Rel-17

### 6.18.2	Common aspects of RACH procedure

A single CR with miscelaneous corrections is encouraged.  Small editorial corrections should be sent directly to rapporteur.  Big open issues can be discussed with contributions with CR in the appendix of the contribution

Move to email discussion

R2-2208988	Report [AT119-e][308][RA Part] UP open issues and CR 38.321 (ZTE)

To agree offline:

- changes proposed in R2-2207905 are not pursued (5/9)

- changes proposed in R2-2207989 are not pursued (6/9)

- changes proposed in R2-2208132 are not pursued (6/8)

- changes proposed in R2-2208400 are not pursued (8/9)

- Agree the changes in R2-2208662 (9/9)

For online discussion:

Proposal 5: changes proposed in R2-2208614 (to clarify that the applicability of specific slice group(s) can be determined by upper layer when the RA procedure is initiated not only by the upper layer but also by the MAC layer) are not pursued (5/9)

R2-2207905	UL carrier selection for RA-SDT	Nokia, Nokia Shanghai Bell	CR	Rel-17	38.321	17.1.0	1353	-	F	NR_SmallData_INACTIVE-Core

=>	The CR is not pursued

R2-2207990	MAC correction to the RACH partitioning	Huawei, HiSilicon	draftCR	Rel-17	38.321	17.1.0	NR_SmallData_INACTIVE-Core, NR_slice-Core, NR_redcap-Core, NR_cov_enh-Core

R2-2208131	Correction to CFRA with additionalRACH-Configs	Ericsson	discussion	Rel-17	NR_redcap-Core, NR_SmallData_INACTIVE-Core, NR_cov_enh-Core, NR_slice-Core

R2-2208132	Correction to CFRA with additionalRACH-Configs	Ericsson	CR	Rel-17	38.321	17.1.0	1372	-	F	NR_redcap-Core, NR_SmallData_INACTIVE-Core, NR_cov_enh-Core, NR_slice-Core

=>	The CR is not pursued

R2-2208400	Correction on fallback cases from CFRA to CBRA in CE-only BWP	LG Electronics Inc.	discussion	Rel-17	NR_SmallData_INACTIVE-Core, NR_slice-Core, NR_redcap-Core, NR_cov_enh-Core

=>	The CR is not pursued

R2-2208614	38.321 CR Correction on the provision of the feature applicability for RACH	Beijing Xiaomi Software Tech	draftCR	Rel-17	38.321	17.1.0	F	NR_SmallData_INACTIVE-Core, NR_cov_enh-Core, NR_redcap-Core, NR_slice-Core

=>	The change is combined in R2-2208662

R2-2208662	Correction on RO Selection with RA Partitioning	vivo	CR	Rel-17	38.321	17.1.0	1398	-	F	NR_SmallData_INACTIVE-Core, NR_cov_enh-Core, NR_redcap-Core, NR_slice-Core

=>	The CR is revised in R2-2208916

R2-2208916	Correction on RO Selection with RA Partitioning	vivo, Xiaomi	CR	Rel-17	38.321	17.1.0	1398	-	F	NR_SmallData_INACTIVE-Core, NR_cov_enh-Core, NR_redcap-Core, NR_slice-Core

# 8	Rel-18

## 8.3	Network energy savings for NR

(xx-Core; leading WG: RAN1; REL-18; WID: RP-213554)

Time budget: 1 TU

Tdoc Limitation: 2 tdocs

### 8.3.1	Organizational

LS, workplan, etc

R2-2208339	Work plan for NR network energy savings	Huawei	Work Plan	Rel-18	FS_Netw_Energy_NR

=>	Noted

R2-2208340	TR 38.864 skeleton for study on network energy savings for NR	Huawei	discussion	Rel-18	FS_Netw_Energy_NR

-	Huawei indicates that RAN3 endorsed the same skeleton

=>	Use skeleton as baseline for further discussion

=>	Noted

R2-2208341	General consideration of RAN2 study	Huawei	discussion	Rel-18	FS_Netw_Energy_NR

### 8.3.2	gNB and UE supporting techniques

Contributions should focus on how to achieve more efficient operation dynamically and/or semi-statically and finer granularity adaptation of transmissions and/or receptions in one or more of network energy saving techniques in time, frequency, spatial, and power domains, with potential support/feedback from UE, and potential UE assistance information

R2-2208431	Discussion on the technical directions for network energy saving	CMCC	discussion	Rel-18

Proposal 1: Spatial and power domain energy saving need to be studied in RAN1 first, and the RAN2 impacts can be studied after RAN1 make further progress. RAN2 can firstly study on the time domain and frequency domain techniques.

-	Nokia thinks that time domain is the most promising domain in RAN2 but for frequency domain we may not be the right group.

=>	We will prioritize time domain and frequency domain techniques until RAN1 further progresses on other techniques.

Proposal 2: RAN2 is kindly asked to study the UE behavior for the carriers with or without SSB/SIB/paging, e.g., whether UE is allowed to receiving SSB/SIB/paging on the anchor carrier, while perform random access on either anchor carrier or non-anchor carriers, similar as NB-IoT.

Proposal 3: RAN2 is kindly asked to study the UE impact if the transmission period of SSB/SIB is increased. Backward compatible issue needs to be considered.

Proposal 4: RAN2 is kindly asked to study the scenario and mechanism for on demand SSB/SIB transmission.

Observation 1: gNB DTX can help reducing the always on signal transmission, such as SSB/SIB transmission and other period RS transmission, and help UE power saving.

Proposal 5: RAN2 is kindly asked to further study on gNB DTX and potential UE impacts.

Questions:

-	Oppo asks if 2,3,4 are complementary or do we prioritize. CMCC indicates that all three proposals are important.

-	Apple is asking if proposal 2-4 are for single or multiple-carrier as the impacts to legacy are quite different.  CMCC indicates that the focus is on multi-carrier.  It is true that single carrier has impact on legacy UEs but we do refarming on some carriers then there will be no legacy.

-	Apple asks if the proposal 3 should focus mainly on SIB1 since for other SIBs we already have flexibility with on demand SIB.  CMCC indicates that this is focusing on all SIBs and also consider solution where we can do RA on another carrier.  Vodafone doesn’t think this is linked to on demand SIB.

-	Vodafone asks about the periodicity as we already have quite a flexible long SIB periodicity.

R2-2207037	Discussion on NW energy saving	KDDI Corporation	discussion

Proposal RAN2 study the mechanism and potential solution on transmit partial SSB and SIB1 of energy saving cell on anchor cell

-	Vodafone thinks that we should ensure that there is no impact to legacy.

-	Apple asks what is anchor cell. KDDI indicates that it is the cell that is transmitting information.

-	CATT indicates Regarding legacy, I understand some cells supporting NES may not be visible to legacy, which will access via "anchor cell".

-	Huawei asks if we should categorize the solutions in single carrier and multiple carrier.  KDDI thinks that we should prioritize multi-carrier.

R2-2207115	Efficient operation of adaptation for network energy saving	Intel Corporation	discussion	Rel-18	FS_Netw_Energy_NR

Proposal#1: RAN2 to further study how to enable a group configuration update mechanism to improve network energy saving.

Proposal#2: RAN2 to further study 2-Step approach (preconfiguring the UEs using dedicated signalling and triggering a configuration change using common group signalling) that allows the network to move between different network energy efficiency levels quickly and efficiently by simultaneously reconfiguring multiple UEs securely.

-	Samsung asks if this assumes using multicast or groupcast signaling.  Intel indicates that this is FFS as it can be DCI with group signaling or paging.  Samsung asks how can we guarantee reliability and should be studied.  Intel asks that we can study reliability but we assume that DCI has been used and can discuss further.  Vivo think that RAN2 should focus RRC/MAC and let RAN1 discuss reliability issue for DCI.

-	CATT thinks that we should take a step back and agree that we should first agree that we should minimize the signalling.

-	CATT also thinks that we should categorize between IDLE and CONNECTED.

Proposal#3: RAN2 to further study using of 2-step approach of group signalling for fast PCell change/handover for the case of turning off booster cells where the candidate (coverage) cell for a UE to handover to is provided in Step 1 while Step 2 provides the indication by the network when UE needs to perform such handover.

-	Apple thinks that group handover may have overlap with other WIs and we should avoid the misalignment.

-	BT thinks that we should study what type of devices can support this and how it impacts legacy.

R2-2208297	Network Energy savings - UE grouping for efficient signaling	Rakuten Mobile, Inc	discussion	Rel-18

=>	Noted

R2-2207246	Time domain NES techniques	InterDigital	discussion	Rel-18	FS_Netw_Energy_NR

Support adaptation of semi-statically configured uplink and downlink data and control resources:

-	UE does not monitor downlink resources when the network is not active due to NES (e.g. for configured PDCCH and PDSCH resources)

-	UE does not transmit on preconfigured uplink resources when the network is not blind decoding due to NES (e.g. PRACH, SRS, PUSCH, and PUCCH resources)

-	Intel asks if this is group or dedicated and if it is similar to the solution from Intel

Proposal 6:	Study techniques for wake-up request transmitted by the UE in connected mode when the serving cell is in sleep state due to NES.

-	Ericsson asks if this is more dependent on RAN1.  InterDigital indicates that this also depends on RAN2 as triggers are more RAN2

-	Qualcomm asks what the UE knows about the state of the network and we should study.

-	Qualcomm thinks that we should take into account UE side.

Proposal 8:	Support cell-specific alternative IDLE/Inactive mode DRX cycles for NES.

Proposal 9:	Study means for paging reception from a non-camped cell.

=>	Noted

R2-2208120	Network Energy Savings Techniques	Qualcomm Incorporated	discussion	Rel-18

Proposal 1: Study how to efficiently align UE DRX to support dynamic DTX on the cell side

Proposal 2: Study how to reduce energy consumption on the cell due to blind monitoring and considering the UE impact.

Proposal 3: Study how UE-controlled target cell selection, including cell selection for CHO mobility, and cell selection/reselection in idle/inactive-mode mobility can be made aware of the cell modes of operation.

Proposal 4: Study how to efficiently configure cell selection by the UE in idle/inactive/CHO mobility to support dynamic ES by the candidate target cells

-	Fujitsu thinks we should also include on-demand MIB

=>	Noted

R2-2207424	On-demand measurement for network energy saving	Apple	discussion	Rel-18	FS_Netw_Energy_NR

Proposal 1: The UE is allowed to notify Network its preference on reference signal pattern for measurements via assistance information. FFS the assistance information is RRC message, MAC-CE or L1 signaling

Proposal 2: The Network can use MAC-CE to activate / deactivate a different pattern of reference signals for measurement

=>	Noted

R2-2208606	Coexistence considerations in network energy saving	MediaTek Inc.	discussion	Rel-18	FS_Netw_Energy_NR

=>	Noted

R2-2207546	NW energy saving in IDLE	Nokia, Nokia Shanghai Bell	discussion	Rel-18	FS_Netw_Energy_NR

=>	Noted

R2-2207414	Efficient PCell and SCell handling for network energy saving	Fujitsu	discussion	Rel-18	FS_Netw_Energy_NR

=>	Noted

R2-2208343	Discussion on network energy saving techniques for multi-carrier case	Huawei, HiSilicon, China Unicom, Deutsche Telekom	discussion	Rel-18	FS_Netw_Energy_NR

=>	Noted

R2-2208342	Discussion on network energy saving techniques for single carrier	Huawei, HiSilicon, Deutsche Telekom	discussion	Rel-18	FS_Netw_Energy_NR

=>	Noted

R2-2207786	discussions on time domain techniques for network energy saving	vivo	discussion	Rel-18

=>	Noted

R2-2207799	Discussion on network energy savings	OPPO	discussion	Rel-18	FS_Netw_Energy_NR

=>	Noted

R2-2207960	Alignment of UE and Network Energy Saving	Fraunhofer IIS, Fraunhofer HHI	discussion	Rel-18

=>	Noted

R2-2208593	Network Energy Saving (NES) Techniques	Samsung	discussion	Rel-18

-	BT asks what is new wrt to BWP.

=>	Noted

R2-2208331	Techniques in various domains and UE assistance information for network energy saving	ZTE corporation, Sanechips	discussion	Rel-18

=>	Noted

UE assistance information

R2-2207512	Consideration on UE Assistance Information	CATT	discussion	Rel-18	FS_Netw_Energy_NR

Proposal 1: If UE assistance information is part of a specific network energy saving technique, it should be discussed and evaluated under the topic of the specific network energy saving technique.

-	Ericsson thinks it should be studied in general

Proposal 2: RAN2 to study further the necessary common UE assistance information for network energy saving.

=>	Noted

R2-2208026	Assistance information from the UE for NW energy savings	Ericsson	discussion

Proposal 1	  RAN2 to start with assistance information example list agreed in RAN1, and to consider extending the list.

Proposal 2 RAN2 to adopt the term “assistance information from the UE” as agreed in RAN1.

=>	Noted

Discussion:

-	Nokia agrees with CATT and we should understand what information is useful for NES and then we can group information based on what we find useful.

-	Apple agrees we should discuss case by case and analyze impact to both UE and gNB

-	Oppo agrees with proposal 1

-	Intel agrees with the general approach but there are some parameters already in UE assistance can be used for NES already and should be used as a baseline.

=> RAN2 to adopt the term “assistance information from the UE” as agreed in RAN1

Solution groups:

- Adaption of MIB/SSB/SIB

-  partial/simplified SSB

2	Increase of SSB/SIB periodicity

3	On demand SSB/SIB1 (FFS if there are enhancements for other SIBs)

- FFS for on-demand MIB

4	Receiving SSB/SIB on one carrier/cell and performing access to another carrier/cell

5	Handover/Fast PCell change for NES

- CHO or new configuration

- group HO

6	Resource adaptation (frequency and time domain)

- Including PRACH, SRS, PUSCH, PUCCH resources and periodicities

- cell DTX/DRX

- measurement

- reference signal type and configuration of reference signal pattern for connected mode

- BWP adaptation

7	Any Cell activation/re-activation or UE wake up request signal (connected/idle)

8	Paging enhancements (includes paging-less solutions)

9	Cell selection/reselection (ie. cell prioritization also including legacy UEs)

Things to study

- Study group configuration and signalling for transitions for different solutions

- pre-configuration and L1/L2 signaling to trigger change of configuration

2	Identify/capture RAN2 impact to legacy for the different solutions

3	Awareness of the NES states at the UE side for the different solutions

4	Aim to minimize DL signalling for NES

5	Consider UE complexity and energy consumption

6	UE assistance information for the specific network energy technique, it’s benefits and impact to UE/NW

[Post119-e][NES] Details of solutions - Huawei

- Capture more RAN2 details/impact/benefit on the solution groups and additional things to study

- Attempt prioritization of solutions

R2-2207116	Additional UE assistance information and UE feedback	Intel Corporation	discussion	Rel-18	FS_Netw_Energy_NR

R2-2207293	Assistance information to support choice of NES configuration	NEC Telecom MODUS Ltd.	Discussion

R2-2208592	Feedback and Assistance Information for NES	Samsung	discussion	Rel-18

R2-2207247	Frequency domain and UE assistance NES techniques	InterDigital	discussion	Rel-18	FS_Netw_Energy_NR

R2-2207292	Finer granularity configuration for NES	NEC Telecom MODUS Ltd.	discussion

R2-2207406	Consideration on network energy saving	Fujitsu	discussion	Rel-18	FS_Netw_Energy_NR

R2-2207423	Initial discussion on RAN2 work of Network energy saving	Apple	discussion	Rel-18	FS_Netw_Energy_NR

R2-2207511	Network energy savings: issues for investigation in RAN2	CATT	discussion	Rel-18	FS_Netw_Energy_NR

R2-2207545	NW energy saving in CONNECTED	Nokia, Nokia Shanghai Bell	discussion	Rel-18	FS_Netw_Energy_NR

R2-2207787	discussion on frequency domain and UE-assisted Network Energy saving techniques	vivo	discussion	Rel-18

R2-2207800	Discussion on the UE assistance information	OPPO	discussion	Rel-18	FS_Netw_Energy_NR

R2-2207919	Discussion on supporting of network energy savings for NR	Lenovo	discussion	Rel-18	FS_Netw_Energy_NR

R2-2207920	Discussion on the state transition in NES	Lenovo	discussion	Rel-18	FS_Netw_Energy_NR

R2-2208031	Miscellaneous mechanisms for network energy savings	Ericsson	discussion

R2-2208233	gNB operation for NES	ETRI	discussion

R2-2208330	Supporting access via assistant cell for network energy saving	ZTE corporation, Sanechips	discussion	Rel-18

R2-2208432	Analysis on power consumption in base station	CMCC	discussion	Rel-18

R2-2208573	Energy saving on system information transmission	Xiaomi	discussion	Rel-18	FS_Netw_Energy_NR

## 8.8	NR support for UAV

(xx-Core; leading WG: RAN1; REL-18; WID: RP-213600)

Time budget: 0.5 TU

Tdoc Limitation: 2

### 8.8.1	Organizational

R2-2207328	Uncrewed Aerial Vehicles in Rel-18 - workplan	Nokia, Nokia Shanghai Bell	Work Plan	Rel-18	NR_UAV-Core

=>  Noted

=> The WP may be revised once the SA2 objective is agreed in next plenary

### 8.8.2	Measurement reporting

Contributions should focus on enhancement to measurement reports, for example UE-triggered measurement report based on configured height thresholds, Reporting of height, location and speed in measurement report, Flight path reporting, Measurement reporting based on a configured number of cells (i.e. larger than one) fulfilling the triggering criteria simultaneously

Note: Work done in LTE is a starting point for this objective. NR-specific enhancements can be considered, if needed, while overall the LTE and NR solutions should be harmonized as much as possible.

R2-2207518	Measurement Reporting for NR UAV	CATT	discussion	Rel-18	NR_UAV-Core

Proposal 1: As in LTE, UE-triggered measurement report based on configured height thresholds is reused for NR UAV.

Proposal 2: For NR UAV, LTE principle i.e. UE location information, reference altitude information and vertical speed are included in the measurement report. Any parameters for reporting can be further discussed.

Proposal 3: A request message for flight path information from serving gNB is introduced. Use UE information request response message to response.

Proposal 4: A location list of waypoints (3D location information) is adopted as the basic content of flight path report.

Proposal 5: The waypoints can optionally include time stamps if available. Absolute time information is used as baseline for time stamp. UE reports up to the maximum number of waypoints configured by gNB.

-	Nokia asks how it works if the timestamp is optional and should be mandatory.

Proposal 6: Introduce the number of triggered cells for interference detection for NR UAV UE.

=>	Noted

R2-2207194	Discussion on NR support for UAV	NTT DOCOMO, INC.	discussion	Rel-18

Proposal3: Introduce numberOfTriggeringCells in ReportConfigNR to control the excessive measurement reporting for NR UAV.

Proposal4: Introduce numberOfTriggeringBeams in ReportConfigNR to control the excessive measurement reporting for NR UAV.

-	Nokia explains that triggeringcells was for interference but the beams are for mobility.  NTT explains that this is to reduce measurement reporting overhead.

Proposal5: Introduce a prohibit timer to control the excessive measurement reporting for NR UAV.

-	Nokia thinks this should be studied

Proposal6: Introduce a NumberOfGoodBeams threshold to take number of good beams of target cell into account for NR UAV mobility.

-	ZTE asks if we support standalone only or DC as well.  Nokia doesn’t think there was anything explicitly stated but we should focus on standalone.  Huawei thought that DC was not included.

-	Intel asks if P3,4 and 6 are individual features or combined.  For P4 do we count individual beams.    NTT explains that 3 and 4 is combined and 6 is individual.

=>	Noted

R2-2207925	NR support for UAV first steps plus Inter RAT aspects	Vodafone GmbH	discussion	Rel-18

Proposal 1: It is proposed to re-use 1 to 1 events H1 and H2 defined in LTE and the height reporting within NR

Proposal 2: The accuracy of 1 m for the Hight information is sufficient and hight IE could be re-used in NR

Proposal 3: The definition of the Location Info seems to be sufficient in LTE, but the definition is often relating to LPP protocol 37.335 LTE Positioning Protocol (LPP) and it needs to be confirmed the location information can be re-used with NR.

-	Qualcomm ask if we reuse the LTE or do we use the NR one.

-	CATT points out that the location needs to be reported to gNB and not the CN.

Proposal 4: The current procedure used for flight path reporting and configuration could be re-used. It is proposed to study if the number of Way Points need to be enhanced.

-	Samsung asks why we would need to increase the number of way points.  Vodafone asks why we have limited in the first place anyways.

Proposal 5: The functionality of the numberOfTriggeringCells should be introduced in NR. Also, a leaving conditions available in LTE should be introduced too.

Proposal 6: It is proposed to discuss if the enhancements to InterRAT mobility mechanism for UAV need to be done.

-	Vodafone clarifies that the scenario is drone is moving to an area where there is no NR.  Is existing mobility sufficient to cover the inter-RAT mobility.   Huawei agrees that inter-RAT mobility is very important to work well, but perhaps we can rely on legacy mobility.

=>	Noted

R2-2207329	On measurement reporting enhancements for UAVs - LTE baseline in NR framework	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_UAV-Core

Proposal 2: RAN2 is asked to consider if the UAV UE may be allowed to scale some of the RRM parameters (e.g. TTT) when event H1 or H2 is met.

Proposal 3: RAN2 to study the use of H1 and H2 event in CHO triggering.

Proposal 4: RAN2 to study the vertical movement and associated mobility for UAV UEs.

Proposal 8: Consider studying the following areas related to flight path plan:

•	Providing flight path plan from the core network to gNB (e.g. UTM to gNB)

•	Waypoints distribution in time and space/location

•	Flight path plan modification

Proposal 10: In measurement reporting based on numberOfTriggeringCells study how to avoid sending the measurement reports mainly due to reportOnLeave.

-	Qualcomm thinks we should study all these proposals.  Huawei explains that CHO was not included on purpose.  Intel explains that it was downscoped.

-	Ericsson agrees with P2-P3 and we should look at specification changes and ensure we limit complexity.  For example vertical location is included in common parameters but we have to be careful what we will report, everything or have something to allow just reporting location.

Proposal 6: If user consent is needed for location reporting in CONNECTED, this shall be clearly indicated to SA3 and other affected WGs already now/as soon as possible.

-	Vodafone thinks we should not bother SA3 everytime.  Nokia explains this was a problem in NTN.

=>	Noted

R2-2207233	Measurement Reports Enhancement for UAV	OPPO	discussion	Rel-18

=>	Noted

Agreements

- Use LTE principle as a baseline, introduce similar event H1 (aerial UE height become higher than threshold) and H2 (aerial UE height become lower than threshold.  FFS if further NR enhancements are needed.  FFS study scaling of RRM parameters (e.g. which parameters and what is the purpose/benefit of the scaling and how)

FFS how to limit excessive measurements and measurement reporting

FFS if user consent is needed for location reporting in CONNECTED

FFS study the vertical movement and associated mobility for UAV UEs

2	Rel-18 NR supports reporting of UAV UE’s height, location and velocity. It is for further study what accuracy and reporting mechanisms are required and if further enhancements are needed.

3	As in LTE, flight path plan reporting will be introduced.  Location list of waypoints (3D location information) and timestamp is adopted as the basic content of flight path report.  FFS if timestamp is mandatory or optional for NR.  FFS if further enhancements are needed

4	Introduce similar functionality to LTE (numberofTriggeringCells).  FFS whether numberoftriggerbeams for NR is required or other enhancements.  FFS study how to avoid sending the measurement reports mainly due to reportOnLeave.

Not treated

R2-2207076	Consideration on measurement reporting of NR support for UAV	DENSO CORPORATION	discussion	NR_UAV-Core

R2-2207154	Considerations on Measurement Reports Enhancements	NEC Europe Ltd	discussion	Rel-18	NR_UAV-Core

R2-2207601	Discussion on measurement reporting enhancement for NR UAV	vivo	discussion	Rel-18	NR_UAV

R2-2207602	Discussion on flight path reporting for NR UAV	vivo	discussion	Rel-18	NR_UAV

R2-2207624	On measurement and reporting enhancements 	Ericsson	discussion	NR_UAV-Core	Revised

R2-2207715	measurement report enhancement for NR UAV	Lenovo	discussion	Rel-18

R2-2207836	UAV measurement reporting	Sony	discussion	Rel-18	NR_UAV

R2-2207935	Discussion on measurement reporting in UAV	Apple	discussion	Rel-18	NR_UAV-Core

R2-2208042	On measurement and reporting enhancements 	Ericsson	discussion	NR_UAV-Core	R2-2207624	Withdrawn

R2-2208098	Measurement and reporting enhancements	Qualcomm Incorporated	discussion	Rel-18	NR_UAV-Core

R2-2208099	Mobility considerations and some performance results	Qualcomm Incorporated	discussion	Rel-18	NR_UAV-Core

R2-2208250	UAV support for NR	Intel Corporation	discussion	Rel-18	NR_UAV-Core

R2-2208279	Measurement reporting for UAV	InterDigital	discussion	Rel-18	NR_UAV-Core

R2-2208335	Measurement Report Enhancement	LG Electronics Finland	discussion

R2-2208336	Flight Path Information Enhancement	LG Electronics Finland	discussion

R2-2208412	Discussion on measurement reporting enhancements for NR UAV	ZTE Corporation, Sanechips	discussion	Rel-18	NR_UAV-Core

R2-2208421	Consideration on subscription-based UAV identification	Huawei, HiSilicon	discussion	Rel-19	NR_UAV-Core

R2-2208445	Consideration on Measurement Reporting for UAV	CMCC	discussion	Rel-18	NR_UAV-Core

R2-2208469	Discussion on measurement reporting for NR UAV	Xiaomi	discussion

R2-2208608	Discussion on enhancements on measurement reports for NR UAV	Samsung Electronics Co., Ltd	discussion	Rel-18	NR_UAV-Core

### 8.8.3	Subscription-based aerial-UE identification

Contributions should focus on signaling required to support subscription-based aerial-UE identification

Note: Work done in LTE is a starting point for this objective. NR-specific enhancements can be considered, if needed, while overall the LTE and NR solutions should be harmonized as much as possible.

R2-2208630	Discussion on subscription-based aerial-UE identification for NR UAV	Samsung Electronics Co., Ltd	discussion	Rel-18	NR_UAV-Core

R2-2207234	Subscription-based Aerial-UE Identification	OPPO	discussion	Rel-18

R2-2207330	On subscription-based UAV UE identification	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_UAV-Core

R2-2207519	Subscription-based Aerial-UE Identification for NR	CATT	discussion	Rel-18	NR_UAV-Core

R2-2208413	Discussion on subscription-based identification for NR UAV	ZTE Corporation, Sanechips	discussion	Rel-18	NR_UAV-Core

R2-2208420	Measurement report enhancement for UAV	Huawei, HiSilicon	discussion	Rel-18	NR_UAV-Core

R2-2208426	Discussion on subscription-based aerial-UE identification	CMCC	discussion	Rel-18	NR_UAV-Core

### 8.8.4	UAV identification broadcast

Study and specify, if needed, enhancements for UAV identification broadcast

NOTE: This Agenda Item will not be treated in this meeting

R2-2207155	Considerations on Enhancements for UAV Identification Broadcast	NEC Europe Ltd	discussion	Rel-18	NR_UAV-Core

R2-2207626	On broadcasting UAV identification	Ericsson	discussion	NR_UAV-Core	Revised

R2-2207716	Discussion on broadcasting remote id for UAV	Lenovo	discussion	Rel-18

R2-2207837	UAV identification broadcast	Sony	discussion	Rel-18	NR_UAV

R2-2208040	On broadcasting UAV identification	Ericsson	discussion	NR_UAV-Core	R2-2207626	Withdrawn

R2-2208162	UAV Identity broadcast and Identification	Beijing Xiaomi Mobile Software	discussion	Rel-18	NR_UAV-Core

## 8.15	R18 Other

Misc Impacts from Other RAN WGs and TSGs (incl MC Enhancements). LS ins for Rel-18 topics that has no RAN WI.

Time budget: 0.5 TU

Tdoc Limitation: -

Low Latency

This topic is handled by UP breakout session (Diana)

R2-2208914	Report for email discussion [AT119-e][310][R18 Others - Low Latency] LS on Low latency (Huawei)

R2-2208688	Draft reply LS on RAN feedback for low latency	Huawei	LS out	Rel-18	FS_5TRS_URLLC	To:SA2	Cc:RAN1, RAN3	Late

=>	The LS is updated in R2-2208913

R2-2208913	Draft reply LS on RAN feedback for low latency	RAN2	LS out	Rel-18	FS_5TRS_URLLC	To:SA2	Cc:RAN1, RAN3

=>	The LS is approved

R2-2206963   LS on RAN feedback for low latency (S2-2201767; contact: Huawei)           SA2   LS in    Rel-18  FS_5TRS_URLLC         To:RAN2          Cc:RAN1, RAN3

R2-2208134   Discussion on RAN feedback for low latency         Ericsson           discussion   Rel-18

R2-2208007   Proposed response to SA2 LS R2-2203930 on low latency Nokia, Nokia Shanghai Bell      discussion        Rel-18  FS_5TRS_URLLC

Moved from 3

R2-2207043   Draft reply LS on RAN feedback for low latency     Qualcomm Incorporated   discussion        Rel-18  FS_NR_XR_enh

Moved from 8.5.1

R2-2207768   Consideration on meeting very low latency requirement in TDD      ZTE Corporation, Sanechips, China Southern Power Grid Co., Ltd         discussion   Rel-17  NR_IIOT_URLLC_enh-Core      R2-2205732

Moved from 6.5.1

R2-2207775   [DRAFT] Reply LS on RAN feedback for low latency          ZTE Corporation, Sanechips           LS out  Rel-17  NR_IIOT_URLLC_enh-Core      R2-2205734   To:SA2 Cc:RAN3

Moved from 6.5.1

R2-22086871	21		Discussion on RAN feedback for low latency enquired by SA2	Huawei	discussion	Late