3GPP TSG-RAN WG2 #119-e	R2-2208710

Online Meeting, Aug 17th – 29th, 2022

Agenda Item:	6.11.1

Source:	Ericsson

Title:	RRC Corrections for Positioning

Document for:	Discussion, Decision

# Introduction

The below papers have been submitted for positioning correction which impacts RRC

R2-2207881 “Correction for inactivePosSRS-TAT upon transitioning to RRC_CONNECTED” Huawei,HiSilicon

R2-2208076 “Miscellaneous correction for Positioning”, Ericsson, Nokia, Nokia Shanghai Bell

R2-2207411 “Change request about PPW configuration” vivo, Ericsson

# Discussion

## R2-2207881	Correction for inactivePosSRS-TAT upon transitioning to RRC_CONNECTED

The CR in R2-2207881 mentions that the UE shall stop the RRC Inactive Positioning SRS Time alignment Timer like CG-SDT when RRC setup/resume msg is received:

For CG-SDT, as specified in 5.3.3.4, UE shall perform the following actions upon reception of the RRCSetup:

1>	if the RRCSetup is received in response to an RRCReestablishmentRequest; or

1>	if the RRCSetup is received in response to an RRCResumeRequest or RRCResumeRequest1:

2>	if sdt-MAC-PHY-CG-Config is configured:

3>	instruct the MAC entity to stop the cg-SDT-TimeAlignmentTimer, if it is running;

The SRS Positioning for Inactive mode is valid only for RRC Inactive mode. If the UE receives RRC setup/resume this implies that the UE is switching to RRC Connected mode.

In positioning session, it was agreed to follow CG-SDT mechanism for SRS for positioning config in RRC Inactivate. Considering this also the same behaviour as defined for CG SDT TAT should be applied also for SRS for Positioning TAT.

CR in R2-2207881 to instruct to MAC layer to stop inactivePosSRS-TimeAlignmentTimer upon receiving RRC Setup/Resume is agreed to be merged into the rapporteur CR

## R2-2208076 Miscellaneous correction for Positioning

The CR adds TS 38.305 as a reference as the definition of UE TxTEGID needs to be cited from TS 38.305.

Further, it consolidates the clause in section 5.7.17 “Derivation of pathloss reference for TA validation of SRS for Positioning transmission and CG-SDT in RRC_INACTIVE” that is mainly polishing the content.

CR in R2-2208076 to add TS 38.305 reference and to consolidate the clause in section 5.7.17 is agreed to be merged into the rapporteur CR

## R2-2207411 “Change request about PPW configuration”

The CR provides correction to the parameter “maxNrofPPW-Config-r17”.

From

Maximum number of Preconfigured PRS processing windows per BWP

To

Maximum number of activated PRS processing windows across all active DL

Since, DL-PRS-ProcessingWindowPreConfigAddModList-r17’ and ‘DL-PRS-ProcessingWindowPreConfigReleaseList-r17’ are defined per BWP-DownlinkDedicated.

So then ‘maxNrofPPW-Config-r17’ is the maximum number of PPW configurations per DL BWP rather than the maximum number of activated PRS processing windows across all active DL BWPs.

CR in R2-2207411 to correct definition of maxNrofPPW-Config-r17 as Maximum number of Preconfigured PRS processing windows per BWP is agreed to be merged into the rapporteur CR.

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	CR in R2-2207881 to instruct to MAC layer to stop inactivePosSRS-TimeAlignmentTimer upon receiving RRC Setup/Resume is agreed to be merged into the rapporteur CR

Proposal 2	CR in R2-2208076 to add TS 38.305 reference and to consolidate the clause in section 5.7.17 is agreed to be merged into the rapporteur CR

Proposal 3	CR in R2-2207411 to correct definition of maxNrofPPW-Config-r17 as Maximum number of Preconfigured PRS processing windows per BWP is agreed to be merged into the rapporteur CR.

# References

[1] AI 6.11