3GPP TSG-RAN WG2 Meeting #119 electronic	R2-2208760

Online, August 17 - 26th, 2022

Agenda Item:	6.10.1.1

Source: 	Thales

Title:  	Summary of [AT119-e][109][NR-NTN] Stage-2 CR (Thales)

Document for:	Discussion and Decision

# Introduction

This document aims to summarize the following discussion which aims at consolidating a CR for TS 38.300 as outcome of RAN2#119-e.

[AT119-e][109][NR-NTN] Stage-2 CR (Thales)

Scope: Draft Stage-2 CR, also considering Stage-2 text proposals in contributions to other AIs

Intended outcome: Agreeable Stage-2 CR

Deadline (for companies' feedback): Thursday 2022-08-25 1000 UTC

Deadline (for rapporteur's summary in R2-2208760 and for Stage-2 CR in R2-2208761): Friday 2022-08-25 1000 UTC

Status: Ongoing

Among the TDOCs submitted under RAN2 NR-NTN agenda, at least the following are considered: R2-227065, 7141, 7144, 7322, 7442, 7597, 8380, 8678 and 8751

# 1st round discussion

## 2.1 TS 38.300 rapporteur’s review

In R2-2207322, a number of corrections are proposed by the rapporteur of TS 38.300 (Nokia) with the email discussion rapporteur enhancements (see draft R2-22xxxxx R17 NTN Stg2 (CR Moderator)_R1_v00 in same folder as the email disc)

Question 2.1.1: Do companies agree with the proposed corrections for chap 3.2 (adding “LEO operates at altitudes between 300 km and 1500 km and MEO at altitudes between 7000 km and 25000 km, approximately. Service link switch refers to a change of the serving satellite.” in the definition of Non-Geosynchronous orbit ?

| Company | Agree/Disagree | Comments/Suggestions |
| --- | --- | --- |
| Thales | Agree |  |
| Intel | partially agree | ok with the first part. But “Service link switch refers to a change of the serving satellite.” is not relevant to the definition of NGSO orbit, this text could be clarified still in 16.14.1. |
| Lenovo | Agree |  |
| Xiaomi | partially agree | This part is not relevant to the definition and should be removed: “Service link switch refers to a change of the serving satellite.” |
| CATT | See the comment | We agree to add “LEO operates at altitudes between 300 km and 1500 km and MEO at altitudes between 7000 km and 25000 km, approximately.” here. But for “Service link switch refers to a change of the serving satellite”, it seems more appropriate to be added in the definition of “Service link”. Additionally, we think the limitation of “In case of NGSO” can be removed, for GSO system, there will also occur service link switch situation caused by UE movement. The modification like: Service link: Wireless link between the NTN payload and UE. Service link switch refers to a change of the serving satellite. |
| Apple | Partially agree | The last sentence which is not relevant to the definition should be removed. Service link switch refers to a change of the serving satellite. |
| Samsung | Partially agree | Agree with Apple |
| Huawei, HiSilicon | partially agree | This sentence is not needed in 38300: “Service link switch refers to a change of the serving satellite.” |
| OPPO |  | Agree with Apple. |
| Qualcomm | Agree partly | Either keep the “service link..” part in the original or move to the section as suggested by CATT. |
| Nokia | Agree | Proponent. If companies think second part is not relevant to the Definitions section then we are fine not to copy it there. But please note that it is also not relevant to the section where it is currently. Thus, our suggested modification. |
| LGE | Partially agree | Agree with CATT. |
| Panasonic | Agree with CATT and … | A service link switch can also happen between one beam and another – both emitted from the same satellite. This to be added to the related definition. Or is that case excluded in Rel.17? |
| Ericsson | Partially agree | 3.2 change: this part “Service link switch refers to a change of the serving satellite.” is NOT part of geosynch definition. Can either be 1) its own definition in 3.2, or 2) part of 16.14.1 (directly after the section that introduces different service links). |

[Rapporteur summary]:

Given that most companies agree to the proposed change except the addition of the Service link switch, , the moderator

Proposal 2.1.1a: Add “LEO operates at altitudes between 300 km and 1500 km and MEO at altitudes between 7000 km and 25000 km, approximately.” to the definition of Non-Geosynchronous orbit in chap 3.2.

Proposal 2.1.1b: Add “Service link switch refers to a change of the serving satellite.” in the chapter 16.14.4.1	Definitions (part of chapter 16.14.4	Switch over)

Question 2.1.2: Do companies agree with the proposed corrections for chap 16.14.1	 Overview ?

| Company | Agree/Disagree | Comments/Suggestions |
| --- | --- | --- |
| Thales | Agree |  |
| Intel | partially agree | It would be better to keep “Service link switch refers to a change of the serving satellite.” Still in 16.14.1. |
| Lenovo | Agree |  |
| Xiaomi | partially agree | Agree with intel that to keep the sentence “Service link switch refers to a change of the serving satellite.” |
| CATT | agree | Agree to move the orbit altitude explanation to section 3.2 Definitions and modify “service links” to “coverage”. |
| Apple | Partially agree | The sentence of the service link switch should be kept. |
| Samsung | Partially agree | Agree to move the orbit altitude explanation. “types of service links” is referred in later text, should not be changed alone. The definition of service link switch “Service link switch refers to a change of the serving satellite” can be moved to 16.14.4.1 (together with feeder link switch definition). |
| Huawei, HiSilicon | Agree | This sentence is not needed in 38300: “Service link switch refers to a change of the serving satellite.” |
| OPPO | Agree |  |
| Nokia | Agree |  |
| LGE | Agree |  |
| Panasonic | Agree |  |
| Ericsson | Partially agree | 16.14.1 do not rename service link scenarios to “coverages”. Instead change the With NGSO satellites, the gNB can provide either quasi-Earth-fixed service link or Earth-moving service link, while gNB operating with GSO satellite can provide Earth fixed service link. |

[Rapporteur summary]:

See summary related to Question 2.1.2

Proposal 2.1.2: Replace coverage by service link and change one sentence as “With NGSO satellites, the gNB can provide either quasi-Earth-fixed service link or Earth-moving service link, while gNB operating with GSO satellite can provide Earth fixed service link.”

Question 2.1.3: Do companies agree with the proposed corrections for chap 16.14.2.1	 Scheduling and Timing + enhancements from Moderator of the present email disc ?

| Company | Agree/Disagree | Comments/Suggestions |
| --- | --- | --- |
| Thales | Agree |  |
| Intel | Agree |  |
| Lenovo | Agree |  |
| Xiaomi | Agree |  |
| CATT | Agree |  |
| Apple | Agree |  |
| Samsung | Partially agree | We are not sure the text from “It is needed for UE action and assumption on….” To the end of 16.14.2.1 is really needed for stage-2 description. Go with majority view. |
| Huawei, HiSilicon | Agree |  |
| OPPO | Agree |  |
| Qualcomm | Agree |  |
| Nokia | Agree | Regarding the explanations from Thales, thanks for these. They should be included in the text, for example below figure 16.14.2.1-1, you can some excerpts relate to 38.821 or 38.213. Please add such reference in the text, so that it is known to all readers where to search for additional details. On the ‘approximately’ – either please insert this text/explanation to the 38.300 description (but it will not be a Stage-2 text) or let’s remove the term entirely. |
| LGE | Agree | We slightly prefer to remove ‘approximately.’ |
| Panasonic | Agree | In figure 16.14.2.1-1 there should be a gateway on the ground – instead of a terrestrial base station. |
| Ericsson | Partly agree | 16.14.2.1 modify Nokia’s proposed changes: Koffset exchange “approximately corresponds” to “shall be larger or equal “: - Koffset is a configured scheduling offset that need to be larger or equal to the sum of the service link RTT and the common TA. - kmac is a configured offset that need to be larger or equal to the RTT between the RP and the gNB. It is needed for UE timing of downlink configuration change indicated by a MAC-CE command in PDSCH. We do not agree with moderators comment on figure 16.14.2.1-2, we agree with Nokia. This is captured in RAN1 spec already, please Remove the whole text part and figure 16.14.2.1-2. |

[Rapporteur summary]:

On the basis of the feedbacks, the moderator proposes the following:

Proposal 2.1.3: Agree with the proposed corrections in draft R2-2208761 v00 (including enhancements from Moderator of the present email disc) for chap 16.14.2.1	 Scheduling and Timing (Note that “approximately” is removed). +

- Change “approximately corresponds” by “need to be larger or equal”

- Correct the sentence “It is needed for UE timing of downlink configuration change indicated by a MAC-CE command in PDSCH.”

- Remove figure 16.14.2.1-2 and related text

Question 2.1.4: Do companies agree with the proposed corrections for chap 16.14.2.2	Pre-compensation by the UE + enhancements from Moderator of the present email disc ?

| Company | Agree/Disagree | Comments/Suggestions |
| --- | --- | --- |
| Thales | Agree |  |
| Intel | Agree |  |
| Lenovo | Agree |  |
| Xiaomi | Agree |  |
| CATT | Agree |  |
| Apple | Agree |  |
| Samsung | Partially agree | The sentence “To achieve synchronisation, before and during connection to an NTN cell, the UE computes the TTA by self-estimating the RTT corresponding to the service link round trip delay and computing the Common TA based on related higher layer parameters if indicated by the network.” Is confusing, it reads like UE computes Common TA, which is not true. One way to reword: “To achieve synchronisation, before and during connection to an NTN cell, the UE computes the service link RTT based on the GNSS position and the satellite ephemeris and autonomously pre-compensates TTA.” |
| Huawei, HiSilicon | Partially agree | Agree with Samsung’s wording. Besides, we think the sentence “The UEs may be configured to report Timing Advance during Random Access procedures or in connected mode” can be kept as a general description of TA reporting mechanism. It is a way to let NW know UE’s pre-compensation. Otherwise, the subsequent sentence seems abrupt:” In connected mode, event-triggered reporting of the Timing Advance is supported”. |
| OPPO | Agree | Regarding Samsung’s comment, indeed common TA is computed by the UE based on common TA drift and drift variant. |
| Qualcomm | Agree | Suggestion provided here is also fine. |
| Nokia | Agree | Samsung’s suggestion seems to be OK. |
| LGE | Agree | Agree with Samsung’s suggestion. Also, Huawei’s suggestion also needs to be handled. We suggest to change the sentence “In connected mode, the UE should be able to continuously update the Timing Advance and frequency pre-compensation.” as “In connected mode, the UE should be able to continuously update the Timing Advance and frequency pre-compensation, which the UE may be configured to event-triggered reporting of the Timing Advance.” |
| Panasonic | Widely agree | Agree to Samsung’s wording. In addition, the key expressions “frequency Doppler shift” and “frequency pre-compensation” are not well aligned. There should be a single better expression, e.g. “Doppler spread” (and hence “Doppler spread pre-compensation”). Furthermore, amend “… the network broadcast blabla” with an “s” resulting in “… the network broadcasts blabla” (first sentence of this clause). |
| Ericsson | Agree |  |

[Rapporteur summary]:

On the basis of the feedbacks, the moderator proposes the following:

Proposal 2.1.4: Agree with the proposed corrections in draft R2-2208761 v00 (including enhancements from Moderator of the present email disc) for chap 16.14.2.2	Pre-compensation by the UE. In addition

- Correct the following sentence as “To achieve synchronisation, before and during connection to an NTN cell, the UE computes the service link RTT based on the GNSS position and the satellite ephemeris and autonomously pre-compensates TTA.the TTA by self-estimating the RTT corresponding to the service link round trip delay and computing the Common TA based on related higher layer parameters if indicated by the network.”

- Restore the sentence “The UEs may be configured to report Timing Advance during Random Access procedures or in connected mode”.

- Correct the following sentence as “In connected mode, the UE should be able to continuously update the Timing Advance and frequency pre-compensation, which the UE may be configured to event-triggered reporting of the Timing Advance.”

Question 2.1.5: Do companies agree with the proposed corrections for chap 16.14.3.1	 Mobility in RRC_IDLE and RRC_INACTIVE ?

| Company | Agree/Disagree | Comments/Suggestions |
| --- | --- | --- |
| Thales | Agree |  |
| Intel | partially agree | the neighbour cell satellite ephemeris can be highlighted in 16.14.3.3 for measurement. |
| Lenovo | Agree |  |
| Xiaomi | Agree |  |
| CATT | Agree |  |
| Apple | Agree |  |
| Samsung | Agree |  |
| Huawei, HiSilicon | Agree |  |
| OPPO | Agree |  |
| Qualcomm | Agee |  |
| Nokia | Agree |  |
| LGE | Agree |  |
| Panasonic | Agree with comment | Agree with Intel’s comment, i.e. elsewhere in this spec both, the serving and neighbouring/target cell satellite(s)’(s) ephemerides, need to be sorted out with related spec text. |
| Ericsson | Agree | We suggest adding a sentence about ephemeris provisioning |

[Rapporteur summary]:

On the basis of the feedbacks, the moderator proposes the following:

Proposal 2.1.5: Agree with the proposed corrections in draft R2-2208761 v00 for chap 16.14.3.1	 Mobility in RRC_IDLE and RRC_INACTIVE

+ adding sentence “The NTN ephemeris is provisioned. It includes serving cell's satellite ephemeris and neighbouring cell's satellite ephemeris.”

Question 2.1.6: Do companies agree with the proposed corrections for chap 16.14.4 	Switch over ?

| Company | Agree/Disagree | Comments/Suggestions |
| --- | --- | --- |
| Thales | Agree |  |
| Intel | partially agree | if 16.14.4 is only for feeder link swich, the section name could be changed to “feeder link switch over”. |
| Lenovo | Agree |  |
| Xiaomi | Agree |  |
| CATT | Agree |  |
| Apple | Agree | We agree with Intel that the title of this section can be changed to “feeder link switch over” |
| Samsung | Agree | The definition of service link switch can be moved to this section |
| Huawei, HiSilicon | Agree |  |
| OPPO | Agree |  |
| Nokia | Agree |  |
| LGE | Agree |  |
| Panasonic | Agree with comments | Agree with Intel’s comment. In addition, do we want to cover Inter-Satellite Links (ILSs) at the feeding end with Rel.17 already or do come to those with Rel.18? |
| Ericsson | Agree |  |

[Rapporteur summary]:

The title should remain “Switch over” since it is a pace holder for both service and feeder link switch been defined by RAN3 covering service link switch

As per suggestion from Samsung the definition of the service link can be moved in this section

ISL are not in the scope of R17 nor R18.

On the basis of the feedback, the moderator suggests to agree the following

Proposal 2.1.6: Agree with the proposed corrections in draft R2-2208761 v00 for chap 16.14.4.1 Definitions

In relation to the summary related to question 2.1.1b, the following is again recommended

Proposal 2.1.1b: Add “Service link switch refers to a change of the serving satellite.” in the chapter 16.14.4.1	Definitions (part of chapter 16.14.4	Switch over)

Question 2.1.7: Do companies agree with the proposed corrections for chap 16.14.5 	NG-RAN signalling ?

| Company | Agree/Disagree | Comments/Suggestions |
| --- | --- | --- |
| Thales | Agree |  |
| Intel | agree |  |
| Lenovo | Agree |  |
| Xiaomi | Agree |  |
| CATT | No strong view |  |
| Apple | Agree |  |
| Samsung | Agree |  |
| Huawei, HiSilicon | Agree |  |
| OPPO | Agree |  |
| Nokia | Agree |  |
| LGE | Agree |  |
| Panasonic | Don’t agree | We’re missing the precision here regarding the formerly listed four Cell Identity types. Nokia proposed to refer to RAN3 specs for those, but this didn’t happen (unless we overlooked it). |
| Ericsson | Agree |  |

[Rapporteur summary]:

As indicated by Panasonic, it is preferable not to remove the list of use cases if it is not reflected in RAN3 stg3 specifications

- The proposed correction is dismissed

## 2.2 Chapter 16.14.3.2.2	Conditional Hand-over

In R2-2207322 and R2-2208380, the following corrections (in red) are proposed:

The same principle as described in 9.2.3.4 applies to NTN unless hereunder specified.

’

NTN supports the following additional triggering conditions upon which UE may execute CHO to a candidate cell, as defined in TS 38.331 [12]:

-	event A4;

-	A time-based trigger condition;

-	A location-based trigger condition.

A time-based or a location-based trigger condition is always configured together with one of the measurement-based trigger conditions (CHO events A3/A4/A5). Location is defined by the distance between UE and a reference location. Time is defined by the time between T1 and T2, where T1 is an absolute time value t1-Threshold and T2 is the time after a duration started at T1, i.e. t1-Threshold + duration, defined in TS 38.331 [12].

For NTN, it is up to UE implementation how the UE evaluates the time- or location-based condition jointly with the RRM event Ax, as long as the UE has RRM measurement results within the time window [T1, T2] or when the location condition is met.

Question 2.2: Do companies agree with the proposed corrections ?

| Company | Agree/Disagree | Comments/Suggestions |
| --- | --- | --- |
| Thales | Agree |  |
| Intel | agree |  |
| Lenovo | Agree |  |
| CATT | Agree |  |
| Apple | Partial agree | RRM event Ax should be updated to “one of the RRM measurement-based CHO event A3, A4, or A5”. |
| Samsung | See comment | NTN supports the following additional trigger conditions for CHO execution condition, as defined in TS 38.331 [12]: - CondEvent A4; - CondEvent D1; - CondEvent T1. CondEvent D1 or CondEvent T1 is always configured together with one of the measurement-based trigger conditions (CHO events A3/A4/A5). For NTN, it is up to UE implementation how the UE evaluates CondEvent D1 or CondEvent T1 jointly with one of the RRM measurement-based CHO event A3, A4, or A5, as long as the UE has RRM measurement results when CondEvent D1 or CondEvent T1 is fulfilled. |
| Huawei, HiSilicon | Agree |  |
| OPPO | Agree |  |
| Nokia |  | We suggest to follow our proposal from 7322. We do not need such details as proposed above. Stage-2 shall be made simpler. |
| LGE | Partially agree | We suggest to modify ‘event A4’ to ‘event A3/A4/A5.’ |
| Panasonic | Agree partly | The definition “Location is defined by the distance between UE and a reference location” sounds clumsy, because a reference point and a distance indication don’t lead to a single, new location, but to a circle around the ref. location. Furthermore, TR38.821 envisages independent use of different triggers for handover execution – opposite to the combined conditions mentioned here, i.e. Ax fulfilled and distance to ref- location or time window reached. Is the TR38.821 concept overruled here? |
| Ericsson | Disagree | This looks like stage 3 details, why include in stage 2? We prefer Nokias proposed changes. |

[Rapporteur summary]:

On the basis of the feedback, and in particular the recommendation of the TS 38.300 to keep the text at stage 2 level, the moderator suggests to agree the following

Proposal 2.2a: Agree with the proposed corrections in draft R2-2208761 v00 for chap 16.14.3.2.2 Conditional Hand-over

Proposal 2.2b: Correct as follow “one of the RRM measurement-based event A3/A4/A5;”

## 2.3 Chapter 16.14.3.3	Measurements

In R2-2207065, R2-2207322 and R2-2208380, the following corrections (in red) are proposed:

The same principle as described in 9.2.4 applies to measurements in NTN unless hereunder specified.

To perform neighbour cell measurement, the UE autonomously pre-compensates the frequency Doppler shift by considering UE position and the satellite position through neighbour cell’s satellite ephemeris.

The network can configure:

-	multiple SMTCs in parallel per carrier and for a given set of cells depending on UE capabilities using propagation delay difference and ephemeris information;

-	measurement gaps based on multiple SMTC.

The adjustment of SMTCs is possible under network control based on UE assistance information if available for connected mode and under UE control based on UE location and satellite assistance information (e.g., ephemeris, common TA parameters) for idle/inactive modes NW-controlled adjustment of SMTCs can be based on UE assistance information reported in Connected mode. For Idle/Inactive mode UE can adjust SMTCs based on its location and satellite assistance information (e.g. ephemeris, common TA parameters).

UE assistance information is in the form of may report service link propagation delay differences between service cell and neighbour cells as the assistance information for the adjustment of SMTCs in connected mode.

In the quasi-earth fixed cell scenario, UE can perform timing related e-based and location related -based measurement in RRC_IDLE/RRC_INACTIVE.

-	The timing and location information associated to a cell are provided via system information;

-	Timing information refers to the time when the serving cell is going to stop serving a geographical area;

-	Location information refers to the reference location of serving cell and a corresponding distance threshold.

Measurement rules for cell re-selection with timing information and location information are is specified in section 5.2.4.2 in 38.304 [10].

Question 2.3: Do companies agree with the above proposal or have further suggestions ?

| Company | Agree/Disagree | Comments/Suggestions |
| --- | --- | --- |
| Thales | Agree |  |
| Intel | agree |  |
| Lenovo | Agree |  |
| Xiaomi | Agree |  |
| CATT | See the comment | We agree with the modifications for the SMTC part and we think the wording of time-based and location-based measurements are not accurate. However the modification of “In the quasi-earth fixed cell scenario, UE can perform timing related e-based and location related -based measurement in RRC_IDLE/RRC_INACTIVE.” Is still not appropriate. For the time-based solution and location-based solution is not “measurement” but are methods for UE to judge when to initiate the measurement on neighbour cells. So we propose the following modification: In the quasi-earth fixed cell scenario, UE can perform time-based solution and location-based solution to judge when to initiate the measurements on neighbour cells in RRC_IDLE/RRC_INACTIVE: |
| Apple | Agree |  |
| Samsung | Agree |  |
| Huawei, HiSilicon | Agree |  |
| OPPO | Agree |  |
| Qualcomm | See comment | Now “UE assistance information is in the form of may report service link propagation delay differences between service cell and neighbour cells” is not correct as UE location is also UE assistance information not only PDD. Either keep original text or add “UE assistance information is in the form of UE location report or service link propagation delay…?” |
| Nokia | Agree | Not sure what QC actually mean? |
| LGE | Agree |  |
| Panasonic | Agree in principle | Replace the second sentence by “To perform neighbour cell measurement, the UE autonomously pre-compensates the Doppler spread on the target service link based on determining the positions of the UE and the target satellite, the latter described with the satellite ephemeris information for the neighbour cell.” Replace “service cell” by “serving cell”. |
| Ericsson | Agree |  |

[Rapporteur summary]:

On the basis of the feedback, the moderator suggests to agree the following

Proposal 2.3: Agree with the proposed corrections except the following sentence which is corrected as follow: “In the quasi-earth fixed cell scenario, UE can perform time-based solution and location-based solution to judge when to initiate the  measurements on neighbour cells in RRC_IDLE/RRC_INACTIVE”. +

- Add the sentence “To perform neighbour cell measurement, the UE autonomously pre-compensates the Doppler effect on the target service link based on determining the positions of the UE and the target satellite, the latter described with the satellite ephemeris information for the neighbour cell.”

- Replace “service cell” by “serving cell”.

## 2.4 Chapter 16.14.8	 UE location aspects

In R2-2207141, R2-2207144 and R2-2207322, the following corrections (in red) are proposed:

16.14.8	UE location reporting aspects

Upon network request (provided that user consent is available), after AS security is established in connected mode, a UE shall can report its coarse UE location information (coarse most significant bits of the GNSS coordinates, ensuring the with accuracy around 2km) to the NG-RAN, if available. If the UE cannot provide its coarse UE location, it may report “no coarse GNSS location available”.

Editor’s note: If/when SA3 will provide feedback that this is not recommended, this above text would be removed. Otherwise RAN2 can further update the above text, e.g. considering any other feedback from SA3.

Question 2.4.1: Do companies agree with the above proposal or have further suggestions ?

| Company | Agree/Disagree | Comments/Suggestions |
| --- | --- | --- |
| Thales | Agree |  |
| Intel | partially agree | the text “(provided that user consent is available)” is not needed, it’s up to NW to handle whether user consent is needed, as SA3 clarified that according to some local regulations, user consent may not be necessary. |
| Lenovo | Agree |  |
| Xiaomi | Agree | We think it is good to capture the requirement on user consent to request coarse location in 38.300:“(provided that user consent is available)”. During the IOT-NTN discussion, people think it is not a good place to capture this requirement in 36.331 because RRC doesn’t have the concept of user consent, but are genally fine to capture it somewhere else, one candidate is 36.300 spec, the other is RAN3 spec. For NR NTN, we prefer to capture it in the 38.300 spec as it is quite simple. For RAN3 spec, we are not sure we can find a suitable place to capture it. Please note that, for MDT, we capture the requirement on user consent for Enb to request MDT activation in TS32.422: “When the management based MDT activation is sent to eNodeB/RNC, eNodeB/RNC shall check the availability of the Management Based MDT Allowed IE before making the UE selection. In case the Management Based MDT Allowed IE is not available, the eNodeB/RNC shall not select the UE. ” For NR NTN, we should also have a place to capture this equirement on user consent. We are open where to capture it. If people suggest to capture it in RAN3, we can have a LS to RAN3 to ask them if they can do so. |
| CATT | Agree |  |
| Apple | Comments | Suggest adding the following description (copied from the SA3 LS) at the beginning: The network should request for the location if there is user consent (based on subscription-based or proprietary mechanisms) and the network should not request for the location if there is no user consent. The UE should provide the location information (if available) if the network requests for it. |
| Samsung | Agree |  |
| Huawei, HiSilicon | See comments | - Title should be : “Coarse UE location reporting” - Suggest to reword as: “If user consent is required, the network can only request coarse UE location reporting provided that user consent is available. Upon network request, after AS…” |
| OPPO | Agree with comments | To address Intel’s comments, perhaps it can be reworded as “(provided that user consent is available if that is required)”. |
| Qualcomm | Comments | As we mentioned earlier, UE does not know user consent. We are not talking about user. If network has requested, then it means network must have already sorted out the user consent issue. The UE must report if available. So RAN2 does not specify user consent in the spec, but other working groups may. We are ok to send LS to RAN3 to capture something. But agree with Apple, the other option could be to capture something in line with SA3 recommendation as this information is only for the network for Rel-17. We suggest: The network should request for the location if there is user consent (based on subscription-based or proprietary mechanisms) depending on local regulations. |
| Nokia | Agree |  |
| Panasonic | Agree with comment | Instead of “… ensuring the accuracy around 2km …” we suggest to write “… ensuring an accuracy in the order of 2 km …”. |
| Ericsson | Partly agree | There shall be no mentioning of user consent in stage 2, it is out of RAN2 scope. Remove “(provided that user consent is available)” |

[Rapporteur summary]:

On the basis of the feedback, the moderator suggests to agree the following

Proposal 2.4.1: Agree with the following corrections

16.14.8	Coarse UE location reporting aspects

The network should request for the location if there is user consent (based on subscription-based or proprietary mechanisms) depending on local regulations.

Upon network request, after AS security is established in connected mode, a UE shall can report its coarse UE location information (coarse most significant bits of the GNSS coordinates, ensuring the with accuracy around 2km) to the NG-RAN, if available. If the UE cannot provide its coarse UE location, it may report “no coarse GNSS location available”.

Editor’s note: If/when SA3 will provide feedback that this is not recommended, this above text would be removed. Otherwise RAN2 can further update the above text, e.g. considering any other feedback from SA3.

In [7]	R2-2207597, the following is proposed:

- Proposal 1: RAN2 to discuss whether the restriction of having user consent should be specified for the Gnb requesting for UE location based on regulations requirement.

- Proposal 2: RAN2 to clarify the different use cases for fine UE location reporting and coarse UE location reporting as both require user consent.

Note in the previous question, the following correction was suggested:

“Upon network request (provided that user consent is available), ..”

Question 2.4.2: Do companies think that some further edits are needed compared to the ones discussed in Question 2.4.1 ?

| Company | Yes/No | Comments/Suggestions |
| --- | --- | --- |
| Thales | No |  |
| Intel | No | same comment as for Question 2.4.1 |
| Lenovo | No |  |
| Xiaomi | Yes for P1 | As we clarified in Q2.4.1, user consent requirement for Gnb should be specified similar to MDT. Regarding coarse location report and fine location report, they belong to different WI, i.e. NTN and MDT respectively. SA3 has agreed that NTN specific user consent will be defined. For MDT, the user consent requirement has already been captured in TS32.422. So, we only need to capture the user consent requirement for NTN. |
| CATT | No |  |
| Apple | Yes | Suggest adding the following description (copied from the SA3 LS) at the beginning: The network should request for the location if there is user consent (based on subscription-based or proprietary mechanisms) and the network should not request for the location if there is no user consent. The UE should provide the location information (if available) if the network requests for it. |
| Huawei, HiSilicon | Yes | See our answer to Question 2.4.1. |
| OPPO | Yes | See our suggestion to Question 2.4.1. |
| Qualcomm | No | See our suggestion in Q 2.4.1. |
| Nokia | No |  |
| Panasonic | See comment | Since the user consent issue is of a more common nature affecting multiple 3GPP specs, it should be solved in a commonly applicable way, i.e. all data whose provision require user consent should be treated the same way (possibly with different degrees of sensitivity). A separate clause in each spec (ideally describing the same generic concept of dealing with user consent) could be a simple and sensible solution. |
| Ericsson | No |  |

[Rapporteur summary]:

On the basis of the feedbacks, no additional corrections compared to the one proposed in relation to question 2.4.1 is needed

In [7]	R2-2207597, the following is proposed:

- Proposal 2: RAN2 to clarify the different use cases for fine UE location reporting and coarse UE location reporting as both require user consent.

Question 2.4.3: What use cases require fine UE location reporting and coarse UE location reporting ? and should that be reflected in the specifications ?

| Company | Comments/Suggestions |
| --- | --- |
| Intel | user consent related description can be transparent in RRC spec. |
| Xiaomi | See our comment to Q2.4.2 |
| CATT | Maybe we need to confirm the issue, due to both of them need the UE consent. |
| Huawei, HiSilicon | Tend to agree with Xiaomi’s understanding. |
| OPPO | Maybe good to clarify in stag-2 spec. |
| Qualcomm | No. see our suggestion in Q2.4.1. In Rel-17, its already too late we have no choice, but depend on some proprietary solution for user consent (that’s what SA2 recommended). In Rel-18, SA3 will have a solution. So we can live with a text clarification in Rel-17. |
| Nokia | We are not convinced this should be covered in R17 stage-2 specs. |
| Panasonic | Location-dependent handover (in the extreme case in the absence of any further accompanying condition). With e.g. many small areas each illuminated by a sharp spot beam one shouldn’t work with the coarse accuracies used so far. Another situation might be border lines between e.g. countries. It might be decisive to know exactly on which side of the border the UE is located – also for emergency cases. |
| Ericsson | Agree with Nokia. Not in RAN2 scope. |

[Rapporteur summary]:

In view on diverse views, this proposal is dismissed.

## 2.5 Chapter 16.14.1	Overview

As per R2-2207442, the following is proposed:

Question 2.5: Do companies agree with the addition of “In this release, CA and DC are not supported in NTN network” in Chapter 16.14.1	Overview ?

| Company | Agree/Disagree | Comments/Suggestions |
| --- | --- | --- |
| Thales | Agree with suggestions | We would prefer to clarify the release: “In this release-17, CA and DC are not supported in NTN network” |
| Intel | Agree | the exact release number is already shown in the spec cover sheet |
| Lenovo | Agree |  |
| Xiaomi | Agree |  |
| CATT | Agree |  |
| Apple | Agree |  |
| Huawei, HiSilicon | Agree |  |
| OPPO | Agree |  |
| Nokia | Agree |  |
| LGE | Agree |  |
| Panasonic | Agree with Thales |  |
| Ericsson | Disagree | There are many other features that are not supported in NTN so why should we just specify CA/DC (and DAPS HO) as not supported in NTN? |

[Rapporteur summary]:

On the basis of the feedback, this proposal is dismissed

## 2.6 Chapter 2 References

In R2-2208678 and R2-2208751, the following corrections (in red) is proposed:

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[51]	NIMA TR 8350.2, Third Edition, Amendment 1, 3 January 2000: "DEPARTMENT OF DEFENSE WORLD GEODETIC SYSTEM 1984", https://gis-lab.info/docs/nima-tr8350.2-wgs84fin.pdf.

- This has been agreed on line so no need to discuss anymore and the corrections is implemented in the draft CR.

## 2.7 Other points

Question 2.7: Are there any other corrections to be discussed ?

| Company | Yes/No | Comments/Suggestions |
| --- | --- | --- |

[Rapporteur summary]:

-

# 4. Summary and Proposals

All proposals have been captured in the draft CR (v01) which is submitted for review.

# 5. References

- 3GPP TS 38.300 “NR; NR and NG-RAN Overall description; Stage-2”, v17.1.0

- R2-2207065	NTN Stage-2 correction (CR)	Oppo, THALES

- R2-2207141	Corrections of UE location aspects in NTN Thales, Xiaomi

- R2-2207144	Correction of UE location aspects in Non-Terrestrial Networks (CR) Thales, Xiaomi

- R2-2207322	Rel-17 NTN related Rapporteur’s corrections to TS 38.300 (CR) Nokia

- R2-2207442   Clarification on the  features supported in NTN network   Apple

- R2-2207597   Discussion on the UE location reporting     Huawei, HiSilicon

- R2-2208380	Miscellaneous corrections on 38.300 (CR)	CATT

- R2-2208678	R17 NR NTN stage 2 corrections	Ericsson

- R2-2208751	Report of [AT119-e][101][NTN] UP corrections InterDigital

# 6. Contact information

| Company | Delegate contact |
| --- | --- |
| COMPANY_NAME | NAME (email@address.com) |
| Thales | nicolas.chuberre@thalesaleniaspace.com |
| OPPO | lihaitao@oppo.com |
| Panasonic | frank.herrmann@eu.panasonic.com |