3GPP TSG RAN WG2 Meeting #119-e                                           R2-2208817

# Electronic meeting, 17- 29 Aug, 2022

Agenda item:			6.11.1

Source:			Intel Corporation

Title:		Report of [AT119-e][409][POS] Rel-17 positioning capabilities (Intel)

Document for:	 	Discussion and decision

# Introduction

This is the report of following offline discussion:

[AT119-e][409][POS] Rel-17 positioning capabilities (Intel)

Scope: Check and update the rapporteur CR in R2-2207385 to take account of decisions of this meeting.  Evaluate the proposals in the following tdocs:

- R2-2208492

Intended outcome: Agreeable CR

Deadline: Tuesday 2022-08-23 1200 UTC

I would like to split the discussion into two phases:

Phase 1: to provide your view on issues;     Deadline:  Saturday 2022-08-20 1800 UTC

Phase 2: To check TPs; Deadline:  Tuesday 2022-08-23 1200 UTC

Following stage 2 changes are discussed in the offline discussion:

R2-2207385	Corrections on LPP capabilies	Intel Corporation

R2-2208492	Change request about UE capability for PRS measurement within a PPW	vivo

P3 of R2-2208792	(Summary of AI 6.11.2.1)	Qualcomm

# Annex: companies’ point of contact

| Company | Point of contact | Email address |
| --- | --- | --- |
| Intel Corporation | Yi Guo | Yi.guo@intel.com |
| vivo | Xiang Pan | panxiang@vivo.com |
| ZTE | Yu Pan | pan.yu24@zte.com.cn |
| CATT | Jianxiang Li | lijianxiang@catt.cn |

# Discussion

## Changes from R2-2207385	Corrections on LPP capabilies	Intel Corporation

Reason for change:

Based on RAN1 latest feature list in R1-2205607 and RAN4 R4-2211189, the Prerequisite feature groups and some clarifications for following LPP capabilities are missing:

27-1-1 nr-UE-RxTEG-ID-MaxSupport-r17

27-1-2/27-1-2a  nr-UE-TxTEG-ID-MaxSupport-r17

27-1-3  nr-UE-RxTxTEG-ID-MaxSupport-r17

27-1-4   measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17

27-1-4a   measureSameDL-PRS-ResourceWithDifferentRxTEGsSimul-r17

27-2-1 maxDL-PRS-FirstPathRSRP-MeasPerTRP-r17 (in addition, restriction “The maximum number of first path PRS RSRP per TRP should be less than or equal to the maximum number of PRS RSRP (27-2-2)” should be added;)

27-2-2 maxDL-PRS-RSRP-MeasurementFR1-r17, maxDL-PRS-RSRP-MeasurementFR2-r17

27-3-1 supportedDL-PRS-ProcessingSamples (In addition, a note should be added as

Note: this feature is supported for both UE-assisted and UE based positioning

)

27-3-2

prs-ProcessingWindowType1A-r17

prs-ProcessingWindowType1B-r17

prs-ProcessingWindowType2-r17

(A note should be added as Note: Within a PRS processing window, UE measurement is inside the active DL BWP with PRS having the same numerology as the active DL BWP)

27-3-3 prs-ProcessingCapabilityOutsideMGinPPW (no change is needed)

27-4-1 nr-los-nlos-AssistanceDataSupport-r17 (in addition, a Note should be added as NOTE:	A single value is reported when both Multi-RTT and DL-TDOA are supported.)

27-6  (no change is needed)

dl-PRS-BufferType-RRC-Inactive-r17

durationOfPRS-Processing-RRC-Inactive-r17

maxNumOfDL-PRS-ResProcessedPerSlot-RRC-Inactive-r17

27-7  (no change is needed)multiMeasInSameMeasReport-r17

27-8 nr-PosCalcAssistanceSupport-r17	(bit 3 DL-TDOA, bit 2 DL-AoD, )

27-9  (no change is needed) lowerRxBeamSweepingThan8-FR2-r17

27-10a mg-ActivationRequest-r17

27-12 nr-los-nlos-IndicatorSupport-r17(no change is needed)

27-13 additionalPathsExtSupport-r17(no change is needed)

27-13a supportOfDL-PRS-FirstPathRSRP-r17

27-14 additionalPathsExtSupport-r17(no change is needed)

27-14a supportOfDL-PRS-FirstPathRSRP-r17

27-15 posSRS-RRC-Inactive-InInitialUL-BWP-r17(no change is needed)

27-15b posSRS-RRC-Inactive-OutsideInitialUL-BWP-r17 (in addition, some notes should be added

Note 2: If component 9 is not signaled, the UE only supports same center frequency  between the SRS for positioning and initial UL BWP

Note 3: If component 5 is not signaled, the UE only supports same numerology between the SRS and the initial UL BWP

Note 4: If component 6 is not signaled, the UE supports only SRS BW that include the BW of the CORESET #0 and SSB.

)

27-15a maxNumOfSemiPeristentSRSposResources-r17										maxNumOfSemiPersistentSRSposResourcesPerSlot-r17

27-15c maxNumOfSemiPeristentSRSposResources-r17										maxNumOfSemiPersistentSRSposResourcesPerSlot-r17

27-16a

olpc-SRS-PosRRC-Inactive-r17

27-18a/27-18b/27-18c dl-PRS-MeasRRC-Inactive-r17

27-19a

spatialRelationsSRS-PosRRC-Inactive-r17

27-20 dl-PRS-ResourcePrioritySubset-Sup-r17 (no change is needed)

27-21 nr-DL-PRS-BeamInfoSup-r17(no change is needed)

27-22 nr-PosCalcAssistanceSupport-r17, beamInfoSup(no change is needed)

14-2 supportedDL-PRS-ProcessingSamples-RRC-Inactive-r17

.

Summary of change::

To add the Prerequisite feature groups for LPP capabilities introduced in Rel-17 and corresponding clarifications based on RAN1/RAN4 latest feature list in R1-2205607 and R4-2211189

Consequences if not approved:

It is unclear on how to set the LPP capabilities.

Moderator’s comments:

- It is to align with latest RAN1/4 feature lists;

Moderator would like to check companies’ view:

Discussion point 3.1: Do you agree the changes in R2-2207385?

| Company’s name | Yes or No? | Comments, if any |
| --- | --- | --- |
| ZTE | Yes |  |
| Qualcomm | Yes | In all field descriptions, the -r16/-r17 suffix should be removed. New field names should be Italic Font in posSRS-RRC-Inactive-InInitialUL-BWP, posSRS-RRC-Inactive-OutsideInitialUL-BWP, olpc-SRS-PosRRC-Inactive, nr-PosCalcAssistanceSupport |
| CATT | Yes | There are still some editorials, e.g. the names of parameters are not italic. Can double check and correct them. |
| Huawei, HiSIlicon | Yes |  |
| Xiaomi | Yes |  |

Summary:

All companies agree the changes. But some editorial changes are still needed, e.g. to remove -r16/-r17 suffix, and make field name italic.

Proposal 1: With the changes “remove -r16/-r17 suffix, and make field name italic”, the changes in R2-2207385 is agreeable, and to be merged into R2-2208802.

## Changes from R2-2208492	Change request about UE capability for PRS measurement within a PPW	vivo

Reason for change:

According to RAN4 LS R2-2206945, RAN2 needs to introduce the UE capability for support of Rx timing difference between the serving cell and non-serving cell for PRS measurement within a PPW in NR-DL-PRS-ProcessingCapability of TS 37.355. The capability type shall be per band.

- The applicability condition on Rx timing difference between the serving cell and a neighbor cell/TRP for PRS measurements within a PPW is max ∆T≤THR, where - ∆T is the time difference between the start of a slot containing PRS from the neighbor cell/TRP and the start of the closest slot from the serving cell, , and - the range of ∆T is determined by the expected RSTD and expected RSTD uncertainty in the assistance data. - THR is the threshold, which is up to UE capability (as already informed in R4-2206981) - The candidate values for THR include {CP length, ¼ symbol length, ½ symbol length, ½ slot length}

Summary of change::

Introduce a new UE capability for support of Rx timing difference between the serving cell and non-serving cell for PRS measurement within a PPW in NR-DL-PRS-ProcessingCapability;

Consequences if not approved:

Incomplete UE capability for PRS measurement.

Moderator’s comments:

The capability is indicated in RAN4 feature list as 14-3, and has been captured in RRC/306 based on latest RAN4 feature list. RAN4 did not agree to add this in LPP. Therefore I would suggest not to add it in LPP unless RAN4 requests us to do so.

| 14. NR_pos_enh | 14-3 | PRS measurement without MG | Capability for the threshold used to be compared against with the Rx timing difference to determine whether the PRS from the non-serving cell satisfy the condition of PRS measurement outside MG. | 27-3-2 | yes |  |  | Per band | No | No | N/A | The candidate threshold values: CP length, 1/4 symbol, 1/2 symbol, half of slot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Moderator would like to check companies’ view:

Discussion point 3.2: Do you agree the changes in R2-2208492?

| Company’s name | Yes or No? | Comments, if any |
| --- | --- | --- |
| vivo |  | In 38.214, the UE behavior related to this capability is as follows: The UE is not expected to measure the DL PRS outside the measurement gap if the expected received timing difference between the DL PRS from the non-serving cell and that from the serving cell, determined by the higher layer parameters nr-DL-PRS-ExpectedRSTD and nr-DL-PRS-ExpectedRSTD-Uncertainty, is larger than maximum Rx timing difference provided by [UE capability].. Only UE and LMF are aware of the expected RSTD of multiple TRPs. The capability can be used for LMF to determine which TRPs shall be included in the PRS configuration. Besides, we already have the prs-ProcessingCapabilityOutsideMGinPPW-r17 in the LPP, it’s confusing why this new capability is excluded. Or do we need to LS to RAN4/RAN1 for clarification? |
| ZTE | Yes | Other RAN4 UE capabilities are captured in LPP so we think this should be included in LPP too |
| Qualcomm | Yes | Agree with Rapporteur that this is not captured in the RAN4 spread sheet for LMF. Agree with vivo on the need. Maybe include this issue in the LS to RAN1/4 [AT119-e][422][POS]? |
| CATT | Yes | We agree to add the capability in LPP to let LMF aware of the whole PRS measurement related capabilities. But it is ok for us to further check with RAN4 if most companies want. |
| Huawei, HiSilicon |  | An LS to RAN4 is OK for the purpose of clarification whether they intend to inform this caapability to the LMF. |
| Xiaomi | Yes | Ok to send LS to RAN4. |

Summary:

All companies agree to send LS to RAN4, to check whether the capability should be informed to the LMF, i.e. also captured in LPP.

5 companies agree to capture the change into LPP.

Proposal 2: Send a quick LS to RAN4, check them whether 14-3	PRS measurement without MG should be reported to the LMF.

## (new added) Agreements on P3 from R2-2208792	(Summary of AI 6.11.2.1)	Qualcomm

During online discussion, RAN2 agreed:

Proposal 3: The LPP CR R2-2207885, "Correction to the number of samples for PRS measurement in RRC_INACTIVE", is an essential correction. RAN2 to discuss and decide: (a) whether to dummify the existing field of supportedDL-PRS-ProcessingSamples-RRC-Inactive-r17, and add a new field for the reducedDL-PRS-ProcessingSamples-RRC-Inactive-r17 as proposed in R2-2207885. (b) extend the ENUMERATED and dummify the existing values 'm1' and 'm2': ENUMERATED { dummy1, dummy2, ..., supported-v17xy } (c) make a non-backwards compatible ASN change as proposed in R2-2207580 [7]: ENUMERATED { supported } Discussion: Huawei proposed option a, but think option b would also be OK; they cannot accept option c. Samsung also support the proposal and prefer option a; they see not much difference between a and b in overhead, and find a more readable. CATT and ZTE prefer the NBC change in c. Apple think either a or b is fine. Intel and OPPO prefer option b. Qualcomm think we could avoid any ASN.1 change and just clarify the field description. They agree that the NBC solution in c is not desired. Agreement: The capability for supportedDL-PRS-ProcessingSamples-RRC-Inactive-r17 is modified to indicate only support in a backward compatible fashion. Details can be discussed in the LPP capability email discussion [409].

Based on the online discussion, following backward compatible approaches are possible:

(a) whether to dummify the existing field of supportedDL-PRS-ProcessingSamples-RRC-Inactive-r17, and add a new field for the reducedDL-PRS-ProcessingSamples-RRC-Inactive-r17 as proposed in R2-2207885.

(b)	extend the ENUMERATED and dummify the existing values 'm1' and 'm2':

ENUMERATED { dummy1, dummy2, ..., supported-v17xy }

(d) (raised by Sven online): we may just clarify the field description. Moderator provides the example as

supportedDL-PRS-ProcessingSamples-RRC-Inactive

Indicates the UE capability for support of measurements based on measuring M=1 or M=2 samples (instances) of a DL-PRS Resource Set in RRC_INACTIVE state. The UE indicates value “m1” or “m2” shall supports measurements based on both measuring M=1 or M=2 samples (instances) of a DL-PRS Resource Set in RRC_INACTIVE state.

(e) others?

Moderator would like to check companies’ view:

Discussion point 3.3: Which option do you prefer?

| Company’s name | Option a, b or c? | Comments, if any |
| --- | --- | --- |
| ZTE | b | We think the field description change in c is a little bit unclear from spec writing perspective that UE can indicate both m1 or m2 for the same purpose. So we prefer change in b without any field description change. |
| Qualcomm | (b) or (d) | 1st preference (d) 2nd preference (b) |
| CATT |  | We do not like to see any Rel-17 dummies in such early Rel-17 spec versions. Since this is capability related and NBC is allowed for this stage, we still prefer to correct it directly. Please note NBC is always allowed if all companies agreed even for a frozen version. But it is acceptable for us to adopt option d as a compromise. |
| Huawei, HiSilicon | (a) | The spec would be clean to reflect intention of the UE capability. We would also be fine to adopt another method to revise m1 to supported, and dummy m2, so that it reads ENUMERATED { supported, dummy1, ... } |
| Xiaomi | (d) |  |

Summary:

Companies’ view are quite diverging.

1 company prefer a;

3 companies prefer or can accept b;

3 companies prefer or can accept d;

1 company cannot accept dummy,i.e. a and b;

Commpanies who do not like d because they would like to have clear way.

Moderator comments:

d seems a good compromise, and would like to check whether it can be acceptable.

Proposal 3: For the P3 from R2-2208792, agree the compromised solution, i.e. clarify in the field description, and merge it into R2-2208802.

supportedDL-PRS-ProcessingSamples-RRC-Inactive

Indicates the UE capability for support of measurements based on measuring M=1 or M=2 samples (instances) of a DL-PRS Resource Set in RRC_INACTIVE state. The UE indicates value “m1” or “m2” shall supports measurements based on both measuring M=1 or M=2 samples (instances) of a DL-PRS Resource Set in RRC_INACTIVE state.

# Phase 2

Companies are invited to check the proposals from phase 1 discussion, and the updated draft CR R2-2208802.

Proposal 1: With the changes “remove -r16/-r17 suffix, and make field name italic”, the changes in R2-2207385 is agreeable, and to be merged into R2-2208802.

Proposal 2: Send a quick LS to RAN4, check them whether 14-3	PRS measurement without MG should be reported to the LMF.

Proposal 3: For the P3 from R2-2208792, agree the compromised solution, i.e. clarify in the field description, and merge it into R2-2208802.

supportedDL-PRS-ProcessingSamples-RRC-Inactive

Indicates the UE capability for support of measurements based on measuring M=1 or M=2 samples (instances) of a DL-PRS Resource Set in RRC_INACTIVE state. The UE indicates value “m1” or “m2” shall supports measurements based on both measuring M=1 or M=2 samples (instances) of a DL-PRS Resource Set in RRC_INACTIVE state.

Discussion point 4.1: Do you agree the proposals made in phase 1 discussion?

| Company’s name | Yes or No? | Comments, if any |
| --- | --- | --- |
| Nokia | Yes to P1 and P2. No to P3 | Regarding P3: Option (a) does seem to be the clean and clear way to us. This option particularly renames the field to reflect the current agreements made in RAN4 and we like that. Option (b) is also acceptable but the field name should be changed to say reducedDL-PRS-ProcessingSamples-RRC-Inactive-r17 Option (d) is the most confusing to us since, in the field description, the newly added sentence is not very clear. Field name kept as it is, is also confusing. The field name should reflect the capability whether UE supports reduced number of samples for measurements in inactive state or not. So, we do not prefer Option (d) |
| ZTE | NO to P3 | Agree with Nokia that the newly added field description has no precedent in UE capability, that UE fills m1 or m2 to both indicate support or not. (a) or (b) is better |

Discussion point 4.2: Do you have any comments on the merged CR R2-2208802?

| Company’s name | Yes or No? | Comments, if any |
| --- | --- | --- |
| CATT | Yes | “not signaled/signalled” is not used before in the spec. Can change to “If the field is absent”……There are several places having the same issue. [Moderator] Updated in v01. |
| Nokia | Yes | - Suggest adding in the CR cover, the RAN2 TDoc reference for the RAN1 and RAN4 UE features list. [Moderator] Updated in v01. - Suggest formatting the information in the Reason for change field on CR cover to keep the feature number on a line by itself and the parameters/fields in a separate line below the feature number for better readability. Also, remove those fields highlighted in grey which requires no change in this CR [Moderator] Updated in v01. |

Moderator would suggest to remove the changes of P3 from R2-2208802 for nowj, and discuss P3 online.

R2-2208802 has been updated in Draft R2-2208802 -Corrections on LPP capablities v01 with following changes:

- Remove the change from p3;

- Update based on comments from CATT and Nokia .

# Summary report and proposals

For agreement:

Proposal 1: With the changes “remove -r16/-r17 suffix, and make field name italic”, the changes in R2-2207385 is agreeable, and to be merged into R2-2208802.

Proposal 2: Send a quick LS to RAN4, check them whether 14-3	PRS measurement without MG should be reported to the LMF.

Online discussion:

Proposal 3: For the P3 from R2-2208792, agree the compromised solution, i.e. clarify in the field description, and merge it into R2-2208802.

supportedDL-PRS-ProcessingSamples-RRC-Inactive

Indicates the UE capability for support of measurements based on measuring M=1 or M=2 samples (instances) of a DL-PRS Resource Set in RRC_INACTIVE state. The UE indicates value “m1” or “m2” shall supports measurements based on both measuring M=1 or M=2 samples (instances) of a DL-PRS Resource Set in RRC_INACTIVE state.

# Reference

- R2-2207385	Corrections on LPP capabilies	Intel Corporation

- R2-2208492	Change request about UE capability for PRS measurement within a PPW	vivo

- R2-2208792	(Summary of AI 6.11.2.1)	Qualcomm