3GPP TSG-RAN2 Meeting #119-e	R2-2208827

# Online, 17th Aug 2022 - 29th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.355 | CR | 0377 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the GNSS Orbit and Clock Integrity Bounds in TS 37.355 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Swift Navigation, ESA, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The resolution and range for the meanOrbitError, varOrbitError, meanClock, stdDevClock parameters are inconsistent with the corresponding values for the meanIonosphere and stdDevIonosphere. At RAN2#117-e (R2-2203515) we agreed to separate the orbit and clock parameters into individual bounds (as per the current specification). The issue is that for each bounding parameter (meanOrbitError, varOrbitError, meanClock, stdDevClock) we adopted the range and resolution values from an earlier proposal by default (R2-2201214), but these values were associated with a combined orbit/clock term. Hence, these fields should have been updated to define the individual orbit and clock resolution instead of the combined resolution. In addition, the varOrbitError and varOrbitRateError parameters are specified as variances whereas all the other bounds use standard deviation. The most practical solution is to ensure that the orbit and clock resolution is consistent with the meanIonosphere and stdDevIonosphere parameters and that all parameters are expressed as a standard deviation, so that each bound can be consistently aligned at the same levels of precision (the current discrepancy also leads to unnecessary complexity having to decode different resolutions for the orbit/clock bounds versus the ionospheric bounds). |  |  |  |  |  |  |  |  |
| Summary of change: |  | The varOrbitError and varOrbitRateError fields are renamed and redefined as stdDevOrbitError and stdDevOrbitRateError, respectively. The range and resolution for meanOrbitError and stdDevOrbitError (under GNSS-SSR-OrbitCorrections) and meanClock, stdDevClock (under GNSS-SSR-ClockCorrections) have been updated to align with the meanIonosphere and stdDevIonosphere values (under GNSS-SSR-STEC-Correction). Impacted functionality Orbit and Clock integrity bounds Inter-operability: If the NW implements this CR but the UE does not: - the orbit or clock bound values will be incorrectly interpreted by the UE. If the UE implements this CR but the NW does not: - the orbit or clock bound values will be incorrectly interpreted by the UE. Mandatory to implement the concerned functionality. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The precision of the orbit and clocks bounds is limited by the specification and it becomes inconsistent with the other bounding parameters. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 6.5.2.2	GNSS Assistance Data Elements

/**Skip unrelated parts**/

#### –	GNSS-SSR-OrbitCorrections

The IE GNSS-SSR-OrbitCorrections is used by the location server to provide radial, along-track and cross-track orbit corrections together with integrity information. The target device may use the SSR-OrbitCorrectionList to compute a satellite position correction to be combined with the satellite position calculated from broadcast ephemeris.

The parameters provided in IE GNSS-SSR-OrbitCorrections – except for ORBIT-IntegrityParameters and SSR-IntegrityOrbitBounds –  are used as specified for SSR Clock Messages (e.g., message type 1057 and 1063) in [30] and apply to all GNSSs.

-- ASN1START

GNSS-SSR-OrbitCorrections-r15 ::= SEQUENCE {

epochTime-r15						GNSS-SystemTime,

ssrUpdateInterval-r15				INTEGER (0..15),

satelliteReferenceDatum-r15			ENUMERATED { itrf, regional, ... },

iod-ssr-r15							INTEGER (0..15),

ssr-OrbitCorrectionList-r15			SSR-OrbitCorrectionList-r15,

...,

[[

orbit-IntegrityParameters-r17	ORBIT-IntegrityParameters-r17	OPTIONAL -- Need ON

]]

}

SSR-OrbitCorrectionList-r15 ::= SEQUENCE (SIZE(1..64)) OF SSR-OrbitCorrectionSatelliteElement-r15

SSR-OrbitCorrectionSatelliteElement-r15 ::= SEQUENCE {

svID-r15							SV-ID,

iod-r15								BIT STRING (SIZE(11)),

delta-radial-r15					INTEGER (-2097152..2097151),

delta-AlongTrack-r15				INTEGER (-524288..524287),

delta-CrossTrack-r15				INTEGER (-524288..524287),

dot-delta-radial-r15				INTEGER (-1048576..1048575)		OPTIONAL, -- Need ON

dot-delta-AlongTrack-r15			INTEGER (-262144..262143) 		OPTIONAL, -- Need ON

dot-delta-CrossTrack-r15			INTEGER (-262144..262143) 		OPTIONAL, -- Need ON

...,

[[

ssr-IntegrityOrbitBounds-r17	SSR-IntegrityOrbitBounds-r17	OPTIONAL  -- Cond Integrity1

]]

}

ORBIT-IntegrityParameters-r17 ::= SEQUENCE {

probOnsetConstFault-r17					INTEGER (0..255),

meanConstFaultDuration-r17				INTEGER (1..3600),

probOnsetSatFault-r17					INTEGER (0..255),

meanSatFaultDuration-r17				INTEGER (1..3600),

orbitRangeErrorCorrelationTime-r17		INTEGER (0..255)			OPTIONAL, -- Need ON

orbitRangeRateErrorCorrelationTime-r17	INTEGER (0..255)			OPTIONAL, -- Cond Integrity2

...

}

SSR-IntegrityOrbitBounds-r17 ::= SEQUENCE {

meanOrbitError-r17				RAC-OrbitalErrorComponents-r17,

stdDevOrbitError-r17				RAC-OrbitalErrorComponents-r17,

meanOrbitRateError-r17			RAC-OrbitalErrorComponents-r17,

stdDevOrbitRateError-r17			RAC-OrbitalErrorComponents-r17,

...

}

RAC-OrbitalErrorComponents-r17	::= SEQUENCE {

radial-r17			INTEGER (0..255),

alongTrack-r17		INTEGER (0..255),

crossTrack-r17		INTEGER (0..255)

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| Integrity1 | The field is mandatory present if ORBIT-IntegrityParameters is present; otherwise it is not present. |
| Integrity2 | The field is mandatory present if orbitRangeErrorCorrelationTime is present; otherwise it is not present. |

GNSS-SSR-OrbitCorrections field descriptions

epochTime This field specifies the epoch time of the orbit corrections. The gnss-TimeID in GNSS-SystemTime shall be the same as the GNSS-ID in IE GNSS-GenericAssistDataElement.

ssrUpdateInterval This field specifies the SSR Update Interval. The SSR Update Intervals for all SSR parameters start at time 00:00:00 of the GPS time scale. A change of the SSR Update Interval during the transmission of SSR data should ensure consistent data for a target device. See table Value of ssrUpdateInterval to SSR Update Interval relation below. NOTE 1.

satelliteReferenceDatum This field specifies the satellite refence datum for the orbit corrections.

iod-ssr This field specifies the Issue of Data number for the SSR data. A change of iod-ssr is used to indicate a change in the SSR generating configuration.

svID This field specifies the satellite for which the orbit corrections are provided.

iod This field specifies the IOD value of the broadcast ephemeris for which the orbit corrections are valid (see IE GNSS-NavigationModel). NOTE 2.

delta-radial This field specifies the radial orbit correction for broadcast ephemeris. NOTE 3. Scale factor 0.1 mm; range ±209.7151 m.

delta-AlongTrack This field specifies the along-track orbit correction for broadcast ephemeris. NOTE 3. Scale factor 0.4 mm; range ±209.7148 m.

delta-CrossTrack This field specifies the cross-track orbit correction for broadcast ephemeris. NOTE 3. Scale factor 0.4 mm; range ±209.7148 m.

dot-delta-radial This field specifies the velocity of radial orbit correction for broadcast ephemeris. NOTE 3. Scale factor 0.001 mm/s; range ±1.048575 m/s.

dot-delta-AlongTrack This field specifies the velocity of along-track orbit correction for broadcast ephemeris. NOTE 3. Scale factor 0.004 mm/s; range ±1.048572 m/s.

dot-delta-CrossTrack This field specifies the velocity of cross-track orbit correction for broadcast ephemeris. NOTE 3. Scale factor 0.004 mm/s; range ±1.048572 m/s.

probOnsetConstFault This field specifies the Probability of Onset of Constellation Fault per Time Unit where a constellation fault is at least two satellites being faulty simultaneously due to the same event. This field specifies the onset probability that the residual range or range rate error exceeds a bound created using the minimum allowed inflation factor Kmin, and bounding parameters as mean + Kmin * stdDev where Kmin = normInv(irMaximum / 2), with irMaximum as provided in IE GNSS-Integrity-ServiceParameters. The probability is calculated by P=10-0.04n [hour-1] where n is the value of probOnsetConstFault and the range is 10-10.2 to 1 per hour.

meanConstFaultDuration This field specifies the Mean Constellation Fault Duration which is the mean duration between when a constellation fault occurs, and the user is alerted by the service through FFS (or the integrity violation is over). Scale factor 1 s; range 1-3600 s.

probOnsetSatFault This field specifies the Probability of Onset of Satellite Fault per Time Unit which is the probability of occurrence of satellite error to exceed the residual error bound for more than the Time to Alert (TTA). This field specifies the onset probability that the residual range or range rate error exceeds a bound created using the minimum allowed inflation factor Kmin, and bounding parameters as mean + Kmin * stdDev where Kmin = normInv(irMaximum / 2), with irMaximum as provided in IE GNSS-Integrity-ServiceParameters. The probability is calculated by P=10-0.04n [hour-1] where n is the value of probOnsetSatFault and the range is 10-10.2 to 1 per hour.

meanSatFaultDuration This field specifies the Mean Satellite Fault Duration which is the mean duration between when a satellite fault occurs, and the user is alerted by the service through FFS (or the integrity violation is over). Scale factor 1 s; range 1-3,600 s.

orbitRangeErrorCorrelationTime This field specifies the Orbit Range Error Correlation Time which is the upper bound of the correlation time of the satellite residual range error due to orbit. The time is calculated using: t=10i, &i≤1801800+100(i-180), 180<&i≤234 7200+1000i-234, &i>234 [s] Range is 1-28,200 s.

orbitRangeRateErrorCorrelationTime This field specifies the Orbit Range Rate Error Correlation Time which is the upper bound of the correlation time of the satellite residual range rate error due to orbit. The time is calculated using: t=10i, &i≤1801800+100(i-180), 180<&i≤234 7200+1000i-234, &i>234 [s] Range is 1-28,200 s.

meanOrbitError This field specifies the Mean Orbit Error bound in satellite radial, along-track and cross-track coordinates, which are the mean values for a set of three overbounding models that bound the residual orbit error in satellite radial, along-track and cross-track directions. Each mean is calculated using: μ=0.0125i, &i≤20052+0.15(i-200), 200<&i≤2340 25+0.52i-2340, &i>2340 [m] Range is 0-17.5 m.

stdDevOrbitError This field specifies the Standard Deviation Orbit Error bound in satellite radial, along-track and cross-track coordinates, which are the standard deviation values for a set of three overbounding models that bound the residual orbit error in satellite radial, along-track and cross-track directions. Each standard deviation is calculated using: σ2=0.0125i, &i≤20052+0.15(i-200), 200<&i≤2340 25+0.52i-2340, &i>2340 [m] Range is 0-17.5 m.

meanOrbitRateError This field specifies the Mean Orbit Rate Error in satellite radial, along-track and cross-track coordinates, which are the mean values for a set of three overbounding models that bound the residual satellite orbit rate error in satellite radial, along-track and cross-track directions. Scale factor 0.001 m/s; range 0-0.255 m/s.

stdDevOrbitRateError This field specifies the Standard DeviationOrbit Rate Error in satellite radial, along-track and cross-track coordinates, which are the standard deviation values for a set of three overbounding models that bound the residual satellite orbit rate error in satellite radial, along-track and cross-track directions. Scale factor 0.001 m/s; range 0-0.255 m/s.

NOTE 1: 	The update intervals are aligned to the GPS time scale for all GNSSs in order to allow synchronous operation for multiple GNSS services. This means that the update intervals may not be aligned to the beginning of the day for another GNSS. Due to the leap seconds, this is generally the case for GLONASS.

NOTE 2:	In the cases that gnss-ID indicates 'gps' or 'qzss', the iod refers to the NAV broadcast ephemeris (GPS L1 C/A or QZSS QZS-L1, respectively, in table GNSS to iod Bit String(11) relation in IE GNSS-NavigationModel).

NOTE 3: 	The reference time t0 is epochTime + ½ × ssrUpdateInterval. The reference time t0 for ssrUpdateInterval '0' is epochTime.

Value of ssrUpdateInterval to SSR Update Interval relation

| Value of ssrUpdateInterval | SSR Update Interval |
| --- | --- |
| 0 | 1 second |
| 1 | 2 seconds |
| 2 | 5 seconds |
| 3 | 10 seconds |
| 4 | 15 seconds |
| 5 | 30 seconds |
| 6 | 60 seconds |
| 7 | 120 seconds |
| 8 | 240 seconds |
| 9 | 300 seconds |
| 10 | 600 seconds |
| 11 | 900 seconds |
| 12 | 1800 seconds |
| 13 | 3600 seconds |
| 14 | 7200 seconds |
| 15 | 10800 seconds |

#### –	GNSS-SSR-ClockCorrections

The IE GNSS-SSR-ClockCorrections is used by the location server to provide clock correction parameters together with integrity information. The target device may use the SSR-ClockCorrectionList to compute a clock correction to be applied to the broadcast satellite clock parameters, identified by iod of corresponding GNSS-SSR-OrbitCorrections.

The parameters provided in IE GNSS-SSR-ClockCorrections – except for CLOCK-IntegrityParameters and SSR-IntegrityClockBounds – are used as specified for SSR Clock Messages (e.g., message type 1058 and 1064) in [30] and apply to all GNSSs.

-- ASN1START

GNSS-SSR-ClockCorrections-r15 ::= SEQUENCE {

epochTime-r15						GNSS-SystemTime,

ssrUpdateInterval-r15				INTEGER (0..15),

iod-ssr-r15							INTEGER (0..15),

ssr-ClockCorrectionList-r15			SSR-ClockCorrectionList-r15,

...,

[[

clock-IntegrityParameters-r17	CLOCK-IntegrityParameters-r17		OPTIONAL   -- Need ON

]]

}

SSR-ClockCorrectionList-r15 ::= SEQUENCE (SIZE(1..64)) OF SSR-ClockCorrectionSatelliteElement-r15

SSR-ClockCorrectionSatelliteElement-r15 ::= SEQUENCE {

svID-r15							SV-ID,

delta-Clock-C0-r15					INTEGER (-2097152..2097151),

delta-Clock-C1-r15					INTEGER (-1048576..1048575)			OPTIONAL, -- Need ON

delta-Clock-C2-r15					INTEGER (-67108864..67108863)		OPTIONAL, -- Need ON

...,

[[

ssr-IntegrityClockBounds-r17	SSR-IntegrityClockBounds-r17		OPTIONAL  -- Need ON

]]

}

CLOCK-IntegrityParameters-r17 ::= SEQUENCE {

clockRangeErrorCorrelationTime-r17		INTEGER (0..255),

clockRangeRateErrorCorrelationTime-r17	INTEGER (0..255),

...

}

SSR-IntegrityClockBounds-r17 ::= SEQUENCE {

meanClock-r17							INTEGER (0..255),

stdDevClock-r17							INTEGER (0..255),

meanClockRate-r17						INTEGER (0..255),

stdDevClockRate-r17						INTEGER (0..255),

...

}

-- ASN1STOP

GNSS-SSR-ClockCorrections field descriptions

epochTime This field specifies the epoch time of the clock corrections. The gnss-TimeID in GNSS-SystemTime shall be the same as the GNSS-ID in IE GNSS-GenericAssistDataElement.

ssrUpdateInterval This field specifies the SSR Update Interval. The SSR Update Intervals for all SSR parameters start at time 00:00:00 of the GPS time scale. A change of the SSR Update Interval during the transmission of SSR data should ensure consistent data for a target device. See table Value of ssrUpdateInterval to SSR Update Interval relation in IE GNSS-SSR-OrbitCorrections.

iod-ssr This field specifies the Issue of Data number for the SSR data. A change of iod-ssr is used to indicate a change in the SSR generating configuration.

svID This field specifies the satellite for which the clock corrections are provided.

delta-Clock-C0 This field specifies the C0 polynomial coefficient for correction of broadcast satellite clock. NOTE 1. Scale factor 0.1 mm; range ±209.7151 m.

delta-Clock-C1 This field specifies the C1 polynomial coefficient for correction of broadcast satellite clock. NOTE 1. Scale factor 0.001 mm/s; range ±1.048575 m/s.

delta-Clock-C2 This field specifies the C2 polynomial coefficient for correction of broadcast satellite clock. NOTE 1. Scale factor 0.00002 mm/s2; range ±1.34217726 m/s2.

clockRangeErrorCorrelationTime This field specifies the Clock Range Error Correlation Time which is the upper bound of the correlation time of the satellite residual range error due to clock. The time is calculated using: t=10i, &i≤1801800+100(i-180), 180<&i≤234 7200+1000i-234, &i>234 [s] Range is 1-28,200 s.

clockRangeRateErrorCorrelationTime This field specifies the Clock Range Rate Error Correlation Time which is the upper bound of the correlation time of the satellite residual range rate error due to clock. The time is calculated using: t=10i, &i≤1801800+100(i-180), 180<&i≤234 7200+1000i-234, &i>234 [s] Range is 1-28,200 s.

meanClock This field specifies the Mean Clock Error bound which is the mean value for an overbounding model that bounds the residual clock error. The bound is meanClock + K * stdDevClock and shall be so that the probability of it to be exceeded shall be lower than IRallocation for irMinimum < IRallocation < irMaximum, where K = normInv(IRallocation / 2) and irMinimum, irMaximum as provided in IE GNSS-Integrity-ServiceParameters. This IRallocation is a fraction of the Target Integrity Risk that represents the integrity risk budget available. The mean is calculated using: μ=0.0125i, &i≤20052+0.15(i-200), 200<&i≤2340 25+0.52i-2340, &i>2340 [m] Range is 0-17.5 m.

stdDevClock This field specifies the Standard Deviation Clock Error bound which is the standard deviation for an overbounding model that bounds the residual clock error. The standard deviation is calculated using: σ=0.0125i, &i≤20052+0.15(i-200), 200<&i≤2340 25+0.52i-2340, &i>2340 [m] Range is 0-17.5 m.

meanClockRate This field specifies the Mean Clock Rate Error bound which is the mean value for an overbounding model that bounds the residual clock rate error. The bound is meanClockRate + K * stdDevClockRate and shall be so that the probability of it to be exceeded shall be lower than IRallocation for irMinimum < IRallocation < irMaximum, where K = normInv(IRallocation / 2) and irMinimum, irMaximum as provided in IE GNSS-Integrity-ServiceParameters. This IRallocation is a fraction of the Target Integrity Risk that represents the integrity risk budget available. Scale factor 0.001 m/s; range 0.000-0.255 m/s.

stdDevClockRate This field specifies the Standard Deviation Clock Rate Error bound which is the standard deviation for an overbounding model that bounds the residual clock rate error. Scale factor 0.001 m/s; range 0.000-0.255 m/s.

NOTE 1: 	The reference time t0 is epochTime + ½ × ssrUpdateInterval. The reference time t0 for ssrUpdateInterval '0' is epochTime.

/**Skip unrelated parts**/