3GPP TSG-RAN WG2 Meeting #119-e                             	R2-2208926

Online, August 17 – August 29, 2022

Agenda item:	6.5.3

Source:	Samsung

Title:	Summary of User Plane Issues in IIOT/URLLC

Document for:	Discussion & Decision

# Introduction

This document provides a summary of user plane issues based on the following contributions:

R2-2207432	Discussion on MAC layer operation at PUSCH cancellation	Apple	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

R2-2207433	Draft CR for MAC layer operation at PUSCH cancellation	Apple, Ericsson	CR	Rel-17	38.321	17.1.0	1316	-	F	NR_IIOT_URLLC_enh-Core

R2-2207506	Consideration on CG-PUSCH cancellation for UCI -only case	CATT	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

R2-2207507	Simultaneous transmission of SR and PUSCH over different PUCCH groups	CATT	CR	Rel-17	38.321	17.1.0	1321	-	F	NR_IIOT_URLLC_enh-Core

R2-2207796	Issue on a CG transmission cancelled by a DG without UL-SCH	OPPO	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

R2-2208013	MAC impact on PHY prioritization	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

R2-2208014	Correction on TB generated for UCI multiplexing	Nokia, Nokia Shanghai Bell	CR	Rel-17	38.321	17.1.0	1361	-	F	NR_IIOT_URLLC_enh-Core

R2-2208061	Discussion on deprioritized CG-PUSCH with UCI only TB	Huawei, HiSilicon	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

R2-2208062	Discussion on simultaneous transmissions of SR and PUSCH	Huawei, HiSilicon	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

R2-2208122	Open Issues in IIOT UP	Qualcomm Incorporated	discussion	Rel-17

R2-2208355	Discussion on SR error handling on PUCCH Cells	ASUSTeK	discussion	Rel-16	38.321	NR_IIOT_URLLC_enh-Core

R2-2208588	Correction for De-prioritizatin of Overlapping Resources	Samsung	draftCR	Rel-17	38.321	17.1.0	F	NR_IIOT_URLLC_enh-Core

For the issue of simultaneous transmission over different PUCCH groups covered by R2-2207507 (CATT) and R2-2208062 (Huawei), the following contributions were submitted to TEI17. This document covers those documents as well.

R2-2207792	Clarification on SR and PUSCH collision-Alt1	OPPO, Samsung	CR	Rel-17	38.321	17.1.0	1341	-	F	TEI17

R2-2207793	Clarification on SR and PUSCH collision-Alt2	OPPO, Samsung	CR	Rel-17	38.321	17.1.0	1342	-	F	TEI17

R2-2207794	Clarification on SR and PUSCH collision-Alt2	OPPO, Samsung	CR	Rel-17	38.331	17.1.0	3315	-	F	TEI17

R2-2207795	Clarification on SR and PUSCH collision-Alt2	OPPO, Samsung	CR	Rel-17	38.306	17.1.0	0778	-	F	TEI17

# Issues

## CG-PUSCH Cancellation by UCI-only DG-PUSCH without UL-SCH

In RAN2#118-e, RAN2 agreed that Rel-16 LCH-based Prioritization is independent of cancellation of CG-PUSCH with low PHY priority by DG-PUSCH with high PHY priority for DG-PUSCH with UL-SCH. A remaining open issue is for DG-PUSCH without UL-SCH (i.e. UCI-only PUSCH), as follows:

4	When a CG-PUSCH transmission is cancelled by a DG-PUSCH with UL-SCH (i.e. MAC PDU is delivered to PHY) in Rel-17, de-prioritization relies on existing Rel-16 LCH-based Prioritization. (no specification change)

5	FFS When a CG-PUSCH transmission is cancelled by a DG-PUSCH without UL-SCH (i.e. MAC PDU is not delivered to PHY) in Rel-17, de-prioritization relies on existing Rel-16 LCH-based Prioritization. The CG is not considered as a de-prioritized uplink grant. (no specification change).  Check issue described for UCI only case

Multiple companies submitted contributions for this issue. Companies’ views are summarized as follows:

- Option 1. The de-prioritization relies on existing Rel-16 LCH-based Prioritization. (no specification change)

  - CATT (7506): When lch-BasedPrioritization is configured, the case where a CG-PUSCH transmission is cancelled by a DG-PUSCH for UCI only is invalid in Rel-17.

  - OPPO (7796): This case is similar to the one that already exists in Rel-16.

  - Nokia/NSB (8013): It will complicate the specification and increase the interactions between PHY and MAC quite a lot.

  - Huawei/HiSilicon (8061): The UE’s behaviour for a CG-PUSCH overlapping with a DG-PUSCH without UL-SCH is undefined in RAN1.

  - Qualcomm (8122): The complexity of sending cancelled PHY PUSCH transmissions back to MAC in their PDU form for de-prioritization is not justified as it slightly breaks the PHY-MAC modularity.

- Option 2. The uplink grant associated with the cancelled CG is considered as a de-prioritized grant.

  - Apple (7432): RAN1 has introduced two new feature groups, FG 25-14 and FG 25-15, for overlapping DG/CG and overlapping CG/DG. Also, PHY specification captures that the UE is expected to cancel a PUSCH transmissions of smaller priority index before the first symbol overlapping with the PUSCH transmission of larger priority index.

Proposal 1. RAN2 to discuss: When a CG-PUSCH transmission is cancelled by a DG-PUSCH without UL-SCH (i.e. MAC PDU is not delivered to PHY) in Rel-17,

- Option 1. de-prioritization relies on existing Rel-16 LCH-based Prioritization. The CG is not considered as a de-prioritized uplink grant. (no specification change).

- Option 2. the uplink grant associated with the cancelled CG is considered as a de-prioritized grant.

## Simultaneous Transmission for Different PUCCH Groups

Since Rel-15, RAN1 assumed that the simultaneous transmission of SR (PUCCH) and UL-SCH data (PUSCH) over different PUCCH groups is supported. However, the MAC specification has been blocking it, since the MAC entity compares resource overlap between SR and UL-SCH in the MAC entity. Only UL-SCH resource can be transmitted in time in Rel-15.

In Rel-16 LCH-based Prioritization, this problem has more impact. This feature assumes more resources can be overlapped with each other. Based on the logical channel priority, only one resource (either SR or UL-SC) can be transmitted in the MAC entity, regardless of same or different PUCCH group. This is not aligned with RAN1 specification and assumption.

Some companies considered this issue as a Rel-15/16 issue. In RAN2#118-e, RAN2 discussed how to capture Rel-15/16 simultaneous transmission for different PUCCH groups, concluded not to change legacy UEs, and postpone Rel-17 change, as follows:

R2-2206468 Offline 014: Rel-15/16 User Plane Samsung. We don’t require change of R15 R16 implementations (i.e. accept UEs impl acc to understanding 1). For Rel-17 postpone the discussion to next meeting.

For this meeting, CATT (7507) and Huawei (8062) submitted contributions to Rel-17 IIOT Correction whereas OPPO/Samsung submitted to TEI17 (R2-2207792/7793/7794/7795). Irrespective of the exact session where the discussion takes place, those contributions should be discussed together. All those companies proposed to support the simultaneous transmission over different PUCCH groups in Rel-17, since Rel-17 IIOT already agreed to support the simultaneous transmission within the same PUCCH group.

There are two discussion points

- 1) Whether the Rel-17 MAC specification supports simultaneous transmission of SR and UL-SCH over different PUCCH groups (CATT (7507), Huawei (8062), OPPO/Samsung(7792-7795))

- 2) If the conclusion of 1) is yes, does all Rel-17 UE mandatorily follow it (CATT (7507), Huawei (8062), OPPO/Samsung(7792) or can it be based on a configuration and UE capability (OPPO/Samsung(7793-7795)).

Proposal 2. RAN2 to discuss the following options for simultaneous transmission of SR and PUSCH over different PUCCH groups:

- Option 1. All Rel-17 UEs mandatorily supports simultaneous transmissions of overlapping SR and PUSCH over different PUCCH groups.

- Option 2. Define a capability and RRC configuration parameter of simultaneous transmissions of overlapping SR and PUSCH over different PUCCH groups.

- Option 3. Rel-17 MAC does not allow simultaneous transmission of SR and UL-SCH over different PUCCH groups. (No specification change)

## Flushing HARQ Buffer for Empty MAC PDU

For UCI-only PUSCH whose uplink grant is de-prioritized, if CGRT is configured, the uplink grant will be considered as that for retransmission and prioritized in the next CG occasion. The transmission of the empty MAC PDU increases the delay for other HARQ processes who serve non-empty MAC PDU. To avoid delaying new data transmission, Nokia (8013) proposed to flush the empty TB only for UCI multiplexing purpose after the transmission and stop CGRT and CGT for the corresponding HARQ process.

Proposal 3. RAN2 to discuss that empty TB generated only for UCI multiplexing purpose should be flushed after the transmission and stop the configuredGrantTimer and cg-RetransmissionTimer when cg-RetransmissionTimer is configured.

## De-prioritization of CG-PUSCH by DL Transmission or Measurement Gap

Huawei (8061) pointed out that CG-PUSCH can be cancelled or partially cancelled by a subset of symbols of the CG-PUSCH have been replaced to receive CSI-RS or PDSCH. Also, in case that the CG occasion encounters a measurement gap, the TB cannot be transmitted. The proposal is use those CG for autonomous transmission, so to consider the configured grant for those cases as a de-prioritized uplink.

Proposal 4. RAN2 to discuss:

- If a subset of symbols of a CG-PUSCH has been replaced to receive downlink transmission, the CG shall be considered as a deprioritized uplink grant.

- If a TB is generated for a CG, but it is not transmitted due to collision with a measurement gap, the CG shall be considered as a deprioritized uplink grant.

## PUCCH Resource Handling for Maximum Number of SR Transmissions on non-SpCell

In the current MAC specification, when the SR transmission reaches the maximum number, the UE shall release UL resources for all Serving Cells and initiate random access procedure on SpCell. Even if the PUCCH resource for the SR is not on SpCell, the MAC entity follows the same procedures. ASUSTek (8355) pointed out, in this case, SpCell may not be problematic and initial access may not be needed. They proposed for the maximum SR transmissions not on SpCell, the UE shall not initiate a random access procedure, cancel pending SRs on the problematic cell, and release UL resource of Serving Cells with pucch-cell set to the cell.

Proposal 5. RAN2 to discuss: For a SR transmitted on PUCCH resources on a Cell other than SpCell, when the SR reaches the maximum number of transmission (sr-TransMax),

- 5a) the UE shall not initiate a random access procedure on SpCell.

- 5b) the UE shall cancel pending SRs that are transmitted via PUCCH resources on the Cell.

- 5c) the UE shall release UL resources of Serving Cells with pucch-cell set to the Cell.

## Clarification on De-prioritization of Overlapping Resources (Clarification)

Considering simultaneous transmission of SR and UL-SCH within the PUCCH group (configured by simultaneousPUCCH-PUSCH or simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup), in LCH-based Prioritization, the MAC entity checks overlap of resources whose the simultaneous transmission is not allowed.

Huawei (8062) and Samsung (8588) pointed out mutual pre-emptions which prohibit the simultaneous transmission. According to the current interpretation, if one resource (either SR or UL-SCH) is prioritized, the other overlapping resource is automatically de-prioritized. The overlapping resource here should be the resource whose simultaneous transmission with the prioritized resource is not allowed, however the current text does not have this restriction. Both contributions proposed the almost same TP, for instance:

1> else if this uplink grant is addressed to CS-RNTI with NDI = 1 or C-RNTI: 2> if there is no overlapping PUSCH duration of a configured uplink grant which was not already de-prioritized, in the same BWP, whose priority is higher than the priority of the uplink grant; and 2> if there is no overlapping PUCCH resource with an SR transmission which was not already de-prioritized and the simultaneous transmission of the SR and the uplink grant is not allowed by configuration of simultaneousPUCCH-PUSCH or simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup, and the priority of the logical channel that triggered the SR is higher than the priority of the uplink grant: 3> consider this uplink grant as a prioritized uplink grant; 3> consider the other overlapping uplink grant(s), if any, as a de-prioritized uplink grant(s); 3> consider the other overlapping SR transmission(s), if any, as a de-prioritized SR transmission(s), except for the overlapping SR transmission(s) whose simultaneous transmission with the uplink grant is allowed as specified in clause 9 of TS 38.213 [6]; 3> if the de-prioritized uplink grant(s) is a configured uplink grant configured with autonomousTx whose PUSCH has already started: 4> stop the configuredGrantTimer for the corresponding HARQ process of the de-prioritized uplink grant(s); 4> stop the cg-RetransmissionTimer for the corresponding HARQ process of the de-prioritized uplink grant(s).

Proposal 6. RAN2 to discuss: if simultaneousPUCCH-PUSCH or simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup is configured,

- 6a) when a SR transmission is considered as a prioritized SR transmission, the overlapping uplink grant(s) which can be transmitted with the SR simultaneously shall not be considered as de-prioritized uplink grant(s).

- 6b) when a PUSCH transmission is considered as a prioritized PUSCH transmission, the overlapping SR(s) which can be transmitted with the PUSCH simultaneously shall not be considered as de-prioritized SR(s).

## Confirmation of Starting Condition of drx-RetransmissionTimerDL (Confirmation of Current Agreement)

In RAN2#118-e, RAN2 agreed not to start drx-RetransmissionTimerDL for maximum allowed deferral time of HARQ feedback, as follows:

9	When the maximum allowed deferral time of HARQ feedback is reached drx-RetransmissionTimerDL is not started. (no specification change)

Qualcomm (8122) proposed to keep the current agreement since they do not see to alter that agreement. However, no counter-proposal was submitted to this meeting. No further discussion is suggested.

Proposal 7. RAN2 to confirm without further discussion: “When the maximum allowed deferral time of HARQ feedback is reached drx-RetransmissionTimerDL is not started. (no specification change)”.

# Conclusion

< FFS points in the last meeting >

Proposal 1. RAN2 to discuss: When a CG-PUSCH transmission is cancelled by a DG-PUSCH without UL-SCH (i.e. MAC PDU is not delivered to PHY) in Rel-17,

- Option 1. de-prioritization relies on existing Rel-16 LCH-based Prioritization. The CG is not considered as a de-prioritized uplink grant. (no specification change).

- Option 2. the uplink grant associated with the cancelled CG is considered as a de-prioritized grant.

Proposal 2. RAN2 to discuss the following options for simultaneous transmission of SR and PUSCH over different PUCCH groups:

- Option 1. All Rel-17 UEs mandatorily supports simultaneous transmissions of overlapping SR and PUSCH over different PUCCH groups.

- Option 2. Define a capability and RRC configuration parameter of simultaneous transmissions of overlapping SR and PUSCH over different PUCCH groups.

- Option 3. Rel-17 MAC does not allow simultaneous transmission of SR and UL-SCH over different PUCCH groups. (No specification change)

< New Issues >

Proposal 3. RAN2 to discuss that empty TB generated only for UCI multiplexing purpose should be flushed after the transmission and stop the configuredGrantTimer and cg-RetransmissionTimer when cg-RetransmissionTimer is configured.

Proposal 4. RAN2 to discuss:

- If a subset of symbols of a CG-PUSCH has been replaced to receive downlink transmission, the CG shall be considered as a deprioritized uplink grant.

- If a TB is generated for a CG, but it is not transmitted due to collision with a measurement gap, the CG shall be considered as a deprioritized uplink grant.

Proposal 5. RAN2 to discuss: For a SR transmitted on PUCCH resources on a Cell other than SpCell, when the SR reaches the maximum number of transmission (sr-TransMax),

- 5a) the UE shall not initiate a random access procedure on SpCell.

- 5b) the UE shall cancel pending SRs that are transmitted via PUCCH resources on the Cell.

- 5c) the UE shall release UL resources of Serving Cells with pucch-cell set to the Cell.

< Clarification of the current text >

Proposal 6. RAN2 to discuss: if simultaneousPUCCH-PUSCH or simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup is configured,

- 6a) when a SR transmission is considered as a prioritized SR transmission, the overlapping uplink grant(s) which can be transmitted with the SR simultaneously shall not be considered as de-prioritized uplink grant(s).

- 6b) when a PUSCH transmission is considered as a prioritized PUSCH transmission, the overlapping SR(s) which can be transmitted with the PUSCH simultaneously shall not be considered as de-prioritized SR(s).

< Confirmation of the current agreement >

Proposal 7. RAN2 to confirm without further discussion: “When the maximum allowed deferral time of HARQ feedback is reached drx-RetransmissionTimerDL is not started. (no specification change)”.