3GPP TSG-RAN WG2 Meeting #119-e	R2-2208978

# Electronic,  2022-08-17 – 2022-08-26

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0551 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | Χ | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR Correction related to RNA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Deutsche Telekom, Huawei, HiSilicon, Nokia (Rapporteur) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2022-08-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The current statement in Section 9.2.2.3 RAN-Based Notification Area “A cell broadcasts one or more RAN area IDs in the system information.” is ambiguous as two and more RAN area IDs can only be broadcast by a (physical) cell in special cases, as specified in 38.331. Therefore, in order to solve this ambiguity, we see the need for a correction on this Stage 2 aspect for RRC_Inactive State to align with stage 3 specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarified that only in case of network sharing with multiple cell ID broadcast, a cell may broadcast more than one RAN area ID in the system information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 2 ambiguity left for broadcast of RAN area IDs in RRC_Inactive State. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | χ | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | χ | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | χ | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 9.2.2.3	RAN-Based Notification Area

A UE in the RRC_INACTIVE state can be configured by the last serving NG-RAN node with an RNA, where:

-	the RNA can cover a single or multiple cells, and shall be contained within the CN registration area; in this release Xn connectivity should be available within the RNA;

-	a RAN-based notification area update (RNAU) is periodically sent by the UE and is also sent when the cell reselection procedure of the UE selects a cell that does not belong to the configured RNA.

There are several different alternatives on how the RNA can be configured:

-	List of cells:

-	A UE is provided an explicit list of cells (one or more) that constitute the RNA.

-	List of RAN areas:

-	A UE is provided (at least one) RAN area ID, where a RAN area is a subset of a CN Tracking Area or equal to a CN Tracking Area. A RAN area is specified by one RAN area ID, which consists of a TAC and optionally a RAN area Code;

-	A cell broadcasts one or, in case of network sharing with multiple cell ID broadcast, more RAN area IDs in the system information.

NG-RAN may provide different RNA definitions to different UEs but not mix different definitions to the same UE at the same time. UE shall support all RNA configuration options listed above.