3GPP TSG-RAN2 WG2 Meeting #119 electronic	R2-2208991

# Online, August 17 – 29, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 0775 | rev | 1 | Current version: | 15.17.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.306 on UE capability of MMSE-IRC receiver |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2022-08-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In RAN4 LS (R2-2204471_R4-2206572), Rel-17 RAN4 UE feature list for NR is sent to RAN2. For the features of NR_demod_enh2-Core, a new UE optional feature of MMSE-IRC receiver for scenarios with inter-cell and intra-cell inter-user interference is introduced by RAN4 with the index of FG 24-6. It is mandatory to meet the new MMSE-IRC receiver requirements without capability signalling from Rel-17, and optional without capability signalling for Rel-15 and Rel-16. To make it clear, the new MMSE-IRC receiver feature without UE radio access capability parameters shall be specified in TS 38.306 for Rel-15 based on RAN4 LS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - A new UE feature of MMSE-IRC receiver is added to clause 5.2, which is optional without capability signalling for Rel-15. Impact analysis Impacted 5G architecture options: NR SA, NR-DC, (NG)EN-DC, NE-DC Impacted functionality: UE receiver feature Inter-operability: - If the network is implemented according to the CR and the UE is not; no inter-operability issue is foreseen. - If the UE is implemented according to the CR and the network is not; no inter-operability issue is foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TS 38.306 for Rel-15 remains unclear on the applicability of MMSE-IRC receiver feature. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2 UE receiver features |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R2-2207640 |  |  |  |  |  |  |  |  |

START OF CHANGE

## 5.2	UE receiver features

Definitions for feature

SU-MIMO Interference Mitigation advanced receiver - R-ML (reduced complexity ML) receivers with enhanced inter-stream interference suppression for SU-MIMO transmissions with rank 2 with 2 RX antennas - R-ML (reduced complexity ML) receivers with enhanced inter-stream interference suppression for SU-MIMO transmissions with rank 2, 3, and 4 with 4 RX antennas UE supporting the feature is required to meet the Enhanced Receiver Type requirements in TS 38.101-4 [18].

MMSE-IRC (Minimum Mean Square Error - Interference Rejection Combining) receiver It is optional for UE to support MMSE-IRC processing for scenarios with inter-cell and intra-cell inter-user interference.

END OF CHANGE