3GPP TSG-RAN WG2 Meeting #119-e	R2-2209069

# Electronic meeting, August 15 – 26, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 0786 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on crossCarrierA-CSI-trigDiffSCS-r16 (38.306) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Core |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In crossCarrierA-CSI-trigDiffSCS-r16 field description it is not clear if “A-CSI RS” is related to the UE report and/or CSI-RS transmission. It should thus be corrected to reflect that the UE capability is related to the UE report and the CSI-RS transmission. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 4.2.7.4 CA-ParametersNR - Clarify crossCarrierA-CSI-trigDiffSCS-r16 - Including that the field is related to aperiodic CSI report and aperiodic CSI-RS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The feature may be interpreted as related to only the aperiodic CSI report, and thus it could be considered that, based on this feature, an aperiodic CSI report could also be triggered in case of periodic CSI-RS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.7.4 CA-ParametersNR |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1: This CR was revised to keep the relation between crossCarrierA-CSI-trigDiffSCS-r16 and csi-RS-IM-ReceptionForFeedback: “A UE supporting this feature shall also indicate support of CSI-RS and CSI-IM reception for CSI feedback using csi-RS-IM-ReceptionForFeedback”, even if the field csi-RS-IM-ReceptionForFeedback is mandatory. |  |  |  |  |  |  |  |  |

START OF CHANGE

| crossCarrierA-CSI-trigDiffSCS-r16 Indicates the UE support of handling cross-carrier aperiodic CSI report with aperiodic CSI-RS where triggering PDCCH and triggered CSI-RS resource are on different cells with different SCS. Value higherA-CSI-SCS indicates the UE support of PDCCH cell of lower SCS and CSI RS cell of higher SCS and value lowerA-CSI-SCS indicates the UE support of PDCCH cell of higher SCS and CSI RS cell of lower SCS, and value both indicates the support of both variations. A UE supporting this feature shall also indicate support of CSI-RS and CSI-IM reception for CSI feedback using csi-RS-IM-ReceptionForFeedback | BC | No | N/A | N/A |
| --- | --- | --- | --- | --- |

END OF CHANGE