3GPP TSG RAN WG2#119-e	R2-2209102

e-Meeting, 17th - 29th August, 2022

3GPP TSG RAN WG1 Meeting #110	R1-2207980

Toulouse, France, 22nd – 26th August 2022

Title:	Reply LS on introduction of an offset to transmit CD-SSB and NCD-SSB at different times

Response to:	R1-2205734 / R2-2206267

Release:	Rel-17

Work Item:	Support of reduced capability NR devices (NR_redcap-Core)

Source:	RAN1

To:	RAN2

Cc:	RAN4

Contact person:	Johan Bergman, johan (dot) bergman (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN1 would like to thank RAN2 for the LS where RAN2 asked the following question:

RAN2 would like to thank RAN1 and RAN4 for their reply LS on introduction of an offset to transmit CD-SSB and NCD-SSB at different times. RAN2 agreed to specify the offset with the following value range {sf5, sf10, sf15, spare5, spare4, spare3, spare2, spare1} and the definition below: ssb-TimeOffset The time offset between CD-SSB of the serving cell and this Non-Cell Defining SSB. Value sf5 means the first burst of Non-Cell Defining SSB is transmitted 5ms later than the first burst of CD-SSB transmitted after the first symbol of SFN=0 of the serving cell, value sf10 means the first burst of Non-Cell Defining SSB is transmitted 10ms later than the first burst of CD-SSB transmitted after the first symbol in SFN=0 of the serving cell, and so on. If the field is absent, RedCap UE considers that the time offset between the first burst of CD-SSB transmitted in the serving cell and the first burst of this Non-Cell Defining SSB transmitted is zero.. RAN2 would like to ask RAN1 and RAN4 to take the above into consideration and provide feedback on the values, i.e., confirm and/or indicate whether additional values are needed.

RAN1 has discussed the question in the LS and would like to provide the following reply:

RAN1 understands RAN4 has defined 20 and 40 ms offset, and RAN1 thinks that the NCD-SSB time offset values {sf5, sf10, sf15} are sufficient from RAN1 perspective, and {sf20, sf40} are also feasible.

# 2	Actions

To RAN2:

ACTION: 	RAN1 respectfully requests RAN2 to take the above into account in their future work.

# 3	Dates of next TSG-RAN WG1 meetings

TSG-RAN WG1 Meeting #110-bis-e	10th – 19th October 2022		Online

TSG-RAN WG1 Meeting #111	14th – 18th November 2022		TBD