3GPP TSG-RAN2 Meeting #119 electronic	R2-2209113

Online, Aug 17th – 26th, 2022

Source: 	     ZTE Corporation, Sanechips

Title: 	     Summary of [ AT119-e][027][NPN] NPN corrections (ZTE)

Agenda item:  6.16

Document for:  Discussion and Decision

1 Introduction

This contribution summarizes the following discussion:

[AT119-e][027][NPN] NPN corrections (ZTE)

Scope: Treat R2-2207163, R2-2207501, R2-2208624. Determine agreeable parts. For agreeable parts, agree CRs

Intended outcome: Report, Agreed CRs

Deadline: Schedule 1

Schedule 1: A first round with Deadline for comments W1 Friday Aug 19th 1400 UTC to settle scope what is agreeable etc A Final round with Final deadline W2 Thursday Aug 25th 1200 UTC to settle details / agree CRs etc.

R2-2207163	CR on the  ims-EmergencySupport  for the SNPN and PLMN RAN sharing scenario	ZTE Corporation, Sanechips	CR	Rel-17	38.331	17.1.0	3224	-	F	NG_RAN_PRN_enh-Core

R2-2207501	Correction to 38.300 on GIN	Huawei, HiSilicon	CR	Rel-17	38.300	17.1.0	0512	-	F	NG_RAN_PRN_enh-Core

R2-2208624	Changing the gins-PerSNPN-List Need Code	Ericsson	discussion	Rel-17	NG_RAN_PRN_enh-Core

Contact person(s) for each participating company:

| Company | Contact Name, Email |
| --- | --- |
| Nokia | gyorgy.wolfner@nokia.com |
| Qualcomm | oozturk@qti.qualcomm.com |
| Intel | seau.s.lim@intel.com |
| ZTE | Li.wenting@zte.com.cn |
| Samsung | sriganesh.r@samsung.com |
| Huawei, HiSilicon | zhenglili4@huawei.com |
| Ericsson | Felipe Arraño Scharager, felipe.arrano.scharager@ericsson.com |
| OPPO | fanjiangsheng@oppo.com |
| Apple | yuqin_chen@apple.com |

2	Discussion

2.1	Phase 1: Intended to determine agreeable parts

2.1.1 ims-EmergencySupport

R2-2207163	CR on the  ims-EmergencySupport  for the SNPN and PLMN RAN sharing scenario	ZTE Corporation, Sanechips	CR	Rel-17	38.331	17.1.0	3224	-	F	NG_RAN_PRN_enh-Core

Q1  Do companies agree with this CR?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Nokia | No | As the imsEmergencySupportForSNPN is per SNPN, it is forwarded to the upper layer with the corresponding SNPN identities. Therefore, the upper layers can never misinterpret the ims-EmergencySupport, which is forwarded to the upper layers without SNPN identity, as an indication that an SNPN supports emergency services. |
| Qualcomm | Maybe | The change is technically correct. But in SNPN mode upper layers can just ignore the legacy IE, which can be done as UE implementation. |
| Intel | No | Agree with Nokia there is no need to filter at the UE access stratum for this case |
| ZTE | Yes(proponent) | We think the current spec is unaligned with RAN2’s understanding. For the PLMN and NPN sharing scenario, the ims-EmergencySupport can only indicate whether the public network support emergency service. ims-EmergencySupport Indicates whether the cell supports public network IMS emergency bearer services for UEs in limited service mode. If absent, IMS emergency call is not supported by the public network in the cell for UEs in limited service mode. |
| Samsung | No | Same opinion as Nokia, The reason for introducing a separate indicator for SNPN was to avoid such confusion. |
| Huawei, HiSilicon | Prefer no | The change could have be adopted if proposed before R17 is frozen. Now that the UE behavior is already specified, and we have some sympathy with Nokia’s reasoning that there is no confusion to the NW even with the current behavior, our preference is to leave the spec as it is. |
| Ericsson | No | We see no need for this change as the spec is not ambiguous as it is. |
| OPPO | Maybe | The change is not essential, as mentioned above, a good NAS implementation will not consider ims-EmergencySupport indicator when in SNPN mode. |
| Apple | Tend to No | We agree with the intent but also see this as a non-essential correction. |

Summary:

9 companies give the feedback, 6 companies say no, 2 companies say “maybe” but don’t think it’s a essential correction. So this CR is suggested to be not pursued.

Proposal 1: R2-2207163 is suggested to be not pursued.

2.1.2	Correction to 38.300 on GIN

R2-2207501	Correction to 38.300 on GIN	Huawei, HiSilicon	CR	Rel-17	38.300	17.1.0	0512	-	F	NG_RAN_PRN_enh-Core

Q2  Do companies see a need to change TS38.300 on GIN as above?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Nokia | No | The introduction of this definition and the wording change in are not necessary, we already have references to 23.501 for NPN features. |
| Qualcomm |  | No strong opinion; it is editorial and seems redundant. |
| Intel | No strong view | Though do not see the change as essential |
| ZTE | No strong view | We can follow the majorities. |
| Samsung | No strong opinion |  |
| Huawei, HiSIlicon | Yes | GIN is mentioned several times in both 38.300 and 38.331, but currently there is no definition in RAN2 spec and the readability is greatly impacted. |
| Ericsson | No strong view |  |
| OPPO | No strong view | Not critical from our side. |
| Apple | Good to have | For the sake of readability of spec, it is good to have the definition. |

Summary:

9 companies give the feedback, 1 companies say no for the current spec has reference to 23.501, 2 companies say Yes for that it can improve the readability. 6 companies don’t have strong view on it but in which 3 companies don’t think it’s an essential correction.

Proposal 2: Merge the modification in R2-2207501 in to the rapporteur CR (R2-2208961 as in offline 035)

2.1.3	Need code for the gins-PerSNPN-Lis

R2-2208624	Changing the gins-PerSNPN-List Need Code	Ericsson	discussion	Rel-17	NG_RAN_PRN_enh-Core

Observation 1	Even with RAN2#118-e rewording, there is still a need to change the Need Code of ginsPerSNPN-List-r17, as there is a special UE behaviour linked to the absence of the field.

Proposal 1	Change the Need Code of ginsPerSNPN-List-r17 from Need R to Need S.

Q3. Do companies agree with the above observation 1 and proposal 1?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Nokia | No | NEED S is to specify special UE behavior when the field is not present. We think this is not the case, this is just the description when the field may be omitted by the network due to signaling optimization. |
| Qualcomm | No | Agree with Nokia. |
| Intel | Yes | There is a description of behaviour on absence and then by definition, it has to be Need S. Just Need R is not sufficient as the configuration is not just released, but also apply the configuration defined in the field description. |
| ZTE | Yes | It would have some impact on the UE behavior, so we tend to agree with this CR |
| Samsung | No | Same view as Nokia. |
| Huawei, HiSilicon | No | Agree with Nokia |
| Ericsson | Yes (proponent) | As per Nokia’s comment above; the field description doesn’t need to capture network recommendations/implementation in the specification. We only need to capture what the UE is supposed to do. And as this is special UE behavior upon absence, the Need Code for this one should be changed to Need S. This has further been stressed by Intel. |
| OPPO | No | We don’t think there is room for misunderstanding when implementing, so prefer not to have this change. |
| Apple | Yes | We tend to agree with Ericsson since the absence of this field does not mean UE should release this configuration, Need S is more suitable here. |

Summary:

9 companies give the feedback, 4 companies say Yes for that the absence of this filed does not mean the UE should release this configuration and it’s about UE special behavior, need R is not sufficient. However, 5 companies say no with the reason that “ this is just the description when the field may be omitted by the network due to signaling optimization” and that there is no room for misunderstanding. Considering that there are more than half companies disagree with this modification, it’s suggested to be not pursued.

Proposal 3: R2-2208624 is suggested to be not pursued.

3	Conclusion

Proposal 1: R2-2207163 is suggested to be not pursued.

Proposal 2: Merge the modification in R2-2207501 in to the rapporteur CR (R2-2208961 as in offline 035)

Proposal 3: R2-2208624 is suggested to be not pursued.