3GPP TSG RAN WG2#119-e	R2-2209122

e-Meeting, 17th - 29th August, 2022

3GPP TSG-RAN WG1 #110			R1-2208072

Toulouse, France, August 22 – 26, 2022

Title:	Reply LS on rate matching patterns and CORESET configuration for MBS

Response to:	R1-2205732/R2-2206608

Release:	Rel-17

Work Item:	NR_MBS-Core

Source:	TSG-RAN WG1

To:	TSG-RAN WG2

Cc:

Contact Person:

#### Name:	Jinhuan XIA

Tel. Number:

E-mail Address:	jinhuan.xia@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

- Overall Description:

RAN1 thanks RAN2 for the LS informing RAN2 agreements regarding the rate matching patterns and CORESET configuration for MBS:

- If the rate match pattern with the same RateMatchPatternId is configured in both the CFR and in the associated BWP, network should include the same resource configuration and they shall be counted as a single pattern in the total number of configured rate matching patterns at the UE. - If the rate match pattern with the same RateMatchPatternId is configured in both ServingCellConfig/ServingCellConfigCommon and in SIB20/MCCH, network should include the same resource configuration and they shall be counted as a single pattern in the total number of configured rate matching patterns at the UE. - Similar to the case of commonControlResourceSet, for MBS broadcast reception in RRC_CONNECTED, the network may configure via dedicated signaling the control resource set with the same ControlResourceSetId as used for commonControlResourceSetExt in CFR configuration in SIB20, and the configuration from PDCCH-Config takes precedence and should not be updated by the UE based on SIB20.

Regarding the RAN2 agreements, the following bullets are necessary clarifications from RAN1 perspective,

- When UE is configured with the same RateMatchPatternId in both the CFR and in the associated BWP, it is expected to have the same set of RBs/REs indicated by the patterns for rate matching around. Otherwise, the different RateMatchPatternId will be configured in the CFR and in the associated BWP.

- For UE not supporting multipleCORESET in FR1, in order to receive MBS multicast in CFR within UE active BWP,

- if a CORESET is not configured within the PDCCH-ConfigMulticast, the other CORESET than CORESET0 configured within UE active BWP for scheduling unicast can also to be used for scheduling multicast, and the CORESET is expected to be included completely within the CFR and the parameters configured in the CORESET are expected to be supported by the UE for multicast.

2. Actions:

To TSG-RAN WG2

ACTION: 	RAN1 respectfully requests RAN2 to take RAN1’s response into account.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #110bis-e	10 – 19 October 2022	online

TSG-RAN WG1 Meeting #111	14 – 18 November 2022	EU