3GPP TSG RAN WG2#119-e	R2-2209169

e-Meeting, 17th - 29th August, 2022

3GPP TSG-RAN WG4 Meeting #104-e	R4-2214924

Electronic Meeting, 15th-26th Aug, 2022

Title:	Reply LS on clarification of dualPA-Architecture capability

Response to:	R2-2206428

Release:	Rel-17

Work Item:	NR_RF_FR1_enh

Source:	TSG RAN WG4

To:	TSG RAN WG2

Cc:

Contact Person:

Name:  Yuanyuan Zhang

E-mail Address:  tina55.zhang@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

- Overall Description:

RAN4 would like to thank RAN2 for the LS to inquire the compatibility and applicability of the required change from RAN4 on dualPA-Architecture capability.

Regarding RAN2’s request to RAN4 for the responses to the following questions:

Q1: During RAN2#117, RAN2 had made the following agreement for the DC location report

[032] It is left to UE implementation whether a UE supporting dualPA-Architecture for a BC always reports two DC locations for the BC.

[032] A UE not supporting dualPA-Architecture for a BC always reports one DC location for the BC. Whether to change the specification can be discussed at next meeting.

Is the required change from RAN4 (i.e., the reporting of dualPA-Architecture also indicates the support of dual-LO) compatible with the RAN2 agreement above (i.e., the reporting of dualPA-Architecture does not mandate the UE to report two DC locations for the BC)?

Q2: In RAN2 specification, there are two dualPA-Architecture as follows: Where the former one is reported for the intra-band CA part of NR, while the latter one is for the intra-band BC part of (NG)EN-DC/NE-DC.

| dualPA-Architecture For band combinations with single-band with UL CA, this field indicates the support of dual PA. If absent in such band combinations, the UE supports single PA for all the ULs. For other band combinations, this field is not applicable. | BC | No | N/A | N/A |
| --- | --- | --- | --- | --- |
| dualPA-Architecture For an intra-band band combination, this field indicates the support of dual PAs. If absent in an intra-band band combination, the UE supports single PA for all the ULs in the intra-band band combination. For other band combinations, this field is not applicable. This capability applies to: - Intra-band (NG)EN-DC/NE-DC combination without additional inter-band NR and LTE CA component; - Intra-band (NG)EN-DC/NE-DC combination supporting both UL and DL intra-band (NG)EN-DC/NE-DC parts with additional inter-band NR/LTE CA component; - Inter-band (NG)EN-DC/NE-DC combination, where the frequency range of the E-UTRA band is a subset of the frequency range of the NR band (as specified in Table 5.5B.4.1-1 of TS 38.101-3 [4]). If this capability is included in an "Intra-band (NG)EN-DC/NE-DC combination supporting both UL and DL intra-band (NG)EN-DC/NE-DC parts with additional inter-band NR/LTE CA component", this capability applies to the intra-band (NG)EN-DC/NE-DC BC part. | BC | No | N/A | N/A |

Is the required change also applicable to the latter one, or only applicable to the former one?

After discussion during RAN4#104-e, RAN4 would like to respectfully provide the following responses for RAN2 consideration.

Response to Q1: Reporting DC location(s) is up to UE implementation which is independent with indicating dualPA-Architecture capability. The RAN4 required change on dualPA-Architecture capability was for the purpose of differentiating two sets of MPR requirements with different UE architectures for intra-band UL non-contiguous CA, it was not intended to use as an indicator for UE to report two DC locations as of now.

Response to Q2: RAN4 required change applicable only to the former one is sufficient as of now.

2. Actions:

To RAN2:

ACTION: RAN4 respectfully ask RAN2 to take above RAN4 responses into consideration.

3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN WG4 Meeting #104-bis-e	10 - 19 Oct 2022	E-meeting