3GPP TSG-RAN WG2 Meeting #119 Electronic                           R2-220xxxx

Online, 17th Aug– 29th Aug, 2022

Agenda item:	8.10.2

Source:	Huawei, HiSilicon

Title:	[DRAFT] Report of [AT119-e] [615] [IDC] FDM solution enhancements

Document for:	Discussion & Decision

# Introduction

- [AT119-e][651][IDC] FDM solution enhancements (Huawei)

Scope: based on companies’ contributions submitted in 8.10.2

A) Identify the use cases or scenarios (e.g. serving/non-serving, E-UTRA frequency, NR frequency and MR-DC) for the FDM solution enhancements.

B) Granularity of frequency indication for identified use case/scenarios

Intended outcome: Report to Wednesday session in R2-2208921

Deadline: Wednesday 2022-08-24 00:30 AM UTC.

# Contact Details

Respondents to the email discussion are kindly asked to fill in the following table.

| Company | Name | Email Address |
| --- | --- | --- |
| Huawei, HiSilicon | Jagdeep Singh | jagdeep.singh6@huawei.com |
| Ericsson | Mattias Bergström | Mattias.a.bergstrom@ericsson.com |
| Xiaomi | Yumin Wu | wuyumin@xiaomi.com |
| Samsung | Weiwei Wang | ww1016.wang@samsung.com |

# Discussion

In Rel-15, the IDC Frequency Division Multiplexing (FDM) solution was introduced to support the EN-DC. In Rel-16, the IDC FDM solution was introduced to support the standalone NR.

In these legacy scheme in NR, the network would indicate a list of “center frequency” in candidateServingFreqList with ARFCN value. The UE would then report the affected frequencies to the network. The network would move the affected NR frequency away from the non-3GPP frequencies.

However, the current IDC solution in NR has the following limitations

- The affected frequencies cannot be adequately indicated via the NR FDM solution

- TDM solution as in LTE is not available

In RAN#94e, the new work item for In-Device Co-existence (IDC) enhancements for NR and MR-DC was approved. In the WID, the following objective for FDM solution enhancement was agreed:

This WI expects to address interference between 3GPP (including various MR-DC architectures, i.e. NR-DC and EN-DC) and non-3GPP RAT (e.g. WiFi). - Enhancements to FDM solution, to allow more granular indication of affected frequencies (e.g. granularity of BWP or PRB level). (RAN2) Note: Enhancements to FDM solution is prioritized.

## Scenarios to be addressed IDC interference in Rel.18

Legacy FDM solution to address the IDC scenarios such as (i) adjacent channel interference between NR and non-3GPP technologies and (ii) Intermodulation Distortion (IMD) interference from simultaneous Tx in MR-DC to non-3GPP have already been supported in the current specification but it is too coarse as the reporting is at the NR carrier frequency level.

Contributions [1], [9]. [11] and [14] discusses the details of these scenarios which can be considered for the FDM enhancement in Rel.18.

### Scenario 1: Adjacent channel interference between NR and non-3GPP

In this scenario IDC interference will be caused by the NR Transmission (Tx) to Non-3GPP Reception (Rx) or by the Non-3GPP Transmission (Tx) to NR Reception (Rx) due to the operational NR bands are adjacent to the ISM bands. An example of this scenario is as shown in Figure 1 below.

Figure 1 - Example of adjacent channel interference between NR and non-3GPP

Depending on whether the NR node is operating in standalone mode or as MN of NR-DC or as SN of MR-DC, this scenario can be further categorized into following two sub scenarios

Scenario 1-1: Adjacent channel interference between NR Stand Alone (SA) or MN of NR-DC and non-3GPP

Scenario 1-2: Adjacent channel interference between SN (NR) of MR-DC and non-3GPP

Q1. Do you agree that Scenario 1-1: Adjacent channel interference between NR Stand Alone (SA) or MN of NR-DC and non-3GPP should be considered for the FDM enhancement in Rel.18?

| Company | Agree/Disagree | Comments (if any) |
| --- | --- | --- |
| Huawei, HiSilicon | - Agree | We think that the legacy FDM solution to address the Adjacent channel interference issue for the scenarios described above has been supported by the current specification, so we think that this scenarios should be considered for the FDM enhancement in Rel.18. |
| Xiaomi | - Agree, but | For NR SA and NR-DC, the non-serving frequency of LTE (which is configured for measurement purpose) also has the adjacent channel interference issue. Then the UE configured with NR should also be able to report the affected frequency of LTE. |
| Samsung | - Agree with comments | We agree scenario 1-1. On the other hand, in our contribution [6], the standalone case may contain another scenario, where the adjacent channel interference may be caused between frequency used at sidelink and non-3GPP. The reason is that the sidelink may use different power level from Uu link. The IDC interference may be different at the PC5 link and Uu link. For example, the Uu link may not be interfered by non-3GPP, while sidelink may be interfered due to high transmission power of Uu link. Thus, we suggest to consider IDC issue over sidelink as well, which can be considered as part of standalone case. |
| ZTE | - See comments | We agree with that the “Adjacent channel interference” shall be considered. But we don’t think we need to separate MN and SN at this stage, so no need to divide to 2 scenarios at this stage. |

Q2. Do you agree that Scenario 1-2: Adjacent channel interference between SN (NR) of MR-DC and non-3GPP should be considered for the FDM enhancement in Rel.18?

| Company | Agree/Disagree | Comments (if any) |
| --- | --- | --- |
| Huawei, HiSilicon | - Agree | Same reason as described in comment for Q1. |
| Xiaomi | - Agree, but | We think that one intention of Scenario 1-2 is to exclude NE-DC from MR-DC. This is aligned with the IDC WID. For MR-DC, the non-serving frequency of either LTE or NR can have adjacent channel interference. |
| Samsung | - Agree |  |
| ZTE | - Same comments as Q1 |  |

### Scenario 2: Intermodulation Distortion (IMD) interference from simultaneous Tx in MR-DC to non-3GPP

In this scenario the inter modulation distortion interference will be caused by the simultaneous transmission (Tx) in the uplink within MCG and SCG (including NR-DC and EN-DC case) to non-3GPP due to the combination of the operational bands used during MR-DC operation. An example of this scenario is as shown in Figure 2.

Figure 2 - Example of IMD interference from simultaneous Tx in MR-DC to non-3GPP

Depending on whether the MR-DC configuration is EN-DC or NR-DC, this scenario can also be further categorized into following two sub scenarios

Scenario 2-1: IMD interference from simultaneous Tx in EN-DC to non-3GPP

Scenario 2-2: IMD interference from simultaneous Tx in NR-DC to non-3GPP

Q3. Do you agree that Scenario 2-1: IMD interference from simultaneous Tx in EN-DC to non-3GPP should be considered for the FDM enhancement in Rel.18?

| Company | Agree/Disagree | Comments (if any) |
| --- | --- | --- |
| Huawei, HiSilicon | - Agree | We think that the legacy FDM solution to address the IMD interference issue for the scenarios described above has been supported by the current specification, so we think that this scenarios should be considered for the FDM enhancement in Rel.18. |
| Xiaomi | - Agree, but | As indicated above, we think that NE-DC is not part of the WID scope. For EN-DC, we could clarify that the simultaneous TX in EN-DC includes the following cases: - Case 1: UL CA within LTE MCG - Case 2: UL CA within NR SCG - Case 3: Multiple UL transmission across LTE MCG and NR SCG Case 1 and Case 3 should be the targeted scenarios for our Rel-18 FDM solution, since Case 2 only impacts the LTE frequency, which can reuse the legacy LTE FDM solution. Since we only include EN-DC and NR-DC, maybe we can clarify that NE-DC is not with the IDC WID scope. |
| Samsung | - Agree |  |
| ZTE | - See comments | We are a little confused with this scenario. Does it mean that the UE only report the EN-DC IMD interference when the UE is working at EN-DC state? Or does it mean that the UE would report IMC interference that cause by Eutra-NR BC. If for the latter case it seems that the network doesn’t care about the EN-DC or NE-DC, it can only see this IMD interference is caused by a BC with both Eutra and NR band. |

Q4. Do you agree that Scenario 2-2: IMD interference from simultaneous Tx in NR-DC to non-3GPP should be considered for the FDM enhancement in Rel.18?

| Company | Agree/Disagree | Comments (if any) |
| --- | --- | --- |
| Huawei, HiSilicon | - Agree | Same reason as described in comment for Q3. |
| Xiaomi | - Agree, but | For NR-DC, we could clarify that the simultaneous TX in EN-DC includes the following cases: - Case 1: UL CA within NR MCG - Case 2: UL CA within NR SCG - Case 3: Multiple UL transmission across MCG and SCG All three cases should be the target of the FDM solution |
| Samsung | - Agree |  |
| ZTE | - Agree |  |

## Principles of existing FDM Solution to be considered for R18 enhancement

In the existing FDM solution, the network would indicate a list of “center frequency”, e.g. candidateServingFreqList with ARFCN value.

IDC-AssistanceConfig-r16 ::= SEQUENCE { candidateServingFreqListNR-r16 CandidateServingFreqListNR-r16 OPTIONAL, -- Need M ...} CandidateServingFreqListNR-r16 ::= SEQUENCE (SIZE (1..maxFreqIDC-r16)) OF ARFCN-ValueNR

The the affected frequencies reported for IDC issue include both serving and non-serving frequencies as can be seen from the quoted text from 36.331 and 38.331.

36.331/38.331: NOTE 2: For the frequencies on which a serving cell or serving cells is configured that is activated, IDC problems consist of interference issues that the UE cannot solve by itself, during either active data exchange or upcoming data activity which is expected in up to a few hundred milliseconds. For frequencies on which a SCell or SCells is configured that is deactivated, reporting IDC problems indicates an anticipation that the activation of the SCell or SCells would result in interference issues that the UE would not be able to solve by itself. For a non-serving frequency, reporting IDC problems indicates an anticipation that if the non-serving frequency or frequencies became a serving frequency or serving frequencies then this would result in interference issues that the UE would not be able to solve by itself.

Some contributions discussed the issue whether the non-serving frequency should also be considered for the finer frequency granularity reporting [1], [5], [8], [9], [10], [11]

Q5. Do you agree that the granular indications of the affected NR frequency reported for IDC issue needs to consider both serving and non-serving frequency as in the legacy FDM solution?

| Company | Agree/Disagree | Comments (if any) |
| --- | --- | --- |
| Huawei, HiSilicon | - Agree | We think that reporting enhanced FDM information for both serving and non-serving frequencies can provide gNB more information about the ongoing IDC issue on the serving frequency as well as the potential issues that may arise on the non-serving frequencies. Such indications will provide gNB with flexibility and reduce latency if gNB decides to use those frequencies for the UE. |
| Ericsson | - Agree | We think this is already supported by current specs (both NR and LTE) and this approach should work also for the higher granularity indications. |
| Xiaomi | - Agree | We share the same view with Huawei and Ericsson. For non-serving frequency reporting, the intention is also to indicate the IDC issue on the measurement for non-serving frequency, since the IDC interference could cause inaccurate measurement result, which will impact the mobility and MDT. |
| Samsung | - Agree | Share the same view as Huawei and Ericsson |
| ZTE | - See comments | Considering the BWP is only configured for serving cells, the FDM enhancement can be discussed for the serving frequency and non-serving frequency separately. For the non-serving frequency, the related frequency info has been reported in the legacy signaling, so we are not sure whether some enhancement are necessary for the non-serving frequency. |

Q6. Do you agree that we should aim to have unified FDM solution with granular indications for both  serving and non-serving frequency in R18 for simplicity as in the legacy FDM solution?

| Company | Agree/Disagree | Comments (if any) |
| --- | --- | --- |
| Huawei, HiSilicon | - Agree | Yes, we should aim to have a common design for reporting both serving frequency and non-serving frequency for the FDM solution in R18 otherwise we may end up with different solutions for serving and no-serving frequency. |
| Ericsson | - Agree | To allow for the outcome of this IDC WI to have a chance of being implemented, it is important to keep the complexity down. To have one solution which can be applied both for serving and non-serving frequencies is less complex than two different solutions. |
| Xiaomi | - Agree, but | We think that it would be better to have unified solution for finer granularity of frequency information. However if most companies consider that some specific enhancements are needed (e.g. for serving frequency), we are also open to discuss. |
| Samsung | - Agree | The intention of this WI is to find a fine granularity of frequency information to solve the ongoing/potential IDC problem. Thus, such intention should be applicable for both serving and non-serving frequencies, and a unified solution can simplify our design. |
| ZTE | - Disagree | Same as above Considering the BWP is only configured for serving cells, the FDM enhancement can be discussed for the serving frequency and non-serving frequency separately. For the non-serving frequency, the related frequency info has been reported in the legacy signaling, so we are not sure whether some enhancement are necessary for the non-serving frequency. If go the the same solution, it seems that the BWP-based scheme has been precluded for that for the non-serving frequency there is no BWP. |

## Frequency Reporting Granularity

With the current FDM solution, it is already possible to send FDM-indication indicating problematic frequencies. The scope of the R18 IDC Enhancement includes that the FDM indications should be made more granular.

To make the FDM indications more granular the following options are proposed in the papers:

- Option 1: Central frequency + Bandwidth of the actual affected frequency range [5], [6], [9], [11],

- Option 2: Starting frequency + Ending frequency of the actual affected frequency range [5], [6]

- Option 2a: starting frequency + Bandwidth of the actual affected frequency range [6]

- Option 3: BWP-based reporting using BWP ID [1], [6], [7], [10], [12], [13]

- Option 4: BWP-based reporting using BWP ID + PRB index [6]. [7], [9]

- Option 5: Measurement object ID

- Option 6: Resource Block Group (RBG) based reporting [8]

Further Details about these options from the papers

For Option 1 and 2 - in addition to configuring the candidate serving frequency list (center frequency), the gNB can configure the candidate bandwidth for each of the candidate serving frequency. The UE can report the actual affected frequency range either in terms of Central frequency + Bandwidth [5], [11] or in terms of Starting frequency + Ending frequency [5]. Option 1 and 2 provides finer granularity than BWP based reporting in Option 3. Additionally, with option 1 and 2, unified reporting approach can be applied for both serving and non-serving frequencies.

For option 3 – It needs to be taken into consideration that currently BWP is only configured for serving cells, thus such FDM enhancement can be applied to the serving frequency only in its current form. If BWP based reporting is to be considered for non-serving frequencies then additional signaling (Location And Bandwidth, Subcarrier Spacing, offsetToCarrier) as described in [10] will be needed to configure candidate BWP IDs on the non-serving frequencies.

For option 4 – This is same as option 3, with additional affected PRBs reported within the BWP. This option will also need additional signaling as in option 3 if it to be extended for non-serving frequencies.

For option 5 - One NR measurement object can include two different frequencies (e.g. ssbFrequency for SSB measurement and refFreqCSI-RS for CSI-RS measurement), and the NR measurement object configuration is also not able to provide the affected frequency range. Using the NR measurement object ID to indicate the NR frequency is not be able to provide the accurate frequency indication for the IDC interference [5].

For option 6 – RBG has the benefit of less signaling overhead, and is also flexible in size [8]. However, it could be possibly only be applied for indicating the affected RBG within the serving cell

Based on the discussion above, companies are invited to indicate their preferred option(s) for frequency reporting granularity and may provide any additional details on how the preferred option could work for both serving and non-serving frequencies with an aim to have unified solution.

| Company | Preferred Option # | Any details on how their preferred option(s) could work for both serving and non-serving frequencies |
| --- | --- | --- |
| Huawei, HiSilicon | - Option 1 | Yes. We think that if the affected frequency is the UE’s serving frequency, then there is no problem for the UE to report the actually affected PRB or BWP. However, if the affected frequency is the UE’s non-serving frequency, the UE will not be able to report the actually affected PRB or BWP unless it is provided this information with additional signalling. To have a common design for both serving frequency and non-serving frequency, we prefer that the UE report the actually affected bandwidth and the central frequency of this bandwidth for each affected frequency as it is more granular than the BWP option and has low signalling overhead. |
| Ericsson | - 3 | We think that option 1, 2 and 3 are on paper very similar and main difference is semantics. One benefit of option 3 is simplicity: - The gNB configures all available BWPs as candidate BWPs for the UE (i.e. similar to how the gNB with the current FDM-solution configures candidate ARFCNs for the UE). - If the UE detects interference on candidate BWPs, the UE reports those BWPs. - gNB avoids that the UE uses the problematic BWPs. We consider option 1 and 2 to be optimizations. They do increase the granularity even further, but the question is if it will be used in reality? That increased granularity would only be useful if the gNB would be able to avoid that the UE is scheduled on, and avoids that the UE sends/receive signals on, the problematic PRBs of a BWP. We do not think this is likely to be implemented as it would be complex for the gNB to do so. So in our mind, option 1 and 2 are most likely just increasing the complexity of the solution without bringing any real gains. |
| Xiaomi | - Option 1 or Option 2 for NR frequency - Option 5 for LTE frequency | For NR frequency, Option 3/4/6 are not valid for non-serving frequency, as non-serving frequency does not have BWP configuration, PRB index and RGB information. Option 3/4/6 are also not valid when the NR frequency of SCG is reported to the LTE MN, as the LTE eNB does not have the BWP configuration, PRB index and RGB information of the NR frequency of SCG, and is not able to understand NR ASN.1. For Option 5, one measurement object of NR can include more than one frequencies, and then we would need to find another solution to indicate which frequency from NR measurement object is affected. For LTE frequency (which could be included for IMD issue or non-serving LTE frequency reporting via NR side), the legacy measurement object ID for LTE frequency can be reused. We understand that the question is only for the NR frequency. However it would be better to confirm that LTE frequency granularity for FDM is not changed. |
| Samsung | - Frequency range related options (1/2/2a), further down-selection in next meeting. | For the above options, we are open for discussion on options 1/2/2a/3/4 as long as a unified solution can be applied. For option 1, we need face a problem on how to indicate the range which is not around the central frequency. Option 2 has the spirit to indicate the exact range of frequency. To achieve such purpose, another choice is Option 2a. The difference may be on the signalling design and the corresponding signalling overhead. In our understanding, Option 2a can be achieved with less signalling overhead. For example, the bandwidth can be represented by percentage of the cell bandwidth. For option 3/4, we need resolve the issue on how to indicate the candidate BWPs on the non-serving frequency. Among those solutions, we slightly prefer to options on providing frequency range, i.e., option 1/2/2a, since it can provide more flexibility than BWP related solutions, i.e., option 3/4 |
| ZTE | - Option 3 | We prefer to adopt BWP scheme for the serving frequency. For the PRB based, our concern is on the CA/MR DC case, the UE would report Band combinations that has IMD interference for the CA/MR DC. With PRB, it means the UE has to report many PRB range combination, we think it’s too complex. |

# Conclusion

In summary, the following are proposed:

TBD

# References

[1] R2-2207162	Consideration on the FDM enhancement	ZTE Corporation, Sanechips

[2] R2-2207469	Discussion on FDM solution enhancements for IDC	OPPO

[3] R2-2207539	Discussion on FDM solution enhancements	Sharp

[4] R2-2207556	Assistance information for FDM	Nokia, Nokia Shanghai Bell

[5] R2-2207804	Discussion on the IDC FDM solutions	Xiaomi

[6] R2-2207844	Discussion on FDM solution for in-device co-existence interference avoidance	Samsung

[7] R2-2207936	Discussion on FDM solution in IDC	Apple

[8] R2-2207968	Enhanced FDM solution for IDC	Intel Corporation

[9] R2-2208116	FDM Solutions in IDC	Qualcomm Incorporated

[10] R2-2208135	 FDM solution for IDC	Ericsson

[11] R2-2208230	 Discussion on FDM enhancement	Huawei, HiSilicon

[12] R2-2208396	 Discussion on FDM solution for R18 IDC	vivo

[13] R2-2208524	 IDC FDM solution	LG Electronics

[14] R2-2207161	 Clarification on the IDC scope	ZTE Corporation, Sanechips