3GPP TSG-RAN WG2 Meeting #119bis electronic                                              	R2-2209572

Online, 10th - 19th October 2022

Source:	CATT

Title:	RACH enhancement for SON

Agenda Item:	8.13.6

Document for:	Discussion and Decision

Introduction

In last meeting, some agreements are made for the SgNB RACH report and RACH partitioning, respectively [1]:

Agreements

RACH report

1	RAN2 to discuss RACH partitioning for RACH report enhancements.

2	RAN2 is asked to discuss the support of (NG)EN-DC and NE-DC scenarios for SN RACH report. Only focus on the leftover issues for completing the whole work which partly done in R17 in RAN3. Draft LS to RAN3 ask for clarification. (Ericsson)

In this contribution, we analyse the appropriate MR-DC scenarios and the solution for SgNB RACH report, and also the issues on RACH report for RACH partitioning.

Discussion

SgNB RACH report for MR-DC scenarios

RAN2 has reached an agreement that only focus on the leftover issues for completing the whole work which partly done in R17 in RAN3. An LS to RAN3 [2] is also sent to RAN3 to ask the support of (NG)EN-DC and NE-DC scenarios for clarification.

To check with RAN3 specification and the discussion in R17, we think it is clear that:

- For NR-DC and NG-EN-DC cases, an IE of UE Assistant Identifier is introduced in R17 TS38.423 to support more UE context capture by the SN node of the dual connection, besides the legacy IE of RACH Report Container (introduced for NR single connection RACH report retrieve in R16) in the ACCESS AND MOBILITY INDICATION message;

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| RACH Report List |  | 0..1 |  |  | YES | ignore |
| >RACH Report List Item |  | 1 .. <maxnoofRACHReports> |  |  | EACH | ignore |
| >>RACH Report Container | O |  | OCTET STRING | RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [10]. | YES | ignore |
| >>UE Assistant Identifier | O |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | YES | ignore |

- For EN-DC case, the IE of UE Assistant Identifier and the IE of RACH Report Container are both introduced within the new introduced ACCESS AND MOBILITY INDICATION message in R17 TS36.423.

Therefore, NE-DC scenario is not in the scope of RAN3 RACH report optimization consideration. To fulfill the RAN3 SgNB RACH report requirement, and to align the RAN2 and RAN3 procedures, it is suggest supporting the SgNB RACH report of (NG)EN-DC scenario in R18.

Proposal 1: Study and support the SgNB RACH report of EN-DC and NG-EN-DC scenarios in R18.

For the EN-DC and NG-EN-DC scenarios, in RAN2 the same UE behaviour will be performed for UE RACH report in the air-interface which only impact TS36.331.

Current RACH Report in TS36.331 only supports simple parameters report such as the number of preambles and whether the contention is detected (For normal UE).

RACH-Report-r16 ::=					SEQUENCE {

numberOfPreamblesSent-r16			NumberOfPreamblesSent-r11,

contentionDetected-r16				BOOLEAN

}

RACH-Report-v1610 ::=	SEQUENCE {

initialCEL-r16 					INTEGER (0..3),

edt-Fallback-r16					BOOLEAN

}

To support the SgNB RACH information and report to the MN, the UE should include the NR SgNB RACH information in LTE specification, the format could be an NR container. Two detailed options could be considered:

- MN(LTE) could be enhanced to decode the NR container to get the target cell ID, and then send the RACH information to the correct SgNB (in which the RACH occurred);

- UE send the cell ID in LTE format together with the NR container to the MN. The MN does not need to decode the NR container, and send the container to the correct SgNB based on the cell ID. This option needs UE to report multiple containers for different SgNBs/cells respectively.

Compare with the 2 options, it is simpler to use option2 to assist the MN sending the NR container to the correct  SN node which does not need the updated MN to decode the NR container [3].

Proposal 2: Includes NR container in LTE RACH Report to enhance the SgNB UE RACH Report for EN-DC and NG-EN-DC scenarios.

Proposal 3: Includes the cell ID in which the RACH occurred in LTE format together with the NR container to assist the network forwarding the SgNB RACH report content for EN-DC and NG-EN-DC scenarios.

Since in LTE, the RA report is almost an immediate report after UE has connected to a cell, the is no variable about RA report. Therefore to support the SgNB RA report mechanism, an additional Variable should be introduced into TS36.331.

Proposal 4: A new UE variable about SgNB RACH report information should be introduced in TS36.331.

For NR-DC scenario, there is no additional UE capability introduced for the SgNB RACH report since the RACH information of MN and SN use the same encode format and share the same mechanism and the UE variable. But for EN-DC and NG-EN-DC scenarios, if the NR container is introduced, additional UE capability may be needed to identify whether the inter-RAT record and report of RACH information could be supported.

Proposal 5: Additional capability may be needed for NR RACH Report enhancement in LTE for EN-DC and NG-EN-DC scenarios.

RACH report about RACH partitioning information

Background

In Rel-17, In order to enable early identification of the RACH feature or RACH feature combination on the network side, RACH partitioning was introduced in Rel-18.

- For RedCap, the UE can indicate the reduced capabilities to the network;

- For SDT, the UE can indicate the SDT transmission to the network, so that the network can perform suitable scheduling for the subsequent transmission;

- For coverage enhancement, the UE can indicate the network that the channel condition is poor. So that, MSG3 repetition can be performed;

- For Slicing, RACH isolation can be achieved and the UE can indicate that the traffic for specific NSAG (Network Slice AS Group) arrives;

- Some features can also be combined, for example, coverage enhancement and slicing. So that the network can identify the feature combinations.

Generally, the basic MAC procedure for RACH partitioning can be summarized as [4]:

- Step 1: RRC indicates to MAC whether SDT, Redcap, Slice group X is applicable for any RACH;

- Step 2: MAC selects the carrier, i.e. SUL or NUL;

- Step 3: MAC selects the BWP;

- Step 4: MAC determines CE applicability, i.e. MAC check whether the RSRP is below the configured RSRP threshold;

- Step 5: MAC selects the RACH partition.

RACH partitioning in MDT

To enable better utilization of the RACH resources, it is necessary for the UE report the triggered RACH feature to the network. For example, the UE can report the RACH feature or feature combination to the network. Hence, the network can reassign the RACH resources based on the triggered RACH feature/feature combination.

Proposal 6: The UE indicates the feature /feature combination to the network to facilitate the RACH resource utilization.

It may happen that multiple RACH features (feature combinations) are available or there is no RACH resource corresponding to the available RACH feature/feature combination for the UE. In order to handle these issues, RACH feature prioritization, i.e. featurePriorities is defined. The UE can select the feature with higher feature priority or feature combination which includes the feature configured with higher feature priority. So it may happen that the triggered RACH feature/feature combination is not the same as the available RACH feature/feature combination for the UE. For example, when the UE triggers NSAG1 and CE, but there is no RACH resource for NSAG 1+ CE. Hence, the UE can compare the feature priority of NSAG1 and CE. Assuming the CE has higher priority, the UE selects the RACH resource of CE. But this does not corresponding to the actual available RACH feature/feature combination, i.e. NSAG1+CE. So RAN2 needs to discuss whether and how to address this issue.

Observation 1: The RACH feature/feature combination which is selected by the UE may not be same as the RACH feature/feature combination that is available for the UE.

Proposal 7: RAN2 to study whether and how to address the issue in above observation 1.

Conclusion

According to the analysis in section 2, we propose:

For SgNB RACH report for MR-DC

Proposal 1: Study and support the SgNB RACH report of EN-DC and NG-EN-DC scenarios in R18.

Proposal 2: Includes NR container in LTE RACH Report to enhance the SgNB UE RACH Report for EN-DC and NG-EN-DC scenarios.

Proposal 3: Includes the cell ID in which the RACH occurred in LTE format together with the NR container to assist the network forwarding the SgNB RACH report content for EN-DC and NG-EN-DC scenarios.

Proposal 4: A new UE variable about SgNB RACH report information should be introduced in TS36.331.

Proposal 5: Additional capability may be needed for NR RACH Report enhancement in LTE for EN-DC and NG-EN-DC scenarios.

For RACH report about RACH partitioning

Proposal 6: The UE indicates the feature /feature combination to the network to facilitate the RACH resource utilization.

Observation 1: The RACH feature/feature combination which is selected by the UE may not be same as the RACH feature/feature combination that is available for the UE.

Proposal 7: RAN2 to study whether and how to address the issue in above observation 1.

Reference

- RAN2#119-e Meeting Report-V1.docx

- R2-2209024 LS on SN RACH report status in R17 RAN2

- R2-2200394 Specification Impact of SgNB RACH Report on TS38.331 and TS36.331

- TS38.321 Medium Access Control (MAC) protocol specification