3GPP TSG-RAN WG2 Meeting #120		R2-2211195

Toulouse, France, November 14-18, 2022

Agenda Item:	8.4.2.1.2

Source: 	MediaTek Inc.

Title:  	Procedures of L1/L2-triggered Mobility

Document for:	Discussion and decision

# Introduction

In previous RAN2 meetings, we analysed the key components contributing to mobility latency in L1/L2-triggered mobility (LTM). A baseline time chart (before any enhancements) was captured in the endorsed running Stage-2 CR [1].

Figure 1.	Baseline time chart for LTM

As for other mobility procedures, the overall procedure of LTM needs to be captured in Stage-2 specifications. In this contribution, we provide draft for overall procedure of LTM, based on the above time chart. Moreover, we discuss potential procedure enhancements specifically for inter-DU LTM.

# Discussion

## Overall procedure of LTM

From the time chart, we observe that even with L1/L2 triggering, inter-cell mobility is not as fast as intra-cell beam switch if UE needs to perform reconfiguration and DL/UL synchronization towards target cell.

Observation 1: Even with L1/L2 triggering, inter-cell mobility is not as fast as intra-cell beam switch if UE needs to perform reconfiguration and DL/UL synchronization towards target cell.

In LTM, to allow UE to quickly switch to target cell once it receives cell switch command, UE may perform synchronization to some potential target cells earlier (before cell switch command). Our understanding is that LTM-capable UE should at least support DL synchronization towards a different cell, and UL synchronization is also possible if UE hardware supports. Although this may require further study in RAN1 and RAN4, we suggest that such “early synchronization” be considered in the overall procedure of LTM.

Proposal 1: Early synchronization towards target cell is specified in the LTM procedure.

Taking inter-DU operation as an example, the overall procedure of L1/L2-basedinter-cell mobility is illustrated in the figure below.

Figure 2.	Overall procedure of LTM

LTM in above figure consists of the following major parts (some steps among CU, source DU, and target DU can be skipped in case of intra-DU. It is left to RAN3 to consider the detailed procedures and signalling to support inter-DU inter-cell mobility):

Pre-configuration

- (Step 1 to 2) UE sends L3 measurement report to gNB-DU based on measurement configurations. The measurement report is forwarded to gNB-CU.

- (Step 3 to 8) The gNB-CU decides the candidate set for UE, sends a preparation request to target DU and receives acknowledgement from target DU (This may be implemented by existing signalling, e.g., UE context setup request/ response). Then gNB-CU sends the RRC reconfiguration to UE and receives complete message, via source gNB-DU.

Early synchronization

- (Step 9 to 11) UE performs L1 measurements and reports for reference signals (SSB or CSI-RS) corresponding to inter-cell beams, following configurations from network. Based on L1 measurement reports, network may activate some TCI states QCL-ed with cells whose PCI is different from serving cell. UE performs DL synchronization (DL and optionally UL) for these cells.

Cell switch

- (Step 12 to 13) Based on further L1 report, gNB-DU may indicate target cell and beam (TCI state). UE applies target cell configurations.

- (Step 14) If TA is not available, UE performs RACH towards the indicated target cell.

- (Step 15 to 16) UE receives PDCCH from target cell using new TCI state.

The detailed procedures may be further elaborated during the WI. The intention here is to provide the full picture of the overall procedure and identify the key enablers to support LTM, i.e., pre-configuration of candidate cell(s), early synchronization, and fast cell switch by L1/L2 signalling.

Proposal 2: The basic LTM procedure is comprised of pre-configuration of candidate cell(s), early synchronization towards the candidate cell(s), and fast cell switch by L1/L2 signalling.

The major steps shown here can be considered as a baseline, starting from which, we can study the design of each step to achieve the goal of fast cell switching.

## Inter-DU operation

In RAN2#119 meeting, we confirmed the support of LTM for inter-DU scenario. Furthermore, the design for intra-DU and inter-DU L1/L2 mobility should share as much commonality as reasonable. But it’s for sure that there are inter-DU specific issues, which need to be addressed.

The main aspect needs to be considered for inter-DU mobility is UP (RLC/MAC) handling and the coordination among CU, source DU and target DU over F1 interface.  However, the impact of additional latency due to UP reset and cell switch preparation delay over F1 interface to the mobility performance is marginal. The mobility performance of LTM for inter-DU is comparable to intra-DU.

In legacy handover mechanism, the handover decision is made in a relatively stable manner based on the RRM measurements/reports.  The network tries to balance the mobility performance of ping-pong effect and HOF rate by adjusting TTT value and thresholds of the triggering events. However, the LTM mechanism is targeting to fast cell switch based on L1 measurements/report to reduce the mobility latency and improve the mobility reliability. The ping-pong rate is much higher than legacy handover mechanism.

If inter-DU mobility also wants to obtain the similar benefits as intra-DU mobility, it also needs to rely on L1 measurements/reports to make mobility decision and take advantages of ping-pong effect instead of minimizing it. But the drawback is increased interaction/signalling overhead over F1 interface due to the high ping-pong rate.

RAN2 should discuss how to handle the ping-pong effects for inter-DU mobility. The opposite option is to minimize the ping-pong rates as legacy handover procedure by setting longer TTT or introducing additional L1-RSRP threshold for cell switch, similar as legacy handover procedure. However, the benefits obtained by LTM mechanism will shrink dramatically. The mobility reliability will degrade, and mobility latency will increase.

Proposal 3: RAN2 discuss how to handle the ping-pong effects for inter-DU mobility:

- Option 1: Minimize ping-pong rates/longer TOS to reduce the interactions between CU and DUs.

- Option 2: Take advantage of ping-pong effect/shorter TOS to improve the mobility reliability.

Compared with intra-DU mobility, the major extra procedures for inter-DU mobility are RLC reestablishment and MAC reset.  The frequent RLC re-establishment will result in more packet loss and longer packet delivery latency just as illustrated in Figure 3.

- Packet loss for RLC-UM bearer: PDCP PDUs which are not successfully transmitted/received to/from source cell are discarded.

- Longer packet delivery latency for RLC-AM bearers: PDCP PDUs which are not successfully transmitted/received to/from the source cell are retransmitted/re-received to/from the target cell.

This is illustrated below.

Figure 3.	Pack loss in RLC-UM and longer latency in RLC-AM

Considering Rel-18 L1/L2-based mobility targets more frequent cell switching (e.g., back-and-forth among cells in a region), UE experience degradation due to more packet loss or longer packet delivery latency is more severe. Therefore, RAN2 should discuss potential enhancements to mitigate packet loss and latency problems due to RLC reestablishment for L1/L2-based inter-DU mobility.

One possible way to minimize packet data loss for RLC-UM bearer is to support PDCP packet retransmission when cell switch occurs, similar as RLC-AM bearer.  Considering PDCP status report has already been supported in Rel-16 DAPS handover upon uplink data switching, it can also be extended to support inter-DU mobility in Rel-18.

Proposal 4: Support PDCP status report and PDCP packet retransmission for UM DRBs to reduce the packet loss for inter-DU LTM.

To support fast cell switch, there are two possible methods to support L1/L2-based inter-DU mobility from UP protocol point of view, i.e., single-protocol or dual-protocols, as illustrated in Figure 4.

RLC

MAC

CU/PDCP

PDCP

Source DU

UE

F1

MAC

RLC

CU/PDCP

PDCP

Target DU

UE

RLC

MAC

F1

MAC

RLC

RLC

MAC

CU/PDCP

PDCP

Source DU

UE

F1

RLC

MAC

F1

MAC

RLC

MAC

RLC

RLC

MAC

CU/PDCP

PDCP

UE

F1

RLC

MAC

F1

MAC

RLC

MAC

RLC

Target DU

Source DU

Target DU

a) Single-protocol                                                                       b) Dual-protocol

Figure 4.	Single-protocol vs. Dual-Protocol for inter-DU mobility

The two methods are further explained below.

- Single Protocol: From UE side, single protocol of one MAC entity and one set of RLC entities is associated with the serving cell. Before cell switch, the single protocol is associated with the source cell/DU and is associated with the target cell/DU after cell switching.

- Dual Protocols: Frome UE side, dual protocols, each protocol comprising one MAC entity and one set of RLC entities, are associated with the source cell/DU and target cell/DU respectively. Only one single protocol is actively in use. Before cell switching, the protocol associated to the source cell/DU is in use; after cell switching, the protocol associated to the target cell/DU is in use.

The single protocol method is considered as the basic solution for inter-DU mobility. MAC reset and RLC re-establishment are always required for cell switching. The issue of frequent packet loss can’t be avoided. Further optimization should be considered, just as mentioned Proposal 6. The dual protocol method is considered as a further optimization, which can further reduce the mobility latency. In dual protocol, the MAC entity and the RLC entities can be prepared upon reception of the reconfiguration message. UE can use the protocol associated to the target cell immediately upon reception of the cell switch command. Furthermore, the architecture of dual protocol is the same as split bearer. It allows that possibility to avoid RLC re-establishment and MAC reset when UE is switched back and forth between the source cell and the target cell. Considering the UP process for dual protocol is similar as split-bearer, there is not additional requirement on UE capability. UE which supporting split-bearer can support dual protocol directly.

Proposal 5: RAN2 consider both the solutions of single protocol and dual protocols for inter-DU LTM.

- Single protocol: single protocol of one MAC entity and one set of RLC entities is associated to the serving cell from UE aspect.

- Dual Protocols: dual protocols with each protocol comprising one MAC entity and one set of RLC entities are associated to the source cell/DU and target cell/DU respectively from UE aspect.

# Conclusion

We have the following observations:

Observation 1: Even with L1/L2 triggering, inter-cell mobility is not as fast as intra-cell beam switch if UE needs to perform reconfiguration and DL/UL synchronization towards target cell.

It is proposed to discuss and decide on the following proposals:

Overall procedure

Proposal 1: Early synchronization towards target cell is specified in the LTM procedure.

Proposal 2: The basic LTM procedure is comprised of pre-configuration of candidate cell(s), early synchronization towards the candidate cell(s), and fast cell switch by L1/L2 signalling.

Inter-DU operation

Proposal 3: RAN2 discuss how to handle the ping-pong effects for inter-DU mobility:

- Option 1: Minimize ping-pong rates/longer TOS to reduce the interactions between CU and DUs.

- Option 2: Take advantage of ping-pong effect/shorter TOS to improve the mobility reliability.

Proposal 4: Support PDCP status report and PDCP packet retransmission for UM DRBs to reduce the packet loss for inter-DU LTM.

Proposal 5: RAN2 consider both the solutions of single protocol and dual protocols for inter-DU LTM.

- Single protocol: single protocol of one MAC entity and one set of RLC entities is associated to the serving cell from UE aspect.

- Dual Protocols: dual protocols with each protocol comprising one MAC entity and one set of RLC entities are associated to the source cell/DU and target cell/DU respectively from UE aspect.

# Reference

- R2-2209255, 38.300 running CR for introduction of NR further mobility enhancements, MediaTek Inc, Aug. 2022