3GPP TSG-RAN WG2 Meeting #120		R2-2211196

Toulouse, France, November 14-18, 2022

Agenda Item:	8.4.2.2

Source: 	MediaTek Inc.

Title:  	RRC Aspects of L1/L2-triggered Mobility

Document for:	Discussion and decision

# Introduction

In last meeting, we discussed RRC aspects for L1/L2-triggered mobility (LTM) and made the following agreements and assumptions.

- General A L1/L2 inter-cell mobility candidate (target) configuration is received within an RRC message before the L1/L2 dynamic switch is triggered. - RRC model of candidates RAN2 continues the discussion on the RRC models by focusing on Model 1 and Model 2 and stage-3 details. - a. Model 1: One RRCReconfiguration message (or FFS RRCReconfiguration IEs) for each candidate target configuration - b. Model 2: One CellGroupConfig IE (FFS additional IEs) for each candidate target configuration RAN2 assumes that sequential L1L2 cell change between Candidates without RRC reconfiguration can be supported. RAN2 assumes that candidate cell configuration can only be modified / released by Network (FFS later whether some optimization should be applied e.g. for release). For L1L2 mobility will support that candidate configurations are delta configurations on top of a reference configuration. FFS if the reference configuration is a separate reference configuration or e.g. the current configuration. For L1L2 mobility, Target Pcell/SCell can be current SCell/PCell, i.e., current SCell/PCell can be configured as candidates.

# Discussion

## Reference configuration and subsequent LTM

In both CHO and LTM, candidates are pre-configured, and UE receives and stores multiple configurations. In CHO, to reduce signalling overhead, these configurations are usually delta configurations on top of current RRC configurations. When one CHO candidate is chosen, its configuration is applied, and other candidates are released because they are no longer applicable after the reconfiguration. Therefore, ‘subsequent CHO’ without RRC reconfiguration is not supported.

Observation 1: In CHO, other candidates are released upon CHO execution, and thus subsequent CHO without RRC reconfiguration is not supported.

Compared to normally “slow” cell change in legacy RRC-based handover, in high frequency, we expect that a UE switches between beams in an area and the beams may belong to different cells. If CHO procedure is used, candidates are configured and released frequently. The following figure shows CHO configuration and releasing for a UE moving among three cells. Each ‘+’ means candidate configured, and each ‘-’ means candidate released.

Figure 1.	Candidate configuration and releasing in CHO

In the last meeting, RAN2 made an important agreement that for LTM, we will support that candidate configurations are delta configurations on top of a reference configuration (FFS if the reference configuration is a separate reference configuration or the current configuration). Moreover, RAN2 assumes that sequential L1/L2 cell change between candidates (subsequent LTM) without RRC reconfiguration can be supported. Subsequent LTM can be naturally supported with the “reference plus delta” candidate configuration, since the UE needs not to “fallback” to original configuration and then apply the delta configuration.

Observation 2: Subsequent LTM can be naturally supported with the “reference plus delta” candidate configuration.

We propose to confirm the assumption to support subsequent LTM (i.e., sequential L1/L2 cell change).

Proposal 1: RAN2 confirm to support sequential LTM between candidates without RRC reconfiguration.

The above RAN2 progress implies that a different candidate configuration from CHO is needed in LTM.

The “reference plus delta” configuration implies a scenario where the reference configuration serves as a kind of “anchor”. The reference configuration can be modified by RRC reconfiguration. In the RRC reconfiguration, the network may also add or release candidate configurations, or modify existing candidate configurations so that they become delta on top of the latest reference configuration.

Proposal 2: The reference configuration in LTM can be modified by RRC reconfiguration.

## RRC modelling options

In last meeting, two RRC models for candidate configuration were left for further discussions:

- Model 1: One RRCReconfiguration message (or FFS RRCReconfiguration IEs) for each candidate target configuration

- Model 2: One CellGroupConfig IE (FFS additional IEs) for each candidate target configuration

We now discuss the details for each model. We provide examples of ASN.1 structure and RRC procedures by considering:

- How to provide “reference” and “delta” configurations

- How to support subsequent LTM

Model 1: RRCReconfiguration

According to the discussion in [1], common understanding is that Model 1 provides full flexibility since everything can be reconfigured. There were concerns about the signalling overhead, but in fact if we limit the IEs that can be changed, the overhead should be reasonable.

Considering the complexity of RRCReconfiguration, we suggest that the reference configuration be the RRC configuration, excluding the part of candidate configuration, and the delta configuration for a candidate is carried by a RRCReconfiguration in container (a structure similar to CHO).

We have the following proposal:

Proposal 3: If LTM candidates are modeled as RRCReconfiguration, the RRC configuration excluding LTM candidate configuration is considered as the reference configuration for LTM, and the delta configuration for a candidate is carried by a RRCReconfiguration in the container.

By imitating the RRC structure for conditional reconfigurations, an ASN.1 example (modified from [1]) is provided below.

RRCReconfiguration-IEs ::=              SEQUENCE {

radioBearerConfig                       RadioBearerConfig                                                      OPTIONAL, -- Need M

masterCellGroup                         OCTET STRING (CONTAINING CellGroupConfig)                              OPTIONAL, -- Cond SCG

measConfig                              MeasConfig                                                             OPTIONAL, -- Need M

lateNonCriticalExtension                OCTET STRING                                                           OPTIONAL,

nonCriticalExtension                    RRCReconfiguration-v1530-Ies                                           OPTIONAL

}

RRCReconfiguration-vXXXX-Ies ::=        SEQUENCE {

candLTM-Reconfiguration-r18             CandLTM-Reconfiguration                                                OPTIONAL,

lateNonCriticalExtension                OCTET STRING                                                           OPTIONAL,

nonCriticalExtension                    RRCReconfiguration-v1530-Ies                                           OPTIONAL

}

CandLTM-Reconfiguration-r18 ::=   SEQUENCE {

candLTM-ReconfigToRemoveList-r18         CandLTM-ReconfigToRemoveList-r18   OPTIONAL,   -- Need N

candLTM-ReconfigToAddModList-r18         CandLTM-ReconfigToAddModList-r18   OPTIONAL,   -- Need N

…

}

CandLTM-ToAddModList-r18 ::= SEQUENCE (SIZE (1.. maxNrofL1L2Candidates-r18)) OF CandLTM-ToAddMod-r18

CandLTM-ToAddMod-r18 ::=     SEQUENCE {

candLTM-ReconfigId-r18         CandLTM-RRCConfigId-r18,

candLTM-RRCReconfig-r18       	OCTET STRING (CONTAINING RRCReconfiguration)          OPTIONAL,

}

Figure 2. Example of configuring a LTM candidate target configuration with RRCReconfiguration

The RRC procedure for LTM, however, may be quite different from that of CHO execution. If subsequent LTM is to be supported, UE must store the reference configuration when receiving RRCReconfiguration carrying LTM candidates and apply candidate configuration on top of the reference configuration upon LTM cell switch. As illustrated in the figure below.

Figure 3.	Reference and delta configurations with LTM candidate modelled as RRCReconfiguration

Example procedural text for LTM with candidate modelled as RRCReconfiguration is as follows.

##### 5.3.5.x.a LTM candidate reconfiguration removal The UE shall: 1> for each candLTM-ReconfigId value included in the candLTM-ReconfigToRemoveList that is part of the current UE candidate configuration in VarCandLTM-Reconfig: 2> remove the entry with the matching candLTM-ReconfigId from the VarCandLTM-Reconfig; ##### 5.3.5.x.b L1/L2-mobility candidate reconfiguration addition/modification Upon receiving RRCReconfiguration containing candLTM-Reconfiguration, the UE shall: 1> store current RRC configuration, excluding candLTM-Reconfiguration, as the reference configuration in LTM. For each candLTM-ReconfigId received in the candLTM-ReconfigToAddModList IE the UE shall: 1> if an entry with the matching candLTM-ReconfigId exists in the candLTM-ReconfigToAddModList within the VarCandLTM-Reconfig: 2> replace candLTM-Reconfig within the VarCandLTM-Reconfig with the value received for this candLTM-ReconfigId; 1> else: 2> add a new entry for this candLTM-ReconfigId within the VarCandLTM-Reconfig; … ##### 5.3.5.x.c L1/L2-mobility candidate reconfiguration execution The UE shall: 1> for the indicated cell of LTM execution: 2> apply the stored candLTM-RRCReconfig of the selected cell on top of reference RRCReconfiguration, and perform the actions as specified in 5.3.5.3; …

Model 2: CellGroupConfig

If LTM candidate configuration is provided at cell-group level, the UE may be configured with one MCG [SCG] configuration, plus other MCG [SCG] configurations for candidates in the container for LTM candidates. In this case, the reference configuration would be the masterCellGroup or secondaryCellGroup. The “delta configuration” for a candidate can be modelled as another cellGroupConig in a container. The concept is illustrated below.

Figure 4.	LTM candidates as CellGroupConfig

We have the following proposals:

Proposal 4: If LTM candidate configuration is provided at cell-group level, the reference configuration would be the masterCellGroup or secondaryCellGroup. The “delta configuration” for a candidate can be modelled as another cellGroupConig in container.

Notice that if candidates are modelled at CellGroupConfig level, we may need to study the feasibility of delta configuration on top of a cellGroupConfig; if infeasible, a full configuration (at cell-group level) will be needed.

An ASN.1 example is provided below.

RRCReconfiguration-IEs ::=              SEQUENCE {

radioBearerConfig                       RadioBearerConfig                                                      OPTIONAL, -- Need M

masterCellGroup                         OCTET STRING (CONTAINING CellGroupConfig)                              OPTIONAL, -- Cond SCG

measConfig                              MeasConfig                                                             OPTIONAL, -- Need M

lateNonCriticalExtension                OCTET STRING                                                           OPTIONAL,

nonCriticalExtension                    RRCReconfiguration-v1530-Ies                                           OPTIONAL

}

RRCReconfiguration-vXXXX-Ies ::=              SEQUENCE {

candLTM-Reconfiguration-r18              CandLTM-Reconfiguration                                                 OPTIONAL,

lateNonCriticalExtension                OCTET STRING                                                           OPTIONAL,

nonCriticalExtension                    RRCReconfiguration-v1530-Ies                                           OPTIONAL

}

CandLTM-Reconfiguration-r18 ::=   SEQUENCE {

candLTM-ReconfigToRemoveList-r18         CandLTM-ReconfigToRemoveList-r18   OPTIONAL,   -- Need N

candLTM-ReconfigToAddModList-r18         CandLTM-ReconfigToAddModList-r18   OPTIONAL,   -- Need N

…

}

CandLTM-ToAddModList-r18 ::= SEQUENCE (SIZE (1.. maxNrofL1L2Candidates-r18)) OF CandLTM-ToAddMod-r18

CandLTM-ToAddMod-r18 ::=     SEQUENCE {

candLTM-ReconfigId-r18                CandLTM-RRCConfigId-r18,

candLTM-CellGroupConfig-r18       	OCTET STRING (CONTAINING CellGroupConfig)          OPTIONAL,

}

Figure 5. Example of candidate CellGroupConfig with reference configuration

The corresponding RRC procedure for L1/L2-mobility execution is as follows, where we assume that configuration ID 0 is reserved for the reference configuration (e.g., masterCellGroup)

##### 5.3.5.x.y LTM execution The UE shall: 1> if the [config ID] is zero: 2> restore the masterCellGroup or [secondaryCellGroup]; 1> else: 2> for the selected configuration for LTM, apply the corresponding CellGroupConfig; …

Before choosing the RRC model (Model 1 or 2), RAN2 should discuss if there is any configuration outside CellGroupConfig that needs to be included in LTM candidate configuration. For example,

- PDCP recovery may be needed for inter-DU LTM, but recoverPDCP is in RadioBearerConfig

- Network may want to update measurement configurations when UE moves to another cell, but measConfig is outside CellGroupConfig.

If any configuration outside CellGroupConfig needs to be provided for a candidate, we may either adopt Model 1, or include the extra configuration in the candidate config (e.g., under CandLTM-ToAddMod in above ASN.1 example).

Proposal 5: Before choosing the RRC model, RAN2 should discuss what configurations outside CellGroupConfig (including but not limited to recoverPDCP and measConfig) need to be included in LTM candidate configuration.

# Conclusion

We have the following observations:

Observation 1: In CHO, other candidates are released upon CHO execution, and thus subsequent CHO without RRC reconfiguration is not supported.

Observation 2: Subsequent LTM can be naturally supported with the “reference plus delta” candidate configuration.

It is proposed to discuss and decide on the following proposals:

Proposal 1: RAN2 to confirm the support sequential LTM between candidates without RRC reconfiguration.

Proposal 2: The reference configuration in LTM can be modified by RRC reconfiguration.

Proposal 3: If LTM candidate configurations are modeled as RRCReconfiguration, the RRC configuration excluding LTM candidate configuration is considered as the reference configuration for LTM, and the delta configuration for a candidate is carried by a RRCReconfiguration in container.

Proposal 4: If LTM candidate configuration is provided at cell-group level, the reference configuration would be the masterCellGroup or secondaryCellGroup. The “delta configuration” for a candidate can be modelled as another cellGroupConig in container.

Proposal 5: Before choosing the RRC model, RAN2 should discuss what configurations outside CellGroupConfig (including but not limited to recoverPDCP and measConfig) need to be included in LTM candidate configuration.

# Reference

- R2-2210329, [Post119-e][048][feMob] Candidate target configurations for L1/L2 mobility, Ericsson, Oct 2022