3GPP TSG-RAN WG2 #120	R2-2211219

Toulouse, France, November 2022

Agenda Item:	5.1.3.3

Source:	OPPO

Title:	Discussion on intra-band ENDC

Document for:	Discussion, Decision

Introduction

This is to discuss the left issue on intra-band ENDC.

RAN2 concludes that the discussed cases are not currently supported by signalling and new signalling is needed.

Case validity is up to RAN4, and if RAN4 concludes they are valid, RAN2 can then attempt to find a signalling solution. RAN4 can also develop a preference as to what release should be applicable.

Discussion

R4 has confirmed the validity of the following cases.

- Case 3: All CCs are contiguous in DL but neither carrier is contiguous to each other in UL:

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_(n)41AB DC_(n)41CA DC_(n)41DA | DC_41A_n41A |
| DC_(n)48CA | DC_48A_n48A |
| DC_(n)48DA | DC_48A_n48A |

- Case 4: LTE and NR adjacent carriers are contiguous but carriers in LTE or NR are non-contiguous, it will have two kinds of UL ENDC configurations:

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_48A_(n)48AA | DC_(n)48AA DC_48A_n48A |

For case-3, R4 is still struggling on the solution for solving this within R16 and R17.

- FFS following solutions in next meeting

  - Option 2b: In Rel-16 and Rel-17, report an additional band combination DC_48A_n48A in both UL and DL to support the Case 3 configurations DL DC_(n)48CA with UL DC_48A_n48A and DL DC_(n)48DA with UL DC_48A_n48A, i.e.

UE indicate “contiguous” capability for DL DC_(n)48CA with UL DC_(n)48AA

UE additionally indicate “non-contiguous” capability for DC_48A_n48A in both UL and DL

    - Interpretation A:

Case 3 is considered as exceptional configuration with DC_48A_n48A in UL and DC_(n)48CA in DL, and a NOTE can be added in RAN4 spec to clarify this.

    - Interpretation B:

DL DC_48A_n48A is not considered as a fallback combination of DC_(n)48CA, but only a lower order combination of DC_(n)48CA.

  - Option 2c: In Rel-16 and Rel-17, UE shall also support non-contiguous operation in the DL (DC_48C_n48A), then the network can configure DL_(n)48CA with the middle LTE cell DL-only and the UL with a gap (non-contiguous)

Although there is an AP left to R2

- Consider design new signalling in Rel-18 for Case3 if necessary and detailed signalling design is up to RAN2. And rely existing signalling to indicate Case 3 before Rel-18.

After R4 has concluded on R16/17 solution, it would impact R18, i.e., if the R16/17 solution can be applied to R18 without introducing new signaling, it should be used directly without further R2 invention on signaling.

- R2 wait for R4 progress on R16/17 solution for case-3 of intra-band ENDC issue before working on R18 solution.

For case-4, there is no AP for R2

- FFS following solutions in next meeting

- Option 2b: Rel-16 and 17 combinations of contiguous and non-contiguous intra-band EN-DC should be limited to two sub-blocks one of which consists of a contiguous EN-DC configuration in table Table 5.3B.0-1 in 38.101-3. For these the UE must support both contiguous and non-contiguous EN-DC in the UL, i.e.

  - UE indicate “both” capability for DL DC_48A_(n)48AA with UL DC_(n)48AA and UL DC_48A_n48A

- Option 2c: In Rel-16 and Rel-17, report an additional band combination DC_48A_n48A in both UL and DL to support the Case 4 configuration DL DC_48A_(n)48AA with UL DC_48A_n48A and DC_(n)48AA with UL DC_48A_n48A, i.e.

  - UE indicate “contiguous” capability for DL DC_48A_(n)48AA with UL DC_(n)48AA

  - UE additionally indicate “non-contiguous” capability for DC_48A_n48A in both UL and DL

- Option 3: New signalling

  - A solution is necessary in RAN2 to address the ambiguity issue for configurations on some intra-band EN-DC band combinations with more than 2 carriers from Rel-15.

- R2 wait for R4 progress on case-4 of intra-band ENDC issue.

Conclusion

We have the following proposals:

Proposal 1	R2 wait for R4 progress on R16/17 solution for case-3 of intra-band ENDC issue before working on R18 solution.

Proposal 2	R2 wait for R4 progress on case-4 of intra-band ENDC issue.

Reference

- R4-2217759, 	WF on IntrabandENDC-Support.