3GPP TSG RAN WG2 #120                            		             R2-2211229

Toulouse, Nov 14th– 18th, 2022

Source:	CATT

Title:	Discussion on RedCap Positioning

Agenda:        8.2.5

Document for:	Discussion and Decision

Introduction

As can be seen in the SID [1], one of the objectives of R18 positioning enhancement is to support the positioning for RedCap UEs.

- Positioning support for RedCap UEs, considering the following: - Evaluate positioning performance of existing positioning procedures and measurements with RedCap UEs[RAN1] - Based on the evaluation, assess the necessity of enhancements and, if needed, identify enhancements to help address limitations associated with for RedCap UEs [RAN1, RAN2]

In RAN2#119bis-e, the following agreements were made on RedCap positioning.

Agreement: - Information on RedCap UE capability from the UE to the LMF (e.g., restricted bandwidth capability) can be discussed in WI phase. - RAN2 wait for RAN1 conclusion on the signalling design for assistance data for frequency hopping.

In this contribution, we would like to discuss the potential enhancement for RedCap positioning.

Discussion

Enhancement for RedCap positioning

# Alignment between the DRX and DL-PRS/UL-SRS

The DRX information applied by UE and the DL-PRS/SRS configured by LMF are independent, which may result in misalignment between PRS or SRS configuration and the DRX configuration. And the scenario shown in Figure 4 may occur, i.e., transmission occasion is in DRX off. In such scenario, if UE wakes up to perform PRS measurement, additional power consumption will be introduced. Instead, if UE does not wake up, UE cannot perform the PRS measurement at the corresponding PRS transmission occasion, which may lead to positioning failure. This issue makes the DL positioning procedure at low efficient and accuracy, it also brings some unnecessary energy consumption.

Figure 1: DL-PRS configuration and DRX period

Proposal 1: RAN2 to agree the enhancement on PRS/SRS configuration in normative work, i.e., to align with the DRX configuration.

# Usage of Carrier phase positioning

Carrier phase positioning technology has been widely used in the field of satellite positioning, which can achieve high measurement accuracy by using a small bandwidth (e.g., the bandwidth of GPS L1 C/A signals is only 1.023MHz). Consider that RedCap UEs can support bandwidth up to 20MHz, the bandwidth of the RedCap UE is not a limitation for supporting NR carrier phase positioning. Thus, using NR carrier phase positioning is one of the promising enhancement methods for RedCap UEs positioning.

Proposal 2: So as to improve the positioning performance, RAN2 to agree that the carrier phase positioning can be reused in RedCap positioning.

TP for RedCap positioning

Based on above analysis and RAN2 agreements, the TP on TS38.859 is provided.

Proposal 3: For RedCap positioning, RAN2 to agree the TP in annex 1 can be used as baseline for TR38.859.

Conclusions

In this contribution, we discussed the solutions for the RedCap positioning. Based on the discussion, we have the following proposals:

Proposal 1: RAN2 to agree the enhancement on PRS/SRS configuration in normative work, i.e., to align with the DRX configuration.

Proposal 2: So as to improve the positioning performance, RAN2 to agree that the carrier phase positioning can be reused in RedCap positioning.

Proposal 3: For RedCap positioning, RAN2 to agree the TP in annex 1 can be used as baseline for TR38.859.

References

- RP-213588, “Revised SID on Study on expanded and improved NR positioning.”

- MCC Support, “Draft Report of 3GPP TSG RAN WG1 #109-e v0.3.0”.

- MCC Support, “Draft Report of 3GPP TSG RAN WG1 #110-e”.

Annex: Text Proposal for TR 38.859

- 6.5	Positioning of UEs with Reduced Capabilities

For the purpose of the study of positioning performance for UEs with Reduced Capabilities (RedCap UEs), the following target performance requirements were considered:

For commercial use cases for both indoor and outdoor scenarios

-	Horizontal positioning accuracy: (< 3 m) for 90% of UEs

-	Vertical positioning accuracy: (< 3 m) for 90% of UEs

For IIoT use cases:

-	Horizontal positioning accuracy: (< 1 m) for 90% of UEs

-	Vertical positioning accuracy: (< 3 m) for 90% of UEs

For the above target requirements for evaluations, it should be noted that the target positioning requirements may not necessarily be achieved for all scenarios and deployments. Further, all positioning techniques may not achieve all positioning requirements in all scenarios.

### 6.5.1	Potential Solutions for Positioning for RedCap UEs

The following potential solutions for sidelink positioning are studied and summarized:

1. Enhancements to align the configuration between DRX and DL-PRS/UL-SRS.

2. Enhancements to use the Carrier phase positioning for RedCap positioning.

### 6.5.2	Summary of Evaluations for Positioning for RedCap UEs

### 6.5.3	Potential Specification Impact for Positioning for RedCap UEs

The potential specification impact for the RedCap positioning are studied in high layer and recommend in normative work, including:

-	Specification of the information on RedCap UE capability from the UE to the LMF (e.g., restricted bandwidth capability;

-	Specification of signalling design for assistance data for frequency hopping.