3GPP TSG-RAN WG2 #120	R2-2211236

Toulouse, France, November, 2022

Agenda Item:	8.15.2

Source:	OPPO

Title:	Discussion on CAPC definition in SL-U

Document for:	Discussion, Decision

Introduction

In RAN2 #119, the following agreements have been made regarding the CAPC definition in SL-U. This paper will further discuss the RAN2 work on the CAPC definition aspect.

Agreement on CAPC: 1: Working assumption: PQI is used to determine the CAPC mapping as in NR-U. FFS whether the same principle is also applied to the UE side. 2: For SL-DRB the CAPC value is (pre)configurable per-DRB as in NR-U. 3: For all SL-SRBs, CAPC value is fixed to the highest priority (i.e., lowest CAPC value). 4: If PQI-based CAPC mapping is agreed, for all SL MAC CEs, CAPC value is fixed to the highest priority (i.e., lowest CAPC value). 5: If PQI-based CAPC mapping is agreed, at least PDB can be used as the criterion to determine the CAPC mapping. FFS if any other additional criterions needed. 6: As in NR-U, if SL CAPC is determined based on PQI, as a baseline, for non-standardized PQI, to use the CAPC of the standardized PQI which best matches the QoS characteristics of the non-standardized PQI. FFS if any specific work needed for RRC_INACTIVE/RRC_IDLE/OOC UEs. 7: If PQI-based CAPC mapping is agreed, as in NR-U, to determine the CAPC of the SL TB when the CAPC is not indicated in the DCI: - If only SL MAC CE(s) are included in the SL TB, the highest priority SL CAPC is used; FFS whether this rule can be extended to the case when SL MAC CE(s) multiplexed with STCH. - If SCCH SDU(s) are included in the SL TB, the highest priority SL CAPC is used; - FFS how to select SL CAPC when SL CAPC of the SL logical channel(s) with MAC SDU multiplexed in the SL TB is used otherwise.

Discussion

PQI-based CAPC table

For the CAPC value configuration in NR-U, gNB considers the 5QIs of all the QoS flows multiplexed in a DRB to ensure fairness between different traffic types and transmissions. Table 1 below shows the mapping of standardized 5QIs and CAPC values.

Table 1: Mapping between Channel Access Priority Classes and 5QI

| CAPC | 5QI |
| --- | --- |
| 1 | 1, 3, 5, 65, 66, 67, 69, 70, 79, 80, 82, 83, 84, 85 |
| 2 | 2, 7, 71 |
| 3 | 4, 6, 8, 9, 72, 73, 74, 76 |
| 4 | - |
| NOTE: lower CAPC value means higher priority - |  |

The above mapping used in NR-U is copied from the LTE LAA conclusion from RAN2.

During LTE discussion (RAN2 #93, [1]), considering the CAPC with a lower number (as compared to one with a higher number) has a higher chance of acquiring the channel, the more delay-sensitive traffic is mapped to classes with lower numbers. Specifically, for the service with standardized 5QI, a stricter Packet Delay Budget is to be mapped to higher priority classes (lower number).

And priority class 4 may be used for services that have even lower priority, e.g. background data. However, there is no standardized 5QI defined for such services and it can be left to the operator to map such services to priority class 4.

The mapping between CAPC and 5QI was defined in NR-U based on the principle that the more delay-sensitive traffic is mapped to classes with lower numbers, i.e., stricter Packet Delay Budget is to be mapped to higher priority classes (lower number).

When we come to SL-U where PQI is used, the following agreements have been made,

1: Working assumption: PQI is used to determine the CAPC mapping as in NR-U. FFS whether the same principle is also applied to the UE side. 5: If PQI-based CAPC mapping is agreed, at least PDB can be used as the criterion to determine the CAPC mapping. FFS if any other additional criterions needed.

For the CAPC table in SL-U, majority of companies agree the PQI based CAPC table as in NR-U can be used, while there are some concerns on the possible issues for this PQI based CAPC table, thus a Working assumption has been made.

For the concerned issues,

- 1) how to (pre)configure CAPC per logical channel of SL-DRB for Mode 2 UE in IDLE/INACTIVE state and OOC UE?

- 2) How to avoid conflicting between CAPC and L1 priority in SCI which both serve intention of resource grabbing?

- 3) In case of non-standardized PQI NOT reported to NW, how to handle Mode 2 UE in IDLE/INACTIVE and OOC UE?

Issue-1 and Issue-2 are general for standardized and non-standardized QoS while Issue-3 is for non-standardized QoS specifically, so they will be separately discussed as follows.

Issue-1: per-logical channel CAPC configuration for IDLE/INACTIVE/OOC UE

For this issue, according to the legacy sidelink design principle, the IDLE/INACTIVE/OOC UE acquires the per-logical configuration (i.e., the priority configuration) from SIB/pre-configuration. The concern for per-CAPC mapping is mostly from the semi-static per-LCH configuration is not enough considering the N-to-1 mapping between QoS flow and logical channel. For this concern,

- The per-logical channel semi-static configuration for IDLE/INACTIVE/OOC UE based on N-to-1 mapping between QoS flow and logical channel is not a SL-U specific issue, but exist since day-1 for OoC sidelink, we understand the root issue is similar  for the per-logical channel priority configuration;

- Thus the concern is not specifically for the PDB-based CAPC table since the root issue is the legacy mechanism.

Per-logical channel pre-configuration for IDLE/INACTIVE/OOC UE based on the N-to-1 QoS flow to LCH mapping is a legacy mechanism.

Issue-2: Conflicting between CAPC and L1 priority in SCI

The conflict here comes from “both serve intention of resource grabbing”, and whether that is a valid issue may need further discussion since the L1 priority and CAPC are for different purposes,

- The L1 priority is used for congestion control/reserve resources inside sidelink system;

- The CAPC is used for channel access/reserve resources outside sidelink system, i.e., to guarantee the fairness among Wifi/NR-U/… system.

L1 priority is used for intra-system collision while CAPC is used for inter-system (with Wifi/BT…) collision.

As for the FFS point on the applicable of the PQI based CAPC mapping at UE side, at least for standardized PQI and RRC_CONNECTED case, the NR-U like mechanism works well, which means UE can rely on NW to configure the specific CAPC for each RB and be blind to how the CAPC configuration is determined (i.e., be blind to the PQI based CAPC table), so the FFS point can be solved/removed for RRC_CONNECTED and for standardized PQI.

Therefore, according to the above analysis for Issue-1, Issue-2 and the FFS point, the Working assumption can be confirmed by removing the FF point at least for standardized PQI.

RAN2 to confirm the WA as “PQI is used to determine the CAPC mapping as in NR-U” as baseline.

Then for the non-standardized PQI, i.e., Issue-3 as follows.

Issue-3: The handling of non-standardized PQI in case UE is in IDLE/INACTIVE/OOC

This issue-3 has been discussed in the following agreement and has been left for FFS

6: As in NR-U, if SL CAPC is determined based on PQI, as a baseline, for non-standardized PQI, to use the CAPC of the standardized PQI which best matches the QoS characteristics of the non-standardized PQI. FFS if any specific work needed for RRC_INACTIVE/RRC_IDLE/OOC UEs.

In the legacy sidelink system, the UE acquires the SLRB-QoS flow mapping through SIB or pre-configuration,

- For standardized QoS or the gNB-aware non-standardized QoS, the UE can use the dedicated SLRB configuration for that QoS;

- For the non-standardized QoS which is not aware by gNB, and the default RB has been configured, UE use the default RB configuration;

- And the UE can enter RRC CONNECTED to get the dedicated configuration if possible.

Table 1: legacy mechanism for QoS flow mapping

| Standardized QoS | gNB-aware Non-standardized QoS | Non-standardized QoS not aware by gNB |
| --- | --- | --- |
| dedicated or default RB SLRB configuration for that QoS from NW | dedicated SLRB configuration or default RB for that QoS from NW | default SLRB configuration for non-standardized QoS from NW or UE enter CONNECTED to get the dedicated configuration |

For the CAPC configuration in SL-U, by following the Working assumption on PQI-based CAPC mapping, the problem is the non-standardized QoS may not be aware by the gNB for the case where UE is in RRC IDLE/INACTIVE or OOC, and thus not included in the pre-configuration or SIB. So the concern here is on the FFS point, i.e., whether it is only NW to apply the PQI based CAPC table and UE just rely on NW configuration (reuse legacy mechanism, e.g., default RB) for non-standardized QoS as well or the PQI based CAPC table is also applicable at UE side, and UE can determine the CAPC value for the non-standardized QoS which is not included in SIB/pre-configurations. And in that case,

- On the one hand, some companies believe that the legacy mechanism works for the CAPC configuration as well, since there are other QoS related configurations (e.g., priority/PDB…) in legacy, which seems similar to CAPC.

- On the other hand, the other companies think the legacy mechanism is not enough considering the various QoS parameters, so would like to have some further enhancement on that, i.e., leave the CAPC determination to the UE.

Therefore, seems 2 directions for the handling of this issue,

- Option-1: PQI-based CAPC table only applicable at network side, and rely on network configuration (via SIB or pre-configuration) for the CAPC of non-standardized QoS flows, i.e., reuse legacy mechanism;

- Option-2: PQI-based CAPC table also applicable at UE side, UE itself to determine the CAPC of non-standardized QoS flows which are not included in SIB/pre-configurations.

For the non-standardized QoS flow, RAN2 to discuss how to determine the CAPC between 1) PQI-based CAPC table only applicable at network side so rely on network configuration of per-LCH CAPC; 2) PQI-based CAPC table also applicable at UE side, so UE can by its implementation determine the CAPC following the criterion that ‘for non-standardized PQI, to use the CAPC of the standardized PQI which best matches the QoS characteristics of the non-standardized PQI’. For 2), FFS applicable to which RRC state, coverage state or default/dedicated bearer.

If the WA is confirmed, the mapping between PQI and CAPC value should be defined, for the detailed rules for determining the table, the following agreement has been made.

5: If PQI-based CAPC mapping is agreed, at least PDB can be used as the criterion to determine the CAPC mapping. FFS if any other additional criterions needed.

According to the above agreement, PDB has been agreed as one criterions.

To sum up the CAPC determination of each 5QI:

Table 2: Mapping between Channel Access Priority Classes and PDB of the 5QIs

| CAPC | 5QI | Packet Delay Budget |
| --- | --- | --- |
| 1 | 1, 3, 5, 65, 66, 67, 69, 70, 79, 80, 82, 83, 84, 85 | Mostly≤ 100ms (except for 70 as a Mission Critical data) |
| 2 | 2, 7, 71 | 100/150ms |
| 3 | 4, 6, 8, 9, 72, 73, 74, 76 | ≥300ms |
| 4 | - |  |
| NOTE: lower CAPC value means higher priority - |  |  |

So basically the 5QI is categorized based on the Packet Delay Budget, and there is one exceptional case where 5QI 70 (PDB=200) is mapped to CAPC 1 due to being Mission Critical data.

In NR-U, Mission Critical data is also considered as a reason to map to higher priority CAPC class (i.e., lower value).

The tables below show the standardized PQIs:

Table 3: Standardized PQI values for Prose defined in TS 23.304[2] that are additionally defined to QoS characteristics mapping

| PQI Value | Resource Type | Default Priority Level | Packet Delay Budget | Packet Error Rate | Default Maximum Data Burst Volume | Default Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 24 | GBR (NOTE 1) | 1 | 150 ms | 10-2 | N/A | 2000 ms | Mission Critical user plane Push To Talk voice (e.g. MCPTT) |
| 25 |  | 2 | 200 ms | 10-2 | N/A | 2000 ms | Non-Mission-Critical user plane Push To Talk voice |
| 26 |  | 2 | 200 ms | 10-3 | N/A | 2000 ms | Mission Critical Video user plane |
| 60 | Non-GBR | 1 | 120 ms | 10-6 | N/A | N/A | Mission Critical delay sensitive signalling (e.g. MC-PTT signalling) |
| 61 |  | 6 | 400 ms | 10-6 | N/A | N/A | Mission Critical Data (e.g. example services are the same as 5QI 6/8/9 as specified in TS 23.501 [4]) |
| 92 | Delay Critical GBR (NOTE 1) | 5 | 5ms | 10-4 | 20000 bytes | 2000 ms | Interactive service - consume VR content with high compression rate via tethered VR headset (See TS 22.261 [6]) |
| 93 |  | 6 | 10ms | 10-4 | 20000 bytes | 2000 ms | interactive service - consume VR content with low compression rate via tethered VR headset; Gaming or Interactive Data Exchanging (See TS 22.261 [6]) |
| NOTE 1: GBR and Delay Critical GBR PQIs can only be used for unicast PC5 communications. |  |  |  |  |  |  |  |

Table 4: Standardized PQI values for V2X defined in TS 23.287[3] to QoS characteristics mapping

| PQI Value | Resource Type | Default Priority Level | Packet Delay Budget | Packet Error Rate | Default Maximum Data Burst Volume | Default Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 21 | GBR | 3 | 20 ms | 10-4 | N/A | 2000 ms | Platooning between UEs – Higher degree of automation; Platooning between UE and RSU – Higher degree of automation |
| 22 | (NOTE 1) | 4 | 50 ms | 10-2 | N/A | 2000 ms | Sensor sharing – higher degree of automation |
| 23 |  | 3 | 100 ms | 10-4 | N/A | 2000 ms | Information sharing for automated driving – between UEs or UE and RSU - higher degree of automation |
| 55 | Non-GBR | 3 | 10 ms | 10-4 | N/A | N/A | Cooperative lane change – higher degree of automation |
| 56 |  | 6 | 20 ms | 10-1 | N/A | N/A | Platooning informative exchange – low degree of automation; Platooning – information sharing with RSU |
| 57 |  | 5 | 25 ms | 10-1 | N/A | N/A | Cooperative lane change – lower degree of automation |
| 58 |  | 4 | 100 ms | 10-2 | N/A | N/A | Sensor information sharing – lower degree of automation |
| 59 |  | 6 | 500 ms | 10-1 | N/A | N/A | Platooning – reporting to an RSU |
| 90 | Delay Critical GBR | 3 | 10 ms | 10-4 | 2000 bytes | 2000 ms | Cooperative collision avoidance; Sensor sharing – Higher degree of automation; Video sharing – higher degree of automation |
| 91 | (NOTE 1) | 2 | 3 ms | 10-5 | 2000 bytes | 2000 ms | Emergency trajectory alignment; Sensor sharing – Higher degree of automation |
| NOTE 1: GBR and Delay Critical GBR PQIs can only be used for unicast PC5 communications. |  |  |  |  |  |  |  |

Based on the tables for PQI, if we categorize the PQI based on the above PDB-based rule for the mapping between 5QI and CAPC in NR-U:

- For PQI 90/91/92/93, they have quite restrictive PDB requirement (i.e., 3/5/10ms), so should be categorized as CAPC class 1;

- For PQI 59/61, they have a loose PDB requirement (400/500ms), so should be CAPC class 3, if following the categorization in NR-U;

- For PQI 21/22/23/55/56/57/58, the PDB requirement of these PQI are all equal to or less than 100ms, so should be CAPC class 1, if following the categorization in NR-U;

- For PQI 25, it has a PDB requirement of 200ms (>100), and it is not Mission Critical Service, so CAPC class 2 seems appropriate;

R2 discusses mapping PQI 90/91/92/93/21/22/23/55/56/57/58 to CAPC priority class 1.

R2 discusses mapping PQI 59/61 to CAPC priority class 3.

R2 discusses mapping PQI 25 to CAPC priority class 2.

Then by following NR-U principle, i.e., consider the Mission Critical data as one criterion to determine the CAPC mapping,

- For PQI 24/26/60, On the one hand, if we determine the CAPC based on PDB, their PDB requirement is not that restrictive (i.e., 120/150/200ms), so seems ok to put them into CAPC class 2; On the other hand, they are Mission Critical Services, and 5QI 70 (as a Mission Critical Service) has been categorized as CAPC class 1 – so some discussion on taking either 1 or 2 would be helpful;

R2 discusses mapping PQI 24/26/60 to CAPC priority class 1 or CAPC priority class 2.

In summary, according to the above analysis, the suggested CAPC for each PQI is as follows:

Table 5: Standardized PQI values and the suggested CAPC level according to the PDB

| PQI Value | Packet Delay Budget | Suggested CAPC level | Use Case |
| --- | --- | --- | --- |
| 24 | 150ms | FFS whether 1 or 2 | ProSe, from 23.304 |
| 25 | 200ms | 2 |  |
| 26 | 200ms | FFS whether 1 or 2 |  |
| 60 | 120ms | FFS whether 1 or 2 |  |
| 61 | 400ms | 3 |  |
| 92 | 5ms | 1 |  |
| 93 | 10ms | 1 |  |
| 21 | 20ms | 1 | V2X, from 23.287 |
| 22 | 50ms | 1 |  |
| 23 | 100ms | 1 |  |
| 55 | 10ms | 1 |  |
| 56 | 20ms | 1 |  |
| 57 | 25ms | 1 |  |
| 58 | 100ms | 1 |  |
| 59 | 500ms | 3 |  |
| 90 | 10ms | 1 |  |
| 91 | 3ms | 1 |  |

So it is suggested the below table for the mapping between PQI and CAPC values:

Table 6: Mapping between Channel Access Priority Classes and PDB of the PQIs

| CAPC | PQI |
| --- | --- |
| 1 | 21,22,23,55,56,57,58,90,91,92,93, [24, 26, 60] |
| 2 | 25 |
| 3 | 59,61 |
| 4 | - |
| NOTE: lower CAPC value means higher priority - |  |

CAPC for MAC PDU

For the CAPC value determination of a MAC PDU, RAN2 has made the following agreement

7: If PQI-based CAPC mapping is agreed, as in NR-U, to determine the CAPC of the SL TB when the CAPC is not indicated in the DCI: - If only SL MAC CE(s) are included in the SL TB, the highest priority SL CAPC is used; FFS whether this rule can be extended to the case when SL MAC CE(s) multiplexed with STCH. - If SCCH SDU(s) are included in the SL TB, the highest priority SL CAPC is used; - FFS how to select SL CAPC when SL CAPC of the SL logical channel(s) with MAC SDU multiplexed in the SL TB is used otherwise.

For the above 2 FFS, they are for different cases 1) MAC CE and STCH are included in the TB; or 2) only STCH is included in the TB. However, the key issue is the same, i.e., when there are both high-priority data and low-priority data, whose CAPC should be used, whether the lowest or the highest priority CAPC.

Before making a decision, it’s better to review the actual impact of CAPC. RAN1 has made the agreement on the CAPC parameter table as follows

Agreement In Type 1 SL channel access procedure, the following table is adopted for channel access priority class (CAPC) for SL. - FFS: the applicability and usage of NOTE1 in the table - FFS: whether mp=1 can be used with p=1, and applicable cases Channel Access Priority Class (p); mp; CWmin,p; CWmax,p; Tslmcot,p; allowed CWp sizes. 1; 2; 3; 7; 2 ms; {3,7}. 2; 2; 7; 15; 4 ms; {7,15}. 3; 3; 15; 1023; 6ms [or 10 ms]; {15,31,63,127,255,511,1023}. 4; 7; 15; 1023; 6ms [or 10 ms]; {15,31,63,127,255,511,1023}. [NOTE1: Forp=3,4, Tslmcot,p=10ms if the higher layer parameter absenceOfAnyOtherTechnology-r14 or absenceOfAnyOtherTechnology-r16 is provided, otherwise,Tslmcot,p=6ms.] NOTE 2: When Tslmcot,p=6ms it may be increased to 8ms by inserting one or more gaps. The minimum duration of a gap shall be 100μs. The maximum duration before including any such gap shall be 6ms..

Each CAPC level has the mapped CW size and allowed COT duration,

- a higher priority CAPC has shorter CW but on the other hand has shorter COT as well, which means the higher priority CAPC can access the channel easier but shorter;

- a lower priority CAPC has longer CW but on the other hand has longer COT as well, which means the lower priority CAPC can access the channel harder but longer;

So the CAPC has been designed as a balance between the pay (duration for LBT) and return (duration for transmission), thus by using a lowest priority CAPC value, UE may need more time for each LBT, but also guarantee there is enough COT to be used, which can also save the times of LBT. So there is no essential technical blocking issue to select either way.

Following NR-U principle, we have a slight preference on using the lowest priority CAPC when there is STCH in the MAC PDU.

A higher CAPC class has shorter CW length but shorter COT as well, while a lower CAPC class has longer CW length but longer COT as well.

RAN2 to discuss how to decide the CAPC of a MAC PDU including STCH in SL-U, i.e.,  use the lowest priority CAPC as in NR-U.

Conclusion

We have the following observations:

Observation 1	The mapping between CAPC and 5QI was defined in NR-U based on the principle that the more delay-sensitive traffic is mapped to classes with lower numbers, i.e., stricter Packet Delay Budget is to be mapped to higher priority classes (lower number).

Observation 2	Per-logical channel pre-configuration for IDLE/INACTIVE/OOC UE based on the N-to-1 QoS flow to LCH mapping is a legacy mechanism.

Observation 3	L1 priority is used for intra-system collision while CAPC is used for inter-system (with Wifi/BT…) collision.

Observation 4	In NR-U, Mission Critical data is also considered as a reason to map to higher priority CAPC class (i.e., lower value).

Observation 5	A higher CAPC class has shorter CW length but shorter COT as well, while a lower CAPC class has longer CW length but longer COT as well.

We have the following proposals:

Proposal 1	RAN2 to confirm the WA as “PQI is used to determine the CAPC mapping as in NR-U” as baseline.

Proposal 2	For the non-standardized QoS flow, RAN2 to discuss how to determine the CAPC between 1) PQI-based CAPC table only applicable at network side so rely on network configuration of per-LCH CAPC; 2) PQI-based CAPC table also applicable at UE side, so UE can by its implementation determine the CAPC following the criterion that ‘for non-standardized PQI, to use the CAPC of the standardized PQI which best matches the QoS characteristics of the non-standardized PQI’. For 2), FFS applicable to which RRC state, coverage state or default/dedicated bearer.

Proposal 3	R2 discusses mapping PQI 90/91/92/93/21/22/23/55/56/57/58 to CAPC priority class 1.

Proposal 4	R2 discusses mapping PQI 59/61 to CAPC priority class 3.

Proposal 5	R2 discusses mapping PQI 25 to CAPC priority class 2.

Proposal 6	R2 discusses mapping PQI 24/26/60 to CAPC priority class 1 or CAPC priority class 2.

Proposal 7	RAN2 to discuss how to decide the CAPC of a MAC PDU including STCH in SL-U, i.e.,  use the lowest priority CAPC as in NR-U.

Reference

[1] R2-161548 Mapping between Channel Access Priority Classes and QCI values	Ericsson, Huawei, HiSilicon, Qualcomm Incorporated

[2] TS 23.304 Proximity based Services (ProSe) in the 5G System (5GS)

[3] TS 23.287 Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services