3GPP TSG-RAN2 Meeting #120	R2-2211260

# Toulouse, France, 14th – 18th Nov., 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 1450 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the MAC spec for pre-configured MG request |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2022-11-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Issue 1: for pre-configured measurement gap for positioning, in R17, we have introduced two approaches for activation/deactivation request (a) UE-based approach by UE sending the MAC CE (b) LMF-based approach by the LMF sending the NRPPa message. However, currently, in the spec when the pre-configured measurement gap configurations are provided to the UE, it is by default assumed that the UE can request the activation/deactivation of the MG. While, it is also possible the gNB does not support the reception of the MAC CE and only support the LMF-based approach |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change1: Add RRC configuration on whether the UE can trigger MAC CE to request activation/deactivation of the pre-configured MG |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Without the change, it is assumed that the gNB has to support the reception of pre-configured MG activation/deactivation request, which is unreasonable since the LMF-based MG activation/deactivation request is also supported and can be another alternative for activation/deactivation request Impact analysis Impacted 5G architecture options: SA, NE-DC, NR-DC Impacted functionality: Pre-configured positioning MG activation/deactivation request Chagne Inter-operability: If the UE is implemented according to the CR while the network is not, the UE will not request the MG by MAC CE when the configuration is not supported. Hence, this results in a deadlock if the network is implemented according to the CR while the UE is not, the UE may not recognize the configuration. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.25 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

============================CHANGE BEGINS=====================================

## 5.25	Positioning Measurement Gap Activation/Deactivation Request

If the UE is configured with pre-configured measurement gap and to send activation/deactivation request for pre-configured positioning measurement gap, the UE may request the network to activate or deactivate the Positioning measurement gap with UL MAC CE for Positioning Measurement Gap Activation/Deactivation Request in clause 6.1.3.40.

RRC controls Positioning Measurement Gap Activation/Deactivation Request by configuring the following parameters:

-	mg-ActivationDeactivationReq.

The MAC entity shall, when triggered by the upper layer to send Positioning Measurement Gap Activation/Deactivation Request, cancel the triggered Positioning Measurement Gap Activation/Deactivation Request, if any and trigger another Positioning Measurement Gap Activation/Deactivation Request according to the upper layer's request.

The MAC entity shall, if mg-ActivationDeactivationReq is configured:

1>if Positioning Measurement Gap Activation/Deactivation Request MAC CE has been triggered, and not cancelled:

2>	if indication from upper layer has been received that the triggered Positioning Measurement Gap Activation/Deactivation Request MAC CE should be cancelled; or

2>	if the pre-configured measurement gap indicated in the Positioning Measurement Gap Activation/Deactivation Request MAC CE has already been activated/deactivated according to clause 5.18.20:

3>	cancel the triggered Positioning Measurement Gap Activation/Deactivation Request MAC CE.

2>	if UL-SCH resources are available for a new transmission and these UL-SCH resources can accommodate the Positioning Measurement Gap Activation/Deactivation Request MAC CE plus its subheader as a result of logical channel prioritization:

3>	instruct the Multiplexing and Assembly procedure to generate the Positioning Measurement Gap Activation/Deactivation Request MAC CE according to the upper layer's request;

3>	cancel triggered Positioning Measurement Gap Activation/Deactivation Request MAC CE.

2>	else:

3>	trigger a Scheduling Request for Positioning Measurement Gap Activation/Deactivation Request MAC CE.

===================================CHANGE ENDS==================================