3GPP TSG-RAN WG2 Meeting #120                                                 R2-2211311

Toulouse, France, 14th Nov. – 18th Nov. 2022

Source:	CATT

Title:	Discussion on the HARQ disabling in IoT NTN

Agenda Item:	8.6.2.1

Document for:	Discussion and Decision

Introduction

For HARQ disabling, some agreements have been achieved in RAN2#119 meeting, RAN2#119bis meeting and RAN1#110bis meeting. In this contribution, we give some discussion on disabling of HARQ feedback and observations as well as proposals are provided.

Discussion

UE capability for disabling HARQ feedback

It was agreed in RAN2#119-e meeting that:

Agreements:

- Disabling DL HARQ feedback is supported for NB-IoT and eMTC NTN. FFS on UE capability

Based on the agreement, FFS is left for UE capability, i.e. it is not clear whether disabling DL HARQ feedback should be mandatory supported by all IoT NTN UEs. And as shown the agreements above, for UL HARQ operation, two HARQ modes was introduced, in which HARQ mode B means disabling HARQ feedback. We think RAN2 also need to discuss the issue that whether disabling UL HARQ feedback should be mandatory supported by all IoT NTN UEs.

In Rel-17, the UE capability for disabling DL/UL HARQ feedback was defined which is captured in TS 38.306 [1]

| harq-FeedbackDisabled-r17 Indicates whether the UE supports disabled HARQ feedback for downlink transmission. A UE supporting this feature shall also indicate the support of nonTerrestrialNetwork-r17. | UE | No | No | No |
| --- | --- | --- | --- | --- |

And

| uplink-Harq-ModeB-r17 Indicates whether the UE supports HARQ Mode B and the corresponding LCP restrictions for uplink transmission. A UE supporting this feature shall also indicate the support of nonTerrestrialNetwork-r17. | UE | No | No | No |
| --- | --- | --- | --- | --- |

So similarly, the same mechanism can be supported as start point, to avoid repetitive discussion.

Proposal 1：Introduce separate UE capability indicating whether UE supporting disabling HARQ feedback for DL/UL transmission.

DRX inactivity timer start/restart

It was agreed in RAN2# 119bis-e meeting that:

Agreements:

- RAN2 agree to take R17 NR NTN DRX solution as baseline for IoT NTN, e.g. for HARQ process with DL HARQ feedback disabled, the UE will not start the corresponding DL HARQ RTT timer.

- For NB-IoT NTN with single HARQ process when the HARQ feedback is disabled, the UE will start/restart drx-inactivity timer in the subframe containing the last repetition of the corresponding PDSCH reception (can still check whether the alternative to set the HARQ RTT timer to 0 also works)

- RAN2 agree to take R17 NR NTN DRX solution as baseline for IoT NTN, e.g. for HARQ process in HARQ mode B, the UE will not start the corresponding UL HARQ RTT timer.

- For NB-IoT NTN with single HARQ process in HARQ mode B, the UE will start/restart drx-inactivity timer in the subframe containing the last repetition of the corresponding PUSCH transmission (can still check whether other alternatives also work)

The first issue is, whether the alternative to set the HARQ RTT timer to 0 also works. In the offline discussion of last meeting, some companies thought the alternative to set the HARQ RTT timer to 0 was also workable. It brings less impact on the specification, e.g. no special handling on UE behavior is needed for DRX. But it seems a bit of unnecessary anymore, for the option of start/restart drx-inactivity timer has been supported. Otherwise, we have to discuss the condition for each option, which will make things unnecessary complex. From this point of view, we think a simpler UE behavior is preferable. Hence, we propose that:

Proposal 2: For NB-IoT NTN with single HARQ process when the HARQ feedback is disabled (in HARQ mode B for UL), the alternative to set the HARQ RTT timer to 0 is not supported.

The following agreement was achieved in RAN1# 110bis meeting:

Agreement

For a DL HARQ process with disabled HARQ feedback in NB-IoT, UE is not required to monitor NPDCCH in a period of Y=12(ms) from the end of reception of the NPDSCH.

The reason is the lower processing capability of NB-IoT UE. To avoid unnecessary NPDCCH monitoring and power consumption, the agreement of RAN1 above should also be considered when UE start/restart drx-inactivity timer. So it is proposed that:

Proposal 3: Update the agreement according to RAN1 agreement:

For NB-IoT NTN with single HARQ process when the HARQ feedback is disabled, the UE will start/restart drx-inactivity timer 12ms after in the subframe containing the last repetition of the corresponding PDSCH reception (can still check whether the alternative to set the HARQ RTT timer to 0 also works)

Agreements made for HARQ disabling/enabling in Rel-17 NR NTN

In Rel-17 NR NTN, disabling HARQ feedback has been discussed deeply, and lots of agreements have been achieved. To avoid the repetitive discussion on the similar topic, we suggest the agreements achieved in Rel-17 NR NTN are supported by default, with identifying the ones that are not applicable to IoT NTN, based on contribution or email discussion.

In the Appendix, we list the agreements of Rel-17 NR NTN separately, the ones we suggested being reused by Rel-18 IoT NTN, and the ones we think more discussion is needed.

Proposal 4：By default, the agreements related with HARQ disabling in Rel-17 NR NTN are applicable to Rel-18 IoT NTN.

- Identify/revise the agreements that do not applicable to Rel-18 IoT NTN UE, based on contribution or email discussion

HARQ feedback disabling/enabling in discontinuous coverage scenario

One main difference between IoT NTN and NR NTN is that IoT NTN may support the feature of discontinuous coverage. So in addition to the agreements having achieved in Rel-17 NR NTN, the impact of discontinuous coverage to HARQ disabling should also be considered in Rel-18 IoT NTN HARQ disabling discussion. For example, for HARQ enabling HARQ process, what to do if there is no enough time to feed back NACK to network because the coverage will be lost.

Proposal 5：The impact of discontinuous coverage on HARQ feedback disabling/enabling should be considered.

Conclusion

In this contribution, we give some discussion the Redcap UE behaviour of SI request on SUL, the following proposal and optional solutions are given:

Proposal 1：Introduce separate UE capability indicating whether UE supporting disabling HARQ feedback for DL/UL transmission.

Proposal 2: For NB-IoT NTN with single HARQ process when the HARQ feedback is disabled (in HARQ mode B for UL), the alternative to set the HARQ RTT timer to 0 is not supported.

Proposal 3: Update the agreement according to RAN1 agreement:

For NB-IoT NTN with single HARQ process when the HARQ feedback is disabled, the UE will start/restart drx-inactivity timer 12ms after in the subframe containing the last repetition of the corresponding PDSCH reception (can still check whether the alternative to set the HARQ RTT timer to 0 also works)

Proposal 4：By default, the agreements related with HARQ disabling in Rel-17 NR NTN are applicable to Rel-18 IoT NTN.

- Identify/revise the agreements that do not applicable to Rel-18 IoT NTN UE, based on contribution or email discussion

Proposal 5：The impact of discontinuous coverage on HARQ feedback disabling/enabling should be considered.

Reference

- 3GPP TS 38.306 V17.1.0 (2022-06) User Equipment (UE) radio access capabilities

Appendix

The following agreements can be reused to Rel-18 IoT NTN:

Note:

  - drx-HARQ-RTT-TimerDL is corresponding to the parameter of HARQ RTT Timer in LTE.

  - drx-HARQ-RTT-TimerUL is corresponding to the parameter of UL HARQ RTT Timer in LTE.

  - drx-RetransmissionTimerDL is corresponding to the parameter of drx-RetransmissionTimer in LTE.

- drx-RetransmissionTimerUL is corresponding to the parameter of drx-ULRetransmissionTimer  in LTE.From a RAN2 perspective, for DL, HARQ feedback can be enabled/disabled in Rel-17 NTN, but HARQ processes remain configured. The criteria and decision to enable/disable HARQ feedback is under network control and is signalled to the UE via RRC in a semi-static manner.

The part in red can be updated DCI based solution is introduced by RAN1.

- From RAN2 perspective, for dynamic grant, one possibility for "enabling"/"disabling" HARQ uplink retransmission at UE transmitter is without introducing an additional mechanism (i.e. gNB can send grant with NDI not toggled/toggled without waiting for decoding result of previous PUSCH transmission).

- From RAN2 perspective, for HARQ processes where gNB can sends UL grant without waiting for decoding result of previous PUSCH transmission, no new network scheduling restrictions are introduced to schedule subsequent grants (i.e. up to network implementation.)

- It is NW scheduling strategy to avoid NTN UE in HARQ stalling state. From RAN2 perspective, the NW can continuously schedule the UE using one or a combination of scheduling strategies, such as without HARQ retransmissions, or with blind retransmissions, or with HARQ retransmissions based on DL HARQ feedback (or UL decoding result).

- RAN2 confirms that in NTN if the UE is in DRX Active Time for any reason, the UE should monitor the PDCCH regardless of whether drx-HARQ-RTT-TimerUL or drx-HARQ-RTT-TimerDL is running or not. No specification change is needed.

- In NTN, The drx-HARQ-RTT-TimerUL is configured per UE DRX group and the behaviour can be configured per HARQ process.

- Repetition transmission based HARQ retransmission is always allowed and is explicitly indicated per HARQ process via DCI (as in legacy).

- Configuration of UL HARQ retransmission state is semi-static, signalled via RRC, and the decision and criteria to configure UL HARQ retransmission state is under network control.

The part in red can be updated DCI based solution is introduced by RAN1.

- If HARQ process has not been configured with an UL HARQ retransmission state, new LCH mapping rule has no effect (i.e. UE applies legacy behaviour).

- For HARQ process(es) not configured with an UL HARQ retransmission state, drx-HARQ-RTT-TimerUL and drx-RetransmissionTimerUL behave as per legacy.

- The network may consider delay and reliability characteristics of ongoing services when choosing to configure an UL HARQ retransmission state.

- RAN2 understanding is that UE behaviour in HARQ state A (i.e. extending the drx-HARQ-RTT-TimerUL by UE-gNB RTT) best supports reception of UL retransmission grant based on UL decoding result. (No RAN2 specification impact)

- RAN2 understanding is that UE behaviour in HARQ state B (i.e. not starting drx-HARQ-RTT-TimerUL) best supports no UL retransmission and/or blind UL retransmission. (No RAN2 specification impact)

- UE configured with an UL HARQ retransmission state (i.e. A or B) will always act as indicated in a grant/assignment provided during a valid occasion (i.e. subject to legacy restrictions in e.g. MAC and RAN1 specifications). (No RAN2 specification impact)

- HARQ feedback shall always be sent for SPS deactivation (i.e. regardless of HARQ feedback enabled/disabled).

SPS deactivation is corresponding to SPS release in LTE.

- For HARQ process(es) not configured with DL HARQ feedback enabled/disabled, drx-HARQ-RTT-TimerDL behaves as per legacy.

- It is up to network implementation to ensure proper configuration of HARQ feedback (i.e. enabled or disabled) for HARQ processes used by an SPS configuration (no Stage 3 specification impact).

- It is up to network implementation to ensure proper configuration of HARQ mode for HARQ processes used by a CG configuration (no Stage 3 specification impact).

- RAN2 understanding is that: in general, all HARQ processes used by an SPS configuration are configured with the same HARQ feedback enabled/disabled state. No specification impact.

- RAN2 understanding is that: in general, all HARQ processes used by a CG configuration are configured with the same HARQ state (e.g. A or B). No specification impact

And the following agreements may need more discussion for Rel-18 NB-IoT NTN UE:

- No new LCP restrictions are introduced for exisiting UL MAC Ces

- For dynamic grants, each LCH can optionally be semi statically configured (by RRC) to be mapped to one or more HARQ processes. If there is no RRC configuration for this, this mapping has no effect (legacy behaviour applies).

- For dynamic grants, each LCH can be optionally mapped to an UL HARQ retransmission state via semi-static RRC configuration. If there is no configuration, the mapping has no effect (legacy behaviour applies)

- An UL HARQ retransmission state is configured per HARQ process to support new LCH mapping restriction and proper configuration of drx-HARQ-RTT-TimerUL behaviour.

- RRC parameter “allowedHARQ-DRX-LCP” is included in LogicalChannelConfig

- If uplinkHARQ-DRX-LCP-Mode-r17 is configured, a HARQ process may be mapped to either ‘HARQ mode A’ or ‘HARQ mode B’.

- uplinkHARQ-DRX-Mode shall be included in PUSCH-ServingCellConfig.

- If uplinkHARQ-DRX-LCP-Mode-r17 is configured, the following LCH to HARQ process mapping rules are supported:

1) LCH is mapped only to a HARQ process configured with HARQ mode A;

2) LCH is mapped only to a HARQ process configured with HARQ mode B;

3) If an LCH is not configured with a mapping rule, it may be mapped to any HARQ process (HARQ mode A or B).

- downlinkHARQ-FeedbackDisabled shall be included in PDSCH-ServingCellConfig.

- uplinkHARQ-DRX-Mode-r17 controls the DRX behaviour of HARQ processes in the same way for configured grants as for dynamic grants.

- AllowedHARQ-DRX-LCP also applies to CG

- It is up to NW implementation to properly configure allowedHARQ-DRX-LCP or allowedCG-List for a LCH (e.g. to avoid conflicting configuration) (Comeback if we find a problem in the implementation in the spec)

- uplinkHARQ-mode and allowedHARQ-mode, if configured, also apply for SRB1 to SRB3