3GPP TSG-RAN WG2 Meeting # 120 	R2-2211421

# Toulouse, France, Nov 14th – 18th 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.355 | CR | 0389 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections of LPP capabilities on DL-RPS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Core |  |  |  |  |  | Date: |  |  | 2022-10-31 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Some capabilities in LPP don't follow the capabilities described in Updated RAN1 UE features list for Rel-16 NR after RAN1#108-e (R1-2202763; Moderator (NTT DOCOMO, INC.)). Reason for change of NR-DL-PRS-ProcessingCapability: #1. IE maxNumOfDL-PRS-ResProcessedPerSlot-r16 in asn.1: the value of 6 ,12 in ENUMERATED is missed according to the LS as below: 13. NR Positioning 13-1 Common DL PRS Processing Capability - Max number of DL PRS resources that UE can process in a slot under it - FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz - FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz However 6 and 12 are not found in the existing asn.1 as below: maxNumOfDL-PRS-ResProcessedPerSlot-r16 SEQUENCE { scs15-r16 ENUMERATED {n1, n2, n4, n8, n16, n24, n32, n48, n64} OPTIONAL, scs30-r16 ENUMERATED {n1, n2, n4, n8, n16, n24, n32, n48, n64} OPTIONAL, scs60-r16 ENUMERATED {n1, n2, n4, n8, n16, n24, n32, n48, n64} OPTIONAL, scs120-r16 ENUMERATED {n1, n2, n4, n8, n16, n24, n32, n48, n64} OPTIONAL, ... }, #2. The value of 6 and 32 in the description of durationOfPRS-Processing is missed. Reason for change of maxNrOfTRP-AcrossFreqs-r16 in NR-DL-PRS-ResourcesCapability: the value of 24 in ENUMERATED is missed. 13. NR Positioning 13-2 DL PRS Resources for DL AoD - Max number of TRPs across all positioning frequency layers per UE. Values = {4, 6, 12, 16, 24, 32, 64, 128, 256} 24 is not found in the existing asn.1 as below: maxNrOfTRP-AcrossFreqs-r16 ENUMERATED { n4, n6, n12, n16, n32, n64, n128, n256, ...}, |  |  |  |  |  |  |  |  |
| Summary of change: |  | To modify the corresponding IEs and clarifications based on RAN1 latest feature list in R1-2202763. Feature number; Field name. 13-1; NR-DL-PRS-ProcessingCapability. 13-2; NR-DL-PRS-ResourcesCapability. Impact analysis Impacted 5G architecture options: NR SA, (NG)EN-DC, NE-DC,NR-DC Impacted functionality: Positioning Inter-operability: If the UE implement the CR and the LMF does not, there is interoperability issue that LMF won't decode the message correctly. If the LMF implement the CR and the UE does not, there is no interoperability issue. |
| Consequences if not approved: |  | The capabilities of NR-DL-PRS-ProcessingCapability and NR-DL-PRS-ResourcesCapability are not aligned with feature list. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

6.4.3	Common NR Positioning Information Elements

/**Skip unrelated parts**/

–	NR-DL-PRS-ProcessingCapability

The IE NR-DL-PRS-ProcessingCapability defines the common DL-PRS Processing capability. In the case of capabilities for multiple NR positioning methods are provided, the IE NR-DL-PRS-ProcessingCapability applies across the NR positioning methods and the target device shall indicate the same values for the capabilities in IEs NR-DL-TDOA-ProvideCapabilities, NR-DL-AoD-ProvideCapabilities, and NR-Multi-RTT-ProvideCapabilities.

The PRS-ProcessingCapabilityPerBand is defined for a single positioning frequency layer on a certain band (i.e., a target device supporting multiple positioning frequency layers is expected to process one frequency layer at a time).

-- ASN1START

NR-DL-PRS-ProcessingCapability-r16 ::= SEQUENCE {

prs-ProcessingCapabilityBandList-r16	SEQUENCE (SIZE (1..nrMaxBands-r16)) OF

PRS-ProcessingCapabilityPerBand-r16,

maxSupportedFreqLayers-r16				INTEGER (1..4),

simulLTE-NR-PRS-r16						ENUMERATED { supported }		OPTIONAL,

...,

[[

dummy									ENUMERATED { m1, m2, ... }		OPTIONAL

]]

}

PRS-ProcessingCapabilityPerBand-r16 ::= SEQUENCE {

freqBandIndicatorNR-r16				FreqBandIndicatorNR-r16,

supportedBandwidthPRS-r16			CHOICE {

fr1										ENUMERATED {mhz5, mhz10, mhz20, mhz40,

mhz50, mhz80, mhz100},

fr2										ENUMERATED {mhz50, mhz100, mhz200, mhz400},

...

},

dl-PRS-BufferType-r16		 		ENUMERATED {type1, type2, ...},

durationOfPRS-Processing-r16		SEQUENCE {

durationOfPRS-ProcessingSymbols-r16	ENUMERATED {nDot125, nDot25, nDot5, n1,

n2, n4, n6, n8, n12, n16, n20, n25,

n30, n32, n35, n40, n45, n50},

durationOfPRS-ProcessingSymbolsInEveryTms-r16

ENUMERATED {n8, n16, n20, n30, n40, n80,

n160,n320, n640, n1280},

...

},

maxNumOfDL-PRS-ResProcessedPerSlot-r16	SEQUENCE {

scs15-r16								ENUMERATED {n1, n2, n4, n8, n16, n24, n32,

n48, n64}					OPTIONAL,

scs30-r16								ENUMERATED {n1, n2, n4, n8, n16, n24, n32,

n48, n64}					OPTIONAL,

scs60-r16								ENUMERATED {n1, n2, n4, n8, n16, n24, n32,

n48, n64}					OPTIONAL,

scs120-r16								ENUMERATED {n1, n2, n4, n8, n16, n24, n32,

n48, n64}					OPTIONAL,

...,

[[

scs15-v16xy								ENUMERATED {n6, n12}					OPTIONAL,

scs30-v16xy								ENUMERATED {n6, n12}					OPTIONAL,

scs60-v16xy								ENUMERATED {n6, n12}					OPTIONAL,

scs120-v16xy							ENUMERATED {n6, n12}					OPTIONAL

]]

},

...,

[[

supportedDL-PRS-ProcessingSamples-RRC-CONNECTED-r17	ENUMERATED { supported }		OPTIONAL,

prs-ProcessingWindowType1A-r17			ENUMERATED { option1, option2, option3}		OPTIONAL,

prs-ProcessingWindowType1B-r17			ENUMERATED { option1, option2, option3}		OPTIONAL,

prs-ProcessingWindowType2-r17			ENUMERATED { option1, option2, option3}		OPTIONAL,

prs-ProcessingCapabilityOutsideMGinPPW-r17

SEQUENCE (SIZE(1..3)) OF

PRS-ProcessingCapabilityOutsideMGinPPWperType-r17

OPTIONAL,

dl-PRS-BufferType-RRC-Inactive-r17		ENUMERATED { type1, type2, ... }			OPTIONAL,

durationOfPRS-Processing-RRC-Inactive-r17	SEQUENCE {

durationOfPRS-ProcessingSymbols-r17			ENUMERATED {nDot125, nDot25, nDot5, n1,

n2, n4, n6, n8, n12, n16, n20, n25,

n30, n32, n35, n40, n45, n50},

durationOfPRS-ProcessingSymbolsInEveryTms-r17

ENUMERATED {n8, n16, n20, n30, n40, n80,

n160,n320, n640, n1280},

...

}																					OPTIONAL,

maxNumOfDL-PRS-ResProcessedPerSlot-RRC-Inactive-r17	SEQUENCE {

scs15-r17								ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64}				OPTIONAL,

scs30-r17								ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64}				OPTIONAL,

scs60-r17								ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64}				OPTIONAL,

scs120-r17								ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64}				OPTIONAL,

...

}																					OPTIONAL,

supportedLowerRxBeamSweepingFactor-FR2-r17	ENUMERATED { n1, n2, n4, n6 }			OPTIONAL

]],

[[

supportedDL-PRS-ProcessingSamples-RRC-Inactive-r17	ENUMERATED { supported }		OPTIONAL

]]

}

PRS-ProcessingCapabilityOutsideMGinPPWperType-r17 ::= SEQUENCE {

prsProcessingType-r17						ENUMERATED { type1A, type1B, type2 },

ppw-dl-PRS-BufferType-r17		 			ENUMERATED { type1, type2, ... },

ppw-durationOfPRS-Processing1-r17			SEQUENCE {

ppw-durationOfPRS-ProcessingSymbolsN-r17

ENUMERATED { msDot125, msDot25, msDot5, ms1, ms2, ms4,

ms6, ms8, ms12, ms16, ms20, ms25, ms30, ms32, ms35,

ms40, ms45, ms50 },

ppw-durationOfPRS-ProcessingSymbolsT-r17

ENUMERATED { ms1, ms2, ms4, ms8, ms16, ms20, ms30, ms40, ms80,

ms160, ms320, ms640, ms1280 }

}																					OPTIONAL,

ppw-durationOfPRS-Processing2-r17			SEQUENCE {

ppw-durationOfPRS-ProcessingSymbolsN2-r17

ENUMERATED { msDot125, msDot25, msDot5, ms1, ms2, ms3, ms4, ms5,

ms6, ms8, ms12 },

ppw-durationOfPRS-ProcessingSymbolsT2-r17

ENUMERATED { ms4, ms5, ms6, ms8 }

}																					OPTIONAL,

ppw-maxNumOfDL-PRS-ResProcessedPerSlot-r17	SEQUENCE {

scs15-r17									ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }

OPTIONAL,

scs30-r17									ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }

OPTIONAL,

scs60-r17									ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }

OPTIONAL,

scs120-r17									ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }

OPTIONAL,

...

},

...,

[[

ppw-maxNumOfDL-Bandwidth-r17		CHOICE {

fr1										ENUMERATED {mhz5, mhz10, mhz20, mhz40,

mhz50, mhz80, mhz100},

fr2										ENUMERATED {mhz50, mhz100, mhz200, mhz400}

}																					OPTIONAL

]]

}

-- ASN1STOP

NR-DL-PRS-ProcessingCapability field descriptions

maxSupportedFreqLayers Indicates the maximum number of positioning frequency layers supported by UE.

simulLTE-NR-PRS Indicates whether the UE supports parallel processing of LTE PRS and NR PRS.

dummy This field is not used in the specification. If received it shall be ignored by the receiver.

supportedBandwidthPRS Indicates the maximum number of DL-PRS bandwidth in MHz, which is supported and reported by UE.

dl-PRS-BufferType Indicates DL-PRS buffering capability. Value type1 indicates sub-slot/symbol level buffering and value type2 indicates slot level buffering.

durationOfPRS-Processing Indicates the duration N of DL-PRS symbols in units of ms a UE can process every T ms assuming maximum DL-PRS bandwidth provided in supportedBandwidthPRS and comprises the following subfields: - durationOfPRS-ProcessingSymbols: This field specifies the values for N. Enumerated values indicate 0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50 ms. - durationOfPRS-ProcessingSymbolsInEveryTms: This field specifies the values for T. Enumerated values indicate 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280 ms. See NOTE.

maxNumOfDL-PRS-ResProcessedPerSlot Indicates the maximum number of DL-PRS resources that UE can process in a slot. SCS: 15 kHz, 30 kHz, 60 kHz are applicable for FR1 bands. SCS: 60 kHz, 120 kHz are applicable for FR2 bands.

supportedDL-PRS-ProcessingSamples-RRC-CONNECTED Indicates the UE capability for support of measurements based on measuring M=1 or M=2 (instances) of a DL-PRS Resource Set. The UE can include this field only if the UE supports prs-ProcessingCapabilityBandList. Otherwise, the UE does not include this field. NOTE: This feature is supported for both UE-assisted and UE based positioning.

prs-ProcessingWindowType1A Indicates the supported DL-PRS processing types subject to the UE determining that DL-PRS to be higher priority for DL-PRS measurement outside MG and in a DL-PRS Processing Window. Type 1A refers to the determination of prioritization between DL-PRS and other DL signals/channels in all OFDM symbols within the PRS Processing Window. The DL signals/channels from all DL CCs (per UE) are affected across LTE and NR. Enumerated value indicates supported priority handing options of DL-PRS: - option1: UE indicates support of two priority states. - State 1: DL-PRS is higher priority than all PDCCH/PDSCH/CSI-RS - State 2: DL-PRS is lower priority than all PDCCH/PDSCH/CSI-RS - option2: UE indicates support of three priority states - State 1: DL-PRS is higher priority than all PDCCH/PDSCH/CSI-RS - State 2: DL-PRS is lower priority than PDCCH and URLLC PDSCH and higher priority than other PDSCH/CSI-RS Note: The URLLC channel corresponds a dynamically scheduled PDSCH whose PUCCH resource for carrying ACK/NAK is marked as high-priority. - State 3: DL-PRS is lower priority than all PDCCH/PDSCH/CSI-RS - option3: UE indicates support of single priority state - State 1: DL-PRS is higher priority than all PDCCH/PDSCH/CSI-RS The UE can include this field only if the UE supports prs-ProcessingCapabilityBandList. Otherwise, the UE does not include this field. NOTE: Within a PRS processing window, UE measurement is inside the active DL BWP with PRS having the same numerology as the active DL BWP.

prs-ProcessingWindowType1B Indicates the supported DL-PRS processing types subject to the UE determining that DL-PRS to be higher priority for DL-PRS measurement outside MG and in a DL-PRS Processing Window. Type 1B refers to the determination of prioritization between DL-PRS and other DL signals/channels in all OFDM symbols within the PRS processing window. The DL signals/channels from a certain band are affected. Enumerated value indicates supported priority handing options of DL-PRS (see prs-ProcessingWindowType1A). The UE can include this field only if the UE supports prs-ProcessingCapabilityBandList. Otherwise, the UE does not include this field. NOTE: Within a PRS processing window, UE measurement is inside the active DL BWP with PRS having the same numerology as the active DL BWP.

prs-ProcessingWindowType2 Indicates the supported DL-PRS processing types subject to the UE determining that DL-PRS to be higher priority for DL-PRS measurement outside MG and in a DL-PRS Processing Window. Type 2 refers to the determination of prioritization between DL-PRS and other DL signals/channels only in DL-PRS symbols within the PRS processing window. Enumerated value indicates supported priority handing options of DL-PRS (see prs-ProcessingWindowType1A). The UE can include this field only if the UE supports prs-ProcessingCapabilityBandList. Otherwise, the UE does not include this field. NOTE: Within a PRS processing window, UE measurement is inside the active DL BWP with PRS having the same numerology as the active DL BWP.

prs-ProcessingCapabilityOutsideMGinPPW Indicates the DL-PRS Processing Capability outside MG and comprises the following subfields: - prsProcessingType: Indicates the DL-PRS Processing Window Type for which the prs-ProcessingCapabilityOutsideMGinPPW are provided. - ppw-dl-PRS-BufferType: Indicates DL-PRS buffering capability. Value 'type1' indicates sub-slot/symbol level buffering and value 'type2' indicates slot level buffering. - ppw-durationOfPRS-Processing1: Indicates the duration of DL-PRS symbols N in units of ms a UE can process every T ms assuming maximum DL-PRS bandwidth provided in ppw-maxNumOfDL-Bandwidth and comprises the following subfields: - ppw-durationOfPRS-ProcessingSymbolsN: This field specifies the values for N. Enumerated values indicate 0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50 ms. - ppw-durationOfPRS-ProcessingSymbolsT: This field specifies the values for T. Enumerated values indicate 1, 2, 4, 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280 ms. - ppw-durationOfPRS-Processing2: Indicates the duration of DL-PRS symbols N2 in units of ms a UE can process inT2 ms assuming maximum DL-PRS bandwidth provided in ppw-maxNumOfDL-Bandwidth and comprises the following subfields: - ppw-durationOfPRS-ProcessingSymbolsN2: This field specifies the values for N2. Enumerated values indicate 0.125, 0.25, 0.5, 1, 2, 3, 4, 5, 6, 8, 12 ms. - ppw-durationOfPRS-ProcessingSymbolsT2: This field specifies the values for T2. Enumerated values indicate 4, 5, 6, 8 ms. - ppw-maxNumOfDL-PRS-ResProcessedPerSlot: Indicates the maximum number of DL-PRS resources that UE can process in a slot. SCS: 15 kHz, 30 kHz, 60 kHz are applicable for FR1 bands. SCS: 60 kHz, 120 kHz are applicable for FR2 bands. - ppw-maxNumOfDL-Bandwidth: Indicates the maximum number of DL PRS bandwidth in MHz, which is supported and reported by UE for PRS measurement outside MG within the PPW. The UE can include this field only if the UE supports one of prs-ProcessingWindowType1A, prs-ProcessingWindowType1B and prs-ProcessingWindowType2. Otherwise, the UE does not include this field. NOTE: A UE that supports one of prs-ProcessingWindowType1, prs-ProcessingWindowType1B or prs-ProcessingWindowType2 defined in TS 38.331 [35] shall always support ppw-dl-PRS-BufferType, ppw-durationOfPRS-Processing1, ppw-durationOfPRS-Processing2, ppw-maxNumOfDL-PRS-ResProcessedPerSlot, and ppw-maxNumOfDL-BandwidthFR1 or ppw-maxNumOfDL-BandwidthFR2.

dl-PRS-BufferType-RRC-Inactive Indicates DL-PRS buffering capability in RRC_INACTIVE state. Value 'type1' indicates sub-slot/symbol level buffering and value 'type2' indicates slot level buffering.

durationOfPRS-Processing-RRC-Inactive Indicates the duration N of DL-PRS symbols in units of ms a UE can process every T ms in RRC_INACTIVE state assuming maximum DL-PRS bandwidth provided in supportedBandwidthPRS and comprises the following subfields: - durationOfPRS-ProcessingSymbols: This field specifies the values for N. Enumerated values indicate 0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50 ms. - durationOfPRS-ProcessingSymbolsInEveryTms: This field specifies the values for T. Enumerated values indicate 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280 ms. See NOTE.

maxNumOfDL-PRS-ResProcessedPerSlot-RRC-Inactive Indicates the maximum number of DL-PRS resources a UE can process in a slot in RRC_INACTIVE state. SCS: 15 kHz, 30 kHz, 60 kHz are applicable for FR1 bands. SCS: 60 kHz, 120 kHz are applicable for FR2 bands.

supportedLowerRxBeamSweepingFactor-FR2 Indicates support of the lower Rx beam sweeping factor than 8 for FR2. Enumerated value indicates the number of Rx beam sweeping factors supported.

supportedDL-PRS-ProcessingSamples-RRC-Inactive Indicates the UE capability for support of measurements based on measuring M=1 or M=2 samples (instances) of a DL-PRS Resource Set in RRC_INACTIVE state. The UE can include this field only if the UE supports prs-ProcessingRRC-Inactive defined in TS 38.331 [35]. Otherwise, the UE does not include this field.

NOTE:	When the target device provides the durationOfPRS-Processing capability (N, T) for any P(≥T) time window defined in TS 38. 214 [45] clause 5.1.6.5, the target device should be capable of processing all DL-PRS resources within P, if

-	N≥K where K is defined in the TS 38.214 [45] clause 5.1.6.5, and

-	the number of DL-PRS Resources in each slot does not exceed the maxNumOfDL-PRS-ResProcessedPerSlot, and

-	the configured measurement gap and a maximum ratio of measurement gap length (MGL) / measurement gap repetition period (MGRP) is as specified in TS 38.133 [46].

NEXT OF CHANGE

–	NR-DL-PRS-ResourcesCapability

The IE NR-DL-PRS-ResourcesCapability defines the DL-PRS resources capability for each positioning method. The UE can include this IE only if the UE supports NR-DL-PRS-ProcessingCapability. Otherwise, the UE does not include this IE.

-- ASN1START

NR-DL-PRS-ResourcesCapability-r16 ::= SEQUENCE {

maxNrOfDL-PRS-ResourceSetPerTrpPerFrequencyLayer-r16

INTEGER (1..2),

maxNrOfTRP-AcrossFreqs-r16					ENUMERATED { n4, n6, n12, n16, n32,

n64, n128, n256, ..., n24-v16xy},

maxNrOfPosLayer-r16							INTEGER (1..4),

dl-PRS-ResourcesCapabilityBandList-r16		SEQUENCE (SIZE (1..nrMaxBands-r16)) OF

DL-PRS-ResourcesCapabilityPerBand-r16,

dl-PRS-ResourcesBandCombinationList-r16		DL-PRS-ResourcesBandCombinationList-r16,

...

}

DL-PRS-ResourcesCapabilityPerBand-r16 ::= SEQUENCE {

freqBandIndicatorNR-r16						FreqBandIndicatorNR-r16,

maxNrOfDL-PRS-ResourcesPerResourceSet-r16	ENUMERATED { n1, n2, n4, n8, n16, n32, n64, ...},

maxNrOfDL-PRS-ResourcesPerPositioningFrequencylayer-r16

ENUMERATED { n6, n24, n32, n64, n96, n128,

n256, n512, n1024, ...},

...

}

DL-PRS-ResourcesBandCombinationList-r16 ::=	SEQUENCE (SIZE (1..maxBandComb-r16)) OF

DL-PRS-ResourcesBandCombination-r16

DL-PRS-ResourcesBandCombination-r16 ::=	SEQUENCE {

bandList-r16							SEQUENCE (SIZE (1..maxSimultaneousBands-r16)) OF

FreqBandIndicatorNR-r16,

maxNrOfDL-PRS-ResourcesAcrossAllFL-TRP-ResourceSet-r16

CHOICE {

fr1-Only-r16							ENUMERATED {n6, n24, n64, n128, n192,

n256, n512, n1024, n2048},

fr2-Only-r16							ENUMERATED {n24, n64, n96, n128, n192,

n256, n512, n1024, n2048},

fr1-FR2Mix-r16							SEQUENCE {

fr1-r16									ENUMERATED {n6, n24, n64, n96, n128,

n192, n256, n512, n1024, n2048},

fr2-r16									ENUMERATED {n24, n64, n96, n128, n192,

n256, n512, n1024, n2048},

...

},

...

},

...

}

-- ASN1STOP

NR-DL-PRS-ResourcesCapability field descriptions

maxNrOfDL-PRS-ResourceSetPerTrpPerFrequencyLayer Indicates the maximum number of DL-PRS Resource Sets per TRP per positioning frequency layer supported by UE.

maxNrOfTRP-AcrossFreqs Indicates the maximum number of TRPs across all positioning frequency layers.

maxNrOfPosLayer Indicates the maximum number of supported positioning frequency layers.

dl-PRS-ResourcesBandCombinationList Provides the capabilities of DL-PRS Resources for the indicated band combination in bandList. This field is provided for all band combinations for which the target device supports DL-PRS.

maxNrOfDL-PRS-ResourcesPerResourceSet Indicates the maximum number of DL-PRS Resources per DL-PRS Resource Set. Value 16, 32, 64 are only applicable to FR2 bands. Value 1 is not applicable for DL-AoD.

maxNrOfDL-PRS-ResourcesPerPositioningFrequencylayer Indicates the maximum number of DL-PRS resources per positioning frequency layer. Value 6 is only applicable to FR1 bands.

maxNrOfDL-PRS-ResourcesAcrossAllFL-TRP-ResourceSet Indicates the maximum number of DL-PRS Resources supported by UE across all frequency layers, TRPs and DL-PRS Resource Sets. fr1-Only: This is applicable for FR1 only band combinations; fr2-Only: This is applicable for FR2 only band combinations; fr1-FR2Mix: This is applicable for band combinations containing FR1 and FR2 bands. fr1 means for FR1 in FR1/FR2 mixed operation, and fr2 means for FR2 in FR1/FR2 mixed operation.

END OF CHANGE