3GPP TSG RAN WG3 Meeting #120			R2-2211422

Toulouse, France, 14th – 18th Nov, 2022

Source: 	CATT

Title: 	Discussion on the LS on GNSS integrity requirement provisioning

Agenda item:	6.11.1

Document for: 	Discussion

Introduction

SA2 sent an LS on GNSS integrity requirement provisioning in R2-2211137(S2-2209966). SA2 has one question to RAN2 and CC SA1. This contribution will discuss this LS from RAN2’s perspective.

Discussion

The remaining issue in SA2 is on what parameters can be provided by the LCS client/AF or UE to the GMLC or AMF, and further forwarded to LMF.

In particular, the remaining issue is on what parameters can be provided by the LCS client/AF or UE to the GMLC or AMF, and further forwarded to LMF. LMF may use the parameters to determine to use GNSS positioning method for integrity calculation/determination, and other behaviour corresponding to TS 38.305 clause 8.1.3.3.

During the discussion, some companies believe the following parameters can be provided by LCS client/AF, or UE, i.e. Target Integrity Risk, Time To Alert and Alert Limit. However, other company would like RAN2 to evaluate whether these parameters are agreeable, and whether there are other parameters can be provided by LCS client/AF, or UE.

According to the definition of Positioning integrity in TS 38.305, the ability to provide associated alerts means the alerts to LCS client or LCS client report alerts itself.

Positioning integrity: A measure of the trust in the accuracy of the position-related data and the ability to provide associated alerts.

So it seems that there are two ways to report the alarm of integrity. One mode is to report from LMF which requires Time to Alert and Alert Limit from LCS client. The other mode is to report the alarm by LCS client itself where Time to Alert and Alert Limit is not required by LMF. In order to support two possible modes, we recommend to provide the parameters by LCS client/AF, or UE to LMF, i.e. Target Integrity Risk, Time To Alert and Alert Limit.

Proposal 1: RAN2 to agree to provide the parameters by LCS client/AF, or UE to LMF, i.e. Target Integrity Risk, Time To Alert and Alert Limit to LMF.

However the statement about “LMF may use the parameters to determine to use GNSS positioning method for integrity calculation/determination, and other behaviour corresponding to TS 38.305 clause 8.1.3.3.” in this LS is not supported by us. There is no proof shown that different GNSS positioning methods will have different integrity service levels which means different Target Integrity Risk, Time To Alert and Alert Limit. Then how LMF can determine some specific GNSS positioning method for integrity calculation/determination with the parameters?

So we propose to clarify the usage of parameters in the reply LS to SA2:

Proposal 2: RAN2 to recommend that LMF may use the parameters to provide associated alerts to LCS client, rather than determine to use GNSS positioning method for integrity calculation/determination

Companies also discussed whether the parameters should be provided to UE as well. According to the analysis above, there is no proof to show that UE needs these parameters for alarm because the alarm will eventually reported by LMF either in LMF-Based or UE-Based protection level calculation.

Proposal 3: RAN2 to agree that these parameters from LCS client won’t be provided to UE, i.e. Time To Alert and Alert Limit, unless there is a clear proof of the usage of these parameters in UE side.

Conclusion

In this contribution, we provided our views on the LS from SA2 in R2-2211137(S2-2209966), and propose to give the below answers to SA2 in the reply LS:

Proposal 1: RAN2 to agree to provide the parameters by LCS client/AF, or UE to LMF, i.e. Target Integrity Risk, Time To Alert and Alert Limit to LMF.

Proposal 2: RAN2 to recommend that LMF may use the parameters to provide associated alerts to LCS client, rather than determine to use GNSS positioning method for integrity calculation/determination

Proposal 3: RAN2 to agree that these parameters from LCS client won’t be provided to UE, i.e. Time To Alert and Alert Limit, unless there is a clear proof of the usage of these parameters in UE side.

References

[1] TS 38.305 Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN V17.2.0

[2] R2-2211137 LS on GNSS integrity requirement provisioning (S2-2209966; contact: Huawei), RAN2 #120, Nov 2022.