3GPP TSG-RAN WG2 Meeting #120                                                        R2-2211436

Toulouse, France, 14th – 18th Nov, 2022

Source:              CATT

Title:	             XR awareness for PDU sets and bursts

Agenda Item:	8.5.2.1

Document for:	Discussion and Decision

Introduction

In RAN2#119bis-e, the following was agreed regarding XR awareness:

1: From RAN2 viewpoint, the following information would be useful for PDU set handling in UL and DL: Semi-static information (from CN to RAN): At least PSER and PSDB. Dynamic information: At least identifying which PDU belongs to which data burst/PDU set is also needed, including means to determine at least PDU set boundaries. Discuss whether to send LS on AS re-ordering once we discuss bearer handling.

In the meantime, SA2 further progressed their design of the set of XR parameters that should be visible by RAN [1], so that by the end of the e-meeting, RAN2 concluded:

1. RAN2 discuss whether additional traffic or QoS related information on downlink traffic beyond what has been agreed by SA2 needs to be provided to RAN for UE power savings. 2. RAN2 study what traffic and QoS related information on uplink traffic (e.g. counterpart of what has been agreed by SA2) should be provided to RAN for UE power savings and how the information may be provided to RAN. Capture in TR that traffic parameters and Jitter are semi-static info. Can capture also SA2 agreements related to how they impact RAN2.

This contribution addresses above points #1, #2, as well as the last agreement point. For the latter point, we first summarize the SA2 progress and draw related conclusions from RAN2 perspective.

Discussion

  - Summary of SA2/SA4 progress and RAN2 impacts

    - QoS flow / burst level

TR 23.700-60 [2] captures the XR information to be provided to RAN at QoS flow level:

8.8 Interim Conclusion for Key Issue #8 The following information, to be provided to the NG-RAN at PDU Session Establishment/Modification via an NGAP Message, is taken as baseline for normative work: - Periodicity for UL and DL traffic of the QoS Flow. In addition to integer periodicity values, non-integer values associated to, e.g., 45FPS, 60 FPS, 90FPS, 120FPS, shall be supported. Such information shall be exchanged by re-using/extending the TSCAI/TSCAC definitions in TS 23.501 clause 5.27.2.1. Editor's Note: If the PDU sets with the Periodicity (e.g., 45FPS, 60 FPS, 90FPS, 120FPS) are mapped into different QoS Flows, it is FFS whether the same Periodicity still exists and whether the same Periodicity can be used for the UL and DL traffic of each QoS Flow. […] - Traffic jitter information associated with each periodicity. […] The following information for DL traffic, to be provided to the NG-RAN with in-band signaling via GTP-U header, is taken as baseline for normative work: - Optional, End of Data Burst indication in the header of the last PDU of the Data Burst.

From the above, one can observe that SA2 supports the following parameters for characterizing a QoS flow carrying bursts of an XR video stream:

At control plane level (semi-static information) in both UL & DL:

- Non-integer periodicity values and jitter associated with each periodicity

At user plane level (dynamic information) in DL only:

- End of Data Burst indication in the header of the last PDU of the Data Burst

It can be noted that the user plane parameter (end of data burst) is only defined in DL. However, the same information can also be useful in UL for letting know UE/RAN about the end of an UL burst. This information can be leveraged e.g. for power saving purpose. Given it is very much dependent from the application, its determination is left to UE implementation.

Observation 1: SA2 supports the following parameters for characterizing a QoS flow carrying bursts of an XR video stream:

At control plane level (semi-static information) in both UL & DL:

- Non-integer periodicity values and jitter associated with each periodicity

At user plane level (dynamic information) in DL only:

- End of Data Burst indication in the header of the last PDU of the Data Burst

Proposal 1: The End of Data Burst indication for a QoS flow is also supported in UL and its determination is left to UE implementation.

It is also interesting to notice that the Editor’s Note points out that, in case an XR video stream is further split into different types of PDU sets, where PDU sets of different types are mapped onto different QoS flows, the periodicity associated with such QoS flows may not be defined.

This makes sense considering the following response from SA4 to SA2 about the characterization of I and P/B frames (typically expected to be mapped on different PDU sets) in their reply LS [3]:

Q5: SA2 would like to ask SA4 whether the following scenario exists for some XR service flow: The non-I frames (e.g., P frame or B frame) transmitted/decoded between two successive I frames directly or indirectly refers to the 1st I frame of the two successive I frames? SA4 response: - Assuming that SA2 is asking about frames encoded between 2 IRAP (Intra Random Acces Point) frames, this configuration may exist if there is a good motivation to provide such restrictions. However, this is not likely to be used for low delay applications such as XR in which; there is no regular insertion of IRAP frames; there is the possibility to reference more IRAP frames from the past (e.g., long term references); the encoding configuration excludes delays in decoding time compared to presentation time (e.g., conventional bi-directional prediction of B frames). - The direct/indirect approach of defining reference frames is outdated by the most recent video codecs in 3GPP specifications (e.g., a picture could be referencing an IRAP picture as well as a number of P pictures). In such a case it could be neither “direct” nor “indirect” according to the suggested terminology. - Generally, in well-operated low-latency XR services SA4 does not expect that IRAP frames are added on regular basis, nor does SA4 expect that any fixed or typical dependence structures are applied. Video encoder operation is left to implementation, 3GPP does not define any detailed requirements.

From the above, it is clear that an XR video stream is periodic and can be a composite of different types of frames, but there is no clear periodicity associated with the different frame types.

Observation 2: An XR video stream is periodic in nature, but there is no clear periodicity associated with the different frame types (e.g. I/P frames) it is composed of.

    - PDU set level

TR 23.700-60 [2] captures the XR information to be provided to RAN at PDU set level in DL. This information is split into control plane and user plane.

For control plane information (semi-static) TR 23.700-60 [2] captures the following:

8.4.1 Control plane enhancements for supporting PDU Set in downlink - 8.4.1.1 PDU Set QoS Parameters PDU Set QoS treatment is determined using dynamic or non-dynamic PCC. The following PDU Set QoS parameters are defined to support PDU Set handling: - PDU Set Error Rate: The PSER defines an upper bound for the ratio between the number of PDU Sets not successfully received and the total number of PDU Sets sent towards a recipient measured over a measurement window. Editor’s Note: the criteria for determining whether a PDU Set is successfully delivered or not are FFS - PDU Set Delay Budget Editor’s Note: The definitions of PSER and PSDB are FFS. For PSDB, it needs further study the impact due to N6 jitter. - Whether all PDUs are needed for the usage of PDU Set by application layer (PDU Set Integrated Indication). Editor’s Note: It is FFS “Whether a PDU Set is still valid in case PSDB is exceeded” is needed. It should be discussed together with the definition of PSDB, specially about the boundary of PSDB. If PDU Set based QoS handling is used, PCF determines the above PDU Set QoS Parameters based on information provided by AF (described in 8.4.2) and/or local configuration. The PDU Set QoS parameters are sent to SMF as part of PCC rule, then SMF sends them to RAN. - 8.4.1.2 AF Information Provisioning PDU Set related assistance information provisioning by AF is supported for dynamic PCC. AF may provision one or more of the following PDU Set related assistance information to NEF/PCF during AF QoS request procedure: - PDU Set QoS parameters listed in clause 8.4.1.1. - Burst periodicity

Observation 3: SA2 supports the following control plane (semi-static information) QoS parameters for characterizing PDU set types from an XR traffic in DL:

- PSER, PSDB, PSII (PDU Set Integrated Indication)

These control plane parameters reflect the QoS requirements of different PDU set types, associated with different types of video frames or slices. Since these types of video frames or slices also require the same QoS differentiation in the UL video stream, these parameters should also be supported for UL PDU set types. These should be determined by the CN, in the same manner as for the DL (e.g. by AF). Alternately, since the application layer for UL traffic is in the UE side, it could be left to UE implementation to determine them, and the UE would inform the network via UAI.

Proposal 2: PSER, PSDB and PSII (PDU Set Integrated Indication) are also supported in UL.

Proposal 3: In UL, PSER, PSDB and PSII are determined, as a baseline, by CN as for DL, or alternately by UE implementation in which case they are communicated to the network via UAI.

It can be observed that, although the AF may provision to NEF/PCF, on top of the above parameters, the Burst periodicity, SA2 preferred to keep this information visible to RAN at QoS flow level only. This makes sense since, per observation #2, the burst periodicity of a PDU set type can only be defined if that PDU set type is mapped on ALL frames (i.e. I&P) of the video traffic.

Observation 4: Burst periodicity is only provided to RAN at QoS flow level, not at PDU set level.

For user plane information (dynamic), TR23.700-60 [2] captures the following:

- 8.4.2.1 PDU Set Information The following PDU Set related information may be identified by UPF to support PDU Set based handling: - PDU Set Identifier NOTE 1: Neighbor PDU Sets in sequence will use different PDU Set identifiers. - Optional, Start PDU and End PDU of the PDU Set - PDU SN within a PDU Set - Optional, PDU Set Size NOTE 2: Either PDU Set Size expressed in bytes or PDU Set Size expressed as number of PDUs, needs further determined. NOTE 3: Either one among Start/End PDU of the PDU Set and Number of PDUs within a PDU Set needs to be supported. - PDU Set Importance Editor’s Note: Which above PDU Set information parameters is optional is FFS. - 8.4.2.2 PDU Set Information identification on UPF and supported N6 protocols The detection and marking of the DL PDU Sets sent to the NG-RAN shall be done by the PSA UPF. PSA UPF may identify the PDU Set based on instruction from SMF and packet header of N6 protocols: - by matching RTP/SRTP header and payload (RFC 3550/3711/6184/7798/draft-ietf-avtcore-rtp-vvc/draft-ietf-avtext-framemarking are supported). Editor’s Note: Whether support PDU Set identification information in new RTP is pending to SA4 5G_RTP WI. NOTE 1: In above cases, it is assumed that the RTP/SRTP header and/or payload necessary for the identification of PDU Set Information is not encrypted. - by UPF implementation, e.g., PDU Set detection based on traffic characteristics. IP header parameters DSCP/TOS, IP port, IPv6 flow label may be used to detect PDU set, however detailed mechanisms in UPF for PDU Set information identification will not be standardized. Editor’s Note: Other N6 protocols, i.e. HTTP/MASQUE, GTP-U, IP/TCP/UDP/QUIC options, carrying PDU Set information are FFS. (Potential SoH) - 8.4.2.3 Delivering PDU Set Information to RAN PDU Set Information (listed in 8.4.2.1) are informed by UPF to RAN via GTP-U header of user plane packet. Editor’s Note: Whether PDU Set importance is used for mapping different QoS Flows, sub-QoS Flows, or included in GTP-U header is FFS. (Potential SoH)

Observation 5: SA2 supports the following user plane (dynamic information) parameters for characterizing PDU sets from an XR traffic in DL:

- (FFS which is optional) PDU Set Identifier, Start PDU and End PDU, PDU set size (FFS bytes or nb of PDUs), PDU set importance

- Above parameters are informed by UPF to RAN via GTP-U header of user plane packet

Same for control plane parameters, these parameters are equally important in UL, as agreed in the last RAN2 e-meeting. And their determination can be left to UE implementation.

Proposal 4: The same user plane (dynamic information) parameters agreed by SA2 for characterizing PDU sets from an XR stream in DL are also supported in UL.

Proposal 5: As a baseline, the user plane (dynamic information) parameters for characterizing PDU sets from an XR stream in UL are determined by UE implementation.

The current version of the TR [6] only describes a subset of the above awareness information agreed by SA2, so we propose complementing it based on the above observations.

Proposal 6: Capture the above observations in the TR with the TP provided in Annex.

  - Need for additional traffic or QoS related information

From RAN perspective, we don’t see the need for any additional information on top of what SA2 agreed for DL. Indeed, we acknowledge that the above XR awareness parameters meet the RAN2 agreed requirements captured in Section 1, and Proposals #1-6 address their UL counterpart. The only clarification that would be needed is related to the “importance” parameter. In the meantime, we suggest taking it, from RAN2 perspective, as a priority value associated with a PDU set.

Proposal 7: RAN2 do not see the need for any additional XR awareness information in DL, on top of what SA2 agreed.

Proposal 8: RAN2 interprets the PDU set importance as a priority value associated with the PDU set, until SA2 provides more clarification about it.

The next question is whether the dynamic PDU set and burst information delivered by CN in the user plane is still required beyond the gNB scheduler or, in other words, whether the UE needs to be aware of DL PDU set and bursts. From our perspective, in DL, PDU set identification within a DRB is only useful for DL scheduling (either dynamic or for mapping onto SPS). Therefore, it seems not useful that the dynamic PDU set information be carried over Uu. Considering this would have a potential big impact on the legacy L2 user plane stack, we would prefer to keep it unchanged.

Proposal 9: Unless it is shown that UE and gNB must be aware of the dynamic information of PDU sets and bursts in DL and UL, respectively, RAN2 should aim at keeping the legacy L2 user plane stack unchanged.

Conclusion

According to the analysis in section 2, we have the following observations and proposals:

Observation 1: SA2 supports the following parameters for characterizing a QoS flow carrying bursts of an XR video stream:

At control plane level (semi-static information) in both UL & DL:

- Non-integer periodicity values and jitter associated with each periodicity

At user plane level (dynamic information) in DL only:

- End of Data Burst indication in the header of the last PDU of the Data Burst

Proposal 1: The End of Data Burst indication for a QoS flow is also supported in UL and its determination is left to UE implementation.

Observation 2: An XR video stream is periodic in nature, but there is no clear periodicity associated with the different frame types (e.g. I/P frames) it is composed of.

Observation 3: SA2 supports the following control plane (semi-static information) QoS parameters for characterizing PDU set types from an XR traffic in DL:

- PSER, PSDB, PSII (PDU Set Integrated Indication)

Proposal 2: PSER, PSDB and PSII (PDU Set Integrated Indication) are also supported in UL.

Proposal 3: In UL, PSER, PSDB and PSII are determined, as a baseline, by CN as for DL, or alternately by UE implementation in which case they are communicated to the network via UAI.

Observation 4: Burst periodicity is only provided to RAN at QoS flow level, not at PDU set level.

Observation 5: SA2 supports the following user plane (dynamic information) parameters for characterizing PDU sets from an XR traffic in DL:

- (FFS which is optional) PDU Set Identifier, Start PDU and End PDU, PDU set size (FFS bytes or nb of PDUs), PDU set importance

- Above parameters are informed by UPF to RAN via GTP-U header of user plane packet

Proposal 4: The same user plane (dynamic information) parameters agreed by SA2 for characterizing PDU sets from an XR stream in DL are also supported in UL.

Proposal 5: As a baseline, the user plane (dynamic information) parameters for characterizing PDU sets from an XR stream in UL are determined by UE implementation.

Proposal 6: Capture the above observations in the TR with the TP provided in Annex.

Proposal 7: RAN2 do not see the need for any additional XR awareness information in DL, on top of what SA2 agreed.

Proposal 8: RAN2 interprets the PDU set importance as a priority value associated with the PDU set, until SA2 provides more clarification about it.

Proposal 9: Unless it is shown that UE and gNB must be aware of the dynamic information of PDU sets and bursts in DL and UL, respectively, RAN2 should aim at keeping the legacy L2 user plane stack unchanged.

References

- R2-2211041/S2-2209979, LS on XR and Media Services, SA2.

- TR 23.700-60, Study on XR (Extended Reality) and media services, V1.2.0

- S4aV220921, Reply LS on further details on XR traffic, SA4.

- R2-2210814, TR 38.335 v030, Nokia

Annex: TP to TR 38.835

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], TS 38.300 [15] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and in TS 38.300 [15].

3DoF	Three Degrees of Freedom

6DoF	Six Degrees of freedom

AR	Augmented Reality

DASH	Dynamic Adaptive Streaming over HTTP

FEC	Forward Error Coding

FoV	Field of view

FPS	Frames Per Second

GBR	Guaranteed Bit Rate

GFBR	Guaranteed Flow Bit Rate

HEVC	High-Efficiency Video Coding

HMD	Head-Mounted Display

HUD	Heads-Up Display

PDB	Packet Delay Budget

PDU	Protocol Data Unit

PER	Packet Error Rate

PSDB	PDU-Set Delay Budget

PSER	PDU-Set Error Rate

PSII	PDU-Set Integrated Indication

QCI	QoS Class Identifier

QFI	QoS Flow ID

QoE	Quality of Experience

QoS	Quality of Service

VR	Virtual Reality

XR	Extended reality

************************* Next change ***********************

4.3.1	Video

XR content may be represented in different formats, e.g. panoramas or spheres depending on the capabilities of the capture systems. Since modern video coding standards are not designed to handle spherical content. projection is used for conversion of a spherical (or 360°) video into a two-dimensional rectangular video before the encoding stage. After projection, the obtained two-dimensional rectangular image can be partitioned into regions (e.g. front, right, left, back, top, bottom) that can be rearranged to generate "packed" frames to increase coding efficiency or viewport dependent stream arrangement.

There are mainly three approaches that can be considered for 360 video delivery (see TR 26.918 [5]):

-	Single stream approach: the single stream approach consists in providing the full 360 video and showing the interesting part only. Solutions that lie within this group have the drawback that either they may not be scalable or they may impose a big challenge in terms of required network resources (high bitrate of high resolution video) and required processing at the client side (decode a very high resolution video).

-	Multi-stream approach: the multi-stream approach consists of encoding several streams, each of them emphasizing a given viewport and making them available for the receiver, so that the receiver decides which stream is delivered at each time instance.

-	Tiled stream approach: the tiled stream approach consists in emphasizing the current user viewport through transmitting non-viewport samples with decreased resolution. The tiles can be provided as one common bitstream (using motion-constrained HEVC tiles) or as separate video streams.

In modern video codecs (see S4-220505 [13]):

-	Complex prediction structures are used that take into account application constraints, encoding complexity, latency and dynamic decisions in the encoding. This may result in irregularities, for example based on sequence properties. In particular for low-latency delivery with error resiliency, different flavours of encoding operations are in use, and the concept of I, P and B pictures is not generally applicable.

-	All PDUs in a PDU Set are needed by the application layer to use the corresponding unit of information in some implementations; while in some others, receivers may use the data up to the first lost fragmentation unit to recover at least parts of the video data and apply error concealment afterward.

-	Furthermore, in motion-compensated predicted video decoding, some frames refer to other frames based on the video encoding configuration but also based on dynamic operational decisions. As a consequence, a PDU Set may “depend” on previously received PDU Sets. However, such dependencies do not necessarily result in discarding dependent information units.

From the above descriptions, it can be derived that, if an XR video stream is periodic in nature, there is no clear periodicity associated with the different frame types (e.g. I/P/B frames) it is composed of.

************************* Next change ***********************

5.1		XR Awareness

5.1.1	General

In both uplink and downlink, XR-Awareness contributes to optimizations of gNB radio resource scheduling and relies at least on the notions of PDU set and Data Burst (see TR 23.700-60 [9]): a PDU Set is composed of one or more PDUs carrying the payload of one unit of information generated at the application level (e.g. a frame or video slice), while a Data Burst is a set of data PDUs generated and sent by the application in a short period of time.

NOTE:	A Data Burst can be composed of multiple PDUs belonging to one or multiple PDU Sets.

In order to handle PDUs efficiently in both UL and DL, the following information is provided by CN to RAN:

Parameters for characterizing a QoS flow carrying bursts of an XR traffic:

Semi-static information provided by the CN (UL and DL):

-	Burst periodicity (possibly non-integer);

-	Traffic jitter information associated with the periodicity.

NOTE:	If an XR video stream is further split into different types of PDU sets, where PDU sets of different types are mapped onto different QoS flows, the periodicity associated with such QoS flows may not be defined.

Dynamic information (DL only):

-	End of Data Burst indication in the header of the last PDU of the Data Burst

Similarly, the End of Data Burst indication is determined for an UL QoS flow by UE implementation.

Parameters for characterizing a PDU set (UL and DL):

Semi-static information provided by the CN:

-	The PDU-Set Delay Budget (PSDB);

-	The PDU-Set Error Rate (PSER).

-	PDU Set Integrated Indication (PSII)

DL dynamic information informed by UPF to RAN via GTP-U header of user plane packet (FFS which is optional):

-	PDU Set Identifier,

-	Start PDU and End PDU,

-	PDU set size (FFS bytes or nb of PDUs),

-	PDU set importance

Similarly, the same dynamic information is determined for each UL PDU set by UE implementation.