3GPP TSG-RAN WG2 Meeting #120		R2-2211447

Toulouse, France, November 14th – November 18th, 2022

Source:	CATT, CMCC

Title:	Discussion on EHC for DAPS

Agenda Item:	6.0.3

Document for:	Discussion and Decision

Introduction

In Rel-16 mobility, DAPS was discussed and specified. And EHC which was introduced for Rel-16 TSN, is beneficial to the header reduction for Ethernet PDU. But it was not agreed to configure EHC for DAPS in Rel-16 [1]-[4].

In this document, we analyze the feasibility on whether EHC can be configured for DAPS in Rel-17. And our observations and proposals are also provided.

Discussion

TSN is a set of standards to define mechanisms for the time-sensitive (i.e. deterministic) transmission of data over Ethernet networks. It is under development by the Time-Sensitive Networking task group of the IEEE 802.1 working group. And TSN which is one promising technique has aroused more and more attention from companies, like operators and network vendors. In Rel-16 IIoT, TSN impacts on RAN were identified and some enhancements related TSN were captured in specification.

In Rel-16 NR mobility enhancement, it was agreed in RAN2#109 meeting that [5]:

Not-supported features with DAPS

Proposal 14.	EHC is not considered for DAPS.

This was agreed considering there were no much mobility concerns in TSN network and there was not enough time left for NR mobility enhancements to further check the feasibility. However, in DAPS which maintains the source gNB connection after reception of RRC message for handover and until releasing the source cell after successful random access to the target gNB, brings great benefits on reducing the transmission interruption time and improving the reliability during handover. This can definitely be applied to TSN to optimize the mobility performance for TSN which has critical latency requirements as well as high reliability requirements.

Observation 1: DAPS is a good technique for addressing both latency and reliability requirements for TSN.

Hence, it is proposed that:

Proposal 1: TSN can be configured together with DAPS in Rel-17.

If it is agreed to support DAPS configured with TSN, EHC is requires to be supported for DAPS. And like ROHC configured for DAPS, the following issues need to be checked.

Issue 1: How many EHC compressor/decompressor instances can be configured for DAPS.

In Rel-16 NR mobility, the UE maintains two ROHC decompressor instances, i.e. one is used for source cell while the other is used for target cell, and one ROHC compressor instance for source cell before uplink data switching and for target cell after uplink data switching. This has been captured in [3].

### 5.7.2 Configuration of ROHC PDCP entities associated with DRBs and MRBs can be configured by upper layers TS 38.331 [3] to use ROHC. Each PDCP entity carrying user plane data may be configured to use ROHC. PDCP entities associated with sidelink DRBs can be configured to use ROHC for IP SDUs. For DRBs and MRBs other than DAPS bearers, the PDCP entity uses at most one ROHC compressor instance and at most one ROHC decompressor instance. For DAPS bearers, the PDCP entity uses at most one ROHC compressor instance (i.e. use the ROHC compressor instance for source cell before uplink data switching, and use the ROHC compressor instance for target cell after uplink data switching) and at most two ROHC decompressor instances.

Similar to ROHC, EHC can follow the design like ROHC. That is one uplink compressor instances and two EHC decompressor instances can be configured for EHC if EHC is supported for DAPS.

Issue 2: Whether EHC continuity is configured for DAPS.

EHC continuity is used to indicate whether the PDCP entity continues or resets the downlink/uplink EHC header compression protocol. It is useful to decrease the resource consumption during handover. In Rel-16 EHC, separate control for EHC continuity was introduced, i.e. drb-ContinueEHC-DL and drb-ContinueEHC-UL are defined to control whether to reset EHC header compression protocol for DL and UL respectively.

For ROHC, ROHC continuity is only configured in case of resuming an RRC connection or reconfiguration with sync, where the PDCP termination point is not changed and the fullConfig is not indicated. This restricts the ROHC continuity that can only be applied to intra-node handover. Some companies think this greatly impacts the scenarios where ROHC continuity can be configured in DAPS. And this was the reason why ROHC continuity was not agreed for DAPS in Rel-16. And this was captured in [4].

drb-ContinueROHC Indicates whether the PDCP entity continues or resets the ROHC header compression protocol during PDCP re-establishment, as specified in TS 38.323 [5]. This field is configured only in case of resuming an RRC connection or reconfiguration with sync, where the PDCP termination point is not changed and the fullConfig is not indicated. The network does not include the field if the bearer is configured as DAPS bearer. This field can be configured for both DRB and multicast MRB.

Like ROHC, EHC continuity is not supported for DAPS in Rel-17.

Issue 3: Packet loss

In order to guarantee the reliability for downlink, the network may perform duplication retransmission for DL DAPS. While there are two ROHC decompressor instances with different algorithms from the source gNB and target gNB, the UE should process two different decompressor procedures for the PDUs from source gNB and target gNB. However, based on the PDCP procedure, the UE performs reordering before decompression. It can be foreseen that plenty of duplicate packets may be discarded in reordering procedure. Hence, the decompressor contexts may be lost for source gNB or target gNB after reordering. Consequently, decompression failure happens frequently in DAPS. And more seriously, this can only be recovered by upper layer retransmission, e.g. TCP retransmission, since the packets have been placed in the PDCP reordering buffer and no PDCP retransmission will be triggered. In order to solve this issue, it is specified in PDCP that the ROHC protocol of the target cell should maintain the IR state if operating in U-mode and O-mode during DAPS handover before release of source cell for DAPS. And this has been captured in [3].

### 5.7.4 Header compression using ROHC If ROHC is configured, the ROHC protocol generates two types of output packets: - ROHC compressed packets, each associated with one PDCP SDU; - standalone packets not associated with a PDCP SDU, i.e. interspersed ROHC feedback. /omitted/ NOTE 2: For downlink, the ROHC protocol of the target cell should maintain the IR state if operating in U-mode and O-mode during DAPS handover before release of source cell.

If EHC is specified for DAPS, the reordering procedure for duplicated downlink packets may also bring EHC decompressor context missing issue. One simple solution is to make sure the state for EHC is kept as initial state during the DAPS handover.

As analysed above, the issues for introducing EHC in DAPS can be fixed by following the similar principles like ROHC. Hence, we see no great specification efforts for allowing it.

Observation 2: Small issues are identified for introducing EHC in DAPS and these issues can be solved by following ROHC configured together with DAPS in Rel-16.

Thus, we propose that:

Proposal 2: EHC can be configured together with DAPS in Rel-17.

The corresponding CRs can be found in [6] and [7].

Conclusion

In this document, we analyze the benefits and difficulties introducing EHC in DAPS. And we find the following two observations:

Observation 1: DAPS is a good technique for addressing both latency and reliability requirements for TSN.

Observation 2: Small issues are identified for introducing EHC in DAPS and these issues can be solved by following ROHC configured together with DAPS in Rel-16.

And we propose that:

Proposal 1: TSN can be configured together with DAPS in Rel-17.

Proposal 2: EHC can be configured together with DAPS in Rel-17.

Reference

- 36.323, V16.0.0, Evolved Universal Terrestrial Radio Access (E-UTRA); Packet Data Convergence Protocol (PDCP) specification

- 36.331, V16.0.0, Evolved Universal Terrestrial Radio Access (E-UTRA);Radio Resource Control (RRC);

- 38.323, V16.0.0, NR; Packet Data Convergence Protocol (PDCP) specification;

- 38.331, V16.0.0, NR; Radio Resource Control (RRC);

- RAN2#109 meeting report;

- R2-2211449, 38323_CR0106_(Rel-17)_R2-2211449 Configuration EHC for DAPS;

- R2-2211448, 38331_CR3601_(Rel-17)_R2-2211448 Configuration EHC for DAPS.