3GPP TSG-RAN WG2 Meeting #120	R2-2211503

# Toulouse, France, 14 – 18 November, 2022                  revision of R2-2209860

Agenda Item:	6.7.3

Source:	Ericsson

Title:	Alignment between remote UE paging DRX and relay UE Uu DRX

Document for:	Discussion, Decision

# 1	Introduction

In this paper, we discuss one remaining paging issue for L2 U2N relay.

# 2	Discussion

- In Rel-17, the network may send paging message intended for a remote UE in RRC IDLE/RRC INACTIVE to a relay UE in RRC CONNECTED via dedicated RRC signaling. The relay UE then forwards the paging message to the intended remote UE via the PC5 interface.

- However, the remote UE may be configured with paging DRX, while the relay UE may be configured with Uu DRX. The network pages the remote UE according to the remote UE’s paging occasions. If the relay UE is in Uu DRX. In this case, the remote UE’s paging DRX and the relay UE’s Uu DRX need to be aligned between each other. Otherwise, when the remote UE is being paged by the network, the relay may be in inactive state. so, the paging message will not be sent to the remote UE via the relay UE in time. This may cause additional latency. In addition, a paging message may be also become invalid if it is delayed too much.

The network may send paging message intended for a remote UE in RRC IDLE/RRC INACTIVE to a relay UE in RRC CONNECTED via dedicated RRC signalling.

Paging message carried via dedicated RRC signalling would be delayed if remote UE paging DRX and relay UE Uu DRX is not aligned.

To address the issue, the following option need to be adopted

Option 1: remote UE’s PO should be considered as DRX active time for relay UE

RAN2 needs to discuss which option to be adopted.

To avoid paging message being delayed for remote UE in RRC IDLE or RRC INACTIVE, remote UE’s POs are defined as DRX active time for relay UE in RRC CONNECTED

In another accompany paper, we have also proposed CR.

Adopt the CR captured in [1].

# 3 Conclusion

In the previous sections we made the following observations:

Observation 1	The network may send paging message intended for a remote UE in RRC IDLE/RRC INACTIVE to a relay UE in RRC CONNECTED via dedicated RRC signalling.

Observation 2	Paging message carried via dedicated RRC signalling would be delayed if remote UE paging DRX and relay UE Uu DRX is not aligned.

Based on the discussion in the previous sections we propose the following:

Proposal 1	To avoid paging message being delayed for remote UE in RRC IDLE or RRC INACTIVE, remote UE’s POs are defined as DRX active time for relay UE in RRC CONNECTED

Proposal 2	Adopt the CR captured in [1].

# 4 References

- R2-2211504 Correction to 38.321 on alignment between remote UE paging DRX and relay UE Uu DRX