3GPP TSG-RAN WG2 Meeting #120	R2-2211619

# Toulouse, France, 14 – 18 November 2022

Agenda item:		8.10.3

Source:	Intel Corporation

Title:	TDM solution for IDC

Document for:		Discussion and Decision

# Introduction

In RAN2#119-e meeting, following was agreed regarding TDM solutions for IDC:

⇒The use cases (e.g. BT voice, BT eSCO and WLAN beacon) as described in 3GPP TR 36.816 for LTE TDM solutions are considered for developing the Rel-18 IDC TDM solution in RAN2. ⇒Rel-18 IDC TDM solution(s) targets at resolving the adjacent channel interference issue and the intermodulation distortion interference issue, as LTE. ⇒As the baseline, the UE reports the TDM assistance information for IDC affected frequency list , as LTE. Note, this does not exclude MUSIM gap like solution. ⇒ RAN2 consider “hardware sharing indication” is out of scope. Company should bring this to RANP if want to support it.

Email discussion “[Post119-e][651][IDC] Comparison of TDM solutions (Xiaomi)” was held to analyze and compare TDM solutions.

In this contribution, we discuss TDM solutions for IDC in more detail.

# Recap on LTE TDM solution

Since LTE IDC solution should be considered as the baseline for NR IDC, it is helpful to review TDM solutions specified in LTE.

There are mainly two aspects in LTE IDC TDM: UE assistance information and network configuration.

- UE assistance information: The UE provides the eNB with desired DRX related parameters (in TDM-AssistanceInfo-r11) or subframe patterns to support TDM with other radios. Detailed ASN.1 is as below:

TDM-AssistanceInfo-r11 ::=	CHOICE {

drx-AssistanceInfo-r11				SEQUENCE {

drx-CycleLength-r11					ENUMERATED {sf40, sf64, sf80, sf128, sf160,

sf256, spare2, spare1},

drx-Offset-r11						INTEGER (0..255)	OPTIONAL,

drx-ActiveTime-r11					ENUMERATED {sf20, sf30, sf40, sf60, sf80,

sf100, spare2, spare1}

},

idc-SubframePatternList-r11			IDC-SubframePatternList-r11,

...

}

IDC-SubframePatternList-r11 ::=	SEQUENCE (SIZE (1..maxSubframePatternIDC-r11)) OF IDC-SubframePattern-r11

IDC-SubframePattern-r11 ::= CHOICE {

subframePatternFDD-r11				BIT STRING (SIZE (4)),

subframePatternTDD-r11				CHOICE {

subframeConfig0-r11					BIT STRING (SIZE (70)),

subframeConfig1-5-r11				BIT STRING (SIZE (10)),

subframeConfig6-r11					BIT STRING (SIZE (60))

},

...

}

There are two kinds of TDM assistance information depending on IDC scenario:

- Desired DRX related parameters: For coexistence with WiFi and Bluetooth Data (ACL, Asynchronous Connection-oriented Link), DRX assistance information (DRX cycle length, DRX starting offset, and DRX active time) is provided. In this scenario, TDM pattern length ranges from 40 ms to 256 ms.

Figure 5.2.1.2.1-1 of TR 36.816: Example of UE suggested TDM pattern

- HARQ subframe pattern: For coexistence with Bluetooth Voice (eSCO, extended Synchronous Connection Orientated), fine TDM pattern between LTE and Bluetooth is required. For Bluetooth eSCO, a clean Tx/Rx pair should be available for Bluetooth within EV3 interval, which is 3.75 ms (Annex B of TR 36.816 [1]).

Figure B.1.1-1: BT Tx/Rx pairs

HARQ subframe pattern is provided to take into account of UL synchronous HARQ. Note that LTE IDC was specified in Rel-11 and asynchronous HARQ was introduced in LTE Rel-13. The HARQ subframe pattern depends on duplex mode and TDD configuration.

Figure 5.2.1.2.2-1 of TR 36.816: Example of HARQ process reservation solution

- Solution configured by eNB to resolve IDC problem:

- Legacy DRX based solution is used as the TDM solution. It should be used in a predictable way, i.e. the eNB should ensure a predictable pattern of unscheduled periods by means of DRX mechanism.  eNB decides and signals the final DRX configuration to the UE (in RRCConnectionReconfiguration).

- Autonomous denial: The network may also configure autonomous denial to the UE, allow the UE to skip the scheduling in cellular part in order to protect important event in WiFi and Bluetooth.

Observation 1: In LTE IDC, DRX assistance information is provided as coarse granularity TDM pattern.

Observation 2: In LTE IDC, HARQ subframe pattern is provided as fine granularity TDM pattern.

Observation 3: In LTE IDC, DRX solution, scheduling avoidance or autonomous denial is configured by eNB when TDM assistance information is either DRX assistance information or HARQ subframe pattern.

# Discussion for NR IDC

## Comparison of solutions

In email discussion “[Post119-e][651][IDC] Comparison of TDM solutions (Xiaomi)”, following options are discussed:

- Option 1: DRX solution

- Option 2: MUSIM gap-like solution

- Option 3: UL and/or DL transmission occasion(s)

- Option 4: Autonomous denial solution

Since Option 4 is a complementary solution to address specific issues (e.g. protection of WiFi beacon), our discussion below is focused on Option 1, 2, and 3. It is better to decouple the assistance information and the applied solution. For example, as in Observation 3 above, in LTE, DRX solution, scheduling avoidance or autonomous denial can be configured by eNB when TDM assistance information is either DRX assistance information or HARQ subframe pattern. The same principle can be applied in NR. After gNB receives the assistance information, it is up to gNB implementation which solution can be applied, e.g. DRX, MUSIM or scheduling avoidance.

Proposal 1: For NR IDC TDM solution, the assistance data and applied solution should be decoupled.

Proposal 2: It is up to gNB implementation to choose which solution should be applied to solve IDC issue after receiving assistance information.

Regarding the assistance data, Option 1 and 2 are similar that a coarse granularity of on/off pattern is reported. Example assistance information for Option 1 in email discussion “[Post119-e][651][IDC] Comparison of TDM solutions (Xiaomi)” is as follows:

DRX-AssistanceInfo-r18 ::=              SEQUENCE {

drx-LongCycleStartOffset            CHOICE {

ms10                                INTEGER(0..9),

ms20                                INTEGER(0..19),

ms32                                INTEGER(0..31),

ms40                                INTEGER(0..39),

ms60                                INTEGER(0..59),

ms64                                INTEGER(0..63),

ms70                                INTEGER(0..69),

ms80                                INTEGER(0..79),

ms128                               INTEGER(0..127),

ms160                               INTEGER(0..159),

ms256                               INTEGER(0..255),

ms320                               INTEGER(0..319),

ms512                               INTEGER(0..511),

ms640                               INTEGER(0..639),

ms1024                              INTEGER(0..1023),

ms1280                              INTEGER(0..1279),

ms2048                              INTEGER(0..2047),

ms2560                              INTEGER(0..2559),

ms5120                              INTEGER(0..5119),

ms10240                             INTEGER(0..10239)

},

drx-SlotOffset-r18                      INTEGER (0..31),

drx-ActiveTime-r18    CHOICE {

subMilliSeconds INTEGER (1..31),

milliSeconds    ENUMERATED {ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40,            ms50, ms60, ms80, ms100, ms200, ms300, ms400, ms500, ms600, ms800, ms1000, ms1200, ms1600, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 }

}

}

Example assistance information for Option 2 in email discussion “[Post119-e][651][IDC] Comparison of TDM solutions (Xiaomi)” is as follows:

IDC-GapPreferenceList-r18 ::= SEQUENCE (SIZE (1..4)) OF IDC-GapInfo-r18

IDC-GapInfo-r18 ::=               SEQUENCE {

idc-Starting-SFN-AndSubframe-r18  IDC-Starting-SFN-AndSubframe-r18  OPTIONAL, -- Cond aperiodic

idc-GapLength-r18                 ENUMERATED {ms3, ms4, ms6, ms10, ms20}         OPTIONAL, -- Need S

idc-GapRepetitionAndOffset-r18    CHOICE {

ms20-r18                            INTEGER (0..19),

ms40-r18                            INTEGER (0..39),

ms80-r18                            INTEGER (0..79),

ms160-r18                           INTEGER (0..159),

ms320-r18                           INTEGER (0..319),

ms640-r18                           INTEGER (0..639),

ms1280-r18                          INTEGER (0..1279),

ms2560-r18                          INTEGER (0..2559),

ms5120-r18                          INTEGER (0..5119),

...

}   OPTION       -- Cond periodic

}

IDC-Starting-SFN-AndSubframe-r18 ::= SEQUENCE {

starting-SFN-r18                       INTEGER (0..1023),

startingSubframe-r18                   INTEGER (0..9)

}

It can be seen that except for the aperiodic gap in Option 2 (which will be discussed in section 3.2 below), Option 1 and 2 both report periodicity, offset, scheduling period of NR (as in Option 1) / unscheduled period of NR (as in Option 2), as in Figure 1 below. Therefore a unified approach for Option 1 and Option 2 is to report assistance information of on/off pattern, with parameters periodicity, offset, and scheduling period of NR repoted.

Figure 1: Coarse granularity TDM pattern (Option 1 and 2)

Proposal 3: Assistance information of on/off pattern is supported for NR IDC, and periodicity, offset, and scheduling period of NR are reported.

Option 3 addresses BT voice (eSCO) use case, which requires fine granularity of TDM pattern between NR and Bluetooth. EV3 interval in Bluetooth eSCO is 3.75 ms. In NR TDD, the TDD UL-DL transmission periodicity (dl-UL-TransmissionPeriodicity) can be of the values of {0.5 ms, 0.625 ms, 1 ms, 1.25 ms, 2 ms, 2.5 ms, 5 ms, 10 ms}. It can be seen that 3.75 ms is a multiple of 0.625 ms and 1.25 ms. It appears that a simple ON/OFF pattern can be defined for coexistence with BT voice case if TDD UL-DL transmission periodicity is 0.625 ms and 1.25 ms. However, there are SCS restriction for TDD periodicity of 0.625 ms and 1.25 ms, as in TS 38.213 clause 11.1, as below:

A value  msec is valid only for .

A value  msec is valid only for  or .

A value  msec is valid only for , or , or .

Therefore 0.625 ms periodicity can only be used for 120 kHz SCS (for FR2 only), while 1.25 ms periodic can only be used for 60 and 120 kHz SCS. Since Bluetooth is operating in 2.4 GHz (which belongs to FR1 band), the usage of 0.625 ms periodicity is not possible for a NR frequency having IDC issue. The usage of 1.25 ms periodicity also puts unnecessary restrictions on numerology (60 kHz SCS only). Therefore a simple on/off pattern with periodicity as a divisor of 3.75 ms is not possible to support BT voice use case.

Observation 4: It is not feasible to use simple on/off pattern with periodicity as a divisor of 3.75 ms to support BT voice use case.

Therefore to support BT voice use case, UL / DL transmission occasion defined with a bitmap within a periodicity (with further details in section 3.2) is needed, similar to LTE HARQ subframe pattern.

Proposal 4: Assistance information of UL / DL transmission occasion defined with a bitmap within a periodicity is supported for NR IDC.

In the following sections, further details are discussed regarding assistance information: 1) simple on/off pattern to solve general use case; 2) UL / DL transmission occasions to solve BT voice use case, as well as network configuration to solve IDC issue.

## Assistance information

For DRX assistance information of on/off pattern, detailed value range should be decided. On/off pattern is mainly used for coexistence with WiFi and Bluetooth Data. The reported assistance information is mainly related to the traffic characteristics of the coexistence use case, and is mostly RAT-independent. Therefore DRX assistance information based on LTE can be largely reused for NR. The LTE signalling values are selected based on coexistence scenarios (clause 4.2 of TR 36.816 [1]), and were agreed in email discussion “[79#33] [LTE/IDC] IDC Open issues” (R2-124404). Reusing NR DRX configuration as IDC assistance information is not suitable since extremely long DRX cycles (longer than 256 ms) is helpful for power saving but cannot satisfy the delay requirements of coexisting with other radios.

Proposal 5: DRX assistance information defined in LTE (drx-AssistanceInfo-r11 in TS 36.331) is the baseline for NR assistance information of on/off pattern, which can also cover MUSIM periodic gap assistance information.

Aperiodic gap information is introduced in MUSIM. The applicability of aperiodic gap for IDC needs further discussion. It might be argued that this might be needed for protection of WiFi beacon whose transmission might be delayed due to contention nature of WiFi. However, there might not be sufficient processing time to take aperiodic gap into account. The required processing time includes the UE internal coordination between WiFi and NR, the transmission of UL RRC message (including potential SR/BSR procedure), and the processing at gNB side (including RRC processing time which could be large compared with L1/L2 processing time). In addition, WiFi beacon issue can be also handled by other mechanisms like autonomous denial (if agreed to be included in Rel-18).

Proposal 6: Assistance information of aperiodic gap is not supported.

Following are further details regarding UL / DL transmission occasion (Option 3) to solve IDC issue in BT voice use case. LTE IDC subframe pattern is based on UL synchronous HARQ, and subframe pattern length is related to FDD/TDD and TDD UL-DL configuration. For example, although the periodicity of TDD UL-DL configuration is 10 ms, the subframe pattern length is 70 ms due to UL synchronous HARQ. In NR, HARQ is asynchronous. Therefore there is no need to report TDM pattern based on synchronous HARQ operation. In NR TDD, the pattern periodicity for IDC can be related to the TDD UL-DL transmission periodicity (dl-UL-TransmissionPeriodicity). The ASN.1 of the TDD UL-DL configuration is as follows as from TS 38.331:

TDD-UL-DL-ConfigCommon ::=          SEQUENCE {

referenceSubcarrierSpacing          SubcarrierSpacing,

pattern1                            TDD-UL-DL-Pattern,

pattern2                            TDD-UL-DL-Pattern                                                       OPTIONAL, -- Need R

...

}

TDD-UL-DL-Pattern ::=               SEQUENCE {

dl-UL-TransmissionPeriodicity       ENUMERATED {ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10},

nrofDownlinkSlots                   INTEGER (0..maxNrofSlots),

nrofDownlinkSymbols                 INTEGER (0..maxNrofSymbols-1),

nrofUplinkSlots                     INTEGER (0..maxNrofSlots),

nrofUplinkSymbols                   INTEGER (0..maxNrofSymbols-1),

...,

[[

dl-UL-TransmissionPeriodicity-v1530     ENUMERATED {ms3, ms4}                                               OPTIONAL -- Need R

]]

}

For example, the pattern periodicity for NR IDC TDM pattern can be the same as dl-UL-TransmissionPeriodicity of pattern1 if pattern2 is not signalled, or the sum of dl-UL-TransmissionPeriodicity in pattern1 and pattern2 if pattern2 is signalled. Alternatively, the pattern periodicity for IDC can be a single value (e.g. 20 ms) irrespective of the actual TDD UL-DL configuration since TDD transmission periodicity is a divisor of 20 ms (at least when pattern2 is configured since TS 38.213 clause 11.1 specifies “A UE expects that  divides 20 msec.” ). The pattern periodicity for FDD can the same as that of TDD, e.g. 20 ms. Note that although 3 ms is a possible value for transmission periodicity, it is introduced for coexistence with LTE (as in CR R2-1813303). Therefore 3 ms periodicity is not considered since it would be complicated to support TDM solution in case of coexistence between NR and LTE.

Regarding pattern unit / granularity, for each bit indicated in the pattern, if the pattern periodicity is the same as TDD DL-UL transmission periodicity, the pattern unit can be slot and each bit in the pattern indicates whether NG-RAN is requested to abstain from using the slot. If fixed periodicity (e.g. 20 ms) is used, the pattern unit can be subframe (ms), i.e. each bit in the pattern indicates whether NG-RAN is requested to abstain from using the subframe (ms).

As to downlink symbols (nrofDownlinkSymbols) and uplinks symbols (nrofUplinkSymbols) in TDD UL-DL pattern, whether NG-RAN is requested to abstain from using those symbols can be either explicitly indicated, or implicitly indicated based on adjacent downlink or uplink slots.

An example ASN.1 signaling for UE assistance information is as follows:

IDC-SubframePatternList-r18 ::=	SEQUENCE (SIZE (1..maxSubframePatternIDC-r18)) OF IDC-SubframePattern-r18

IDC-SubframePattern-r18 ::= BIT STRING (SIZE (1..20)

Proposal 7: Assistance information of UL / DL transmission occasion is defined with a bitmap pattern within a 20 ms periodicity, with each bit in the pattern indicates whether NG-RAN is requested to abstain from using the subframe (ms).

## Newtwork configuration to solve IDC issue

As in Proposal 2, it is up to gNB implementation to choose which solution should be applied to solve IDC issue after receiving assistance information.

For TDM solution, it is preferred to have predictable on-off pattern although DRX behavior is rather dynamic. The reason is that both NR and other RATs are coordinated in time domain and NR can only transmit when the other RAT is not receiving, and vice versa. In NR DRX, reception of DRX Command MAC CE and/or Long DRX Command MAC CE can only stop drx-onDurationTimer, drx-InactivityTimer, and drx-ShortCycleTimer, but not drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL, drx-RetransmissionTimerDL, or drx-RetransmissionTimerUL. Therefore UE still needs to receive PDCCH for HARQ retransmissions. To provide predictable on-off pattern gNB needs to terminate DL/UL transmissions earlier within the intended Active time, which might not result in efficient usage of radio resource.

One potential enhancement to DRX for NR IDC is that RRC signalling can configure whether timers drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL, drx-RetransmissionTimerDL, and drx-RetransmissionTimerUL are stopped when receiving DRX Command MAC CE and/or Long DRX Command MAC CE. When those timers are stopped, there are no NR transmission/reception, and other RAT can be utilized.

Proposal 8: RAN2 to discuss potential enhancement to DRX for NR IDC, e.g. whether it is configurable that timers drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL, drx-RetransmissionTimerDL, and drx-RetransmissionTimerUL are stopped when receiving DRX Command MAC CE and/or Long DRX Command MAC CE.

# Conclusion

In this contribution, we discuss TDM solutions for IDC in more detail, and have following observations:

Observation 1: In LTE IDC, DRX assistance information is provided as coarse granularity TDM pattern.

Observation 2: In LTE IDC, HARQ subframe pattern is provided as fine granularity TDM pattern.

Observation 3: In LTE IDC, DRX solution, scheduling avoidance or autonomous denial is configured by eNB when TDM assistance information is either DRX assistance information or HARQ subframe pattern.

Observation 4: It is not feasible to use simple on/off pattern with periodicity as a divisor of 3.75 ms to support BT voice use case.

We propose the following:

Proposal 1: For NR IDC TDM solution, the assistance data and applied solution should be decoupled.

Proposal 2: It is up to gNB implementation to choose which solution should be applied to solve IDC issue after receiving assistance information.

Proposal 3: Assistance information of on/off pattern is supported for NR IDC, and periodicity, offset, and scheduling period of NR are reported.

Proposal 4: Assistance information of UL / DL transmission occasion defined with a bitmap within a periodicity is supported for NR IDC.

Proposal 5: DRX assistance information defined in LTE (drx-AssistanceInfo-r11 in TS 36.331) is the baseline for NR assistance information of on/off pattern, which can also cover MUSIM periodic gap assistance information.

Proposal 6: Assistance information of aperiodic gap is not supported.

Proposal 7: Assistance information of UL / DL transmission occasion is defined with a bitmap pattern within a 20 ms periodicity, with each bit in the pattern indicates whether NG-RAN is requested to abstain from using the subframe (ms).

Proposal 8: RAN2 to discuss potential enhancement to DRX for NR IDC, e.g. whether it is configurable that timers drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL, drx-RetransmissionTimerDL, and drx-RetransmissionTimerUL are stopped when receiving DRX Command MAC CE and/or Long DRX Command MAC CE.

# References

[1] 3GPP TR 36.816, "Study on signalling and procedure for interference avoidance for in-device coexistence"