# 3GPP TSG RAN WG2 Meeting #120      	                           R2-2211680
Toulouse, France, 14th – 18th Nov. 2022

Agenda item:	8.3.3

Source:	Apple

Title:	Discussion and comparison of SSB-less and SIB-less solutions

WID/SID:	FS_Netw_Energy_NR– Release 18

Document for:	Discussion and Decision

# 1 Introduction

In RAN2#119b-e [1], below high level agreements on SSB-less and SIB-less solutions for NES were made:

Agreements:

- For SSB/SIB-less solution, RAN2 starts with multi-carrier case

- RAN2 assumes that the SSB-less solution for inter-band CA in connected mode we can consider to use the intra-band CA mechanism as a baseline/starting point. FFS whether there are other impacts for RAN2 according to other WGs discussion

- For SIB-less/SSB-less, capture the solutions in more details over the email discussion and clarify the definition on anchor cell.  (e.g. 1) non-anchor NES cell doesn’t transmit SSB and SI 2) non-anchor cell doesn’t transmit SIB) FFS for paging in both mechanisms.

List of remaining issues on NES cell without SIB:

The following two scenarios will be studied:

- The anchor cell transmit SIs for NES cells, and NES cells transmit neither SSBs nor SIs;

- The anchor cell transmit SIs for NES cells, and NES cells transmit SSBs but not SIs.

Aspects to be addressed:

  - the detailed solution and potential specification impacts for each direction;

  - the benefits for energy saving and constraints for each direction (baseline is the anchor cell + SIB-less cell);

  - impact on the UE behaviour, e.g. whether the UE always camp on the anchor cell, or can also camp on the NES cells (this is rather dependent on specific directions), how the UE will determine which cell to perform RACH; the applicable RRC state, e.g. whether it only applies to idle mode, or also applies to connected mode;

  - applicability of existing solutions, e.g. how much we can reuse from NB-IoT solution and what needs to be enhanced compared with NB-IoT

  - how to handle paging:

    - detailed solution description, benefits and potential specification impact (baseline is the anchor cell + SIB-less cell);

    - impact on UE behaviour on cell camping;

    - applicability of existing solutions

    - whether a common solution can be applied to both SSB-less and SIB-less solutions

Meanwhile, in post-meeting email discussion#303 [2], draft TP was agreed in [3] and below open issue list was agreed for further discussion.

In this contribution, we share our view on these open issues. We also provide draft TP in Appendix.

# 2 Discussion

During online / offline discussion in RAN2#119b-e [1], multi-carrier operation in NB-IoT was mentioned by multiple companies as baseline of SSB-less / SIB-less solution, but it seems different companies have different understanding on NB-IoT solution. Thus, we fist have a brief recap of multi-carrier operation of NB-IoT specified in Rel-13 / Rel-14. Then, we discuss SSB-less solution and SIB-less solution, respectively.

## 2.1 Multi-carrier operation of NB-IoT

To ensure each company in the same page, we summarize multi-carrier operation of NB-IoT in Table 1.

|  | NB-PSS/SSS/PBCH and SIB | RACH for initial access | Paging | PDCCH ordered RACH | Non-anchor carrier details | After release to IDLE |
| --- | --- | --- | --- | --- | --- | --- |
| Rel-13 multi-carrier of NB-IOT | Only in anchor carrier | Only in anchor carrier | Only in anchor carrier (return to last stayed non-anchor carrier after paging reception) | Only in anchor carrier (return to last stayed non-anchor carrier after RACH) | 1 DL and UL non-anchor carrier can be configured in RRC for unicast data traffic | Retune to anchor carrier |
| Rel-14 multi-carrier of NB-IOT | Only in anchor carrier | Only in anchor carrier | In either anchor carrier or one non-anchor carrier (carrier selection for paging based on UE ID and weight parameter) | In either anchor carrier or one non-anchor carrier (random carrier selection for RACH) | Up to 15 DL and 15 UL non-anchor carriers can be configured in RRC for unicast data traffic | Retune to anchor carrier |

Table.1: Summary of multi-carrier operation of NB-IoT specified in Rel-13 / 14

We need to emphasize 3 key points on multi-carrier operation in NB-IoT:

- It is applied to the UE without CA capability (as NB-IoT UE doesn't support CA).

- IDLE / INACTIVE UEs stay in anchor carrier while only CONNECTED UE can be configured to monitor unicast data traffic in one non-anchor carrier.

- There is no CONNECTED mobility design because NB-IoT doesn't support CONNECTED mobility.

Observation 1: 3 key points on multi-carrier operation in NB-IoT:

- It is applied to the UE without CA capability (as NB-IoT UE doesn't support CA).

- IDLE / INACTIVE UEs stay in anchor carrier while only CONNECTED UE can be configured to monitor unicast data traffic in one non-anchor carrier.

- There is no CONNECTED mobility design because NB-IoT doesn't support CONNECTED mobility

And we also want to highlight mechanism of paging and RACH in non-anchor carrier specified in Rel-14:

- Carrier selection for paging is based on UE ID and a configured weight parameter (i.e. distribute PO across anchor and non-anchor carriers)

- Random carrier selection for RACH

Observation 2: Paging and RACH can be performed in non-anchor carrier in Rel-14 with carrier selection:

- Carrier selection for paging is based on UE ID and a configured weight parameter

- Random carrier selection for RACH

## 2.2 SSB-less design

As discussed from beginning, one key confusing issue of SSB-less discussion is that some companies assume the UE has CA capability while some companies assume the UE doesn't have CA capability (i.e. same assumption as NB-IoT). Thus, separate discussion is necessary.

### 2.2.1 UE with CA capability

In this scenario, we don't think any spec impact are required if SSB-less SCell is supported in inter-band CA, which will depend on RAN1/RAN4 conclusion. Specifically, all the SCells are allowed to not transmit SSB and SIB, and the UE rely on dedicated RRC signaling to receive SIB update.

Proposal 1: For the UE with CA capability in SSB-less solution, no spec impact is needed if SSB-less SCell is supported in inter-band CA.

### 2.2.2 UE without CA capability

In this scenario, we think it is similar to multi-carrier of NB-IoT from the perspective of omitting SSB/SIB transmission in non-anchor cells. Thus, it should be the design baseline.

However, we need to emphasize one difference between LTE and NR is that RACH and paging resource selection depend on the selected SSB in NR but SSB is absent in non-anchor cells.

Observation 3: One difference between LTE and NR is that RACH and paging resource selection depend on the selected SSB in NR but SSB is absent in non-anchor cells for SSB-less design.

Thus, we propose to use multi-carrier operation of Rel-13 NB-IoT as baseline, i.e. paging and RACH can only be performed in anchor cell due to lack of SSB in NES SSB-less cells.

Proposal 2: For the UE without CA capability in SSB-less solution, multi-carrier operation of Rel-13 NB-IoT is baseline:

- NW transmit SSB, SIB and paging only in anchor cell, to reduce NW power consumption

- The UE performs initial access, RACH and monitors paging only in anchor cell

- After entering CONNECTED state, the UE can be configured to monitor unicast DL/UL traffic in one NES cell without SSB via RRC message from anchor cell.

- Upon entering IDLE / INACTIVE state, the UE retunes back to anchor cell

- If RACH is triggered, the UE retunes to anchor cell to perform RACH, and then retunes back to last stayed SSB-less cell

Then, another issue needs further discussion is how to perform RRM / RLM / BFD in this scenario. As we mentioned before, because CONNECTED mobility is not supported in NB-IoT, multi-carrier operation do not consider RRM / RLM / BFD. So, this is a NES specific issue to discuss. We think there are two alternatives:

- Alt-1: Rely on CSI-RS based RRM / RLM / BFD in NES SIB-less cell

  - Please note that although CSI-RS based RRM/RLM/BFD is specified, it seems none of them are deployed yet. So, we suggest to not rely on CSI-RS based RRM/RLM/BFD in SSB-less design.

- Alt-2: The UE is configured to periodicity retune to anchor cell for SSB based RRM/RLM/BFD

  - The issue is that the UE throughput performance will be decreased due to periodic retuning to anchor cell.

We suggest RAN2 to discuss and down-select between Alt-1 and Alt-2 for this NES specific issue.

Proposal 3: For the UE without CA capability in SSB-less solution, RAN2 discuss below two alternatives on how to perform RRM / RLM / BFD:

- Alt-1: Rely on CSI-RS based RRM / RLM / BFD in NES SIB-less cell

- Alt-2: The UE is configured to periodicity retune to anchor cell for SSB based RRM / RLM / BFD

## 2.3 SIB-less design

Similar to SSB-less discussion, we think the discussion should be separate for UE with CA capability and UE without CA capability.

### 2.3.1 UE with CA capability

In this scenario, we don't think any spec impact are required because the UE can already rely on dedicated RRC signaling to receive SIB update.

Proposal 4: For the UE with CA capability in SIB-less solution, no spec impact is needed because legacy system supports reception of SIB update via dedicated RRC signaling.

### 2.3.2 UE without CA capability

In this scenario, we think there are basically 3 alternative mechanisms can be considered:

- Alt-1: Same as SSB-less solution in Proposal 2. The UE first camps in anchor cell. In CONNECTED state, it can be configured to monitor unicast DL/UL traffic, and perform SSB based RRM/RLM/BFD in one NES SIB-less cell.

  - The UE acquires SIB, performs RACH and monitors paging only in anchor cell

  - After entering CONNECTED state, the UE can be configured to monitor unicast DL/UL traffic in one NES cell without SIB via RRC, and the UE performs SSB based RRM/RLM/BFD in this cell.

  - Upon entering IDLE/INACTIVE state, the UE retunes to anchor cell

- Alt-2: The UE camps in anchor cell first, and may select one SIB-less cell to perform RACH upon MO traffic arrival or reception of paging.

  - The UE first acquires SIB with “piggybacked” SIB-less cell info in anchor cell. And then, it performs RACH for initial registration and monitors paging only in anchor cell.

    - The RACH is used for the UE to complete initial registration, so that it can be paged by NW.

  - Upon MO traffic arrival or reception of paging, the UE may select one SIB-less cell to perform RACH, and then enter CONNECTED for MO / MT traffic.

  - Upon entering IDLE/INACTIVE state, the UE retunes to anchor cell.

- Alt-3: The UE selects one SIB-less cell to perform RACH and camp, upon acquisition of “piggybacked” SIB-less cell info in anchor cell’s SIB.

  - The UE acquires SIB with “piggybacked” SIB-less cell info.

  - The UE may select one SIB-less cell to perform RACH and camp. And it monitors paging in the selected SIB-less cell.

  - Upon MO traffic arrival or reception of paging, the UE performs RACH in the selected SIB-less cell to enter CONNECTED for MO / MT traffic.

  - Upon reception of short message for SIB update, it retunes to anchor cell to receive SIB update, and returns back to camp in last stayed SIB-less cell.

The signaling procedures of the 3 alternatives are illustrated in Figure.1 / Figure.2 / Figure.3, respectively. And orange fonts highlight the spec impact. In addition, we summarize the differences of these 3 alternatives in Table.1, and their comparison in Table.2.

As can be observed in Table 2:

- Alt-1 and Alt-2 have benefit of more NES gain and less spec impacts.

- On the other hand, Alt-2 and Alt-3 have some benefit of RACH load balancing over Alt-1 because RACH resources are distributed in anchor cell and SIB-less cell in Alt-2 and Alt-3.

Observation 4: For UE without CA capability in SIB-less solution, Alt-1 and Alt-2 have benefit of more NES gain and less spec impacts. On the other hand, Alt-2 and Alt-3 have some benefit of RACH load balancing over Alt-1 because RACH resources are distributed in anchor cell and SIB-less cell in Alt-2 and Alt-3.

Figure.1 Illustration of Alt-1 of SIB-less solution

Figure.2 Illustration of Alt-2 of SIB-less solution

Figure.3 Illustration of Alt-3 of SIB-less solution

|  | SIB acquisition for SIB-less info | Which cell to camp? | Which cell to monitor paging | When retune to SIB-less cell? | Release to IDLE/INACTIVE | SIB update |
| --- | --- | --- | --- | --- | --- | --- |
| Alt-1 | No need (The UE uses SIB-less cell only in CONNECTED) | Only in anchor cell | Only in anchor cell | Upon reception of RRC configuration from anchor cell (i.e. after entering CONNECTED state in anchor cell) | Retune to anchor cell | Via SIB of anchor cell |
| Alt-2 | From “piggybacked” SIB-less cell info in anchor cell's SIB | Only in anchor cell | Only in anchor cell | Upon reception of MO traffic or paging in anchor cell | Retune to anchor cell | Via SIB of anchor cell |
| Alt-3 | From “piggybacked” SIB-less cell info in anchor cell's SIB | In either anchor cell or SIB-less cell | In either anchor cell or SIB-less cell | Upon acquisition of “piggybacked” SIB-less info from anchor cell's SIB | Stay in SIB-less cell | Retune to anchor cell for SIB update |

Table.1 Illustration of 3 alternatives of SIB-less solution

|  | NES gain | Spec impacts | UE impacts |
| --- | --- | --- | --- |
| Alt-1 | High - No transmission of “piggybacked” SIB-less cell info in anchor cell - Transmit paging only in anchor cell | Low - Allow RRC signaling from anchor cell to configure the UE to monitor DL/UL data traffic in SIB-less cell. - Upon reception of RRC release, the UE retunes to anchor cell, and enter IDLE / INACTIVE state. | Low - Upon reception of RRC message from anchor cell, the UE retunes to the indicated SIB-less cell. - Upon reception of RRC release, the UE retunes to anchor cell. |
| Alt-2 | Medium - Transmission of “piggybacked” SIB-less cell info in anchor cell - Transmit paging only in anchor cell | Medium - Contents of “piggybacked” SIB-less cell info in anchor cell. - Carrier selection solution. - Upon reception of RRC release, the UE retunes to anchor cell, and enter IDLE / INACTIVE state. | Medium - Reception of “piggybacked” SIB-less cell info in anchor cell. - Carrier selection upon MO or reception of paging - Upon reception of RRC release, the UE retunes to anchor cell. |
| Alt-3 | Low - Transmission of “piggybacked” SIB-less cell info in anchor cell - Transmit paging in both anchor cell and SIB-less cell. | High - Contents of “piggybacked” SIB-less cell info in anchor cell. - Carrier selection solution. - Upon reception of short message in SIB-less cell, retune to anchor cell for SIB update. - New UE behavior in cell reselection. | High - Reception of “piggybacked” SIB-less cell info in anchor cell. - Carrier selection after obtaining “piggybacked” SIB-less cell info. - Upon reception of short message in SIB-less cell, retune to anchor cell for SIB update. |

Table.2 Comparison of 3 alternative SIB-less solutions

Thus, we propose to capture the 3 solutions in TR:

Proposal 5: For UE without CA capability in SIB-less solution, RAN2 capture below 3 alternatives in TR:

- Alt-1: Same as SSB-less solution, the UE camps in anchor cell, and configured by RRC from anchor cell to monitor unicast DL/UL data traffic in one NES SIB-less cell.

- Alt-2: The UE camps in anchor cell first, and may select one NES SIB-less cell to perform RACH to enter CONNECTED state upon MO traffic or reception of paging.

- Alt-3: The UE selects one SIB-less cell to camp upon acquisition of “piggybacked” SIB-less cell info in anchor cell’s SIB. Upon reception of short message, it needs to retune to anchor cell for SIB update

Finally, we share some view on two major spec impacts of Alt-2 and Alt-3:

- Contents of “piggybacked” SIB-less cell info in anchor cell’s SIB

Our understanding is that below information is needed for each SIB-less cell:

  - SFN, ARFCN

  - Uu PHY common resource related fields: subCarrierSpacingCommon, ssb-SubcarrierOffset, dmrs-TypeA-Position

  - CORESET#0: pdcch-ConfigSIB1

  - Cell Access related fields: cellAccessRelatedInfo (RACH configuration is included)

  - Common PHY parameters: downlinkConfigCommon, uplinkConfigCommon, n-TimingAdvanceOffset, ssb-PositionsInBurst, ssb-PeriodicityServingCell, tdd-UL-DL-ConfigurationCommon, ss-PBCH-BlockPower

  - UAC parameters: uac-BarringInfo

  - Timer: ue-TimersAndConstants

  - Others: useFullResumeID, ims-EmergencySupport, eCallOverIMS-Support

- Carrier selection solution

We think there are two solutions:

  - Alt-1: random selection as multi-carrier operation in Rel-14 NB-IoT

  - Alt-2: random selection among carriers whose RSRP is larger than one threshold, similar to SUL.

Proposal 6: For UE without CA capability in SIB-less solution, the “piggybacked” SIB-less cell info in anchor cell’s SIB includes SFN, ARFCN, Uu PHY common resource, CORESET#0, cellAccessRelatedInfo, Common PHY parameters, uac-BarringInfo and ue-TimersAndConstants.

Proposal 7: For UE without CA capability in SIB-less solution, RAN2 discuss below two alternatives on carrier selection for RACH:

- Alt-1: random selection similar to multi-carrier operation in Rel-14 NB-IoT

- Alt-2: random selection among carriers whose RSRP is larger than one threshold, similar to SUL

## 2.4 Single carrier SSB-less/SIB-less solution

As can be observed, we still have a lot of remaining issues on SSB-less / SIB-less in multi-carrier. Thus, we don't think RAN2 have sufficient time to discuss single carrier solution. Then, we suggest to leave it to RAN1.

Proposal 8: As SI needs to be concluded in this RAN2 meeting, RAN2 have no sufficient time to discuss SSB-less / SIB-less in single carrier. Thus, RAN2 leave single carrier discussion to RAN1.

# 3 Conclusion

In this contribution, we further discuss SSB-less/SIB-less solution in multi-carrier. Our observations are:

Observation 1: 3 key points on multi-carrier operation in NB-IoT:

- It is applied to the UE without CA capability (as NB-IoT UE doesn't support CA).

- IDLE / INACTIVE UEs stay in anchor carrier while only CONNECTED UE can be configured to monitor unicast data traffic in one non-anchor carrier.

- There is no CONNECTED mobility design because NB-IoT doesn't support CONNECTED mobility

Observation 2: Paging and RACH can be performed in non-anchor carrier in Rel-14 with carrier selection:

- Carrier selection for paging is based on UE ID and a configured weight parameter

- Random carrier selection for RACH

Observation 3: One difference between LTE and NR is that RACH and paging resource selection depend on the selected SSB in NR but SSB is absent in non-anchor cells for SSB-less design.

Observation 4: For UE without CA capability in SIB-less solution, Alt-1 and Alt-2 have benefit of more NES gain and less spec impacts. On the other hand, Alt-2 and Alt-3 have some benefit of RACH load balancing over Alt-1 because RACH resources are distributed in anchor cell and SIB-less cell in Alt-2 and Alt-3.

Based on observations, our proposals are list below. We also provide draft TP in Appendix.

SSB-less in multi-carrier

Proposal 1: For the UE with CA capability in SSB-less solution, no spec impact is needed if SSB-less SCell is supported in inter-band CA.

Proposal 2: For the UE without CA capability in SSB-less solution, multi-carrier operation of Rel-13 NB-IoT is baseline:

- NW transmit SSB, SIB and paging only in anchor cell, to reduce NW power consumption

- The UE performs initial access, RACH and monitors paging only in anchor cell

- After entering CONNECTED state, the UE can be configured to monitor unicast DL/UL traffic in one NES cell without SSB via RRC message from anchor cell.

- Upon entering IDLE / INACTIVE state, the UE retunes back to anchor cell

- If RACH is triggered, the UE retunes to anchor cell to perform RACH, and then retunes back to last stayed SSB-less cell

Proposal 3: For the UE without CA capability in SSB-less solution, RAN2 discuss below two alternatives on how to perform RRM / RLM / BFD:

- Alt-1: Rely on CSI-RS based RRM / RLM / BFD in NES SIB-less cell

- Alt-2: The UE is configured to periodicity retune to anchor cell for SSB based RRM / RLM / BFD

SIB-less in multi-carrier

Proposal 4: For the UE with CA capability in SIB-less solution, no spec impact is needed because legacy system supports reception of SIB update via dedicated RRC signaling.

Proposal 5: For UE without CA capability in SIB-less solution, RAN2 capture below 3 alternatives in TR:

- Alt-1: Same as SSB-less solution, the UE camps in anchor cell, and configured by RRC from anchor cell to monitor unicast DL/UL data traffic in one NES SIB-less cell.

- Alt-2: The UE camps in anchor cell first, and may select one NES SIB-less cell to perform RACH to enter CONNECTED state upon MO traffic or reception of paging.

- Alt-3: The UE selects one SIB-less cell to camp upon acquisition of “piggybacked” SIB-less cell info in anchor cell’s SIB. Upon reception of short message, it needs to retune to anchor cell for SIB update

Proposal 6: For UE without CA capability in SIB-less solution, the “piggybacked” SIB-less cell info in anchor cell’s SIB includes SFN, ARFCN, Uu PHY common resource, CORESET#0, cellAccessRelatedInfo, Common PHY parameters, uac-BarringInfo and ue-TimersAndConstants.

Proposal 7: For UE without CA capability in SIB-less solution, RAN2 discuss below two alternatives on carrier selection for RACH:

- Alt-1: random selection similar to multi-carrier operation in Rel-14 NB-IoT

- Alt-2: random selection among carriers whose RSRP is larger than one threshold, similar to SUL

SSB-less/SIB-less in single-carrier

Proposal 8: As SI needs to be concluded in this RAN2 meeting, RAN2 have no sufficient time to discuss SSB-less / SIB-less in single carrier. Thus, RAN2 leave single carrier discussion to RAN1.

# 4 References

[1] RAN2#119b-e, Chair Notes.

[2] R2-2211427, TP on cell selection／reselection and SSB／SIB-less

[3] R2-2210995, Report of [Offline-302][NES] Cell Selection/Reselection and SSB/SIB-less (Huawei)

# Appendix: draft TP

## 6.1	Techniques in time domain

### 6.1.2	SCell without SSB in inter-band CA

#### 6.1.2.x	Higher layer procedures

The SCell without SSB in intra-band CA is considered as baseline, i.e., for a serving cell without transmission of SS/PBCH blocks, a UE acquires time and frequency synchronization with the serving cell based on receptions of SS/PBCH blocks on the SpCell or the SCell, of the cell group.

Editor's note: impacts in RAN2 may need further analysis pending on other WGs progress.

For the UE with CA capability, no spec impact is needed if SSB-less SCell is supported in inter-band CA.

For the UE without CA capability, multi-carrier operation of Rel-13 NB-IoT is baseline:

- NW transmit SSB, SIB and paging only in anchor cell, to reduce NW power consumption

- The UE performs initial access, RACH and monitors paging only in anchor cell

- After entering CONNECTED state, the UE can be configured to monitor unicast DL/UL traffic in one NES cell without SSB via RRC.

- Upon entering IDLE/INACTIVE state, the UE retunes back to anchor cell

- If RACH is triggered, the UE retunes to anchor cell to perform RACH, and then retunes back to last stayed SSB-less cell

And because SSB is absent in SSB-less cell, the UE is configured to periodicity retune to anchor cell for SSB based RRM / RLM / BFD.

#### 6.1.2.z	Impacts on network interfaces

Editor's note: will be updated once more agreements are made.

### 6.1.3	NES Cell without SIB

#### 6.1.3.x	Higher layer procedures

NES cell without SIB is only applicable in multi-carrier scenario, where there is an anchor cell and one or multiple NES cell(s) without SIB in different carrier(s).

Anchor cell is a cell where NES-capable UE assumes SSB, system information and paging are transmitted.

Editor's note: FFS: The system information transmitted by anchor cell also includes the necessary information for NES-capable UEs to access via an NES cell without SIB.

An NES cell without SIB omits the transmission of SIB, or omits the transmission of both SSB and SIB.

For the UE with CA capability, no spec impact is needed because legacy system supports reception of SIB update via dedicated RRC signaling.

For UE without CA capability, below 3 alternative mechanism can be considered:

- Alt-1: Same as SSB-less solution, the UE camps in anchor cell, and configured by RRC from anchor cell to monitor unicast DL/UL data traffic in one NES SIB-less cell.

- Alt-2: The UE camps in anchor cell first, and may select one NES SIB-less cell to perform RACH to enter CONNECTED state upon MO traffic or reception of paging.

- Alt-3: The UE selects one SIB-less cell to camp upon acquisition of “piggybacked” SIB-less cell info in anchor cell’s SIB. Upon reception of short message, it needs to retune to anchor cell for SIB update

The signaling procedures of the 3 alternatives are illustrated in Figure.6.1.3-1 / Figure. 6.1.3-2 / Figure.6.1.3-3 with spec changes highlighted, respectively. In addition, their comparison is summarized in Table. 6.1.3-1.

Figure.6.1.3-1: Illustration of Alt-1 of SIB-less solution

Figure.6.1.3-2 Illustration of Alt-2 of SIB-less solution

Figure.6.1.3-3 Illustration of Alt-3 of SIB-less solution

|  | NES gain | Spec impacts | UE impacts |
| --- | --- | --- | --- |
| Alt-1 | High - No transmission of “piggybacked” SIB-less cell info in anchor cell - Transmit paging only in anchor cell | Low - Allow RRC signaling from anchor cell to configure the UE to monitor DL/UL data traffic in SIB-less cell. - Upon reception of RRC release, the UE retunes to anchor cell, and enter IDLE / INACTIVE state. | Low - Upon reception of RRC message from anchor cell, the UE retunes to the indicated SIB-less cell. - Upon reception of RRC release, the UE retunes to anchor cell. |
| Alt-2 | Medium - Transmission of “piggybacked” SIB-less cell info in anchor cell - Transmit paging only in anchor cell | Medium - Contents of “piggybacked” SIB-less cell info in anchor cell. - Carrier selection solution. - Upon reception of RRC release, the UE retunes to anchor cell, and enter IDLE / INACTIVE state. | Medium - Reception of “piggybacked” SIB-less cell info in anchor cell. - Carrier selection upon MO or reception of paging - Upon reception of RRC release, the UE retunes to anchor cell. |
| Alt-3 | Low - Transmission of “piggybacked” SIB-less cell info in anchor cell - Transmit paging in both anchor cell and SIB-less cell. | High - Contents of “piggybacked” SIB-less cell info in anchor cell. - Carrier selection solution. - Upon reception of short message in SIB-less cell, retune to anchor cell for SIB update. - New UE behavior in cell reselection. | High - Reception of “piggybacked” SIB-less cell info in anchor cell. - Carrier selection after obtaining “piggybacked” SIB-less cell info. - Upon reception of short message in SIB-less cell, retune to anchor cell for SIB update. |

Table.6.1.3-1 Comparison of 3 alternative SIB-less solutions

Editor's note: FFS the details on how to support NES cell without SIB or without SSB and SIB.

#### 6.1.3.z	Impacts on network interfaces

Editor's note: will be updated once more agreements are made.