# 3GPP TSG RAN WG2 Meeting #120      	                           R2-2211682
Toulouse, France, 14th – 18th Nov. 2022

Agenda item:	8.3.5

Source:	Apple

Title:	Further discussion on mobility enhancement for Network energy saving

WID/SID:	FS_Netw_Energy_NR– Release 18

Document for:	Discussion and Decision

# 1 Introduction

SI of Network energy saving (RP-213554) was agreed in RAN#94e [1]. The related SID objectives involving RAN2 are summarized below.

- Study and identify techniques on the gNB and UE side to improve network energy savings in terms of both BS transmission and reception, which may include:

- How to achieve more efficient operation dynamically and/or semi-statically and finer granularity adaptation of transmissions and/or receptions in one or more of network energy saving techniques in time, frequency, spatial, and power domains, with potential support/feedback from UE, and potential UE assistance information [RAN1, RAN2]

- Information exchange/coordination over network interfaces [RAN3]

Note: Other techniques are not precluded

The study should prioritize idle/empty and low/medium load scenarios (the exact definition of such loads is left to the study), and different loads among carriers and neighbor cells are allowed.

In RAN2#119b-e [2], there were some discussions on CONNECTED mobility enhancement for NES. And below agreements were made:

=>	Scenario 1: UEs are HO’ed due to switch of SOURCE cell to NES mode is considered for further study.  FFS whether any enhancements is needed.

=>	FFS Scenario 2: UEs are HO’ed due to source link degradation, where TARGET cell is selected based on its mode of operation

=>	As a first priority, discussion on RAN2 group handover are confined to the CHO framework

In this contribution, we further discuss mobility enhancement for Network energy saving. We also provide draft TP in Appendix.

# 2 Discussion

We discuss scenario 1 and scenario 2, respectively.

## 2.1 Scenario 1

RAN2#119b-e [1] agreed scenario 1 as below:

=>	Scenario 1: UEs are HO’ed due to switch of SOURCE cell to NES mode is considered for further study.  FFS whether any enhancements is needed.

For this scenario, we think its handover enhancement should cover below two motivations:

- Reduced HO commands by UE group common signaling trigger

- Allow the network to go into NES mode timely by handover a group of UEs simultaneously to another cell (if the gNB in NES mode can’t serve these UE or can’t satisfy their QoS requirement)

Observation 1: The handover enhancement for scenario 1 should over below two aspects:

- Reduced HO commands by UE group common signaling trigger

- Allow the network to go into NES mode timely by handover a group of UEs simultaneously to another cell (if the gNB in NES mode can’t serve these UE or can’t satisfy their QoS requirement)

We think above motivation 2) is important because HO command is UE dedicated RRC message with a typical large payload size. As consequence, it’s difficulty for network to send many HO commands to many UEs at the same time. Thus, when the source cell enters NES mode, it can't handover all the UEs to another cell in a short duration, which results in that the source cell can't immediately enter NES mode.

Observation 2: As HO command is UE dedicated RRC message with a typical large payload size, it is difficulty for network to send many HO commands to many UEs immediately when it decides to enter NES mode.

During online discussion of RAN2#119b-e [1], one of main concern for this solution is that UEs may have different capabilities and measurements towards different cells, so it is hard to NW to handover all UEs together. However, we think it is a misunderstanding:

- gNB still needs to send UE dedicated HO commands to a group of UEs, based on UE capability.

- Upon reception of HO command, the UE just stores it but doesn't execute HO immediately.

- When gNB decides to go to NES mode, it sends L1/L2 UE group common signaling to trigger the group of UEs to execute HO.

Please note that the intention of Step 1) is to provide a large number of HO commands in advance distributed over a wider time period to avoid signaling surge.

Since it is SI phase, we think this solution can be captured in TR for further study.

Proposal 1: For scenario 1, capture UE group handover triggered by L1/L2 UE group common signaling in TR:

- gNB sends UE dedicated HO commands to a group of UEs in advance distributed over a wider time period to avoid signaling surge.

- Upon reception of HO command, the UE just stores it but doesn't execute HO immediately.

- When gNB decides to enter NES mode, it sends L1/L2 UE group common signaling to trigger the group of UEs to execute HO.

During the online discussion, some company raised concern that this UE group handover may be duplicated with L1/L2 mobility in Rel-18 WI of further mobility enhancement. However, we believe this is a misunderstanding: L1/L2 mobility in Rel-18 WI of further mobility enhancement is target for UE dedicated L1/L2 signaling to trigger handover. Group handover is not in its WID objective.

Proposal 2: Confirm UE group handover triggered by L1/L2 UE group common signaling is not duplicated with L1/L2 mobility in Rel-18 WI of further mobility enhancement, which is target for UE dedicated L1/L2 signaling to trigger handover.

Another issue is whether new reporting is required to support this solution. Some companies proposed to report UE location and mobility status. However, please note that NR Rel-16 MDT/SON has specified the reporting of UE location and mobility status.

Observation 3: NR Rel-16 MDT/SON has specified the reporting of UE location and mobility status.

Thus, we don't think these new measurements should be introduced for NES purpose.

Proposal 3: For UE group handover triggered by L1/L2 UE group common signaling, no need to introduce new measurement and reporting (e.g. UE location and mobility status).

## 2.2 Scenario 2

RAN2#119b-e [1] discussed scenario 2, but no consensus was achieved. So, an FFS is left:

=>	FFS Scenario 2: UEs are HO’ed due to source link degradation, where TARGET cell is selected based on its mode of operation

During the discussion, we think there were two concerns for this scenario:

- The NW NES mode may change dynamically, and thereby the UE may need to read SIB of the target cell to identify its NES mode.

- Selection of target cell based on NES mode requires the UE to make the decision rather than NW.

Observation 4: There were two concerns for scenario 2:

- The NW NES mode may change dynamically, and thereby the UE may need to read SIB of the target cell to identify its NES mode.

- Selection of target cell based on NES mode requires the UE to make the decision rather than NW.

To address the above two issues, we propose an enhancement for CHO with candidate target cell (de)prioritization.

First, please note that in legacy CHO, all candidate target cells are assumed with same priority. Therefore, when more than 1 candidate target cells satisfy CHO condition (i.e. A3 or A5 event), it is up to UE implementation to select which cell to execute HO.

Observation 5: In legacy CHO, all candidate target cells are assumed with same priority. Therefore, when more than 1 candidate target cells satisfy CHO condition (i.e. A3 / A5 like event), it is up to UE implementation to select which cell to execute HO.

Then, if one or more candidate target cells are NES cell, we think one general enhancement for CHO can be considered:

- On top of existing CHO mechanism, gNB can configure a priority value for each candidate target cell.

- The setting of priority value is up to gNB implementation (e.g. a low priority value for a NES cell), and its logic behind the value is transparent to the UE.

- If more than 1 candidate target cell satisfy the existing CHO condition (i.e. A3 / A5 like event), the UE selects target cell for HO execution based on their priority value configured by source gNB.

With this enhancement, NW is allowed to configure a low priority value for a candidate target cell which is in NES mode, and the UE doesn't need to read SIB of the cell to identify its NES mode. What's more, this enhancement provides more flexibility for NW in CHO when the candidate target cells are with different cell types (e.g. NES cell and mobile IAB cell).

Thus, we propose:

Proposal 4: Capture Scenario 2 with below wording changes in TR:

- Scenario 2: UEs are HO’ed due to source link degradation, where TARGET cell is selected with consideration of whether it is NES cell or not

Proposal 5: For scenario 2, capture below CHO enhancement with candidate target cell (de)prioritization in TR:

- On top of existing CHO mechanism, gNB can configure a priority value for each candidate target cell.

- The setting of priority value is up to gNB implementation (e.g. a low priority value for a NES cell), and its logic behind the priority value is transparent to the UE.

- If more than 1 candidate target cell satisfy the existing CHO condition (i.e. A3 / A5 like event), the UE selects target cell for HO execution based on their priority value configured by source cell.

# 3 Conclusion

In this contribution, we further discuss mobility enhancement for Network energy saving. Our observations are:

Observation 1: The handover enhancement for scenario 1 should over below two aspects:

- Reduced HO commands by UE group common signaling trigger

- Allow the network to go into NES mode timely by handover a group of UEs simultaneously to another cell (if the gNB in NES mode can’t serve these UE or can’t satisfy their QoS requirement)

Observation 2: As HO command is UE dedicated RRC message with a typical large payload size, it is difficulty for network to send many HO commands to many UEs immediately when it decides to enter NES mode.

Observation 3: NR Rel-16 MDT/SON has specified the reporting of UE location and mobility status.

Observation 4: There were two concerns for scenario 2:

- The NW NES mode may change dynamically, and thereby the UE may need to read SIB of the target cell to identify its NES mode.

- Selection of target cell based on NES mode requires the UE to make the decision rather than NW.

Observation 5: In legacy CHO, all candidate target cells are assumed with same priority. Therefore, when more than 1 candidate target cells satisfy CHO condition (i.e. A3 / A5 like event), it is up to UE implementation to select which cell to execute HO.

Based on observations, our proposals are list below. We also provide draft TP in Appendix.

Scenario 1

Proposal 1: For scenario 1, capture UE group handover triggered by L1/L2 UE group common signaling in TR:

- gNB sends UE dedicated HO commands to a group of UEs in advance distributed over a wider time period to avoid signaling surge.

- Upon reception of HO command, the UE just stores it but doesn't execute HO immediately.

- When gNB decides to enter NES mode, it sends L1/L2 UE group common signaling to trigger the group of UEs to execute HO.

Proposal 2: Confirm UE group handover triggered by L1/L2 UE group common signaling is not duplicated with L1/L2 mobility in Rel-18 WI of further mobility enhancement, which is target for UE dedicated L1/L2 signaling to trigger handover.

Proposal 3: For UE group handover triggered by L1/L2 UE group common signaling, no need to introduce new measurement and reporting (e.g. UE location and mobility status).

Scenario 2

Proposal 4: Capture Scenario 2 with below wording changes in TR:

- Scenario 2: UEs are HO’ed due to source link degradation, where TARGET cell is selected with consideration of whether it is NES cell or not

Proposal 5: For scenario 2, capture below CHO enhancement with candidate target cell (de)prioritization in TR:

- On top of existing CHO mechanism, gNB can configure a priority value for each candidate target cell.

- The setting of priority value is up to gNB implementation (e.g. a low priority value for a NES cell), and its logic behind the priority value is transparent to the UE.

- If more than 1 candidate target cell satisfy the existing CHO condition (i.e. A3 / A5 like event), the UE selects target cell for HO execution based on their priority value configured by source cell.

# 4 References

[1] RP-213554, New SI: Study on network energy savings for NR.

[2] RAN2#119b-e, Chair Notes.

# Appendix: draft TP

## 6.x	Higher layer aspects for network energy savings

Editor's note: This section includes common aspects of higher layers deduced from the above candidate directions.

### 6.X.2	Connected mode mobility

The following two scenarios for connected mode mobility enhancement for NES can be considered:

- Scenario 1: UEs are HO’ed due to switch of SOURCE cell to NES mode.

- Scenario 2: UEs are HO’ed due to source link degradation, where TARGET cell is selected with consideration of whether it is NES cell or not.

To address Scenario 1, below UE group handover triggered by L1/L2 UE group common signaling allows the Network to go into NES mode timely by handover a group of UEs simultaneously to another cell. Thus, it can be further studied:

- gNB sends UE dedicated HO commands to a group of UEs in advance distributed over a wider time period to avoid signaling surge.

- Upon reception of HO command, the UE just stores it but doesn't execute HO immediately.

- When gNB decides to enter NES mode, it sends L1/L2 UE group common signaling to trigger the group of UEs to execute HO.

To address Scenario 2, below CHO enhancement with candidate target cell (de)prioritization allows the Network to configure NES capable UEs to de-prioritize NES cell in CHO execution. Thus, it be further studied:

- On top of existing CHO mechanism, gNB can configure a priority value for each candidate target cell.

- The setting of priority value is up to gNB implementation (e.g. a low priority value for a NES cell), and its logic behind the priority value is transparent to the UE.

- If more than 1 candidate target cell satisfy the existing CHO condition (i.e. A3 / A5 like event), the UE selects target cell for HO execution based on their priority value configured by source cell.