3GPP TSG-RAN WG2 Meeting #120	R2-2211773

Toulouse, France, 14 – 18 November 2022

Agenda item:	6.9.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on ignoring PDCCH skipping

WID/SID:	NR_UE_pow_sav_enh-Core - Release 17

Document for:	Discussion and Decision

# 1	Introduction

An LS has been received from RAN1 in R2-2209152/R1-2208210 [1] on PDCCH skipping which we did not manage to treat in the previous RAN2 meeting, asking for clarification on what is the meaning of “ignores” in the sentence “UE ignores PDCCH skipping while contention resolution timer is running.”

In addition, RAN1 respectfully asks RAN2 what is the meaning of “ignores” in the sentence “UE ignores PDCCH skipping while contention resolution timer is running.” quoted from the previous RAN2 LS to RAN1 on PDCCH skipping in RRC_CONNECTED (R2-2201960).

It concerns the following text we have in 38.300:

UE power saving may also be achieved through PDCCH monitoring adaptation mechanisms when configured by the network, including skipping of PDCCH monitoring and Search space set group (SSSG) switching. In this case UE does not monitor PDCCH during the PDCCH skipping duration or monitors PDCCH according to the search space sets applied in SSSG. However, in the following cases, UE ignores PDCCH skipping: - on all serving cells of the corresponding Cell Group when SR is sent and is pending; - on SpCell while contention resolution timer is running; - on SpCell during monitoring of the RAR/MsgB window.

In this contribution we provide our views on the issue.

# 2	Discussion

The issue was discussed in RAN2 #116bis-e meeting [2] when we made the agreement of UE ignores PDCCH skipping command when SR is pending or during the RAR/MsgB/contention resolution timer running and an LS was sent to RAN1 informing the agreements [3].

From RAN2 point of view, UE ignores PDCCH skipping while the SR is pending. From RAN2 point of view, if PDCCH skipping is applied to RNTI(s) monitored during RAR/MsgB window, the UE ignores PDCCH skipping during the RAR/MsgB window. From RAN2 point of view, UE ignores PDCCH skipping while contention resolution timer is running.

We understood the intention was that regardless of PDCCH skipping command received before or during the period of SR pending or when RAR/MsgB/contention resolution timer is running, the UE does not apply the skipping command during and after the window. Since RA can be triggered due to UL initiated activities and the NW might send PDCCH skipping command to the UE based on DL activities before receiving MSG3/MSGA, upon reception success contention resolution, it makes sense for the UE to keep awake to monitor following scheduling, which was also the intention of using “ignore PDCCH skipping” in the RAN2 agreement. In other words, skipping duration from an earlier received PDCCH skipping command should not continue after successful contention resolution.

Current text in stage 2 is indeed a bit unclear though, we propose to clarify it as when the UE shall monitor PDCCH regardless of the PDCCH skipping command, instead of saying “ignore the PDCCH skipping”.

Proposal 1: clarify “ignore PDCCH skipping” means the UE monitors PDCCH when SR is pending/during RAR/MSGB window/when contention resolution timer is running, as well as after successful contention resolution regardless of the received PDCCH skipping command.

Proposal 2: agree the following TP to 38.300 and reply to RAN1:

# 11 UE Power Saving UE power saving may also be achieved through PDCCH monitoring adaptation mechanisms when configured by the network, including skipping of PDCCH monitoring and Search space set group (SSSG) switching. In this case UE does not monitor PDCCH during the PDCCH skipping duration or monitors PDCCH according to the search space sets applied in SSSG. However, in the following cases, UE shall monitor PDCCH regardless of the received PDCCH skipping command: - on all serving cells of the corresponding Cell Group when SR is sent and is pending; - on SpCell while contention resolution timer is running; - on SpCell during monitoring of the RAR/MsgB window; - on SpCell after successful contention resolution for the RA procedure.

# 3	Conclusion

Ignoring PDCCH skipping is discussed in this contribution with the following proposal proposed:

Proposal 1: clarify “ignore PDCCH skipping” means the UE monitors PDCCH when SR is pending/during RAR/MSGB window/when contention resolution timer is running, as well as after successful contention resolution regardless of the received PDCCH skipping command.

Proposal 2: agree the following TP to 38.300 and reply to RAN1:

# 11 UE Power Saving UE power saving may also be achieved through PDCCH monitoring adaptation mechanisms when configured by the network, including skipping of PDCCH monitoring and Search space set group (SSSG) switching. In this case UE does not monitor PDCCH during the PDCCH skipping duration or monitors PDCCH according to the search space sets applied in SSSG. However, in the following cases, UE shall monitor PDCCH regardless of the received PDCCH skipping command: - on all serving cells of the corresponding Cell Group when SR is sent and is pending; - on SpCell while contention resolution timer is running; - on SpCell during monitoring of the RAR/MsgB window; - on SpCell after successful contention resolution for the RA procedure.

# Reference

[1] R2-2209152, LS on PDCCH skipping, RAN1

[2] R2-2201915, Summary of [AT116bis-e][057][ePowSav] PDCCH Skip (Samsung), Samsung

[3] R2-2201960, LS on PDCCH Skipping in RRC_CONNECTED, RAN2