3GPP TSG-RAN WG2 Meeting #120                                                        R2-2211785

Toulouse, 14th - 18th November, 2022

Source:	Qualcomm Incorporated

Title:	U2U Relay open issues and coexistence with U2N Relay

Agenda Item:	8.9 Enhanced NR Sidelink Relay

Document for:	Discussion and Decision

# Introduction & Background

According to WID [1] and RAN2 agenda for UE-to-UE relay, this contribution discusses RAN2 part for UE-to-UE relay discovery and (re-)selection.

- Specify mechanisms to support single-hop Layer-2 and Layer-3 UE-to-UE relay (i.e., source UE -> relay UE -> destination UE) for unicast [RAN2, RAN3, RAN4].

  - Common part for Layer-2 and Layer-3 relay to be prioritized until RAN#98

    - Relay discovery and (re)selection [RAN2, RAN4]

    - Signalling support for Relay and remote UE authorization if SA2 concludes it is needed [RAN3]

In the last RAN2 meeting, there is a bit discussion about whether the gNB should be involved into U2U Relay operation and whether U2U Relay can coexist with U2N Relay.

Agreements:

RAN2 will strive to simplify the gNB involvement in U2U-relay-specific operation as compared to the U2N case.  Details are FFS, including whether some gNB control is needed for the in-coverage scenario and how/whether the gNB involvement can be simplified compared to U2N.

Rel17 SI assumptions on RRC state and coverage scenarios can be re-used.

In this contrition, it further discusses whether and how the U2U Relay can coexist with U2N Relay on aspects of discovery and Relay (re)selection from gNB and UE side.

# Discussion

2.1 UE-to-UE Relay discovery

In the last RAN2 meeting, U2U relay discovery AS aspects were discussed and agreed to reuse almost all of Rel-17 U2N discovery mechanism.

Proposal 1.1 (modified): In UE-to-UE relay, the remote/relay UE in RRC_IDLE/RRC_INACTIVE or OOC can acquire discovery configuration as in Rel17 (i.e., cell-specific configuration/preconfiguration).  FFS if any restrictions specific to UE-to-UE relay are introduced for in-coverage UE in RRC_CONNECTED.

Proposal 2.1: Protocol stack for U2N Relay discovery is re-used for U2U Relay Discovery

Proposal 2.2: U2U Relay re-uses SL-SRB4 (with associated PDCP, RLC procedures and configuration) to carry discovery messages

Proposal 4.1: Both shared and dedicated resource pool can be used for U2U discovery transmission and Rel-17 pool selection principle is re-used.

One issue needs to be further discussed is how to provide related configuration for the U2U Relay, i.e. should configure separately or share the same configuration with U2N Relay discovery.

In Rel-17, there are two discovery types i.e. U2N discovery and non-relay discovery message. But there is no separate configurations including resource pools configured for the two types of discovery message. Similarly, for the U2U discovery message introduced in Rel-18, there is no motivation to configure separate resource pools for U2U and U2N discovery.

Another issue is the frequency used for U2U discovery message. Currently, the frequency used for sidelink communication (including U2N discovery message) is configured by RRC dedicated message or SIB12, U2U discovery should reuse the same frequency configured for U2N Relay discovery message.

Proposal 1: U2U discovery message shares the same frequency and resource pools configured for U2N Relay discovery message.

Since U2U Relay discovery reuse almostly all the U2N mechanisms except for the configured Uu radio link threshold for U2N Relay UE and Remote UE to transmit discovery message, there is no additional behavior for a gNB capable of U2N Relay disovery to support U2U Relay discovery.

Observation 1: A gNB supporting U2N Relay discovery can support U2U Relay discovery natively.

On the other hand, if a gNB supports U2U Relay discovery, then it can also support U2N Relay discovery by adding the function of configuring Uu radio link threshold to the U2N Relay UE and U2N Remote UE.

Observation 2: A gNB supporting U2U Relay discovery can support U2N Relay discovery with the additional function of configuring Uu radio link threshold to the U2N Relay UE and the U2N Remote UE for U2N Relay operation.

From the above two observations, it can be seen there is no big difference for gNB to support U2N and U2U discovery message.Therefore, it is proposed that a gNB supporting U2N/U2U Relay discovery should support U2U/U2N Relay discovery and the gNB is not aware of the discovery type.

Proposal 2: gNB shares the same capability for U2U Relay and U2N Relay on discovery, and does not need to be aware of U2N Relay and U2U Relay discovery type.

In Rel-17 U2N relay, the gNB will configure a Uu link threshold to the U2N Relay UE, when the Uu radio link quality of the Relay UE is above the threshold, the Relay UE can be in U2N relay operation and transmit discovery message. For Rel-18 U2U relay, since all the UEs can be out of coverage, and RAN2 agreed there is no Uu link threshold to be configured to the U2U Relay UE for discovery transmission.

If a UE is capable of both U2N relay UE and U2U relay UE functionality, the Relay UE AS layer needs to be aware of the discovery type and then determines whether to consider the configured Uu link threshold if configured when transmitting discovery message.

Similar issue with the Remote UE, and the Remote UE AS layer needs to be aware of the discovery type and then determines whether to consider the configured Uu link threshold when transmitting discovery message.

Observation 3: If a UE is capable of both U2N relay/remote and U2U relay/remote, UE AS layer needs to be aware of discovery type, e.g. U2N discovery or U2U discovery to determine whether to consider the configured Uu link threshold when transmit discovery message.

Then is proposed that the Relay UE and the Remote UE upper layer indicates to AS layer discovery type when forwarding discovery message to AS layer.

Proposal 3: Relay UE or the Remote UE upper layer indicates to AS layer discovery type when forwarding discovery message to AS layer.

In the last RAN2 meeting, some contributions proposed to introduce the radio link condition between the soruce UE and the Relay for the Relay UE to determine whether to forward discovery message received from the source UE. The reason is to follow the similar mechansim in U2N discovery that the U2N Relay UE will determine whether to transmit discovery message based on the Uu coverage. But these two scenarios are different. For U2U Relay, once the U2U Relay UE recieves a discovery message from the source UE and decodes the discovery message successfully, it should forward the discovery message, and it relies on Relay UE (re)selection criterion to ensure the link between the source UE and the Relay UE is suitable for U2U Relay communication.

Proposal 4: It is proposed not to introduce radio link condition between the source UE and the Relay UE for Relay UE to forward discovery message.

2.2 UE-to-UE Relay (re-)selection

In the last meeting, some agreements were achieved on U2U Relay (re)selection as following.

Proposal 5.1: SL-RSRP and SD-RSRP can be used for relay selection/reselection criteria.  FFS when each of the two quantities are used and whether to re-use the criteria in Rel17.

Proposal 7.1a: Relay selection triggers include at least 1) Upper layer trigger; 2) PC5 signal strength conditions.  RAN2 further discuss details for trigger 2).

Proposal 7.1b (modified):  Relay reselection triggers include at least 1) Upper layer trigger; 2) PC5-RLF detection at the remote UE; 3) PC5-RLF indication received from the relay; 4) PC5 signal strength conditions; 5) PC5 link release message from relay to remote.  RAN2 further discuss details for trigger 4), potentially including T400 expiry.  FFS if some of the conditions could be indicated to upper layer instead of directly causing reselection.

For the measurement quantity SL-RSRP and SD-RSRP, the exisitng Rel-17 criteria on which one of the two quantities should be used can be reused to U2U Relay. That is if SL-RSRP is available, use SL-RSRP ; otherwise, use SD-RSRP.

Proposal 5: For U2U Relay (re)selection, the Rel-17 criteria are resued on selecting which one of SL-RSRP and SD-RSRP should be used.

For the PC5 signal strengh conditions which trigger U2U Relay UE selection, similar as REL-17, a set of parameters should be configured or preconfigured to the Remote UE.

In existing U2N Relay selection, the parameters SL-ReselectionConfig-r17 is configured to the U2N Remote UE as following. For U2U Relay, the same parameters can be configured to U2U Remote UE for Relay (re)selection.

-- ASN1START

-- TAG-SL-REMOTEUE-CONFIG-START

SL-RemoteUE-Config-r17::=           SEQUENCE {

threshHighRemote-r17                RSRP-Range                                       OPTIONAL,     -- Need R

hystMaxRemote-r17                   Hysteresis                                       OPTIONAL,     -- Cond ThreshHighRemote

sl-ReselectionConfig-r17            SL-ReselectionConfig-r17                         OPTIONAL      -- Need R

}

SL-ReselectionConfig-r17::=         SEQUENCE {

sl-RSRP-Thresh-r17                  SL-RSRP-Range-r16                                OPTIONAL,     -- Need R

sl-FilterCoefficientRSRP-r17        FilterCoefficient                                OPTIONAL,     -- Need R

sl-HystMin-r17                      Hysteresis                                       OPTIONAL      -- Cond SL-RSRP-Thresh

}

-- TAG-SL-REMOTEUE-CONFIG-STOP

-- ASN1STOP

Proposal 6: The REL-17 U2N Relay (re)selection parameters (i.e. included in SL-ReselectionConfig-r17) are used as PC5 signal strength conditions for the Remote UE to (re)select a U2U Relay UE.

One additional issue needs to be discussed is whether the U2U Relay (re)selection parameters can share the same parameter configured for U2N Relay (re)selection or should be configured separately.

It is understood there is no big difference between the U2U Relay communication and U2N Relay communication on the sidelink radio condition requirement, and then no motivation to configure separate Relay (re)selection parameters. And it is also beneficial to save SIB12 signalling overhead if U2N relay and U2U relay can share the same parameters.

Proposal 7: U2U Relay (re)selection can share the same configuration with U2N Relay (re)selection provided in SIB12.

Similarly, for CONNECTED Remote UE and Relay UE, the existing U2N Relay (re)selection parameters configured in dedicated signaling can be reused to U2U Relay (re)selection.

Proposal 8: U2U Relay (re)selection can share the same configuration with U2N Relay (re)selection parameters provided in dedicated signaling.

From gNB point of view, gNB is only involved to provide Relay (re)selection parameters for U2U Relay and U2N Relay. Based on the above discussion, it can be seen there is no difference. And then gNB should be unaware of U2U Relay and U2N relay on relay (re)selection.

Proposal 9: gNB shares the same capability for U2U Relay and U2N Relay on Relay (re)selection, and does not need to be aware of U2N Relay and U2U Relay (re)selection.

# Conclusion

This contribution discusses RAN2 part for UE-to-UE relay discovery and (re-)selection, and provides the following proposals.

UE-to-UE Relay discovery

Observation 1: A gNB supporting U2N Relay discovery can support U2U Relay discovery natively.

Observation 2: A gNB supporting U2U Relay discovery can support U2N Relay discovery with the additional function of configuring Uu radio link threshold to the U2N Relay UE and the U2N Remote UE for U2N Relay operation.

Observation 3: If a UE is capable of both U2N relay/remote and U2U relay/remote, UE AS layer needs to be aware of discovery type, e.g. U2N discovery or U2U discovery to determine whether to consider the configured Uu link threshold when transmit discovery message.

Proposal 1: U2U discovery message shares the same frequency and resource pools configured for U2N Relay discovery message.

Proposal 2: gNB shares the same capability for U2U Relay and U2N Relay on discovery, and does not need to be aware of U2N Relay and U2U Relay discovery type.

Proposal 3: Relay UE or the Remote UE upper layer indicates to AS layer discovery type when forwarding discovery message to AS layer.

Proposal 4: It is proposed not to introduce radio link condition between the source UE and the Relay UE for Relay UE to forward discovery message.

UE-to-UE Relay (re-)selection

Proposal 5: For U2U Relay (re)selection, the Rel-17 criteria are resued on selecting which one of SL-RSRP and SD-RSRP should be used.

Proposal 6:The REL-17 U2N Relay (re)selection parameters (i.e. included in SL-ReselectionConfig-r17) are used as PC5 signal strength conditions for the Remote UE to (re)select a U2U Relay UE.

Proposal 7: U2U Relay (re)selection can share the same configuration with U2N Relay (re)selection provided in SIB12.

Proposal 8: U2U Relay (re)selection can share the same configuration with U2N Relay (re)selection parameters provided in dedicated signaling.

Proposal 9: gNB shares the same capability for U2U Relay and U2N Relay on Relay (re)selection, and does not need to be aware of U2N Relay and U2U Relay (re)selection.

# Reference

[1]    RP-221262: New WID on NR sidelink relay enhancements

[2]    R2-2211101 RAN2#119bis-e Meeting Report	MCC