3GPP TSG-RAN WG2 Meeting #120	R2-2211843

Toulouse, France, November 14th –18th, 2022

Agenda item:	6.9.1

Source: 	Fujitsu

Title: 	BFD relaxation for serving cell with mTRP

Document for:	Discussion and decision

- Introduction

This contribution will discuss BFD relaxation for serving cell with mTRP.

- Discussion

Currently, requirements for TRP specific SSB/CSI-RS based BFD have been captured in section 8.18 of TS38.133 [1]. For the TRP specific BFD, the UE shall assess the downlink radio link quality of a serving cell based on the reference signal provided in the two sets q0,0 and q0,1.

Observation 1: Requirements for TRP specific SSB/CSI-RS based BFD have been captured in section 8.18 of TS38.133.

In addition, requirement on Link Recovery Procedures for UE configured with relaxed measurement criteria is specified in section 8.5 of TS38.133 [1] where multiple resources can be configured in set q0 on a serving cell for BFD evaluation.

Observation 2: Requirements for SSB/CSI-RS based BFD (including multiple resources) for UE fulfilling relaxed measurement criteria have been captured in section 8.5 of TS38.133.

For a serving cell with mTRP, two BFD-RS sets will be configured to the UE and each BFD-RS set corresponds to one TRP including multiple resources.

For the serving cell with mTRP, the following issues are observed regarding BFD relaxation:

- Whether BFD relaxation applies the serving cell with mTRP or not

- Whether TRP or cell specific BFD relaxation is supported for the serving cell with mTRP

  - If cell specific BFD relaxation is supported, requirements on Link Recovery Procedures for UE configured with relaxed measurement criteria captured in section 8.5 of TS38.133 should take two BFD-RS sets into account;

  - If TRP specific BFD relaxation is supported, BFD relaxation state reporting needs to be enhanced.

    - TRP specific BFD relaxation implies that the evaluation for relaxed measurement criterion, the BFD relaxation and BFD relaxation state reporting will be performed per TRP, i.e. per BFD-RS set

Proposal 1: RAN2 can support BFD relaxation for the serving cell with mTRP.

Proposal 2: If P1 is agreed, RAN2 is asked to discuss and determine whether TRP or cell specific BFD relaxation is supported for the serving cell with mTRP.

In addition, RAN4 should be involved if P1 is agreed. RAN2 sends LS to RAN4 to inform the agreements on BFD relaxation for the serving cell with mTRP and asks RAN4 to specify corresponding requirements.

If TRP specific BFD relaxation is supported, BFD relaxation state reporting needs to be enhanced. For the serving cell without mTRP, current mechanism to report BFD relaxation state is feasible. However, for the serving cell with mTRP, separate BFD relaxation state should be provided per TRP. The corresponding CR is provided in [2].

Proposal 3: RAN2 is proposed to agree the CR provided in [2] if TRP specific BFD relaxation is supported.

- Conclusion

In this contribution, we have some discussions on BFD relaxation for serving cell with mTRP and the following observations and proposals are made:

Observation 1: Requirements for TRP specific SSB/CSI-RS based BFD have been captured in section 8.18 of TS38.133.

Observation 2: Requirements for SSB/CSI-RS based BFD (including multiple resources) for UE fulfilling relaxed measurement criteria have been captured in section 8.5 of TS38.133.

Proposal 1: RAN2 can support BFD relaxation for the serving cell with mTRP.

Proposal 2: If P1 is agreed, RAN2 is asked to discuss and determine whether TRP or cell specific BFD relaxation is supported for the serving cell with mTRP.

Proposal 3: RAN2 is proposed to agree the CR provided in [2] if TRP specific BFD relaxation is supported.

References

- 3GPP TS 38.133: "NR; Requirements for support of radio resource management" v17.7.0

- R2-2211844 BFD relaxation for serving cell with mTRP, Fujitsu, CR3649 TS 38.331 17.2.0, RAN2#120