3GPP TSG RAN WG2 #120                                R2-2211851

Toulouse, France, November 14th – 18th, 2022

Agenda item:	8.16.3

Source:	Fujitsu

Title:	Discussion on use case specific aspects of AIML via air interface

Document for:	Discussion/Decision

1. Introduction

The AI/ML for Air Interface study item has been setup in RAN plenary meeting #95 and the corresponding SID [1] indicated the potential use cases study in AI/ML topic, the detailed paragraphs can be found below:

Use cases to focus on: - Initial set of use cases include: o CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] o Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] o Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98. o The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project.

And for the selected use cases, the tasks for further specification impacts have been described as the following:

Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1 - Consider aspects related to, e.g., capability indication, configuration, and control procedures (training/inference), and management of data and AI/ML model, per RAN1 input - Collaboration level specific specification impact per use case

# 2. Discussion

In the following sections, we give our views on the RAN2 potential works for the 3 use cases (CSI feedback enhancement, beam management and positioning accuracy enhancement) in the initial set of use cases, based on the RAN1 agreements study achieved in previous three RAN1 meetings.

RAN2 will focus on high layer mechanism and signaling study of the procedures in model life cycle management, such as data collection, model monitoring and model switching issues according to the conclusions and agreements achieved in each sub agenda of three use cases in RAN1. In the following discussions, we share our view on RAN2 study scope, for the 3 use cases, it is necessary to study the agreements made on these aspects, respectively.

## 2.1 CSI Feedback Enhancement

From our point of view, RAN2 has specification impacts in at least the following two LCM functions for CSI feedback enhancements.

### 2.1.1 Model training

Agreement on model training made by RAN1 can be found below:

Agreement (R1-110) In CSI compression using two-sided model use case, the following AI/ML model training collaborations will be further studied: - Type 1: Joint training of the two-sided model at a single side/entity, e.g., UE-sided or Network-sided. - Type 2: Joint training of the two-sided model at network side and UE side, respectively. - Type 3: Separate training at network side and UE side, where the UE-side CSI generation part and the network-side CSI reconstruction part are trained by UE side and network side, respectively. - Note: Joint training means the generation model and reconstruction model should be trained in the same loop for forward propagation and backward propagation. Joint training could be done both at single node or across multiple nodes (e.g., through gradient exchange between nodes). - Note: Separate training includes sequential training starting with UE side training, or sequential training starting with NW side training [, or parallel training] at UE and NW - Other collaboration types are not excluded. Conclusion (R1-110) If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, a one-sided structure is considered as a starting point, where the AI/ML inference is performed at either gNB or UE.

There will be RAN2 specification impacts such as additional signaling and procedure to facilitate the model training for all the three collaboration types, and the detailed specification impacts may vary according to the specific properties of each type, e.g., Type1 training may need specification enhancement to support model transfer, while Type3 training may need specification enhancement to support dataset transfer. Therefore, based on agreements made in RAN1, we have the following proposal:

Proposal 1 For CSI compression using two-sided model use case, study potential RAN2 signaling and procedures for supporting the three AI/ML model training collaboration types agreed in RAN 1.

### 2.1.2 Model monitoring

Agreement on model monitoring made by RAN1 can be found below:

Agreement (R1-110bis-e) In CSI compression using two-sided model use case, study potential specification impact for performance monitoring including: - NW-side AI model performance monitoring: NW monitors the performance metrics and make decisions of model activation/ deactivation/updating/switching - UE-side AI model performance monitoring: UE monitors the performance metrics and reports to Network, NW will further make decisions of model activation/ deactivation/updating/switching Agreement (R1-110bis-e) In CSI compression using two-sided model use case, further study potential specification impact related to assistance signalling and procedure for model performance monitoring. Agreement (R1-110bis-e) In CSI compression using two-sided model use case, further study potential specification impact related to potential co-existence and fallback mechanisms between AI/ML-based CSI feedback mode and legacy non-AI/ML-based CSI feedback mode. Agreement (R1-110bis-e) In CSI compression using two-sided model use case, further study at least the following options for performance monitoring metrics/methods: - Intermediate KPIs as monitoring metrics (e.g., SGCS) - Eventual KPIs (e.g., Throughput, hypothetical BLER, BLER, NACK/ACK). - Legacy CSI based monitoring: schemes using additional legacy CSI reporting - Other monitoring solutions, at least including the following option: - Input or Output data based monitoring: such as data drift between training dataset and observed dataset and out-of-distribution detection

Both the network and the UE can play the monitoring role for CSI feedback two-sided AI/ML model, the potential RAN2 specification impacts can be the information exchanged between these two sides for supporting model monitoring, e.g., for network-side monitoring, the monitoring metric and threshold can be decided by network implementation, the decisions based on monitoring results need to be transmitted from NW to UE with signaling for indicating follow-up actions of the model at UE side such as model updating, model switching or fallback. On the other hand, if UE oversees monitoring, the corresponding potential signaling will be different. The UE needs report the monitoring results to NW and the final decision on the following-up actions still going to NW side.

Therefore, based on the agreements made in RAN1, we have the following proposal for model monitoring of CSI feedback.

Proposal 2 For CSI compression using two-sided model use case, study model monitoring (e.g., performance monitoring) related signalling and procedures, taking at least the following aspects into account:

- NW-side AI model performance monitoring including NW monitors the performance metrics and make decisions of model activation/ deactivation/updating/switching

- UE-side AI model performance monitoring including UE monitors the performance metrics and reports to Network, NW will further make decisions of model activation/ deactivation/updating/switching

- potential co-existence and fallback mechanisms between AI/ML-based CSI feedback mode and legacy non-AI/ML-based CSI feedback mode.

### 2.1.3 Data collection

Agreement (R1-110) In CSI compression using two-sided model use case, further discuss at least the following aspects, including their necessity/feasibility/potential specification impact, for data collection for AI/ML model training/inference/update/monitoring: - Assistance signalling for UE’s data collection - Assistance signalling for gNB’s data collection - Delivery of the datasets.

Data collection is important to model training, model inference, model update and model monitoring. Assistance signalling for UE side or gNB side data collection is needed such as signalling for label data. Based on the agreements made in RAN1, we have the following proposal for data collection:

Proposal 3 For CSI compression using two-sided model use case, study necessity, feasibility, procedures and potential specification impact for data collection for AI/ML model training/inference/update/monitoring, taking the following aspects into account:

- Assistance signalling for UE’s data collection

- Assistance signalling for gNB’s data collection

- Delivery of the datasets.

### 2.1.4 Model inference

The unique feature of two-sided models is its CSI generation model part at UE side should work with its paired CSI reconstruction model part at network side together. In this way, the CSI information compressed at UE side can be recovered at network side.

Therefore, there is a need to have a mechanism let UE and NW to match or pair the model part of each side before running the two-sided model.

Proposal 4 For CSI compression using two-sided model use case, study potential mechanism of model pairing, so that CSI generation model part at UE side can work with its paired CSI reconstruction model part at network side together.

## 2.2 Beam Management

Agreements on sub use cases made by RAN1 can be found below:

Agreement (R1-109-e) For AI/ML-based beam management, support BM-Case1 and BM-Case2 for characterization and baseline performance evaluations • BM-Case1: Spatial-domain DL beam prediction for Set A of beams based on measurement results of Set B of beams • BM-Case2: Temporal DL beam prediction for Set A of beams based on the historic measurement results of Set B of beams • FFS: details of BM-Case1 and BM-Case2 • FFS: other sub use cases Note: For BM-Case1 and BM-Case2, Beams in Set A and Set B can be in the same Frequency Range

For the above sub use cases in beam management, we share our views on RAN2 study focus with respect to model training, model inference and monitoring.

### 2.2.1 Data collection for model training

For model training, the agreement on data collection has been achieved as follows

Agreement (R1-110) For the data collection for AI/ML model training (if supported), study the following aspects as a starting point for potential necessary specification impact: - Signalling/configuration/measurement/report for data collection, e.g., signalling aspects related to assistance information (if supported), Reference signals - Content/type of the collected data - Other aspect(s) is not precluded

For data collection for AI/ML model training, the configuration of measurement report should be enhanced if large volume of data is collected. It is recommended that RAN2 study the specification impacts on configuration of measurement report for data collection until RAN1 has sufficient study outcome.

Proposal 5: For beam management use cases, wait for RAN1 study on data collection signaling for potential necessary specification impact.

### 2.2.2 Model inference

For model inference, the agreement has been achieved as follows:

Agreement (R1-110) In order to facilitate the AI/ML model inference, study the following aspects as a starting point: - Enhanced or new configurations/UE reporting/UE measurement, e.g., Enhanced or new beam measurement and/or beam reporting - Enhanced or new signalling for measurement configuration/triggering - Signalling of assistance information (if applicable) - Other aspect(s) is not precluded

For spatial-domain beam prediction, the subset of beams is measured to predicted one or several optimal beams. How to configure the subset of beams on gNB and report the measurement results from UE should be specified in RAN2. It is recommended that RAN2 study the specification impacts on configuration of RS resource and measurement report for model inference until RAN1 has sufficient study outcome.

Proposal 6: For beam management use cases, wait for RAN1 study on configuration of RS resource and measurement report for model inference.

### 2.2.3 Model monitoring

For model monitoring, the agreement has been achieved as follows:

Agreement (R1-110) Regarding the model monitoring for BM-Case1 and BM-Case2, to investigate specification impacts from the following aspects - Performance metric(s) - Benchmark/reference for the performance comparison - Signalling/configuration/measurement/report for model monitoring, e.g., signalling aspects related to assistance information (if supported), Reference signals - Other aspect(s) is not precluded

In order to facilitate the agreed aspects in RAN1, it is recommended that RAN2 study the specification impacts on configuration and report for model monitoring until RAN1 has sufficient study outcome.

Proposal 7: For beam management use cases, wait for RAN1 study on configuration and report for model monitoring.

And for the mechanism for realizing the follow-up actions after model monitoring is worthy of study.

Proposal 8: For beam management use cases, study the mechanism for model activation/deactivation/switching/selection/fallback operation.

## 2.3 Positioning Accuracy Enhancement.

The positioning use case is different from CSI feedback and beam management in following aspects:

- LMF, as a high-layer entity, is inevitably involved into the positioning procedure. Two higher layer protocols, NRPPa and LPP are being used for overseeing and managing the signalling and information between LMF and gNB, UE respectively.

- The model label is not fully available during model inference or online-training, which requires additional information or frameworks to support it.

- There are two sub use cases of AI/ML positioning, direct and assisted, for the assisted positioning method, the output of the AI/ML model need additional processing for deducing the final positioning results.

From our knowledge, the current specification NRPPa and LPP are existing positioning framework for the signaling exchange between gNB/LMF and UE/LMF respectively.

Proposal 9: Study enhanced LPP and NRPPa signaling to support AI/ML positioning model lifecycle management.

Agreements on sub use cases and collaboration levels made by RAN1 can be found below:

Agreement (R1-110bis-e) - Study and provide inputs on benefit(s) and potential specification impact at least for the following cases of AI/ML based positioning accuracy enhancement - Case 1: UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning - Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

Agreement on model monitoring made by RAN1 can be found below:

Agreement (R1-110bis-e) Regarding AI/ML model monitoring for AI/ML based positioning, to study and provide inputs on potential specification impact for the following aspects - Assistance signalling and procedure at least for UE-side model - Report/feedback and procedure at least for Network-side model - Note1: study is applicable to both of the following cases - Model inference and model monitoring at the same entity - Entity to perform the model monitoring is not the same entity for model inference - Note2: other aspects are not precluded

Agreement on model training and data collection made by RAN1 can be found below:

Agreement (R1-110bis-e) Regarding data collection for AI/ML model training for AI/ML based positioning, at least for each of the agreed cases (Case 1 to Case 3b) - Study whether (and if so how) an entity can be used to obtain ground truth label and/or other training data - Companies are requested to report their assumption of the entity (or entities) used to obtain ground truth label and/or other training data for each case (Case 1 to Case 3b) - Companies are requested to report their assumption of applicable ground truth label (e.g., location or other information) and/or other training data (e.g., measurement) for each case (Case 1 to Case 3b) - Feasibility study on the entity to obtain ground truth label and/or other training data takes into account at least - availability of the entity to obtain label and/or other training data - Note: further discussion and decision of the entity (or entities) used to obtain ground truth label and/or other training data for each case (Case 1 to Case 3b) is not precluded based on companies’ input - Study potential signaling and procedure to enable data collection - Potential specification impact on the details of request/report of label and/or other training data, and to enable delivering the collected label and/or other training data to the training entity when the training entity is not the same entity to obtain label and/or other training data - Potential specification impact on assistance signaling indicating reference signal configuration(s) to derive label and/or other training data

Agreement of model identification/configuration made by RAN1 can be found below:

Agreement (R1-110bis-e) Regarding AI/ML model indication[/configuration], to study and provide inputs on potential specification impact at least for the following aspects on conditions/criteria of AI/ML model for AI/ML based positioning accuracy enhancement - Validity conditions, e.g., applicable area/[zone/]scenario/environment and time interval, etc. - Model capability, e.g., positioning accuracy quality and model inference latency - Conditions and requirements, e.g., required assistance signalling and/or reference signals configurations, dataset information - Note: other aspects are not precluded

From the above-mentioned agreements, it is noted that:

- Model monitoring, model training and model indication are agreed specification impacts for AI/ML positioning accuracy enhancement.

- For all the aspects mentioned above, detailed analysis should be based on the agreed sub use cases (case1 to case 3b) because the there may be huge differences among the specification impacts of different sub use cases which the locations of the AI/ML model and the positioning functions.

Therefore, we would like to analyze the RAN2 specification impacts for AI/ML positioning in data collection for model monitoring, model training and model indication.

### 2.3.1 Data collection for model training

Similar to all AI/ML model training procedure, both input datasets and ground truth are necessary to run the model training /fine-tuning. The potential specification impacts for model training, as agreed above, are data collection and ground truth obtaining. From our point of view, the data collection and ground truth (label) obtaining can be arranged as follows:

- Data collection:

  - Data requirements information exchange.

  - Data measurement and reporting.

  - Dataset transfer.

- Label obtaining:

  - Assistance information for supporting label calculation.

  - Label transfer.

  - Label quality check.

For part a) of data collection, the model training entity must know the data properties which will be fed into the model for training, the properties will not be the same all the time and may vary according to different factors such as application QoS requirements, system capabilities or model properties. If these variations are triggered by the network entities where model training module does not deploy, then the signaling must be designed for transmitting this information across air interface and these signaling are supposed to be carried by RAN2.

Part b) includes reference signal configuration, channel measurements and reporting. There will be specification impacts such as reference signal pattern indication or enhanced reporting. RAN1 needs to be responsible for most of the actions but pattern indication or enhanced reporting may relate to RAN2 as well if LMF needs to be involved.

Dataset transfer is currently being discussed in RAN1 meeting and the final agreement are yet to be released so propose to defer the discussion before RAN1 finalize this part. There may not be any RAN-related spec impacts if the dataset will be transmitted by user applications.

For label collection part, no matter how to get the values of the labels, channel measurement and assistance information transfer are essential, e.g., gNB can transmit the LOS/NLOS indicator information associated with each link or path to UE for calculating the TOA between gNB and UE. Label transfer is necessary if the label calculation and AI/ML model deploy in different entities. Similar to data collection, label quality check may be based on metrics or additional information transmitted from other entities rather than label calculation entity itself.

From the above analysis, we can have the following observation and proposal.

Observation 1 The model training for AI/ML positioning involves both RAN1 and RAN2 efforts. Procedures and signaling of both data collection and label collection are in the scope of RAN2 work.

Proposal 10 Study the potential specification impacts on model training of AI/ML positioning in at least the following aspects:

- Data collection, e.g., additional information for data requirements.

- label collection, e.g., additional information for label quality.

### 2.3.2 Model monitoring

Model monitoring is not easy for positioning, because the perfect values of the UE locations or related TOA/RSTD are not available in the live network, therefore the model monitoring will probably be based on the observation of the model input data and other environmental parameters. Therefore, multiple signaling or information exchange among network entities are necessary to indicate the required input data statistics, the monitoring metrics and monitoring feedback, e.g., if the AI/ML model deploys in LMF, and the input data are generated in UE, then LMF may trigger the model monitoring procedure and request UE to send the live channel measurement, or the LMF may need the gNB to send it the environmental parameters to assist the model monitoring via LPP and NRPPa protocols, both requires RAN2 framework to realize.

Proposal 11 It is suggested to study the model monitoring related signaling and framework for positioning in at least the following aspects:

- Ground truth label obtaining

- Ground truth label quality check

### 2.3.3 Model identification

Model indication is used to indicate specific models if multiple models are available for model switching, model selection or model monitoring. For AI/ML positioning, thhe models are supposed to be trained under specific scenarios or conditions, therefore model or training associated information are necessary for model identification.

According to RAN1 discussion, the following two solutions may be used for model identification management:

- UE capability request and reporting.

- Model registration

Both of the two solutions require RAN2 signaling enhancement to support, e.g., new capabilities design for AI/ML model or specific training, or model ID assigned from the network to complete the model registration with model ID. The details will be studied in general aspect (8.16.2) because both UE capability and model registration are general for all use cases although these two topics are also being discussed in RAN1.

For positioning, one consideration is to figure out how to embed the UE capabilities or model registration into LPP or NRPPa protocols.

Proposal 12 Study model identification for positioning using UE capabilities or model registration procedure.

# 3. Conclusion

Proposal 1 For CSI compression using two-sided model use case, study potential RAN2 signaling and procedures for supporting the three AI/ML model training collaboration types agreed in RAN 1.

Proposal 2 For CSI compression using two-sided model use case, study model monitoring (e.g., performance monitoring) related signalling and procedures, taking at least the following aspects into account:

- NW-side AI model performance monitoring including NW monitors the performance metrics and make decisions of model activation/ deactivation/updating/switching

- UE-side AI model performance monitoring including UE monitors the performance metrics and reports to Network, NW will further make decisions of model activation/ deactivation/updating/switching

- potential co-existence and fallback mechanisms between AI/ML-based CSI feedback mode and legacy non-AI/ML-based CSI feedback mode.

Proposal 3 For CSI compression using two-sided model use case, study necessity, feasibility, procedures and potential specification impact for data collection for AI/ML model training/inference/update/monitoring, taking the following aspects into account:

- Assistance signalling for UE’s data collection

- Assistance signalling for gNB’s data collection

- Delivery of the datasets.

Proposal 4 For CSI compression using two-sided model use case, study potential mechanism of model pairing, so that CSI generation model part at UE side can work with its paired CSI reconstruction model part at network side together.

Proposal 5: For beam management use cases, wait for RAN1 study on data collection signaling for potential necessary specification impact.

Proposal 6: For beam management use cases, wait for RAN1 study on configuration of RS resource and measurement report for model inference.

Proposal 7: For beam management use cases, wait for RAN1 study on configuration and report for model monitoring.

Proposal 8: For beam management use cases, study the mechanism for model activation/deactivation/switching/selection/fallback operation.

Proposal 9: Study enhanced LPP and NRPPa signaling to support AI/ML positioning model lifecycle management.

Observation 1 The model training for AI/ML positioning involves both RAN1 and RAN2 efforts. Procedures and signaling of both data collection and label collection are in the scope of RAN2 work.

Proposal 10 Study the potential specification impacts on model training of AI/ML positioning in at least the following aspects:

- Data collection, e.g., additional information for data requirements.

- label collection, e.g., additional information for label quality.

Proposal 11 It is suggested to study the model monitoring related signaling and framework for positioning in at least the following aspects:

- Ground truth label obtaining

- Ground truth label quality check

Proposal 12 Study model identification for positioning using UE capabilities or model registration procedure.

# 4. Reference

- RP-221348, Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface