3GPP TSG RAN WG2 #120				R2-2211870

Toulouse, France, Nov 14 – 18, 2022

Agenda Item:	6.1.4	  UP corrections (NR Multicast)

Source:		ASUSTeK

Title:				Discussion on MBS DRX and SPS issues

Document for: Discussion and Decision

- Introduction

In the last RAN2 meeting, several UP issues were discussed in [1]. In this document, we discuss some remaining relevant issues for MBS DRX and SPS.

- Discussion

2.1 Expected PTP retransmission

According to rapporteur’s proposal and agreements [1] in the last meeting as below, details of not starting drx-HARQ-RTT-TimerDL for PTP retransmission are FFS.

Proposal 4 (10/14): RAN2 will try to clarify the MAC entity does not start drx-HARQ-RTT-TimerDL after receiving a PTM transmission if the UE does not support PTP retransmission via C-RNTI for the initial PTM transmission. FFS: Detail (to be discussed in RAN2#120)

According to RAN1’s understanding in 38.213 as shown below, PTP retransmission can be scheduled if the first HARQ-ACK reporting mode (i.e. HARQ feedback option = ack-nack) is configured. It means that drx-HARQ-RTT-TimerDL should be started for this case. For the second HARQ-ACK reporting mode (i.e. HARQ feedback option = nack-only), PTP retransmission would not happen and PTM retransmission should be always scheduled.

A PDSCH reception providing an initial transmission of a transport block is scheduled only by a multicast DCI format. For the first HARQ-ACK reporting mode, a PDSCH reception providing a retransmission of the transport block can be scheduled either by a multicast DCI format using a same G-RNTI as the G-RNTI of the initial transmission of the transport block, or by a unicast DCI format using a C-RNTI [6, TS 38.214]. An activation for SPS PDSCH receptions using a G-CS-RNTI for a corresponding SPS PDSCH configuration is provided only by a multicast DCI format as described in clause 10.2 by replacing CS-RNTI with the G-CS-RNTI. A release for SPS PDSCH receptions using a G-CS-RNTI for a corresponding SPS PDSCH configuration is provided by a multicast DCI format as described in clause 10.2 by replacing CS-RNTI with the G-CS-RNTI, or by a DCI format with CRC scrambled by CS-RNTI. For the first HARQ-ACK reporting mode and for a transport block that a UE received in a SPS PDSCH, a PDSCH reception providing a retransmission of the transport block can be scheduled either by a unicast DCI format using a CS-RNTI or by a multicast DCI format using a same G-CS-RNTI as the G-CS-RNTI of the initial transmission of the transport block [6, TS 38.214].

Proposal 1:  Based on TS 38.213, after receiving a PTM transmission, drx-HARQ-RTT-TimerDL is started for PTP retransmission if the first HARQ-ACK reporting mode (i.e. ack-nack) is configured.

Besides, according to RAN2 #116bis agreements, SPS retransmission via PTP is supported if CS-RNTI is configured.

RAN2#116bis-e: Agreements on MAC open issues - … - If MBS SPS is configured and CS-RNTI is not configured, the retransmission of SPS via PTP is not supported and MBS SPS deactivation via CS-RNTI is not supported.

Proposal 2: After receiving a PTM transmission scheduled by configured downlink multicast assignment, drx-HARQ-RTT-TimerDL is started for PTP retransmission if CS-RNTI is configured.

2.2 HARQ feedback for SPS and activation

According to the current procedure, MAC entity would not instruct physical layer to generate ACK/NACK for the following two cases:

- if HARQ feedback is disabled for G-RNTI or G-CS-RNTI transmission; or

- if HARQ feedback is NACK-only and the data is successfully decoded

#### 5.3.2.2 HARQ process … The MAC entity then shall: … 1> if the HARQ process is equal to the broadcast process; or 1> if the HARQ process is associated with a transmission indicated with a MCCH-RNTI or a G-RNTI for MBS broadcast; or 1> if the HARQ process is associated with a transmission indicated with a G-RNTI or a G-CS-RNTI for MBS multicast and HARQ feedback is disabled; or 1> if the HARQ process is associated with a transmission indicated with a G-RNTI or a G-CS-RNTI for MBS multicast and NACK only HARQ feedback is configured and the data for this TB is successfully decoded; or 1> if the timeAlignmentTimer, associated with the TAG containing the Serving Cell on which the HARQ feedback is to be transmitted, is stopped or expired and if the cg-SDT-TimeAlignmentTimer, if configured, is not running; or 1> if the HARQ process is configured with disabled HARQ feedback: 2> not instruct the physical layer to generate acknowledgement(s) of the data in this TB. 1> else: 2> instruct the physical layer to generate acknowledgement(s) of the data in this TB.

However, it has been agreed in RAN1#107-e that for multicast SPS activation/deactivation, only ACK/NACK based feedback is supported. It means that ACK/NACK should be always sent for the very first new transmission corresponding to the SPS activation addressed to G-CS-RNTI.

Proposal 3:   According to RAN1 agreement, HARQ feedback disabled or NACK-only is applied to “G-CS-RNTI for retransmission” and should not be applied to the very first transmission (i.e. new transmission) after G-CS-RNTI activation.

Besides, according to RRC spec, the configuration of HARQ feedback disabled or NACK-only can be also applied to “configured downlink assignment for MBS multicast” activated by a G-CS-RNTI. This case is missing.

harq-FeedbackOptionMulticast-r17 ENUMERATED {ack-nack, nack-only}

harq-FeedbackOptionMulticast Indicates the feedback mode for MBS multicast dynamically scheduled PDSCH or SPS PDSCH.

Proposal 4:  According to RRC spec, in addition to G-RNTI or G-CS-RNTI (i.e. dynamically scheduling), HARQ feedback disabled or NACK-only can be also applied to “configured downlink assignment for MBS multicast”

2.3 DRX Command

Unicast

According to the current procedure, gNB can only schedule DRX Command MAC CE for unicast by PDCCH addressed to C-RNTI. Since DL transmissions periodically on configured downlink assignments could occur within a periodic running drx-onDurationTimer or could start drx-InactivityTimer due to SPS activation (considered as a new transmission), it’s quite possible for gNB to also include DRX Command MAC CE for unicast into MAC PDU to stop these two times for saving UE power.

## 5.7 Discontinuous Reception (DRX) … When DRX is configured or when multicast DRX is configured, the MAC entity shall: … 1> if a DRX Command MAC CE indicated by PDCCH addressed to C-RNTI for unicast transmission or a Long DRX Command MAC CE is received: 2> stop drx-onDurationTimer for each DRX group; 2> stop drx-InactivityTimer for each DRX group.

Proposal 5:   In addition to scheduling PDCCH addressed to C-RNTI, it’s allowed for gNB to include DRX Command MAC CE for unicast into MAC PDU scheduled by PDCCH addressed to CS-RNTI or by configured downlink assignment.

Multicast

Similar to Proposal 5, PDCCH addressed to G-CS-RNTI and configured downlink multicast assignment should be able to schedule DRX Command MAC CE for multicast.

Proposal 6:   In addition to scheduling PDCCH addressed to G-RNTI, it’s allowed for gNB to include DRX Command MAC CE for multicast into MAC PDU scheduled by PDCCH addressed to G-CS-RNTI or by configured downlink multicast assignment.

Based on [2], some text proposal are shown below.

Text proposal of TS 38.321

#### 5.3.2.2 HARQ process When a transmission takes place for the HARQ process, one or two (in case of downlink spatial multiplexing) TBs and the associated HARQ information are received from the HARQ entity. … The MAC entity then shall: … 1> if the HARQ process is associated with a transmission indicated with a Temporary C-RNTI and the Contention Resolution is not yet successful (see clause 5.1.5); or 1> if the HARQ process is associated with a transmission indicated with a MSGB-RNTI and the Random Access procedure is not yet successfully completed (see clause 5.1.4a); or 1> if the HARQ process is equal to the broadcast process; or 1> if the HARQ process is associated with a transmission indicated with a MCCH-RNTI or a G-RNTI for MBS broadcast; or 1> if the HARQ process is associated with a transmission indicated with a G-RNTI or a G-CS-RNTI for retransmission, or a configured downlink assignment for MBS multicast and HARQ feedback is disabled; or Proposal 3 & Proposal 4 1> if the HARQ process is associated with a transmission indicated with a G-RNTI or a G-CS-RNTI for retransmission, or a configured downlink assignment for MBS multicast and NACK only HARQ feedback is configured and the data for this TB is successfully decoded; or Proposal 3 & Proposal 4 1> if the timeAlignmentTimer, associated with the TAG containing the Serving Cell on which the HARQ feedback is to be transmitted, is stopped or expired and if the cg-SDT-TimeAlignmentTimer, if configured, is not running; or 1> if the HARQ process is configured with disabled HARQ feedback: 2> not instruct the physical layer to generate acknowledgement(s) of the data in this TB. 1> else: 2> instruct the physical layer to generate acknowledgement(s) of the data in this TB. The MAC entity shall ignore NDI received in all downlink assignments on PDCCH for its Temporary C-RNTI when determining if NDI on PDCCH for its C-RNTI has been toggled compared to the value in the previous transmission. NOTE: If the MAC entity receives a retransmission with a TB size different from the last TB size signalled for this TB, the UE behavior is left up to UE implementation. ## 5.7 Discontinuous Reception (DRX) … When DRX is configured or when multicast DRX is configured, the MAC entity shall: … 1> if a DRX Command MAC CE indicated by PDCCH addressed to C-RNTI or CS-RNTI, or by a configured downlink assignment for unicast transmission or a Long DRX Command MAC CE is received: Proposal 5 2> stop drx-onDurationTimer for each DRX group; 2> stop drx-InactivityTimer for each DRX group. … 1> if a DRX Command MAC CE indicated by PDCCH addressed to C-RNTI or CS-RNTI, or by a configured downlink assignment for unicast transmission is received: Proposal 5 2> if the Short DRX cycle is configured: 3> start or restart drx-ShortCycleTimer for each DRX group in the first symbol after the end of DRX Command MAC CE reception; 3> use the Short DRX cycle for each DRX group. 2> else: 3> use the Long DRX cycle for each DRX group. ## …. ## 5.7b Discontinuous Reception (DRX) for MBS Multicast … When multicast DRX is configured for a G-RNTI or G-CS-RNTI or when unicast DRX is configured, the MAC entity shall for this G-RNTI or G-CS-RNTI: 1> if a MAC PDU is received in a configured downlink multicast assignment: 2> if HARQ feedback is enabled: 3> start the drx-HARQ-RTT-TimerDL-PTM for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback; 3> if the first HARQ-ACK reporting mode (i.e. ack-nack) is configured as specified in TS 38.213 [6]; and Proposal 1 3> if CS-RNTI is configured: Proposal 2 43> start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback. 2> stop the drx-RetransmissionTimerDL-PTM for the corresponding HARQ process; 2> stop the drx-RetransmissionTimerDL for the corresponding HARQ process. … 1> if a DRX Command MAC CE indicated by PDCCH addressed to a G-RNTI or G-CS-RNTI, or by a configured downlink multicast assignment is received: Proposal 6 2> stop drx-onDurationTimerPTM of the DRX for this G-RNTI or G-CS-RNTI; Proposal 6 2> stop drx-InactivityTimerPTM of the DRX for this G-RNTI or G-CS-RNTI. Proposal 6 1> if [(SFN × 10) + subframe number] modulo (drx-LongCycle-PTM) = drx-StartOffset-PTM: 2> start drx-onDurationTimerPTM after drx-SlotOffsetPTM from the beginning of the subframe. 1> if the MAC entity is in Active Time for this G-RNTI or G-CS-RNTI; or 1> if multicast DRX is not configured (i.e., only unicast DRX is configured): 2> monitor the PDCCH for this G-RNTI or G-CS-RNTI as specified in TS 38.213 [6]; 2> if the PDCCH indicates a DL multicast transmission: 3> if HARQ feedback is enabled: 4> start the drx-HARQ-RTT-TimerDL-PTM for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback; 4> if the first HARQ-ACK reporting mode (i.e. ack-nack) is configured as specified in TS 38.213 [6]: Proposal 1 54> start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback. 3> stop the drx-RetransmissionTimerDL-PTM for the corresponding HARQ process; 3> stop the drx-RetransmissionTimerDL for the corresponding HARQ process. 2> if the PDCCH indicates a new multicast transmission for this G-RNTI or G-CS-RNTI: 3> start or restart drx-InactivityTimerPTM in the first symbol after the end of the PDCCH reception. NOTE: A PDCCH indicating activation of multicast SPS is considered to indicate a new transmission. The MAC entity needs not to monitor the PDCCH for a G-RNTI or a G-CS-RNTI if it is not a complete PDCCH occasion (e.g. the Active Time for a G-RNTI or a G-CS-RNTI starts or ends in the middle of a PDCCH occasion).

- Conclusion

In this document, we discuss some issues for MBS DRX and SPS.

Proposal 1:  Based on TS 38.213, after receiving a PTM transmission, drx-HARQ-RTT-TimerDL is started for PTP retransmission if the first HARQ-ACK reporting mode (i.e. ack-nack) is configured.

Proposal 2: After receiving a PTM transmission scheduled by configured downlink multicast assignment, drx-HARQ-RTT-TimerDL is started for PTP retransmission if CS-RNTI is configured.

Proposal 3:   According to RAN1 agreement, HARQ feedback disabled or NACK-only is applied to “G-CS-RNTI for retransmission” and should not be applied to the very first transmission (i.e. new transmission) after G-CS-RNTI activation.

Proposal 4: According to RRC spec, in addition to G-RNTI or G-CS-RNTI (i.e. dynamically scheduling), HARQ feedback disabled or NACK-only can be also applied to “configured downlink assignment for MBS multicast”

Proposal 5:   In addition to scheduling PDCCH addressed to C-RNTI, it’s allowed for gNB to include DRX Command MAC CE for unicast into MAC PDU scheduled by PDCCH addressed to CS-RNTI or by configured downlink assignment.

Proposal 6:   In addition to scheduling PDCCH addressed to G-RNTI, it’s allowed for gNB to include DRX Command MAC CE for multicast into MAC PDU scheduled by PDCCH addressed to G-CS-RNTI or by configured downlink multicast assignment.

Reference

[1]  R2-2210873		Report of Offline 603: UP Correction for Rel-17 MBS		Samsung

[2]  R2-2211060		Corrections for MBS								OPPO