3GPP TSG RAN WG2 Meeting #120               	R2-2211919

# Toulouse, France, 14th 18th November 2022

Agenda Item: 	8.2.4

Source:		Sony

Title:			Considerations on some aspects for LPHAP

Document for:	Discussion & Decision

# Introduction

A new release-18 study item on expanded and improved NR positioning was approved in RANP#94e meeting in December 2021 [1]. The study item has several objectives, in which one of them is related to supporting Low Power High Accuracy Positioning (LPHAP).

- Study the requirements on LPHAP as developed by SA1 and evaluate whether existing RAN functionality can support these power consumption and positioning requirements. Based on the evaluation, and, if found beneficial, study potential enhancements to help address any limitations [RAN2, RAN1] - Study is limited to a single representative use case (use case 6 as defined TS 22.104). The choice of selected use case can be reviewed at the start of the study. - Study is limited to enhancements to RRC_INACTIVE and/or RRC_IDLE state

At RAN1, a set of agreements related to evaluation assumptions and reference scenarios for evaluation of LPHAP techniques was made., It was also confirmed in an agreement that use case 6 defined in TS 22.104 is the single representative use case for the study of LPHAP.

At RAN2#119bis e meeting [5], the following agreements were made;

RAN2 do not introduce a new device type for positioning only.

Proposal 2: RAN2 agree that support of MT-SDT in Rel-18 positioning is treated as low priority in SI. (14/15)

Proposal 3 (modified): RAN2 agree to study enhancements on SRS configuration (12/15). Further study the following candidate enhancements on SRS configuration, including the possible interference and changes of spatial relations problems.

- 	a) Validity area mechanism; (12/13)

- 	b) SRS update mechanism; (10/13)

- 	c) Pre-configure multiple SRS, which could include broadcast transmission of configurations with UE-specific determination along with the network of a configuration; (9/13)

FFS if item c would require network nodes to measure multiple SRS configurations for the same UE simultaneously.

LS to RAN1 to ask them about interference issues with SRS configurations across multiple cells and about the validity of SRS parameters.

RAN2 will study the following candidate enhancements on DL-PRS configuration after there is progress in RAN1 and potentially RAN4.

-	a) Simplified PRS configuration; (2/15)

-	b) PRS is configured close to SSBs; (2/15)

-	c) Limit PRS reception in a time period; (3/15)

RAN2 can consider the feasibility of configuration alignment between PRS and DRX (at least paging DRX).

Exposure of LPHAP information to the gNB and/or LMF (e.g., as a UE capability) can be discussed in normative work if any enhancement for LPHAP is agreed, taking into account any guidance from SA2.

Further discuss the two candidate solutions on how to report measurements taken in RRC_IDLE as below:

Alt1: measurement is performed in IDLE and reported in CONNECTED, including the concerns:

- Whether the mechanism of measurement in IDLE and report in CONNECTED is more beneficial for power saving than legacy mechanism, i.e. RRC_INACTIVE positioning.

- Whether the CN can handle the measurement reports from the UE in RRC_CONNECTED, while the positioning was performed in RRC_IDLE.

Alt2: measurement is performed in IDLE and report is carried with initial access messages, including the concern:

- Is there AS context/security issue on sending the measurements to LMF?

In this contribution, we discuss aspects related to the potential positioning methods needed to support LPHAP devices based on agreements from the last meeting [5] .

# Discussion on the study and upper layer enhancements

At the last meeting, a large number of agreements were made, but there is still some further remaining issues. In this document we address some of these issues, particularly we highlight our views related to MT-SDT support, measurement reporting and DRX alignment.

MT-SDT

At the last meeting [5] it was agreed to down prioritize MT-SDT. However, it would still be relevant to capture some agreements related to MT-SDT in the study item report [6].

For more details on the SDT procedure, we, would have to wait for the progress in the MT-SDT Work Item [8]

One aspect is how to configure the UE to perform positioning measurements and reporting, even though the principles of “Low Power Periodic and Triggered 5GC-MT-LR Procedures” [7], needs to be taken into account.

For these LCS procedures there are a number of variants like “deferred” or scheduled location time, however, network triggered positioning procedure, for UE´s in Idle/Inactive is currently not supported.

Observation 1: Network triggered MT-LR procedures for UEs in Idle/Inactive is currently not supported.

The MT-SDT procedure would be used when network needs to trigger and potentially also configure a UE in RRC Idle mode/RRC-Inactive mode) to perform measurement and/or measurement reporting.

In some scenarios the UE has been configured when in RRC Connected mode, or when being released from Connected mode to Idle/Inactive mode. In some other scenarios the UE  in Idle/Inactive is not configured at all, and hence needs both being triggered as well as configured in order to perform the positioning measurements and/or reporting.

Observation 2: Depending on the scenario the UE may have to be configured in Idle/Inactive mode, if not configured while released from Connected mode.

Proposal 1: Both alternatives, configuration of a UE in Idle/Inactive mode triggered via MT-SDT or configured via RRC release should be considered.

Typically and in most cases, the UE will be configured separately and prior to doing the actual positioning procedure. Then for initiating the positioning procedure the UE needs to be triggered.  Here we believe some wake-up mechanism would be suitable, e.g., using PEI, or paging DCI. For this likely legacy functionality can be used, but some positioning trigger indication would be needed.

Proposal 2: Further discuss wake-up mechanism to trigger UE in idle/inactive mode to initiate positioning procedure or if legacy procedures can be used.

Irrespectively, some sorts of indication would be needed on the triggering message to indicate that the MT-SDT procedure is to trigger the MT-LR procedure.

Proposal 3: Introduce new parameter “positioning trigger indication”, in relation to the MT-SDT procedure to initiate/trigger the positioning procedure/measurements.

Measurement reporting

Two alternatives are discussed, the UE enters connected mode, or it can use RACH procedure in Idle/(Inactive) mode to perform the reporting. For the RACH alternative, the MO-SDT procedure would be a relevant procedure to use, but depending on size of the measurement report, either the UE remains in Idle/Inactive after SDT procedure, or it may resume the connection and enter RRC-connected mode.

Proposal 4: Further discuss power saving options to use MO-SDT for positioning measurement reporting.

DRX alignment

Additionally, the operation of DRX in RRC_IDLE/RRC_INACTIVE and positioning procedure are performed independently from each other. This, however, can result in an unnecessary UE power consumption as the UE needs to power on and off its receiver several times to perform both paging related and positioning related operations. There is therefore a need for enhancements, where the operations of positioning and paging procedure can be aligned, resulting in a reduced UE power consumption.

In latest meeting [5] it was agreed to consider, alignment between PRS and DRX.

There can be two types of PRS transmissions, these are periodic and aperiodic (on-demand) PRS transmissions.

- In case of aperiodic/event triggered PRS transmission, the PRS transmission (by gNB) is initiated  by LMF, but triggered by a defined event such as movement, change of temperature etc. However, LMF is not aware of DRX occasion configuration, e.g., occasion time, periodicity, etc. We propose LMF to assign and optionally configure a PRS pattern to relevant gNB(s), and UE, that is aligned with the DRX pattern. In order to make LMF aware of UE’s DRX pattern, UE reports to LMF, where the report contains the UE-ID, and/or its calculated DRX cycle/Paging Frame,  i.e. in which SFN it will wake up to monitor Paging Occasion (PO) . This report can be in a NEW information Element IE, in the On-demand PRS request message. The LMF will then assign and configure a PRS pattern to relevant gNB, and UE that is aligned with the DRX pattern, see figure 2.

- In case of periodic PRS transmission, we propose the gNB to adjust UE’s DRX occasion based on UE request so that it is aligned with PRS transmission. To facilitate this, we propose the UE calculates the distance in time between PRS occasion and DRX occasion. Then, the UE proposes a DRX offset to be used by the gNB (or AMF). Once, it is granted (by gNB of AMF), the UE is provided with the new (adjusted) DRX.. see figure 3.

Proposal 5: In case of aperiodic/event triggered PRS transmission let the LMF align the PRS transmission to the UE DRX cycle/PO,

Proposal 6: In case of periodic PRS transmission let the AMF (or gNB) align the UE DRX/PO to the Periodic PRS transmissions by adding an Offset to the UE-ID for PO calculation, similar to the solution in LTE for MuSIM as specified in 36.304 and 23.502 for avoiding paging collision.

# Summary

In this contribution, we have discussed our view on potential positioning enhancements in NR. Our observations and proposals are listed below:

Observation 1: Network triggered MT-LR procedures for UEs in Idle/Inactive is currently not supported.

Observation 2: Depending on the scenario the UE may have to be configured in Idle/Inactive mode, if not configured while released from Connected mode.

Proposal 1: Both alternatives, configuration triggered via MT-SDT or configured via RRC release should be considered.

Proposal 2: Further discuss wake-up mechanism to trigger UE to initiate positioning procedure or if legacy procedures can be used.

Proposal 3: Introduce new parameter “positioning trigger indication”, in relation to the MT-SDT procedure to initiate/trigger the positioning procedure/measurements.

Proposal 4: Further discuss power saving options to use MO-SDT for positioning measurement reporting.

Proposal 5: In case of aperiodic/event triggered PRS transmission let the LMF align the PRS transmission to the UE DRX cycle/PO,

Proposal 6: In case of periodic PRS transmission let the AMF (or gNB) align the UE DRX/PO to the Periodic PRS transmissions by adding an Offset to the UE-ID for PO calculation, similar to the solution in LTE for MuSIM as specified in 36.304 and 23.502 for avoiding paging collision.

# References

- RP-213588, “Revised SID on Study on expanded and improved NR positioning”, RANP #94e, December 2021

- TS 22.104, “Service requirements for cyber-physical control applications in vertical domains; Stage 1, (Release 18)”

- -

- -

- RAN2#119bis-e meeting

- TR 38.859, TR Study on expanded and improved NR positioning

- 23.273, 5G System (5GS) Location Services (LCS);Stage 2(Release 17)

- RP-213583, Mobile Terminated-Small Data Transmission (MT-SDT) for NR