3GPP TSG-RAN WG2 #120	R2-2211959

Toulouse, France, November 2022

Agenda Item:	8.5.2.3

Source:	OPPO

Title:	Discussion on PDU discard

Document for:	Discussion, Decision

Introduction

This paper will provide our considerations on PDU discard, for which the previous RAN2 meetings reached the following agreements.

- RAN2 assumes that PDU Set based parameters and PDU Set related information may be used for better support of XR services. RAN2 can consider both UL and DL directions.

- RAN2 will study PDU Set based parameters and PDU Set related information handling in Network and UE

- RAN2 to adopt the current SA2 definition of PDU Set as an application media unit as working assumption, subjected to further guidance from SA2 and SA4.

- XR awareness discussion in RAN2 should consider PDU set characteristics and how to use the information available on those (for UL and/or DL). Can also consider how to handle data bursts.

- RAN2 can study e.g. periodicity, arrival time, jitter and frame-size variations for XR awareness to enable power savings and capacity enhancements. Can study also how often such parameters change (i.e. how dynamic they are).

- RAN2 can consider how PDU sets can be mapped to DRBs (FFS if SA2 discussion on PDU set mapping to QoS (sub-)flows impacts this)

- 1. For UE transmitter, the PDCP discard should be performed per PDU set basis.

- 2. For UE transmitter, The PDCP discard is managed per SDU for PDU set, the PDCP entity discards all PDCP SDUs associated with the PDU set.

Discussion

For the XR service, SA2 studies KI#4&5 which are related to the PDU Set support and concludes to deliver the following PDU Set QoS information to the AS layer for the PDU Set handling.

- 8.x.1.1	PDU Set QoS Parameters

PDU Set QoS treatment is determined using dynamic or non-dynamic PCC.

The following PDU Set QoS parameters are defined to support PDU Set handling:

-  PDU Set Error Rate: The PSER defines an upper bound for the ratio between the number of PDU Sets not successfully received and the total number of PDU Sets sent towards a recipient measured over a measurement window.

Editor’s Note: the criteria for determining whether a PDU Set is successfully delivered or not are FFS

-  PDU Set Delay Budget

Editor’s Note: The definitions of PSER and PSDB are FFS. For PSDB, it needs further study the impact due to N6 jitter.

-  Whether all PDUs are needed for the usage of PDU Set by application layer (PDU Set Integrated Indication).

Editor’s Note: It is FFS “Whether a PDU Set is still valid in case PSDB is exceeded” is needed. It should be discussed together with the definition of PSDB, specially about the boundary of PSDB.

If PDU Set based QoS handling is used, PCF determines the above PDU Set QoS Parameters based on information provided by AF (described in 8.X.2) and/or local configuration. The PDU Set QoS parameters are sent to SMF as part of PCC rule, then SMF sends them to RAN

Although the PDU Set QoS parameters above are described from the downlink perspective, we understand such PDU Set QoS parameters also apply to the uplink, i.e. PDU Set Integrated Indication can be applicable for both uplink and downlink to determine whether the remaining PDUs of a specific PDU Set should be discarded.

As the RAN can obtain PDU Set Integrated Indication from the CN and considering RAN2 already agrees UL PDU discard is operated in the UE transmitter, we understand that the UE can obtain such PDU Set Integrated Indication from the gNB for the uplink PDU discard mechanism. For example, if PDU Set Integrated Indication is set as False, the UE should try its best to deliver all PDUs of a PDU Set, otherwise, the UE can discard the remaining PDUs of one PDU Set if some PDU of this PDU Set is considered lost.

- SA2 has concluded that PDU Set Integrated Indication is introduced to indicate whether all PDUs of a PDU Set are needed. Such indication in our understanding can be used for the PDU Set discard.

As the uplink transmitter, the UE can obtain PDU Set Integrated Indication from the gNB to determine whether to enable PDU Set discard. In detail, if PDU Set Integrated Indication is set as True, the UE can discard the remaining PDUs of a PDU set if some PDU of that PDU Set is known as lost, otherwise, the UE can continue the PDU transmission.

In the RAN2#119bis-e meeting, RAN2 agrees to support the PDU discard. The PDU discard is operated in the PDCP transmitting entity. According to the current spec, both the PDCP layer and the RLC layer need to respond/be triggered once there is a need to discard PDU.

In TS 38.323

In TS 38.322

Based on the above, although the PDCP layer indicates the RLC layer to discard the associated SDU, the discard of the corresponding RLC SDU actually happens only when neither the RLC SDU nor a segment has been submitted to the lower layer. We understand the current mechanism may not benefit as we expected on efficient resource usage but it reduces the impact as much as possible on both the transmitter and receiver. From this perspective, we prefer RAN2 to evaluate whether the current SDU discard is indeed insufficient, i.e. whether the RLC entity needs to discard the SDU even if this packet has been delivered to the lower layer. If RAN2 agrees on the RLC enhancement, RAN2 would further discuss the impact on the receiving window.

RAN2 discusses whether to keep the current SDU discard rule, i.e. the RLC SDU to be discarded should be the one that has not been yet submitted to the lower layer.

Conclusion

We have the following observations:

Observation 1	SA2 has concluded that PDU Set Integrated Indication is introduced to indicate whether all PDUs of a PDU Set are needed. Such indication in our understanding can be used for the PDU Set discard.

We have the following proposals:

Proposal 1	As the uplink transmitter, the UE can obtain PDU Set Integrated Indication from the gNB to determine whether to enable PDU Set discard. In detail, if PDU Set Integrated Indication is set as True, the UE can discard the remaining PDUs of a PDU set if some PDU of that PDU Set is known as lost, otherwise, the UE can continue the PDU transmission.

Proposal 2	RAN2 discusses whether to keep the current SDU discard rule, i.e. the RLC SDU to be discarded should be the one that has not been yet submitted to the lower layer.

Reference

[1] RP-220285  New SID “Study on XR Enhancements for NR”

[2] 3GPP TR 23.700-60 Study on XR (Extended Reality) and media services