3GPP TSG-RAN WG2 Meeting #120	R2-2211966

Toulouse, France, 14 – 18 November 2022

Agenda item:	8.3.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Common Channel Updates for NES

WID/SID:	FS_Netw_Energy_NR - Release 18

Document for:	Discussion and Decision

# 1	Introduction

RAN2 is starting to work on study item on NW energy saving (RP-220297) with the scope:

The objectives of the study are the following:

- Definition of a base station energy consumption model [RAN1]

- Adapt the framework of the power consumption modelling and evaluation methodology of TR38.840 to the base station side, including relative energy consumption for DL and UL (considering factors like PA efficiency, number of TxRU, base station load, etc), sleep states and the associated transition times, and one or more reference parameters/configurations.

- Definition of an evaluation methodology and KPIs [RAN1]

- The evaluation methodology should target for evaluating system-level network energy consumption and energy savings gains, as well as assessing/balancing impact to network and user performance (e.g. spectral efficiency, capacity, UPT, latency, handover performance, call drop rate, initial access performance, SLA assurance related KPIs), energy efficiency, and UE power consumption, complexity. The evaluation methodology should not focus on a single KPI, and should reuse existing KPIs whenever applicable; where existing KPIs are found to be insufficient new KPIs may be developed as needed.

Note: WGs will decide KPIs to evaluate and how.

- Study and identify techniques on the gNB and UE side to improve network energy savings in terms of both BS transmission and reception, which may include:

- How to achieve more efficient operation dynamically and/or semi-statically and finer granularity adaptation of transmissions and/or receptions in one or more of network energy saving techniques in time, frequency, spatial, and power domains, with potential support/feedback from UE, and potential UE assistance information [RAN1, RAN2]

- Information exchange/coordination over network interfaces [RAN3]

Note: Other techniques are not precluded

The study should prioritize idle/empty and low/medium load scenarios (the exact definition of such loads is left to the study), and different loads among carriers and neighbor cells are allowed.

The following example scenarios (mapping between scenarios and network loads is left to the study) including single-carrier and multi-carrier deployments are used as the starting point for discussion on prioritized scenarios for the study.

The following example scenarios are listed in no particular order.

- Urban micro in FR1, including TDD massive MIMO (note: this scenario can also model small cells)

- FR2 beam-based scenarios (note: this scenario can also model small cells)

- Urban/Rural macro in FR1 with/without DSS (no impact to LTE expected in case of DSS)

- EN-DC/NR-DC macro with FDD PCell and TDD/Massive MIMO on higher FR1/FR2 frequency

Note 1: legacy UEs should be able to continue accessing a network implementing Rel-18 network energy savings techniques, with the possible exception of techniques developed specifically for greenfield deployments.

Note 2: the study of energy savings specifically for IAB is not part of the scope.

The study should coordinate with RAN4 as needed.

RAN2 objectives are to identify possible techniques to improve NW energy efficiency for base station transmission and reception. The work item does not mention separately CONNECTED and IDLE/INACTIVE techniques but targeting different states will likely have quite different techniques. It is also notable that the SI will focus on low load scenarios.

In this paper we discuss what could be done for limiting NW energy consumption when considering UEs in RRC_IDLE/INACTIVE states.

# 2 	Discussion

Generally, it is quite challenging to modify existing IDLE/INACTIVE procedures due to already existing population of UEs in the networks and difficulty to have different behaviour of UEs for different releases of UEs. Even if one would have different behaviour for release 18 UEs allowing network to save energy, one would need to take into account also the possibility for legacy UEs to camp in the system. But of course it is always possible for an operator to make the decision that in order to achieve maximum energy saving gains it would not support part of the UE populations in 5G system e.g. in certain cells. In other paper [R2-2210129] we discuss how to handle mobility and access control for legacy UEs as well NES capable UEs.

## 2.1	Paging

In 3GPP normally we design paging in such a way that all the UEs paging occasions are distributed evenly in time. This is an excellent approach to maximize the capacity of the system for paging but inherently this will cause also “maximum” energy consumption for paging for the network.

For example, if there are 10 UEs monitoring for paging in a cell it would not be problematic for the base station to serve all 10 UEs simultaneously even if they are all paged. However, due to the UE ID based distribution of users within the paging cycle, it may happen that the network has to transmit 10 individual paging messages spread across the paging cycle. This prohibits effective base station sleep opportunity as the base station has to be active for 10 short periods during the paging cycle.

Observation 1: The existing paging design distributes paging occasions evenly in time which minimizes the possibility for a base station to sleep between paging occasions.

Thus, it will be beneficial for RAN2 to discuss if the paging mechanism can be enhanced to provide better base station sleep opportunities, at least when the paging load is low. This can be achieved by clustering the paging in PF(s)/PO(s) that are confined in cell-specific paging window(s) and that are common to the UEs in the cell. This is shown in Figure 1. Of course, it would be even better if the paging occasions would be collocated in time with other downlink signals (e.g. MIB/SIBs/SSBs). Difficulties in doing this is the need for backward compatibility, i.e.  the solution should work for legacy UEs.

Proposal 1: Study possibilities to save base station energy via time domain enhancements of the paging mechanism e.g. by clustered paging in cell-specific paging window(s).

| Solution: Clustered paging in cell-specific paging window(s) |  |
| --- | --- |
| Introduction | Cluster UEs to monitor paging in PF(s)/PO(s) confined in cell-specific paging window(s) to reduce network wake-up due to paging transmissions spread in time over distributed PFs |
| Scenario | Single-carrier or multi-carrier; UEs in RRC Idle/Inactive/Connected |
| NES gain | The confining in time of the paging transmissions results in increased sleep duration between the paging transmissions. |
| Impact to legacy UEs | No impact: In multi-carrier: the legacy UE can receive paging from the anchor cell. In single-carrier, either the legacy UEs continue to monitor their legacy PFs/POs or legacy means for clustering legacy UEs can be used (e.g. UE-ID remapping) |
| UE assistance needed | No |
| RAN2 impact | Indicating paging configuration to the UE, etc. |

Figure 1 Example of clustered paging in cell-specific paging window(s) (bottom) compared to legacy paging (top).

## 2.2	SSB/MIB/SIB

In general the study item is focusing on low load scenarios and in such a scenario the most effective way to reduce power consumption is to turn off the cells and steer UEs to a carrier i.e. the operator would minimize the amount of concurrent active carriers in low load scenarios. In such a scenario it is imperative that at least one carrier can provide service to all UEs – including legacy UEs. So RAN2 should focus work on handling such a scenario when considering omission of SSB/MIB/SIB transmission from the carrier.

Proposal 2: RAN2 should focus on scenario where most of carriers are turned off and single carrier is usable by legacy UEs as well as NES capable UEs

SSB/SIB1 transmission reduction/adaptation can be achieved by increasing the SSB burst periodicity and SIB1 periodicity and/or repetition frequency so that the SSB and/or SIB1 are transmitted less often (e.g. during low load), thus saving energy.

In fact, it is already possible for the gNB to change the SSB settings in the current system, but the existing mechanism is slow as it requires SI change and requires the UE to re-acquire the SIB1, which may take several tens of seconds. Similarly, SIB1 is transmitted with the configured periodicity e.g. with a periodicity of 160ms, and can be repeated within the 160ms based on NW implementation. For example, for SSB and CORESET multiplexing pattern 1, the SIB1 repetition transmission period is 20ms.

Support of more dynamic adaptation means could be considered, to quickly adapt to the current traffic load, as it can increase the opportunities for the base station to apply (µ)DTX/DRX and in turn increase energy saving potentials. It is noted that increasing the SSB period may impact certain operations of legacy UEs, which for example as per TS38.213 assume that SSBs are transmitted with a periodicity of 20 ms for initial cell selection. Therefore, impact on legacy UEs should be taken into account in the study. For RRC Connected UEs, there may not be much impact from reduced SSB/SIB1 transmissions if, for example, UE-specific CSI-RSs are provisioned as an alternative to the reduced SSBs. Instead, RRC Idle/Inactive UEs may be impacted from reduced SSB/SIB1 transmissions, e.g. the initial access delay may increase.

Observation 2: The impact on RRC Connected UEs from reduced SSB/SIB1 transmissions may be limited if UE-specific reference signals (TRS/CSI-RS) are provisioned by the network to allow proper time and frequency synchronization and compliance with legacy measurement requirements.

Observation 3: The impact on RRC Idle/Inactive UEs from reduced SSB/SIB1 transmissions should be studied.

Proposal 3: RAN2 to study procedures and signalling to enable dynamic SSB/SIB1 transmission reduction/adaptation and consider the impact to RRC Idle/Inactive UEs.

In RAN2 there has been discussion on solutions that UE would be able to camp on a cell that does not transmit SIs and UE would receive SIs from another cell (so called anchor cell). Then when UE needs to perform access to a cell one would start RACH possibly on the cell which did not provide SIs. It was not clear from the solution proposal what is gained compared to just handing over a UE to the NES cell once access has been performed on the “anchor cell”. This is already allowed with existing specifications thus it would be important to see what is the benefit for this SI focusing on low load scenarios.

Proposal 4: Study the gain of “anchor cell” approach against just handing over UE after the access to the NES cell

## 2.2	Cell/node activation by UE

A more flexible control of the cell/network node (re)-activation whenever a UE requiring service appears may potentially increase the usage of cell deactivation for network energy saving when the load decreases. To this end, the cell/network node activation request by the UE is in scope of the RAN1 study (e.g. to identify a suitable signal/channel from UE for gNB’s wake-up request). An aspect that RAN2 could consider, in parallel to the RAN1 study, is whether the UE should be always allowed to send a cell/node wake-up request or not.

Proposal 5: RAN2 to study cell/network node activation request by the UE including the conditions for the UE to trigger such request.

# 3	Conclusion

Observation 1: The existing paging design distributes paging occasions evenly in time which minimizes the possibility for a base station to sleep between paging occasions.

Proposal 1: Study possibilities to save base station energy via time domain enhancements of the paging mechanism e.g. with clustered paging.

Proposal 2: RAN2 should focus on scenario where most of carriers are turned off and single carrier is usable by legacy UEs as well as NES capable UEs

Observation 2: The impact on RRC Connected UEs from reduced SSB/SIB1 transmissions may be limited if UE-specific reference signals (TRS/CSI-RS) are provisioned by the network to allow proper time and frequency synchronization and compliance with legacy measurement requirements.

Observation 3: The impact on RRC Idle/Inactive UEs from reduced SSB/SIB1 transmissions should be studied.

Proposal 3: RAN2 to study procedures and signalling to enable dynamic SSB/SIB1 transmission reduction/adaptation and consider the impact to RRC Idle/Inactive UEs.

Proposal 4: Study the gain of “anchor cell” approach against just handing over UE after the access to the NES cell

Proposal 5: RAN2 to study cell/network node activation request by the UE including the conditions for the UE to trigger such request.