3GPP TSG-RAN WG2 Meeting #120	 										    R2-2211985

Toulouse, France, 14 - 18 November, 2022

Source: 	Xiaomi

Title:  	The scenarios supported for LTM

Agenda Item:	8.4.2.1.1

Document for:	Discussion and Decision

# Introduction

According to the previous RAN2 discussion on the Rel-18 LTM, RAN2 made the following agreements on the scenarios supported for LTM:

RAN2#119-e meeting agreements: - Confirm to Support L1/L2-based inter-cell mobility for inter-DU scenario (as well as intra-DU scenarios). - The design for intra-DU and inter-DU L1/L2-based mobility should share as much commonality as reasonable. FFS which aspects need to be different. - R2 will initially focus on PCell mobility. - R2 assumption: Rel-18 L1/L2 mobility includes both non-CA (PCell only) and CA scenarios (PCell and SCell). This includes the following cases - a) the target PCell/target SCell(s) is not a current serving cell (CA à CA scenario with PCell change) - b) FFS the target PCell is a current SCell - c) FFS the target SCell is the current PCell. - DC scenarios are FFS (e.g. PSCell mobility may be a low hanging fruit FFS).

RAN2#119bis-e meeting agreements: - L1L2 based mobility supports the following CA scenarios: - PCell change without SCell change - PCell change with SCell change - Support NR-DC scenario in L1L2 based mobility, at least for the PSCell change without MN involvement case, i.e. intra-SN. - For L1L2 mobility, Target Pcell/SCell can be current SCell/PCell, i.e., current SCell/PCell can be configured as candidates. - FFS how the UE determine the BWPs (for DL and UL) to be used upon the execution of L1/L2 inter-cell mobility

In this contribution, we intend to clarify further on the scenarios to be supported for LTM

# Discussion

## CG type of the candidate configuration

When the gNB provides the candidate (CG, Cell Group) configuration to the UE, the candidate configuration can be either MCG or SCG. Then we could have the following options for the CG type applicable for each candidate configuration

- Option 1: MCG-only or SCG-only

- Option 2: both MCG and SCG (when the MN and the SN is the same node.)

From our understanding, Option 2 is only applicable for the case when the MN and the SN is the same node. When the MN and the SN are different nodes, if a candidate configuration can be activated as either MCG or SCG, this means that the Node providing the candidate configuration can become MN or SN at the transmission of the cell switch MAC CE. Then we need to support inter-CU cell switching, which is not included in the scope of the work item. To simplify the standard work in Rel-18, we consider that Option 1 should be supported, and Option 2 can be considered in the later release when inter-CU LTE is supported.

Proposal 1: One candidate configuration can be MCG-only or SCG-only, not both.

## Node of sending cell switch MAC CE

Regarding the transmission of cell switch MAC CE, although RAN2 agreed to support “the PSCell change without MN involvement case”, the scenario of DC is restricted to the intra-DU scenario. One may argue that even for intra-DU we could have SCG and MCG configured for the same node, and then the gNB can send the cell switch MAC CE via MCG to change the SCG configuration. However, from our understanding, if the SCG and the MCG are configured for the same node, the gNB can always use the SCG MAC to send the cell switch MAC CE for the SCG configuration change. Then the extra complexity of using different CG to switch the CG configuration from another MAC entity is not essential. As such RAN2 is kindly requested to confirm that the cross-CG cell switch should not be supported in Rel-18. Same as the argument for the discussion on the “CG type of the candidate configuration” in 2.1, the cross-CG cell switch can be supported when inter-CU LTE is supported.

Proposal 2: The cell switch MAC CE transmitted via MCG is only for the cell switch of the MCG.

Proposal 3: The cell switch MAC CE transmitted via SCG is only for the cell switch of the SCG.

# Conclusions

According to the analysis given above, we have the following Proposals:

Proposal 1: One candidate configuration can be MCG-only or SCG-only, not both.

Proposal 2: The cell switch MAC CE transmitted via MCG is only for the cell switch of the MCG.

Proposal 3: The cell switch MAC CE transmitted via SCG is only for the cell switch of the SCG.

# Reference

[1] RAN2#119-e meeting minutes

[2] RAN2#119bis-e meeting minutes