3GPP TSG RAN WG2 #120     	          			  R2-2212000

Toulouse, FR, November 14th – 18th, 2022

Source:         TCL Communication

Title:           Discussions on General Aspects of AI/ML Framework

Agenda item:    8.16.2

Document for:   Discussion and Decision

- Introduction

The study item on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface has been established in RAN plenary #94e meeting for Rel.18 [1]. The three use cases to be studied are respectively the enhanced CSI feedback, the beam management, and the positioning. In this SID, the objectives of RAN2 perspective are mentioned and given as below:

- Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1 - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference), and management of data and AI/ML model, per RAN1 input - Collaboration level specific specification impact per use case

At the latest RAN2 119bis e-meeting, based on the RAN1 process and agreements, some initial assumptions have been noted as following:

Assume that RAN2’s work can be somewhat split: A) use-case-centric configuration, signalling and control procedures, B) management of data and AI/ML models (where part of discussion may overlap between use cases Assume that e.g. for the management of data and AI/ML models, RAN2 could start by focusing on data collection, model transfer, model update, model monitoring and model selection/(de)activation/switching/fallback (to the extent needed), whether UE capabilities has a role in this.

Based on the RAN1 meeting, the study on the common aspects of AI/ML framework begin to shape up. In this contribution, according to the lasted RAN2 meeting, we will provide our views on the general aspects of AI/ML framework in RAN2 perspective. Especially, collaboration level, model transfer, model selection and model switching, and AI/ML model during UE mobility will be discussed.

- Discussions

  - Collaboration levels

During the RAN1#109 meeting, one candidate category of Network-UE collaboration levels has been made an agreement, while at the RAN1#110bis meeting, the working assumptions and agreements of the boundary for above different collaboration levels was achieved . The related agreements and working assumptions are shown as following:

RAN1#109e meeting Agreement Take the following network-UE collaboration levels as one aspect for defining collaboration levels - Level x: No collaboration - Level y: Signaling-based collaboration without model transfer - Level z: Signaling-based collaboration with model transfer RAN1#110bis e meeting Working Assumption - Define Level y-z boundary based on whether model delivery is transparent to 3gpp signalling over the air interface or not. - Note: other procedures than model transfer/delivery are decoupled with collaboration level y-z - Clarifying note: Level y includes cases without model delivery. Agreement Clarify Level x/y boundary as: - Level x is implementation-based AI/ML operation without any dedicated AI/ML-specific enhancement (e.g., LCM related signalling, RS) collaboration between network and UE. (Note: The AI/ML operation may rely on future specification not related to AI/ML collaboration. The AI/ML approaches can be used as baseline for performance evaluation for future releases.)

In our understanding, different Network-UE collaboration levels involve different AI/ML functionalities and actions, hence, the different collaboration levels may have different RAN2 specification impacts.  Based on the RAN1 discussion, for level x, right now, it is clear that there is no dedicated AI/ML-specific enhancement collaboration between network and UE, and it fully depends on implementation. Hence, the AI/ML operation of one entity is transparent to other entities. Thus, the level x has no RAN2 specification impact.

Comparatively, the current collaboration levels are categorized based on the Signaling-based collaboration, both level y and level z have the signaling requirement which relates AI/ML operations between Network and UE. Thereby, both level y and level z have RAN2 specification impact.

Observation 1: Level x has no RAN2 specification impact. Level y and level z have RAN2 specification impact.

Furthermore, considering the difference between level y and level z is the aspect of model transfer. Meanwhile, the boundary for level y and level z is whether model delivery is transparent to 3gpp signaling or not. In other words, the level y includes not only AI/ML assistant information signaling, but also model transfer which is transparent 3gpp signaling. As for level z, besides the AI/ML assistant information signaling, it also includes model transfer which carries the 3gpp signaling. Hence, RAN2 should study the AI/ML assistant information and the related signaling for level y and level z, and should also study the model transfer solutions which is in 3gpp scope and AI/ML model information including description of parameters and/or structure of AI/ML model for level z.

Proposal 1: RAN2 should study the assistant information and related signaling for level y and z.

Proposal 2: RAN2 should study the model transfer solutions which is in 3gpp scope and AI/ML model information for level z.

2.2 Model transfer

According to the RAN1 #110bis, it can mainly be categorized two mechanisms of model transfer:

- Mechanism1: Model transfer is transparent 3gpp signaling(level y）

- Mechanism2: Model transfer is based on 3gpp signaling(level z)

For the mechanism1, it is implied that the AI/ML model are located in servers, such as OTT and OAM, which model transfer is out of 3gpp scope. As for mechanism 2, the solutions of model transfer are CP-based solution and UP-based solution, the signaling which carries AI/ML model can be RRC signaling, NAS signaling and UP traffic, the related analysis are summarized as follows:

- CP - based solution

- Solution1: If the AI/ML model transfer between UE and RAN, it may be carried in RRC signaling, the latency of model transfer is low. Besides, it is flexible to manage AI/ML model, such as AI/ML model (re)configuration, AI/ML model update and so on. However, usually, since the size of AI/ML model is large, it will bring a big overhead for AI/ML model transfer. And due to limitation of RRC message size, it is may unable to carry the large size AI/ML model at a time, the segmentation of AI/ML model may also should be studied.

- Solution2: If the AI/ML model transfer between UE and CN, it may be carried in NAS signaling. Thus, the long latency is unavoidable. Meanwhile, the network only forwards the AI/ML model transparently to CN, hence, the network does not know the structure and parameters of AI/ML model.

- UP - based solution

- Solution3: If the AI/ML model transfer between UE and RAN, it may be carried in UP traffic, although there is no need to concern  the size of AI/ML model for transferring, the latency of model delivery should be a problem. Furthermore, the AI/ML model is invisible for network, hence, there is no management the AI/ML model by network.

Actually, AI/ML model is able to be trained and then deployed in different entities. The model transfer may occur between different entities in different scenarios. Besides, different scenarios may have different requirements. Therefore, we think that all the above solutions for model transfer may be supported in different scenarios.

Proposal 3: All the solutions of model transfer may be supported in different scenarios.

Proposal 4: RAN2 should study the solutions of model transfer over air interface.

2.3 Model selection and Model switching

In latest RAN1 meeting, there is an agreement for model selection, activation, deactivation, switching, and fallback, which is shown as below:

Agreement For model selection, activation, deactivation, switching, and fallback at least for UE sided models and two-sided models, study the following mechanisms: - Decision by the network - Network-initiated - UE-initiated, requested to the network - Decision by the UE - Event-triggered as configured by the network, UE’s decision is reported to network - UE-autonomous, UE’s decision is reported to the network - UE-autonomous, UE’s decision is not reported to the network FFS: for network sided models FFS: other mechanisms

Based on RAN1 process, in our understanding, for UE sided model and two-sided model, combined with model training, the scenarios may include:

- UE-sided model with UE- sided training

- UE- sided model with Network-sided training

- UE- sided model with Joint training

- UE-sided model with Separated training

- Two-sided model with UE-sided training

- Two-sided model with Network-sided training

- Two- sided model with Joint training

- Two sided model with Separated training

Model selection, which is a process of selecting one among multiple alternative models; while for model switching, it is that deactivating a currently active AI/ML model and activating a different AI/ML model for a specific use case. It is noted that in the model selection, the alternative candidate models have the same specific function, which is a little different from model switching. In model switching, there is  no constraint that the specific function for the alternative candidate models to switch to should be same as the original model. Furthermore, in the model selection, the node may not adopt AI/ML working method. In other words, model selection may be occurred in the case that the node decides to use non AI/ML methods instead of AI/ML methods. While for model switching, the node is using AI/ML operation currently, if the system performance become worse based on model monitoring, the node may switch to a more suitable AI/ML model to replace the old one. Hence, in our views, model selection and model switching may occur in the same or different scenarios. RAN 2 is suggested to study the trigger event for model selection and model switching, respectively.

Proposal 5: RAN2 is suggested to study the trigger event for model selection and model switching, respectively.

Among the above scenarios, whatever the locations of model inference function and model training, if a UE or a network decide to do the model selection and model switching, the related signaling and procedure should be studied, which may be as followings:

- Configuration signaling message from network to UE,

- Request message from UE to network

- Response message from network to UE

- Reporting message from UE to network

Besides the mentioned signaling and procedure, since there may be more than one trained AI/ML model. How to select and switch to the most suitable AI/ML model among the various AI/ML models for a specific function should be considered. Hence, the conditions of model selection and model switching should also be studied, such as AI/ML model structure, common KPI, generalization of an AI/ML model and so on.

Proposal 6: RAN2 should study the signaling and procedure of model selection and model switching.

Proposal 7: RAN2 should study the conditions of model selection and model switching.

It is possible that system deploys different AI/ML models for different functions at the same time. However, more ML models mean more resource consumption, and higher UE capability requirements. Thus, we think RAN2 should also study whether there is an upper bound on the maximum number of AI/ML models for simultaneously model selection or model switching at a time.

Proposal 8: RAN2 should also study whether there is an upper bound on the maximum number of AI/ML models for simultaneously model selection or model switching at a time.

2.4 AI/ML Model during UE mobility

In a wireless communication system, the channel may become worse due to the UE movement. When a UE cross cell boundary, it may result in handover. When UE move into a new serving gNB, the system can adopt traditional working method and AI/ML-based working method in some use cases. If the AI/ML-based method is adopted, in order to ensure the service continuity, model selection or model switching may to be triggered Furthermore, since UE moves into a new serving cell, the AI/ML model which used in the old serving gNB may be aligned between the serving cell and target cell,, during UE handover. The alignment may involve model transfer. Besides, considering the size of AI/ML model will be an overhead, model reconfiguration or fine-tuning are possible ways to avoid encountering this overhead. Hence, during handover , to  maintain AI/ML model proper operation, it may involve model selection, model transfer, model reconfiguration, or fine-tuning.

Proposal 9: RAN2 should study the methods for maintaining AI/ML model proper operation during UE mobility.

- Conclusions

In this contribution the following proposals been proposed:

Observation 1: Level x has no RAN2 specification impact. Level y and level z have RAN2 specification impact.

Proposal 1: RAN2 should study the assistant information and related signaling for level y and z.

Proposal 2: RAN2 should study the model transfer solutions which is in 3gpp scope and AI/ML model information for level z.

Proposal 3: All the solutions of model transfer may be supported in different scenarios.

Proposal 4: RAN2 should study the solutions of model transfer over air interface.

Proposal 5: RAN2 is suggested to study the trigger event for model selection and model switching, respectively.

Proposal 6: RAN2 should study the signaling and procedure of model selection and model switching.

Proposal 7: RAN2 should study the conditions of model selection and model switching.

Proposal 8: RAN2 should also study whether there is an upper bound on the maximum number of AI/ML models for simultaneously model selection or model switching at a time.

Proposal 9: RAN2 should study the methods for maintaining AI/ML model proper operation during UE mobility.

- References

- RP-213599, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- Chairman notes of 3GPP TSG RAN WG1 #110 bis-e

- Chairman notes of 3GPP TSG RAN WG2 #119 bis-e